3GPP TSG-RAN WG2 Meeting #127bis		R2-2408125

Hefei, China, 14th – 28th Oct. 2024

Source:	vivo

Title:	Discussion on UE-initiated/event-driven beam management

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- Introduction

According to the previous RAN1 meetings, some progress on UE-initiated/event-driven beam management were made in RAN1, e.g..

Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

In this contribution, we will discuss the RAN2 aspects related to UE-initiated/event-driven beam management, including MAC and RRC impacts.

- Discussion

  - UE-initiated/event-driven beam report - MAC impacts

UE-initiated/event-driven beam reporting is mainly used to assist the network in determining TCI state update or TCI state activation/deactivation. The corresponding measurement results are reported to the network based on event trigger. According to RAN1’s progress, the trigger event evaluation, report procedure, and measurement RS determination of new beam mainly have specification impacts on the MAC layer.

Regarding the trigger event evaluation, the evaluation procedure for Event-2, i.e., the quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam, has been agreed in RAN1#116bis meeting [1].  But whether this conclusion is captured in RAN1 or RAN2 specification is still being discussed in RAN1.

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

In our understanding, it is more proper to evaluate the trigger event for beam report on the MAC layer. If it is performed in the MAC layer, the concepts of timer and counter in the MAC layer can be reused directly. The timer can be started/restarted when the corresponding condition(s) is satisfied, and it is flexible. However, if it is performed in the physical layer, the start and duration of the time window are RRC-configured generally and it cannot accommodate the beam change automatically. In addition, similar evaluation procedures, like beam failure evaluation procedure and event-triggered reporting evaluation procedure for LTM, are performed in the MAC layer. If trigger event evaluation for UE-initiated/event-driven beam reporting is also captured in the RAN2 specification, the above procedures can be the baseline. If it is captured in the RAN1 specification, it would be a totally new procedure, which has big impacts on RAN1 specification. Therefore, it is much simpler if the evaluation of trigger event for beam report is performed and capture in the MAC layer. Furthermore, to simplify the specification design in the MAC layer, a common design for the procedure of trigger event evaluation for beam report in Rel-19 MIMO and Rel-19 LTM could be considered.

Proposal 1: It is up to MAC layer to perform the evaluation of trigger event for beam report. The corresponding procedure should be captured in MAC specification.

Regarding the report procedure, two reporting modes, i.e., Mode A and Mode B have been agreed in the RAN1#116bis meeting [1]. Regardless of Mode A and Mode B, the first step is to transmit a 1-bit indication on a PUCCH resource to the network.

RAN1#1 116bis Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details.

In RAN1#118 meeting [2], RAN1 discussed the UCI type of the 1-bit indication and two alternatives were proposed as follows.

Proposal 3.1.2A (Mode-A): On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following - Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. Proposal 3.1.2B (Mode-B): On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following - Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above is at least applied to a single CC case.

In Mode A, when the condition of trigger event is satisfied, the UE will transmit an indication in PUCCH (first channel) and then transmit a beam report in the second channel as indicated/scheduled by the DCI. It is yet to be agreed whether the indication in the first channel is an SR or a new UCI type. The 1-bit indication is used to request an uplink resource, which achieves the same functionality as SR. In the current SR-related specifications, e.g., RRC configuration, encoding, transmission strategy (including counter and prohibit timer), priority rule, multiplexing and dropping rule, are already supported. If the indication is a dedicated SR, the above mechanism can be reused mostly and minimal spec effort is required. However, if it is a new UCI type, the above details may need to be discussed and re-designed. In addition, if the procedure of event evaluation is performed in the MAC layer as mentioned in Proposal 1, SR is more suitable, as it is MAC procedure.

Thus, once the condition of the trigger event is satisfied, a dedicated SR for UE-initiated/event-driven beam reporting will be triggered. In Mode B, the physical resources for both the 1-bit indication and beam reporting is pre-configured by the network. The 1-bit indication in Mode B is also used to notify the network that Event-2 is satisfied and the network will detect the corresponding pre-configured resource for the second UL channel. Although the functionality of the indication in Mode B is different from what in Mode A, we prefer a unified design to simplify the specification design. Therefore, we prefer the 1-bit indication carried in the PUCCH resource to be a dedicated SR for both Mode A and Mode B.

Proposal 2: Dedicated SR is used for the 1-bit indication carried in the PUCCH resource for both Mode A and Mode B.

Regarding the measurement RS determination of new beam(s), RAN1 has agreed that the measurement RS(s) for new beam(s) are explicitly configured in one RS resource set associated with a CSI reporting configuration in RAN1#118 meeting [2].  But whether the RS in the RS resource set can be updated by MAC-CE is still being discussed in RAN1.

Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE

In our view, new beam(s) changes as the current beam changes, so it is necessary to update the measurement RS(s) of new beam(s). In addition to dynamic update by MAC CE, RRC reconfiguration is another approach. However, as RRC reconfiguration is a long-time operation, it is not suitable for UE-initiated/event-driven beam reporting, which is intended to reduce TCI switch delay. To avoid the update of measurement RS(s) of new beam(s), all of the current beam(s) applied for transmission and reception, the activated TCI states other than the indicated one, and inactive TCI states should be included in the measurement resource set. However, it may result in a large measurement overhead and increase the UE computation complexity. In summary, the dynamic update of measurement RS within the new beam resource set by MAC CE is more reasonable. But anyway, the final decision should be up to RAN1 discussion.

Proposal 3: RAN2 assume to support dynamic update of the measurement RS in the measurement resource set for new beam(s) by MAC CE, and final decision is up to RAN1.

  - Configuration – RRC impacts

In RAN1#116 bis meeting[1], RAN1 has agreed to introduce Event-2 as the trigger event for beam report.

Agreement On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - At least L1-RSRP is supported as quality metrics used for Event-2 - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient) - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event - Regarding RS measurement for the new beam for Event-2, down-select one or more of the following: - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s). - Note-1: ‘New/current beam’ is for discussion purpose. - Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded. - Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable.

In our understanding, the trigger event should be configured by RRC for beam report. Thus, the corresponding configuration as well as the definition of the trigger event should be captured in RRC specification. If there is configuration for trigger event in RRC, the corresponding measurement report should be triggered by the configured event; otherwise, it will follow the legacy periodic report.

In addition to the trigger Event 2 defined as above, RAN1 has working assumption on Event 1 and Event 7 in RAN1#118 meeting [2] as below, which has not been final decided:

Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise The additionally supported events will be lower priority compared to event 2.

If these events are agreeable, it should be up to RRC to configure which trigger event(s) should be applied for beam report.

Proposal 4: It is up to RRC to configure which/whether trigger event(s) should be applied for beam report.

Proposal 5: The corresponding RRC configuration as well as the definition of trigger events should be captured in RRC specification.

- Conclusion

In this contribution, we discuss the RAN2 aspects on UE-initiated/event-driven beam management, including RRC and MAC impacts. Based on the discussion, we have the following observations and proposals.

MAC impacts:

Proposal 1: It is up to MAC layer to perform the evaluation of trigger event for beam report. The corresponding procedure should be captured in MAC specification.

Proposal 2: Dedicated SR is used for the 1-bit indication carried in the PUCCH resource for both Mode A and Mode B.

Proposal 3: RAN2 assume to support dynamic update of the measurement RS in the measurement resource set for new beam(s) by MAC CE, and final decision is up to RAN1.

RRC impacts:

Proposal 4: It is up to RRC to configure which/whether trigger event(s) should be applied for beam report.

Proposal 5: The corresponding RRC configuration as well as the definition of trigger events should be captured in RRC specification.

- References

- Draft_Minutes_report_RAN1#116bis

- Draft_Minutes_report_RAN1#118