3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408129

Hefei, China, Oct 14 – 18, 2024

Agenda item:	8.7.3

Source:	Qualcomm Incorporated

Title:	Discussion on overriding measurement gaps

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In the Rel-19 XR SID, the following objective is specified:

Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

In this paper, we discuss RAN2 enhancements for supporting this objective.

# Discussion

In legacy, if a measurement gap (MG) overlaps with PDCCH/PDSCH/PUSCH/PUCCH/CSI-RS/SRS reception or transmission, it always has higher priority except during RACH procedures. Since the duration of a MG can be a few msec long, it can have non-negligible impact on XR traffic, whose periodicity typically is short (e.g. 16 msec) and have strict delay requirements.

Since UE performs measurements on DL reference signals during MGs, time locations of MGs follow those of DL reference signals, which are configured per-cell and periodic (e.g. once every 20 msec). XR traffic is also periodic but has non-integer periodicity. As a result, there are always some occasions in a MG configuration overlapping with data bursts in XR traffic, regardless of how DL reference signals are configured (see Figure 1 for an example). On the other hand, since both MGs occasions and XR traffic are periodic, the time locations where they may overlap, at least those overlap with the start of data bursts, with jitter also taken into account, are also periodic and can be pre-determined once their periodicities and relative start offsets are known.

Observation 1. 	Measurement gap occasions that overlap with the start of data bursts (with jitter considered), are periodic and can be identified deterministically based on their periodicities and relative start offsets.

To mitigate the impact of MGs on XR traffic, among the occasions where MGs overlap with data, network can choose a subset of them and allow them to be overridden by data. This selection can be periodic, e.g. once every N overlapping MG occasions. And this selection can be relatively static, because they can be determined based on the periodicities of a MG configuration and XR traffic, and their relative start offset. It may need to be adjusted only when UE’s mobility status changes and so on.

Given the periodic and relatively static nature of these occasions, it is more efficient to indicate them to UE by RRC configuration instead of dynamic signaling methods such as DCI. That not only reduces signaling overhead, but also greatly simplifies UE implementation, because pre-determined occasions give UE more time to prepare for RF tuning and related procedures.

Observation 2. 	For measurement gaps that are known to overlap with data, using RRC configuration to specify which ones can be overridden by data reduces signaling and simplifies UE implementation.

At RAN1#118 meeting, RAN1 made a working assumption to support DCI based indication to allow data to override a MG. No additional signaling options were agreed. However, based on the above analysis, we think RRC configuration is a valuable option that can co-exist with DCI based option. They do not need to exclude each other. That is because each of them has distinct use cases where they are better at. For example,

- For MG occasions where overlap with data can be pre-determined, RRC configuration is clearly more advantageous than DCI-based signaling, as previously analysed;

- Due to the randomly varying size of XR data bursts, it is impossible to predict based solely on the periodicities of data traffic and MG configuration whether some MG occasions may overlap with data (e.g., those located near the end of a large data burst). In such cases, it is not possible to use RRC configuration to indicate them ahead of time. DCI-based signaling must be used instead.

The differences between these two types of MG occasions can be illustrated by an example (Figure 1). In this example, data bursts arrive at 60 fps, and a MG configuration has a periodicity of 20 msec and a duration of 4 msec each. It can be predetermined that MG occasion #1 and #3 will overlap with data burst #1 and #4, respectively, because the overlapping part is located at start of a traffic periodic and is shorter than average duration of a data burst. Hence whether they can be overridden by data can be configured by RRC.  On the other hand, whether MG occasion #2 overlaps with data burst #2 depends on the size of data burst #2, because it starts in the latter part of a traffic period. Therefore, for such cases, it makes more sense to use dynamic signaling to indicate whether a MG occasion can be overridden by data.

Figure 1. MG occasions vs data bursts.

Observation 3. 	RRC configuration and DCI-based signaling can complement each other in indicating whether a measurement gap occasion may be overridden by data.

Based on the above analysis, we think RAN2 should discuss the option of using RRC configuration to indicate whether a measurement gap can be overridden by data. And RAN2 is well qualified to do so, because RRC configuration is within RAN2’s scope.

Proposal 1. 	RAN2 discuss the option of using RRC configuration to indicate whether a measurement gap can be overridden by data.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Observation 1.	Measurement gap occasions that overlap with the start of data bursts (with jitter considered), are periodic and can be identified deterministically based on their periodicities and relative start offsets.

Observation 2. 	For measurement gaps that are known to overlap with data, using RRC configuration to specify which ones can be overridden by data reduces signaling and simplifies UE implementation.

Observation 3. 	RRC configuration and DCI-based signaling can complement each other in indicating whether a measurement gap occasion may be overridden by data.

Proposal 1. 	RAN2 discuss the option of using RRC configuration to indicate whether a measurement gap can be overridden by data.