3GPP TSG-RAN WG2 Meeting #127bis	R2-2408132

Hefei, China, Oct 14 – 18, 2024

Agenda item:	8.7.5

Source:	Qualcomm Incorporated

Title:	Discussion on RLC enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss enhancements for timely RLC retransmissions and avoiding unnecessary retransmissions. For both issues, we analyze pros and cons of different options and then propose way-forwards.

# Discussion

## Timely retransmission

At RAN2#127 meeting, the following agreement was made:

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

In the following, we analyze the pros and cons of the identified options.

The first observation we’d like to make is that enhanced polling needs to be considered together with enhanced status report. That is because a receiver does not act on a received poll immediately if the status prohibit timer at the receiver is running. As a result, for a given duration of the t-statusProhibit timer, whether to send more polls or send polls earlier does not have much impact on the RLC round trip time.

Observation 1.	Enhancements to polling should be considered together with enhancements to status reporting, which is controlled by the timers at receiver.

Now let us check how frequent polls and status reports can be sent. Currently,

- The minimum pollPDU is 4, and pollByte is 1KB. With the typically high data rate of XR applications, these thresholds correspond to very short durations already. For example, if the data rate is 2 Mbps, and pollByte is 1 KB, then polls are triggered once every 4 msec.

- The minimum non-zero length of the status prohibit timer is 5 msec.

We think these values are already very close to the shortest durations that may be configured on FR1 carriers, without increasing the risk of the resource waste and false RLF. That is because the round-trip time of a DL HARQ transmission on a FR1 carrier with 30KHz SCS takes 2 msec and the typical number of HARQ transmissions is 3. Therefore, the legacy already allows network to configure very frequent polls and status reports, if needed.

In addition, the above analysis also gives us an upper bound on how much delay reduction enhanced polling or enhanced status reporting can achieve, which is ~5 msec. Compared to the typical RLC round-trip time, this reduction is marginal at its best.

Observation 2.	Network currently can configure very frequent polls and status reports. Further enhancements only result in marginal reductions in RLC round-trip time.

If one breaks the total RLC round-trip time into individual steps in each protocol layer, it is not difficult to see that wait time before sending polls and status reports only contribute a small portion of the total delay (5 msec or less). The longest component is the wait time for the receiver to send back a status report (10~20ms), which it includes at least the delay for UE to get UL grant and UL HARQ transmission. Among the three identified options, only autonomous retransmission can eliminate that longest component. Therefore, RAN2 should focus more on autonomous retransmission and deprioritize study on enhancements to polling and status reporting.

Proposal 1. 	Deprioritize options for enhanced polling and enhanced status reporting.

For autonomous retransmission, there was proposals to reuse/enhance the existing mechanism of the legacy t-PollReTx timer. Since the length of the t-PollReTx timer should match at least the typical delay required to get a status report back from the receiver, from delay performance point of view, it is no different from other options based on enhanced polling. For this reason, we do not think this needs to be discussed further.

Observation 3.	From a delay performance perspective, autonomous retransmission based on t-PollReTx timer is not different from options based on enhanced polling and therefore can be deprioritized.

For autonomous retransmission, there are several options proposed in [1]:

- after the remaining time of the PDU has dropped below a configured threshold; or

- after the PDU has failed a configured number of HARQ transmissions; or

- if the PDU is in the RLC retransmission buffer and there are spare PUSCH resources available after the LCP procedure.

Option #1 is useful because it gives the transmitter one more chance to send the PDU because its PDB depletes. If the threshold is configured to be compatible with the one-way delay from the transmitter to the receiver, then in the worst case, it creates only one extra transmission. Hence the impact on capacity is limited.

In typical network implementations, a HARQ process is terminated after a MAC PDU has gone through several failed transmissions. The remaining of the delivery is handled by RLC retransmission. Option #2 is designed based on this observation, i.e. if network lets UE know about its max number of HARQ retransmission via configuration of the trigger threshold, UE can timely trigger RLC retransmission without waiting for status reports. Even if in some network implementation maximum number of HARQ retransmissions varies across HARQ processes, network can choose a threshold such that it covers a large percentage of HARQ processes. Moreover, since HARQ failure typically has very low probability (e.g. 1% after 2 HARQ transmissions), this scheme will introduce only a very small number of retransmissions, and more importantly, this size of this increase is full controllable by network through the trigger threshold.

Option #3 uses only spare resources in a TB to retransmit RLC PDUs. Therefore, there is no impact on capacity at all.

Observation 4.	Autonomous retransmission, if configured properly, has no or very limited impact on capacity.

Based on the above analysis, we propose that RAN2 should focus its discussion on autonomous retransmission, e.g. the three options discussed above.

Proposal 2.	RAN2 focus the discussion on autonomous retransmission, especially the following options:

- after the remaining time of a PDU has dropped below a configured threshold; or

- after a PDU has failed a configured number of HARQ transmissions; or

- if a PDU is in the RLC retransmission buffer and there are spare PUSCH resources available after the LCP procedure.

## Avoid unnecessary retransmissions

At RAN2#127 meeting, the following agreements were made on the options for avoiding unnecessary retransmissions:

- Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first. - In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer. - In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

In the following, we compare the impacts of the above options on transmitter and receiver procedures, as well as the control-plane signaling.

A key advantage of the transmitter-based approach is that a transmitter has the knowledge of exact remaining time of an SDU. After the remaining time of an SDU expires, it needs to discard the SDU from its retransmission buffer and then notify the receiver of the discard. Hence a new control PDU from the transmitter needs to be introduce. At the receiver, it needs to update its state variables based on the SN of the discarded PDU.

In summary, the transmitter-based approach has impact on both the transmitter and the receiver procedures, as well as the control-plane signaling.

Observation 5.	Transmitter based approach requires a new control PDU and has impact on both transmitter and receiver procedures.

In the receiver-based approach, because a receiver does not have the knowledge of an SDU’s remaining time, it has to estimate that through a timer. This timer can work in a way similar to t-reassembly timer in the RLC UM, i.e. it is started when the receiver detects a gap in sequence numbers. After this timer expires, the receiver stops its attempt to receive the corresponding SDU and sends back a “fake ACK” to the transmitter, so that the transmitter can advance its transmission window.

The advantage of the receiver-based approach is its simplicity compared to the transmitter’s approach. On the receiver side, the only change required is a new timer and state variable updates based on the expiry of this new timer. There is no impact on the legacy transmitter procedure and the control-plane signaling.

However, this timer may not be able to detect obsolete SDUs in all cases. For example, if an obsolete SDU is the last one in a data burst, or the arrival jitter of the next SDU is longer than the new timer, then the timer will not be started timely, even after the SDU has already become obsolete.

Observation 6.	Receiver based approach has impact only on the receiver procedure. However, it may not be able to detect obsolete SDUs in all cases (e.g. last one in a data burst).

This shortcoming with the receiver-based approach can be avoided by a minor enhancement on the transmitter side. For example, the transmitter can stop retransmission of an SDU after it becomes obsolete.

There can be different options to terminate obsolete SDUs. For example, PDCP can send an indication to RLC when an SDU has no more remaining time left. Or network can configure the maxReTxThreshold to match with the PDB of the corresponding DRB.

Either option requires some changes to the legacy procedure:

- The first option requires removal of SDUs from retransmission buffer upon the indication from PDCP. That may have impact on the existing UE implementation of the RLC user-plane procedures.

- The second option requires a new trigger for RLF to be defined for DRBs enabled with the remaining time based retransmission limit. For example, the trigger could be that RLF is triggered after N or more SDUs have reached maxReTxThreshold in the last T msec. The advantage of this option is that it does not have any impact on the legacy user-plane procedures at RLC transmitters. Therefore, the second option is more preferred.

Observation 7.	The combined approach avoids the shortcoming of the receiver-based approach without significant increase in complexity.

In this combined approach, the receiver should still send back “fake ACKs” after an SDU is deemed obsolete, because this feedback tells the transmitter that the receiving window has advanced and it is safe for it to advance its transmission window. This explicit feedback is more robust than relying on network configuration to ensure the synchronization between transmission and receiving windows.

Observation 8.	In the combined approach, the synchronization between transmission and receiving windows is more robust if the receiver sends back “fake ACKs” rather than relying only on network implementation.

Therefore, based on the above analysis, we propose to adopt the following combined approach:

Proposal 3. 	Adopt the following combined approach, in which

-	transmitter stops retransmission of an obsolete SDU after reaching maxReTxThreshold. FFS how to re-define the RLF trigger.

- 	receiver stops reception of an obsolete SDU based on expiry of a new timer and sends back a “fake ACK” in status report.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Timely retransmission

Observation 1.	Enhancements to polling should be considered together with enhancements to status reporting, which is controlled by the timers at receiver.

Observation 2.	Network currently can configure very frequent polls and status reports. Further enhancements only result in marginal reductions in RLC round-trip time.

Proposal 1. 	Deprioritize options for enhanced polling and enhanced status reporting.

Observation 3.	From a delay performance perspective, autonomous retransmission based on t-PollReTx timer is not different from options based on enhanced polling and therefore can be deprioritized.

Observation 4.	Autonomous retransmission, if configured properly, has no or very limited impact on capacity.

Proposal 2.	Focus the discussion on autonomous retransmission, e.g. the following options:

- after the remaining time of a PDU has dropped below a configured threshold; or

- after a PDU has failed a configured number of HARQ transmissions; or

- if a PDU is in the RLC retransmission buffer and there are spare PUSCH resources available after the LCP procedure.

Avoid unnecessary retransmissions

Observation 5.	Transmitter based approach requires a new control PDU and has impact on both transmitter and receiver procedures.

Observation 6.	Receiver based approach has impact only on the receiver procedure. However, it may not be able to detect obsolete SDUs in all cases (e.g. last one in a data burst).

Observation 7.	The combined approach, in which transmitter and receiver independently terminates retransmission and reception, avoids the shortcoming of the receiver-based approach without significant increase in complexity.

Observation 8.	In the combined approach, the synchronization between transmission and receiving windows is more robust if the receiver sends back “fake ACKs” rather than relying only on network implementation.

Proposal 3. 	Adopt the following combined approach, in which

-	transmitter stops retransmission of an obsolete SDU after reaching maxReTxThreshold. FFS how to re-define the RLF trigger.

- 	receiver stops reception of an obsolete SDU based on a new timer and sends back a “fake ACK”.

# References

- R2-2402390, Discussion on RLC enhancements, Qualcomm Incorporated, April 2024.