3GPP TSG-RAN WG2 Meeting #127bis	R2-2408133

Hefei, China, Oct 14 – 18, 2024

Agenda item:	8.7.6

Source:	Qualcomm Incorporated

Title:	Discussion on signaling enhancements for XR rate control

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In the latest WID for Rel-19 XR SID, the following objective is added:

Objective: Specify uplink congestion signaling [RAN2]: - Specify in MAC layer XR rate control signaling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion

In this paper, we discuss whether a new signaling mechanism is needed or the legacy Recommended Bit Rate procedure can be enhanced to meet the objective.

# Discussion

Many implementations of XR applications are capable of rate adaptation, i.e. an application can adjust its bit rate based on the quality of its network connection, such as packet error, available bit rate, etc. With wireless link often being the bottleneck in a network connection, if an application can obtain such information from RAN, it will be able to achieve faster and more efficient rate adaptation.

NR already has such a mechanism since Rel-15, which is the Recommend Bit Rate procedure (clause 5.18.10, TS 38.321). Network can use this procedure to indicate the recommended bit rate for the UE for a specific logical channel and a specific direction. UE can use this procedure to request the gNB to indicate the recommended bit rate for a specific logical channel and a specific direction.

It is not difficult to see that the existing Recommended Bit Rate (RBR) procedure can meet the basic requirements of XR rate control. Therefore, instead of developing a new signaling mechanism from scratch, it is more sensible to take the existing RBR procedure as the baseline for the XR rate control enhancement.

Observation 1.	The legacy recommended bit rate procedure meets the basic requirements of XR rate control signaling.

Proposal 1. 	Use the legacy recommended bit rate procedure and the Recommended Bit Rate MAC CE as the baseline for XR rate control signaling.

On the other hand, the legacy RBR procedure and RBR MAC CE are not 100% ready for XR applications. In the following, we discuss a few aspects that may require enhancements.

A recommended bit rate is indicated in a 6-bit field in the current RBR MAC CE. The encoding of bit rates is defined in Table 6.1.3.20-1 in the MAC spec. A closer look at the table may reveal that each code point has a different quantization error, which varies from 6.7% to 30.8% in a non-monotonic manner. This level of granularity and variation does not meet the requirements of many rate control algorithms. For example, Google congestion control algorithm, described in clause 6.18.2.2 of TR 26.822, along with a few other rate control algorithms, requires more precision. If one follows the formular specified in that TR and works out a few numerical examples, one may find that those algorithms require a granularity of 8% for bit rate increase and 1% for bit rate decrease.

Therefore, we think the granularity of the bit rate table needs to be improved. To achieve that, we may need to refine the range of the bit rates and add more code points. For example, the current maximum bit rate of 1.6 Gbps clearly is much higher than typical XR bit rates and hence unnecessary, and 64 code points are not enough to encode a range of 1.6 Gbps with only 1% quantization error.

Observation 2. 	The bit rates used by the legacy Recommend Bit Rate MAC CE have large quantization errors (6.7% to 30.8%), which may not be adequate for XR applications.

But before RAN2 dive into design details of the new table, we think it is more sensible for RAN2 to first consult with SA4 what the proper range and granularity for the bit rates of typical XR application should be.

Proposal 2.  	Send an LS to SA4 requesting the required range and acceptable quantization error for the bit rate table.

One of the special characteristics of XR applications is that it generates multiple flows with different QoS requirements, e.g. video, audio, haptic, control, etc. When an application performs rate adaptation, it typically adjusts the rates of individual flows simultaneously. The legacy Recommended Bit Rate MAC CE includes indication for only one logical channel. To better support simultaneous rate adaptation of multiple flow in XR applications, the MAC CE can be extended to include recommended bit rates of multiple logical channels.

Observation 3. 	XR applications may adopt bit rates of multiple flows simultaneously.

Proposal 3. 	Enhance the Recommended Bit Rate MAC CE to include bit rates of multiple logical channels.

There were different views at the RAN plenary meeting whether the granularity of rate indication should be per logical channel (as in legacy) or per QoS flow. The motivation for the latter option is that network may multiplex different QoS flows in some network implementations, e.g. video and audio, in a single DRB.

However, in our understanding, if an XR application is given a total available bandwidth for its traffic, it should be able to determine how to distribute the available bandwidth among its flows. Moreover, the application should have better knowledge than network in how to distribute the total available bandwidth among different types of flows based on its own operation state. For example, during congestion, it may prefer to maintain the bit rate of its audio flow but reduce the bit rate of its video flows. Or when there is extra bandwidth available, it may give this extra bandwidth to only video flows.

Therefore, we think it is fine to keep the rate indication per logical channel as in legacy. It can be left to XR applications to decide how to distribute the available bit rate among QoS flows within a logical channel.

Proposal 4. 	Keep the granularity of bit rate indication per logical channel as in legacy.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Observation 1.	The legacy recommended bit rate procedure meets the basic requirements of XR rate control signaling.

Proposal 1. 	Use the legacy recommended bit rate procedure and the Recommended Bit Rate MAC CE as the baseline for XR rate control signaling.

Observation 2. 	The bit rates used by the Recommend Bit Rate MAC CE have large quantization errors (6.7% to 30.8%), which may not be adequate for many XR applications.

Proposal 2.  	Send an LS to SA4 requesting the required range and acceptable quantization error for the bit rate table.

Observation 3. 	XR applications may adopt bit rates of multiple QoS flows simultaneously.

Proposal 3. 	Enhance the Recommended Bit Rate MAC CE to include bit rates of multiple logical channels.

Proposal 4.	Keep the granularity of bit rate indication per logical channel as in legacy.