3GPP TSG-RAN WG2 Meeting #127bis	R2-2408135

Hefei, China, 14th – 18th October, 2024

Agenda Item:	8.7.4.2

Source: 	OPPO

Title:  	Discussion on delay-aware DSR enhancements for XR

Document for:	Discussion and Decision

# Introduction

In Rel-19 XR WID [1] revised in the last RANP#105 meeting, the detailed objectives on enhancement for Scheduling are updated as follows.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This contribution would like to provide our considerations on delay-aware DSR enhancements for scheduling.

# Discussion on delay-aware DSR enhancements

In the last few meetings, RAN2 discussed and agreed to enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. In RAN2#127 meeting, the following agreements were made. Here we further discuss the leftover issues as highlighted below.

Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In legacy DSR, for each LCH only one remaining time threshold is used for both DSR triggering and reporting. At the same time, the remaining time threshold is also used to differentiate the definitions of delay-critical data and non-delay-critical data.

For enhanced DSR in Rel-19 XR, caused by the introduced multiple reporting thresholds, the relationship between the single DSR triggering threshold and mutiple reporting thresholds is not clear, for example,

- Case-1: All reporting thresholds are larger than or equal to DSR triggering threshold, e.g., ThreshDSR <= ThreshReport1 < ThreshReport2 < ThreshReport3,

- Case-2: All reporting thresholds are smaller than or equal to DSR triggering threshold, e.g., ThreshReport1 < ThreshReport2 < ThreshReport3 < = ThreshDSR,

- Case-3: DSR triggering threshold is in the middle of multiple reporting thresholds, e.g., ThreshReport1 < ThreshReport2 <= ThreshDSR < ThreshReport3.

Besides, the definition of delay-critical data is also needed to be clarified caused by the introduced multiple reporting thresholds.

In our understanding, no need to have any constraints on how the NW configures DSR triggering and reporting thresholds. And even though multiple reporting thresholds are introduced, the delay critical data could be still defined based on the single DSR trigger threshold as legacy. In this way, for Case-1 NW can rely on the multiple reporting thresholds to obtain finer delay status information for the data that is going to be delay-critical. For Case-2, NW can rely on the multiple reporting thresholds to obtain finer delay status information for the data that has already been delay-critical. And Case-3 is the combination of both Case-1 and Case-2. Therefore, up to NW implementation, NW could be flexible to obtain the delay status information it would like to know to assist the scheduling better.

- Flexible configuration of DSR triggering and reporting thresholds allows NW to obtain finer delay status information it would like to know, including both delay critical data (i.e., the data that has already been delay-critical) and non-delay critical data (i.e., the data that is going to be delay-critical).

- No need to restrict how NW configures DSR triggering and reporting thresholds.

- Delay critical data is still defined based on the single DSR trigger threshold, regardless of the setting of multiple reporting thresholds.

Regarding the FFS issue whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. In our understanding, changing the buffer size calculation for DSR to include non-delay critical data ahead of delay critical data in the buffer is one kind of solutions on DSR enhancement to include information of non-delay critical data. Based on the proper configuration of DSR trigger threshold and multiple reporting thresholds, to some extent the information of non-delay critical data has already been included by reporting multiple pairs of remaining time and buffer sizes, NW can collect sufficient information to perform proper scheduling for UE. Therefore, we can live without any optimization for on-delay critical data ahead of delay critical data in the buffer.

- Thanks to the enhancement of multiple reporting thresholds, to some extent the information of non-delay critical data has already been captured by reporting multiple pairs of remaining time and buffer sizes.

- RAN2 not pursue optimization for non-delay critical data ahead of delay critical data in the buffer.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- Flexible configuration of DSR triggering and reporting thresholds allows NW to obtain finer delay status information it would like to know, including both delay critical data (i.e., the data that has already been delay-critical) and non-delay critical data (i.e., the data that is going to be delay-critical).

- No need to restrict how NW configures DSR triggering and reporting thresholds.

- Delay critical data is still defined based on the single DSR trigger threshold, regardless of the setting of multiple reporting thresholds.

- Thanks to the enhancement of multiple reporting thresholds, to some extent the information of non-delay critical data has already been captured by reporting multiple pairs of remaining time and buffer sizes.

- RAN2 not pursue optimization for non-delay critical data ahead of delay critical data in the buffer.

# 4. Reference

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia