3GPP TSG-RAN WG2 Meeting #127bis	R2-2408136

Hefei, China, 14th – 18th October, 2024

Agenda Item:	8.5.4

Source: 	OPPO

Title:  	Discussion on adaptation of common signal/channel transmission

Document for:	Discussion and Decision

# Introduction

In Rel-19 NES WID [1], the detailed objectives on adaptation of common signal/channel transmissions are captured as follows.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

This contribution would like to provide our considerations on this objective of Rel-19 NES.

# Discussion

# Adaptation of SSB in time domain

The SSB configuration in time domain includes SSB periodicity and time domain positions of the transmitted SSB in an SSB burst. For legacy RRC_IDLE/INACITVE UE, CD-SSB is configured by the parameters ssb-periodicityServingCell and ssb-PositionsInBurst in ServingCellConfigCommonSIB in SIB1. Network may adapt SSB in time domain relying on system information modification, and a modification period is used as effective time. For legacy RRC_CONNECTED UE, both CD-SSB and NCD-SSB could be configured and adapted by network via dedicated RRC signalling.

Based on R1 minutes, R1 has made some agreements on this.

R1#117 Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

From R2 perspective, there are no sufficient conclusions made by R1 on RACH adaptation, so R2 still needs to wait for further progress of R1 on this topic.

Note that in NES O1, i.e., on-demand SSB SCell supporting, a MAC CE has already been introduced for periodicity change. For adaptation of SSB in time domain, whether we can rely on it or not is unclear.

- For adaptation of SSB in time domain, it is not clear whether to reuse the MAC CE introduced for SSB periodicity change of SCell in NES O1 or not.

- For adaptation of SSB in time domain, R2 waits R1 for further progress.

# Adaptation of RACH in time domain

In legacy, the time domain configuration of RACH occasion (RO) is configured using an index via SIB or dedicated RRC signalling, i.e., by prach-ConfigurationIndex, where the periodicity and time domain positions are mapped in TS 38.211. Besides, the parameters PRACH mask indexes (i.e., ra-ssb-OccasionMaskIndex) are also used to define RO associated with an SSB in which the MAC entity may transmit a Random-Access Preamble, and the mapping table is defined in TS 38.321.

Similarly, most work on the enhancement of adaptation mechanisms is also within R1 scope. Based on R1 minutes shown, R1 has made some agreements on this.

R1#117 Agreement For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE), - At least DCI based adaptation is supported. No introduction of new DCI format.

For the adaptation mechanism of RACH in time domain, R1 has already decided to support DCI-based adaptation, which is fully up to R1 to further specify the detailed design. No R2 work is foreseen.

- For the adaptation mechanism of RACH in time domain, R1 has already decided to support DCI-based adaptation.

- For adaptation of RACH in time domain, R2 relies on R1 to design DCI-based adaptation.

Another leftover issue raised by companies in the email discussion is RACH bundling, i.e., given the PO bundles, whether to also adapt PRACH resources so that RO can be bundled with PO bundles. In this case, from network perspective, network can perform both paging transmission and RACH reception in a shorter time range to obtain additional power saving gain.

Currently, it is still not clear whether the resulted R1 conclusions for DCI-based RACH adaptation can support this kind of RACH bundling or not. Further check and confirmation are needed. In our understanding, if there is a need to go for RACH bundling, it is preferred that R1 lead the work.

- Currently it is still not clear whether the resulted R1 conclusions for DCI-based RACH adaptation can support this kind of RACH bundling or not.

- Up to R1 to evaluate and lead the work for RACH bundling if needed.

# Adaptation of Paging

In R2#125bis meeting, the following agreements on paging adaptation were made.

Agreements on paging adaptation: - For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE: - Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

Regardless of which option is adopted for paging adaptation, the compatibility of legacy RRC_IDLE/INACTIVE UE needs to be considered. As agreed in the last meeting, the following options are included:

- Option 1: prevent the access of legacy UE via barring,

- Option 2: separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs).

Legacy UE impact – barring mechanism

For Option 1, a new barring bit, e.g., cellBarredNES-paging, specific to Rel-19 NES UE supporting paging adaptation enhancement is needed to introduce on top of the legacy barring bit cellBarred. In this case, no legacy UE, i.e., only Rel-19 NES UEs, could access the cell supporting paging adaptation, therefore no need to handle legacy UE-related issues, and the network power saving gain would be maximum. However, it leads to a negative impact on legacy UEs.

- If Option 1 (i.e., prevent the access of legacy UE via barring) is adopted, a new barring bit specific to Rel-19 NES UE supporting paging adaptation is introduced on top of the legacy barring bit cellBarred.

Here we further discuss the barring mechanism of the new barring bit cellBarredNES-paging. In legacy, cellBarred is mandatory present with two values “barred” or “not barred”, while the presence and setting for cellBarredNES-paging as well as the UE interpretation on barring bits should be considered case by case, as analyzed in the following table. Note that what we discuss here is the barring mechanism only for the paging adaptation feature, and whether other features for Rel-19 NES UE could share a common barring mechanism or not is FFS.

| Presence and setting of cellBarred and “cellBarredNES-paging” | UE interpretation on the barring bits |  | Use case for such setting |
| --- | --- | --- | --- |
|  | Rel-19 UE supporting paging adaptation (both barring bits could be understood) | UE not supporting paging adaptation (only cellBarred could be understood) |  |
| cellBarred: “barred”; cellBarredNES-paging: absent | The cell is barred. | The cell is barred. | Legacy cell would like to bar all UEs. |
| cellBarred: “barred”; cellBarredNES-paging: “not barred” | The cell is not barred. |  | Rel-19 NES cell only allows Rel-19 UE supporting paging adaptation to camp. |
| cellBarred: “barred”; cellBarredNES-paging: “barred” | The cell is barred. |  | Rel-19 NES cell would like to bar all UEs. => Not needed. |
| cellBarred: “not barred”; cellBarredNES-paging: absent | The cell is not barred. | The cell is not barred. | Legacy cell would like to allow all UEs to camp. |
| cellBarred: “not barred”; cellBarredNES-paging: “not barred” | The cell is not barred. |  | Rel-19 NES cell would like to allow all UEs to camp. |
| cellBarred: “not barred”; cellBarredNES-paging: “barred” | The cell is barred. |  | Rel-19 NES cell bar Rel-19 UE supporting paging adaptation, but allow UE not supporting paging adaptation to camp. => Not a reasonable case. |

What we can observe based on the table above is that

1/ for the cell that cellBarred is set to “barred”, regardless cellBarredNES-paging is absent or set to “barred”, the cell would like to bar all UEs. No need to differentiate whether the cell is a legacy cell or a Rel-19 NES cell.

2/ for the cell that cellBarred is set to “not barred” while cellBarredNES-paging is set to “barred”, it is not a reasonable setting since in this case Rel-19 NES cell would bar Rel-19 UE supporting paging adaptation but allow Rel-19 UE not supporting paging adaptation and legacy UE to camp.

Therefore, we may conclude that for the new barring bit specific to Rel-19 NES UE supporting paging adaptation enhancement, it is not necessary to set cellBarredNES-paging to value “barred”, keeping “not barred” and absent for cellBarredNES-paging is sufficient. It is similar to what we have done for cellBarredNES in Rel-18 NES which is also optionally present with only one value “not barred”.

- Setting new barring bit specific to Rel-19 NES UE supporting paging adaptation enhancement to value “barred” is not necessary.

- In Rel-18, cellBarredNES is also optionally present with only one value “not barred”.

Regarding the UE interpretation on the barring bits, we may also mimic the barring solution for Rel-18 NES as baseline, as shown as follows,

1> if the UE supports nes-CellDTX-DRX and it is in RRC_IDLE or in RRC_INACTIVE, or if the UE supporting nes-CellDTX-DRX is in RRC_CONNECTED while T311 is running: 2> if cellBarred in the acquired MIB is set to barred: 3> if cellBarredNES is absent in the acquired SIB1: 4> consider the cell as barred in accordance with TS 38.304 [20]; 4> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

i.e., for UE supporting paging adaptation, if cellBarred is set to barred and cellBarredNES-paging is absent, UE considers the cell as barred.

- For Rel-19 UE supporting paging adaptation enhancement, if cellBarred is set to barred and the new barring bit specific to Rel-19 NES UE supporting paging adaptation enhancement is absent (which optionally presents with only one value “not barred”), UE considers the cell as barred.

Legacy UE impact – separate paging resources

If legacy UEs are not prevented from the cell supporting paging adaptation enhancement, Option 2, i.e., separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs), would be adopted. In this case, legacy UE-related issues need to be considered.

For Option 2, it needs to be clarified further that the separate paging resources mean different paging configurations for Rel-19 NES UE and legacy UE respectively, rather than whether paging resources (i.e., PF/PO/PDCCH-monitoring occasion) are separate completely, i.e., it allows partial overlapping paging resources between Rel-19 NES UE and legacy UE. In the shared PO/PF, both Rel-19 NES UE and legacy UE can be paged by networks.

- Allow separate configuration of paging resources for legacy UE and Rel-19 NES UE, and up to network to decide whether to use shared paging resources (e.g., PF/PO/PMO).

Another leftover issue needed to be further discussed is the monitoring behaviour of Rel-19 NES UE. For legacy UE (i.e., the UE not supporting paging adaptation enhancement), it naturally only monitors paging resources based on the current paging configuration. In contrast, for Rel-19 NES UE supporting paging adaptation enhancement, whether it can monitor legacy paging resources and/or new paging resources is not clear.

From UE perspective, monitoring both legacy and new paging resources would lead to unnecessary power consumption of Rel-19 NES UE. For Rel-19 NES UE, it is straightforward to only monitor new paging resources if configured. If new paging resources are configured, there is no need to introduce a configurable mechanism to indicate Rel-19 NES UE to use legacy paging resources instead, since the benefit of the configurable mechanism is not justified.

- From UE perspective, monitoring both legacy and new paging resources would lead to unnecessary power consumption of Rel-19 NES UE.

- The benefit of a configurable mechanism to allow network to indicate whether Rel-19 NES UE to use legacy paging resources or new paging resources is not justified.

- Rel-19 NES UE supporting paging adaptation enhancement only monitors the new paging resources, if configured. Otherwise (i.e., if the new paging resource is NOT configured), it only monitors legacy paging resources.

Capability for paging adaptation enhancement

The paging adaptation enhancement introduced in Rel-19 NES should be an optional feature for a Rel-19 UE to support, thus needing an optional capability reported to network to keep a common understanding between UE and network. R2 can confirm the introduction of the optional capability with signalling.

However, it is unclear from R2 perspective whether the capabilities of paging adaptation enhancement for each frequency band of UE are coincident or not. It is worth referring that for PEI enhancement in Rel-17, per-band capability (i.e., the band list supporting PEI) is introduced. R2 may further discuss the granularity of this UE capability (e.g., per UE, per band, etc.). In case that R2 is not confident to make the decision, R1/4 may need to be involved on this issue.

- Introduce a UE AS capability on whether the UE supports paging adaptation enhancement for Rel-19 NES, which is an optional capability with signalling.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

Adaptation of SSB in time domain

- For adaptation of SSB in time domain, it is not clear whether to reuse the MAC CE introduced for SSB periodicity change of SCell in NES O1 or not.

- For adaptation of SSB in time domain, R2 waits R1 for further progress.

Adaptation of RACH in time domain

- For the adaptation mechanism of RACH in time domain, R1 has already decided to support DCI-based adaptation.

- For adaptation of RACH in time domain, R2 relies on R1 to design DCI-based adaptation.

- Currently it is still not clear whether the resulted R1 conclusions for DCI-based RACH adaptation can support this kind of RACH bundling or not.

- Up to R1 to evaluate and lead the work for RACH bundling if needed.

Adaptation of Paging

- If Option 1 (i.e., prevent the access of legacy UE via barring) is adopted, a new barring bit specific to Rel-19 NES UE supporting paging adaptation is introduced on top of the legacy barring bit cellBarred.

- Setting new barring bit specific to Rel-19 NES UE supporting paging adaptation enhancement to value “barred” is not necessary.

- In Rel-18, cellBarredNES is also optionally present with only one value “not barred”.

- For Rel-19 UE supporting paging adaptation enhancement, if cellBarred is set to barred and the new barring bit specific to Rel-19 NES UE supporting paging adaptation enhancement is absent (which optionally presents with only one value “not barred”), UE considers the cell as barred.

- Allow separate configuration of paging resources for legacy UE and Rel-19 NES UE, and up to network to decide whether to use shared paging resources (e.g., PF/PO/PMO).

- From UE perspective, monitoring both legacy and new paging resources would lead to unnecessary power consumption of Rel-19 NES UE.

- The benefit of a configurable mechanism to allow network to indicate whether Rel-19 NES UE to use legacy paging resources or new paging resources is not justified.

- Rel-19 NES UE supporting paging adaptation enhancement only monitors the new paging resources, if configured. Otherwise (i.e., if the new paging resource is NOT configured), it only monitors legacy paging resources.

- Introduce a UE AS capability on whether the UE supports paging adaptation enhancement for Rel-19 NES, which is an optional capability with signalling.

# 4. Reference

[1] RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson