3GPP TSG-RAN WG2 Meeting #127-bis                                 R2-2408137

Hefei, China, 14th – 18th October, 2024

Source: 			OPPO

Title: 	Discussion on paging procedure for Ambient IoT

Agenda item:		8.2.3

Document for: 	Discussion and Decision

- Introduction

In the last RAN2 meeting, regarding paging procedure for Ambient IOT, following agreement has been achieved.

Agreements:

1 Support mechanism for reader to be able to trigger multiple/subsequent A-IOT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

In this contribution, we aim to provide our views on the paging procedure for the A-IOT system.

2. Discussion

- 2.1 avoiding duplicate response from devices

There are two scenarios that multiple/subsequent A-IOT paging messages are associated with the same request from the CN:

- The same CN request is sent towards more than one reader for enhancing the possibility of reaching the target A-IOT devices

- The reader sends more than one initial A-IOT paging messages at different time to avoid the case that A-IOT devices miss a particular paging message due to low battery level.

In order to let A-IOT device do not respond to the repeated paging messages to save energy and radio resource, an ID identifying the CN request may be needed to be included in the paging message. Some companies claimed that the terminology paging area could be defined, and the device will only respond to the A-IOT paging message originated from the current camping paging area. But this solution could only deal with the first bullet. The A-IOT device still needs to respond to consecutive paging messages from the same reader corresponding to the same CN request. In addition, the A-IOT device needs to be aware of its current paging area information, which complicates the A-IOT device implementation. As a result, we propose to include an ID identifying the CN request in the A-IOT paging message.

Observation 1: repeated A-IOT paging messages corresponding to the same CN request are possible in two scenarios. Firstly, the CN request is sent towards more than one reader for enhancing the coverage of CN request. Secondly, A-IOT paging messages are sent consecutively to avoid the case that A-IOT devices miss a particular paging message due to low battery level.

Proposal 1: include an ID identifying the CN request in the A-IOT paging message to avoid A-IOT device making duplicated responding to A-IOT paging messages corresponding to the same CN request.

- 2.2 Determination of transmission/access occasions

In the RAN2 #126 meeting, it was agreed to consider four different cases covering different number of targeted A-IOT devices to be paged:

- a message containing an ID of a single A-IoT device.

- a message containing a group ID that maps to multiple A-IoT devices.

- a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message.

- a message containing multiple IDs of A-IoT devices.

Subsequently, we will categorize them into two classes: a paging message for a single A-IOT device, and a paging message for more than one A-IOT devices.

- 2.2.1 a paging message containing an ID of a single A-IOT device

In such case, the paged A-IOT device could access to the network immediately, after reception of the A-IOT paging message, subject to the restriction of any potential message processing delay. The paging message should just indicate the D2R message scheduling information, i.e., the time-frequency radio resource for the A-IOT device to send msg1, considering that RAN1 is discussing the multiple-access scheme for the A-IOT device.

Observation 2: when receiving of a A-IOT paging message containing only one ID of a single A-IOT device, the A-IOT device could access to the network immediately, after reception of the A-IOT paging message, subject to the restriction of any potential message processing delay.

Proposal 2: to trigger the one-device CFRA procedure, A-IOT paging message should indicate the D2R message scheduling information for msg1 transmission.

- 2.2.2 a message for more than one A-IOT devices

In such case, (potentially) more than one A-IOT devices are paged by the A-IOT paging message, each of them should know in which transmission/access occasion they can access to the network. In RFID system, the reader sends a number Q to the target A-IOT devices in the inventory message. Subsequently, each targeted A-IOT device will generate a random integer number between 0 and 2Q-1. Only when the integer number equals to 0, it will perform access towards the network. Otherwise, it will wait for subsequent QueryRep command to decrease the integer number by 1, or re-roll the integer number, when it receives the QueryAdjust command. In our opinion, such principle of randomizing the A-IOT device transmission/access occasion could be re-used for the 5G NR contention-based A-IOT procedure. Moreover, presence of such parameter implies the requirement towards the paged A-IOT devices to apply the contention-based access procedures.

Observation 3: in the RFID system, the reader sends a number Q to the target A-IOT devices in the inventory message. Subsequently, each targeted A-IOT device will generate a random integer number between 0 and 2Q-1. Only when the integer number equals to 0, it will perform access towards the network.

Proposal 3: To trigger a contention-based random access procedure, A-IOT initial paging message needs to include a parameter to randomize the access occasions for different A-IOT devices and also indicate the total number of A-IOT access occasions.

On the other hand, for multi-device contention free random access procedure, paging message should indicate the mapping relationship between distinguished device IDs and access occasion indexes. For instance, the most naïve approach to indicate such mapping information is in the manner of one-by-one, though, this approach will suffer from overwhelmingly high signalling overhead and device high scanning load, if a large group of A-IOT devices shall be triggered by one message.

Hence, we also propose RAN2 to study how to reduce signalling overhead of the paging message to indicate dedicated access/transmission occasions for the multi-device random access procedure. For example, the access occasions could be allocated to A-IOT devices considering the order of the IDs, either in ascending or descending order, potentially depending on the reader’s indication included in the A-IOT paging message or the default setting. In addition, the PF/PO-like solution could be also considered. A targeted A-IOT device is distributed to access occasion matched with the remainder of the device ID mod a new parameter defining the number of the initial set of access occasions. In such way, the collision is restricted to A-IOT devices sharing the same ID modulo operation result. Therefore, the device shall only need to find its order in the group of A-IOT devices with same modulo operation result and accordingly adjust its modulo operation result somehow to derive its true access occasion.

An example is provided as follows. If the divisor is set to 10, A-IOT devices ID modulo results will be distributed in the range of [0,9]. Assuming that devices with ID 1, 11, 21 are the target devices, their ID modulo results are the same. To avoid collision, each of them needs to adjust the result with a different adjusting quantity, respectively. Such quantity could be derived from the knowledge of accumulated adjusting quantity and the order of each particular device in the collided ID list. Hence, the ID 1, 11, 21 will be distributed to the first, second, and third access occasion, respectively. Subsequently, the reader could update the accumulated adjusting quantity to 2, considering the ID 11 and 22 occupy the 2rd and 3rd accession occasion. Accordingly, the devices with ID 2 and 12, will be distributed to the 4th to 5th access occasion, and so on. The overall procedure is illustrated in the following figure.

Figure 1: using PF/PO similar approach to compute the access occasion indexes for the target A-IOT devices

Observation 4: indicating the association of consecutive access/transmission occasions and different A-IOT device IDs in the A-IOT paging message explicitly for contention-free based random access procedure could suffer from overwhelmingly large signalling overhead if the A-IOT paging message is targeted for a large group of A-IOT devices, eg., more than 100 A-IOT devices.

Proposal 4: to trigger A-IOT devices to find their correct access occasions in A-IOT contention-free RACH procedure, three options of different information can be discussed to be embedded in the A-IOT paging message:

- One-to-one mapping relationship between the A-IOT device ID and the access occasion index.

- Mask ID defining the group of targeted A-IOT devices and the ID for the initial access occasion and indication of the ascending or descending order of the IDs for the subsequent access occasions

- modulo operation related parameters such as the divisor and lists of targeted A-IOT devices.

Moreover, In RAN1, D2R scheduling information is being discussed. Generally, it consists of time domain resources and frequency domain resources, i.e., both TDM and FDM are in consideration, as indicated as follows:

Agreement

For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH:

-	Time domain resources

-	Frequency domain resources

-	MCS-like information

-	Chip duration

-	ID associated with device(s)

-	Repetitions

FFS: other information

FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

As a result, we think that it is necessary to indicate the mapping relationship between the access occasion index and the time-frequency resource in practice. An example is provided as follows:

Figure 2: an example of mapping relationship between access occasion and time-frequency resource

Proposal 5: Include the mapping relationship between the time-frequency resources and access occasions in the A-IOT initial paging message, of which details depends on RAN1 further progress.

In addition, as mentioned earlier, for the RFID, an asynchronous system, the indication of the current access occasion slot is managed by the network, similar mechanism should also be studied by RAN2. We think this function should be enabled by paging message.

Proposal 6: A-IOT Paging message should be able to indicate the current access occasion related information.

- 2.3 Need of 2-step/3-step/CFRA Access type indication

Regarding the differentiation between the CFRA and the CBRA, it can be achieved by observing if one-to-one mapping relationship between A-IOT devices and the number of access occasion is included in the A-IOT Initial paging message. Naturally, if the mapping relationship is one-to-one, then CFRA shall be performed. Otherwise, CBRA procedure shall be applied. So implicit way is enough.

In addition, regarding differentiation of 2-step CBRA vs. 3-step CBRA, noted that for the 2-step CBRA, the device needs to include the permanent ID in the msg1, different from the 3-step CBRA wherein only a 16-bit random ID is needed. As a result, the TBS provided in the msg1 scheduling is different. So the explicit indication of the 2-step/3-step CBRA type is not needed.

Observation 4: differentiation between the CFRA and CBRA could be achieved by observing if one-to-one mapping relationship between A-IT device and access occasion is included in the A-IOT Initial paging message.

Observation 5: differentiation between the 2-step CBRA and 3-step CBRA could be achieved by observing the TBS of the msg1 scheduling resource.

Proposal 7: there is no need to include access type indication in the A-IOT initial paging message to differentiate 2-step/3-step/CFRA access type.

- Conclusion and proposals

In the contribution, following proposals and observations are made:

Avoiding duplicate response from devices:

Observation 1: repeated A-IOT paging messages corresponding to the same CN request are possible in two scenarios. Firstly, the CN request is sent towards more than one reader for enhancing the coverage of CN request. Secondly, A-IOT paging message is sent consecutively to avoid the case that A-IOT devices miss a particular paging message due to low battery level.

Proposal 1: include an ID identifying the CN request in the A-IOT paging message to avoid A-IOT device making duplicated responding to A-IOT paging messages corresponding to the same CN request.

Determination of transmission/access occasions:

Observation 2: when receiving of a A-IOT paging message containing only one ID of a single A-IOT device, the A-IOT device could access to the network immediately, after reception of the A-IOT paging message, subject to the restriction of any potential message processing delay.

Proposal 2: to trigger the one-device CFRA procedure, A-IOT paging message should indicate the D2R message scheduling information for msg1 transmission.

Observation 3: in the RFID system, the reader sends a number Q to the target A-IOT devices in the inventory message. Subsequently, each targeted A-IOT device will generate a random integer number between 0 and 2Q-1. Only when the integer number equals to 0, it will perform access towards the network.

Proposal 3: To trigger a contention-based random access procedure, A-IOT initial paging message needs to include a parameter to randomize the access occasions for different A-IOT devices and also indicate the total number of A-IOT access occasions.

Observation 4: indicating the association of consecutive access/transmission occasions and different A-IOT device IDs in the A-IOT paging message explicitly for contention-free based random access procedure could suffer from overwhelmingly large signalling overhead if the A-IOT paging message is targeted for a large group of A-IOT devices, eg., more than 100 A-IOT devices.

Proposal 4: to trigger A-IOT devices to find their correct access occasions in A-IOT contention-free RACH procedure, three options of different information can be discussed to be embedded in the A-IOT paging message:

- One-to-one mapping relationship between the A-IOT device ID and the access occasion index.

- Mask ID defining the group of targeted A-IOT devices and the ID for the initial access occasion and indication of the ascending or descending order of the IDs for the subsequent access occasions

- modulo operation related parameters such as the divisor and lists of targeted A-IOT devices.

Proposal 5: Include the mapping relationship between the time-frequency resources and access occasions in the A-IOT initial paging message, of which details depends on RAN1 further progress.

Proposal 6: A-IOT Paging message should be able to indicate the current access occasion related information.

Need of 2-step/3-step/CFRA Access type indication:

Observation 4: differentiation between the CFRA and CBRA could be achieved by observing if one-to-one mapping relationship between A-IT device and access occasion is included in the A-IOT Initial paging message.

Observation 5: differentiation between the 2-step CBRA and 3-step CBRA could be achieved by observing the TBS of the msg1 scheduling resource.

Proposal 7: there is no need to include access type indication in the A-IOT initial paging message to differentiate 2-step/3-step/CFRA access type.