3GPP TSG-RAN WG2 Meeting #127bis	R2-2408148

Hefei, China, October 14th –18th, 2024

Agenda item:	8.2.5

Source: 	Fujitsu

Title: 	Discussions on topology 2 related aspects

Document for:	Discussion and decision

1. Introduction

According to the New SID [1], the overall objective shall be to study a harmonized air interface design with minimized differences for Ambient IoT for Topologies 1 and 2.

This contribution will provide more consideration on Ambient IoT Topology 2.

2. Discussion

Resource Allocation

According to TR38.848, Topology 2 is shown as below:

Figure 4.2.1.2-1: Topology 2 [2]

For the UE as intermediate node, communication with the network, e.g. gNB or CN, needs further study in RAN2.

Communication with the network takes the following aspects into account:

- Notification of UE as intermediate node

The network needs to provide the configuration for reader and thus it is necessary for the network to identify if a UE connects to the network as an intermediate node. This can be notified by the UE as a capability or a status.

- Allocation/management of resource over A-IoT air interface

The reader communicates with served A-IoT devices over the A-IoT air interface. The used resources should be allocated by the gNB and thus the configuration from the network needs to be considered.

- Inventory/command procedure

Inventory/command procedure should be supported for both topologies. When the UE connects to the network as an intermediate node, related signaling or data transmission, e.g. inventory/command request and corresponding response between the UE and the network should be supported.

Based on these, the communication between the UE as intermediate node and the network including notification of UE as intermediate node, resource allocation and inventory/command procedures will be considered.

Observation 1: Communication with the network to support UE as a reader/intermediate node should consider the following aspects:

- UE as intermediate node notification

- Allocation/management of resources over A-IoT air interface

- Inventory/command forwarding

Regarding the first aspect, RAN2 assumes that the intermediate UE authorization is performed by upper layers and RAN2 impact can be studied after further SA3/RAN3 progress [3].

Based on SA2 solutions [4] and RAN2 agreements [3], the following protocol stack options for UE as an intermediate node can be studied:

- NAS based solution: A-IoT upper layer information is transmitted over NAS message of intermediate UE and forwarded via gNB transparently. If this architecture is selected for the Topology 2 scenario, SA2 and RAN3 will work on the NAS enhancements and RAN2 still needs to consider possible UE configuration by gNB.

- UP based solution: A-IoT upper layer information can be transmitted as data of the intermediate UE

- RRC based solution: A-IoT upper layer information can be contained in RRC messages between the gNB and intermediate UE

Therefore, inventory/command forwarding, i.e. A-IoT upper layer information is transmitted over NAS, user plane or RRC of the intermediate UE based on the protocol stack options for UE as an intermediate node.

Observation 2: To support Inventory/command forwarding, A-IoT upper layer information is transmitted over NAS, user plane or RRC of the intermediate UE.

RAN2 has agreed that the radio resources used by A-IoT air interface between the A-IoT device and the UE reader are controlled by the network. One possibility is the radio resources can be allocated by the gNB when the UE is selected/identified as the reader, i.e. the resources are allocated per UE reader. Alternatively, the radio resources used by A-IoT air interface can be allocated by the gNB when the gNB receives A-IoT upper layer information, e.g. inventory request or command. It means the resources are allocated per A-IoT service request.

Proposal 1: RAN2 to discuss the following options for allocation/management of radio resources used by the A-IoT air interface:

- Option 1: allocation/management of radio resources per UE reader

- Option 2: allocation/management of radio resources per A-IoT service request

In Option 1, when A-IoT upper layer information is received, e.g. over NAS or RRC of the intermediate UE, the UE reader performs inventory/command procedure over A-IoT air interface and the radio resource(s) used for the inventory/command procedure are determined by the UE reader from the radio resources allocated for the UE reader. For Option 2, the radio resources used by A-IoT air interface for D2R and R2D transmissions can be allocated by the gNB when the gNB receives A-IoT upper layer information from, e.g. the CN. Alternatively, the radio resources for R2D transmission(s) can be allocated by the gNB when the gNB receives A-IoT upper layer information from, e.g. the CN while the radio resources for D2R transmission(s) can be allocated by the gNB when the gNB receives pre-emptive BSR-like information from the intermediate UE.

“Temporarily” out of connection

RAN2 has agreed that in-coverage cases will be supported and to study cases for reader “temporarily” out of connection, e.g. RLF, HO [3].

First, RAN2 needs to identify the cases for reader “temporarily” out of connection. Legacy UE supports different RRC states, including RRC idle, inactive and connected state. Therefore, the UE as intermediate node may enter RRC idle state and data transmission is not performed. RAN2 considers the UE reader “temporarily” out of connection when the intermediate UE enters RRC idle state. Also, the UE may initiate RRC reestablishment when a failure occurs. During the recovery, the UE cannot communicate with the network and thus a failure or recovery can be considered as “temporarily” out of connection. Besides, if the UE performs the cell switch, some interruption will be incurred. RAN2 needs to study this.

Proposal 2: RAN2 to discuss the following “temporarily” out of connection cases for the intermediate UE:

- when RRC state changes

- when a failure or recovery occurs

- when a cell switch is performed

To handle cases for reader “temporarily” out of connection, two directions can be considered. One is to stop or suspend the inventory/command procedure over A-IoT air interface, i.e. between the UE reader and the A-IoT devices. Another is that the inventory/command procedure over A-IoT air interface can be continued. The radio resource allocated before the reader “temporarily” out of connection can be used or separate resources can be allocated for the reader “temporarily” out of connection.

Proposal 2bis: RAN2 to discuss the following options to handle “temporarily” out of connection:

- Option 1: stop/suspend communication between the UE reader and A-IoT devices

- Option 2: continue communication between the UE reader and A-IoT devices using radio resources which are same as or different from that used before the “temporarily” out of connection

3. Conclusion

In this contribution, we had some discussions on the Ambient IoT Topology 2 and made the following observations and proposals:

Observation 1: Communication with the network to support UE as a reader/intermediate node should consider the following aspects:

- UE as intermediate node notification

- Allocation/management of resources over A-IoT air interface

- Inventory/command forwarding

Observation 2: To support Inventory/command forwarding, A-IoT upper layer information is transmitted over NAS, user plane or RRC of the intermediate UE.

Proposal 1: RAN2 to discuss the following options for allocation/management of radio resources used by the A-IoT air interface:

- Option 1: allocation/management of radio resources per UE reader

- Option 2: allocation/management of radio resources per A-IoT service request

Proposal 2: RAN2 to discuss the following “temporarily” out of connection cases for the intermediate UE:

- when RRC state changes

- when a failure or recovery occurs

- when a cell switch is performed

Proposal 2bis: RAN2 to discuss the following options to handle “temporarily” out of connection:

- Option 1: stop/suspend communication between the UE reader and A-IoT devices

- Option 2: continue communication between the UE reader and A-IoT devices using radio resources which are same as or different from that used before the “temporarily” out of connection

Reference

- RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#103, March 18-21, 2024

- 3GPP TR 38.848: "Technical Specification Group Radio Access Network; Study on Ambient IoT (Internet of Things) in RAN"

- RAN2#127 chair notes

- 3GPP TR 23.700-13: “Study on Architecture support of Ambient power-enabled Internet of Things”, v0.4.0