3GPP TSG-RAN WG2 Meeting #127bis	R2-2408149

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.5.4

Source: 	Fujitsu

Title: 	Adaptation of common signal or channel

Document for:	Discussion and decision

Introduction

Rel-19 NES WI has been approved in RANP#102 [1]. The WID describes the following objective on the adaptation of common signal/channel transmissions.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In RAN2#125-bis meeting, there are following agreements on paging adaptation [3]:

Agreements on paging adaptation:

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

In this contribution, we would like to discuss and provide our views on the adaptation of common signal/channel transmissions.

Discussion

Adaptation of SSB in time domain

In RAN1#116-bis meeting, there are the following agreements on the adaptation of SSB in time domain [2].

Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode Note: Impact to idle/inactive UEs shall be minimized

For the adaptation of SSB, it is necessary to determine applicable scenarios before discussing how to adapt SSB in time domain. As shown in the agreement, it has been agreed that adaptation of SSB can be applied to PCell and SCell for Rel-19 NES capable UEs in RRC_CONNECTED mode.

As specified in TS38.213, for initial cell selection, a UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 2 frames. Thus, adaptation of SSB in time domain, e.g., adaptation of SSB burst periodicity or skipping some SSB transmission, would cause initial access latency or access failure for UEs in RRC_IDLE/RRC_INACTIVE mode.

To avoid negative impact on both legacy and Rel-19 NES-capable UEs in RRC_IDLE/RRC_INACTIVE mode, legacy UEs can be prevented from accessing the network using cell barring, and Rel-19 NES-capable UEs can be provided SSB assumption with an enlarged periodicity to satisfy the requirement of adaptation of SSB periodicity. From RAN2 perspective, it is possible to support SSB adaptation for the UEs in RRC_IDLE/RRC_INACTIVE mode.

Proposal 1: RAN2 confirms it is possible to support adaptation of SSB in both RRC_CONNECTED mode and RRC_IDLE/RRC_INACTIVE mode:

    - Rel-19 NES UEs can be provided assumed SSB with a periodicity larger than 20ms.

    - Legacy UEs can be barred from accessing the NES cell.

For adaptation of CD-SSB, or NCD-SSB, RAN2 can wait for RAN1’s discussion result in the next meeting.

Adaptation of PRACH in time domain

In RAN1#116-bis meeting, there are the following agreements on the adaptation of PRACH in time domain [2].

Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211 Agreement For adaptation of PRACH in time-domain, support the following: - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any) - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources - FFS: SSB-RO mapping for the additional PRACH resources Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode Agreement For adaptation of PRACH in spatial domain, - Study possibility of scenarios with non-uniform distribution of UEs in different beams - Note 6: Companies are encouraged to provide details on how they map UEs to different beams - Study network energy savings gain achieved by non-uniform PRACH resource allocation across SSBs for scenarios with non-uniform distribution of UEs in different beams (if any), - Assume the following framework for network energy evaluation in FR1 and companies to report at least the below settings used in the evaluation/simulation - 20ms SSB period - 30kHz SCS, DDDSU TDD pattern - Setting A: SIB1 period (20ms/40ms/160ms) - Setting B1: Cell load (Empty/low/medium) - Setting B2: Traffic model - Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2 - Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11 - Setting E1: PRACH configurations - (legacy) PRACH resources according to the following PRACH configuration for all transmitted SSBs - Case A1-1: PRACH configuration #5 (20ms) - Case A1-2: PRACH configuration #17 (10ms) - Case A2-1: PRACH configuration #0 (160ms) - (time-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration for all transmitted SSBs - Case B1: PRACH configuration #17 (10ms) - Case B2: PRACH configuration #0 (160ms) - Companies to report details of assumed time domain adaptation mechanism - (spatial-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration - Case C1: PRACH configuration #17 (10ms) - Case C2: PRACH configuration #0 (160ms) - Companies to report details of assumed spatial domain adaptation mechanism, including details of non-uniform PRACH resource allocation across SSBs - Setting F: Cat 1/Cat 2 BS as defined in TR38.864 - Setting G1: Number of SSB beams: 4,8 SSBs in a SSB burst with SSB pattern case C - Note 1: Baseline to compare is Case C1 vs Case B1/A1-1/A1-2, Case C2 vs Case B2/A2-1 - Note 2: It is up to company to report the SSB-RO mapping ratio and FDMed RO number, etc - Note 3: Other PRACH configuration index with different PRACH format other than format 0 is not precluded - Note 4: Other SSB/SIB1/RACH periodicity/PRACH resource/configuration assumptions are not precluded (up to companies to report) Other frameworks for network energy evaluation are not precluded, e.g. including for FR2

As shown above, the adaptation mechanism of additional PRACH resources for NES-capable UEs are agreed. NES-capable UEs can use both additional PRACH resources and legacy PRACH resources for legacy UEs. In high load or medium load case, the legacy and additional RPACH resources are both active, and in case of low load and low number of UE accesses, additional PRACH resources are deactivated to achieve energy saving gain.

For random access in additional PRACH resource, before discussing additional PRACH resource configuration/indication, the random access type should be discussed first.

For 4-step and 2-step random access, there are two possible options. Option 1 is to support both 4-step and 2-step random access for additional RPACH resources, and option 2 is to support 4-step random access first and deprioritize 2-step random access. 2-step random access is an optional function, and it can be applied only in good RSRP, and it may cause larger signaling overhead, such as PUSCH configuration. Therefore, Option1 has full flexibility and no restriction for random access while Option 2 has a small signaling overhead and specification impacts.

Proposal 2: RAN2 to discuss whether the following types of random access should be supported for additional PRACH resource for NES UEs.

    - Option1: Support both 4-step RA and 2-step RA.

    - Option2: Support only 4-step RA.

For CBRA and CFRA, there are also two possible options. Option 1 is to support both CBRA and CFRA, and Option 2 is to support only CBRA. CBRA can be applied in both in RRC_IDLE/RRC_INACTIVE mode and RRC_CONNECTED mode, however CFRA can only be applied in RRC_CONNECTED mode. PRACH adaptation for CBRA may achieve more energy saving gain than PRACH adaptation for CFRA. Furthermore, multiple PRACH configuration signalling is used for CFRA which is more complicated than CBRA. Thus, supporting only CBRA can be a straightforward approach with larger energy saving gain and minimum specification effort. However, NR supports both CBRA and CFRA procedure, and the normative work to support random access in additional RPACH resources should cover CBRA and CFRA. Therefore, the two options are provided for further discussion where option 1 has more flexibility and no restriction while Option 2 has less spec impact.

Proposal 3: RAN2 to discuss whether the following contention types should be supported for additional PRACH resource for NES UEs.

    - Option1: Support both CBRA and CFRA.

    - Option2: Support only CBRA.

Adaptation of paging occasions

Options for paging occasion adaptation

According to the legacy specification, the PFs/POs of different UEs are uniformly distributed in the time domain. An example is shown in the following figure, assuming that T = 16 radio frames, N=T/4, Ns=2, and PF_offset=0. By the current PF/PO determination scheme, the gNB needs to wake up frequently to transmit paging messages which consumes a lot of network energy.

Figure 1 Example of legacy PF/PO

In the RAN2#125bis meeting, RAN2 agreed that RAN2 will study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. In the post-127 email discussion, option a) and b) are further discussed for better understanding, and most companies prefer option b).

In our understanding, both a) and b) can be used for the adaptation of paging occasions. From our perspective, b) is slightly preferred, since b) has smaller specification impact and is simpler without the need of changing the formula for PF determination. Therefore, we prefer b).

Proposal 4: For the adaptation of paging occasions in time domain, b) is adopted, i.e., extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

In the legacy PO formula, the index i_s of the legacy PO is determined by: i_s = floor (UE_ID/N) mod Ns. For the PO determination in option b), companies have discussed the alternative that the NES POs are provided by the extended Ns values in PCCH-Config, as shown in Figure 2, where N is reduced from T/4 to T/8, and the value of Ns is increased from 2 to 4.

Figure 2 Example of confining POs in the time domain

This option is simple but may have higher RRC signalling overhead, as raised by companies in the Post-127 email discussions, since the number of firstPDCCH-MonitoringOccasionOfPO parameters configured for each PO may also be increased excessively with the increased value of Ns.

To save the RRC signalling overhead due to extending Ns values, another alternative for option b) could be that the NES UEs are configured to use an offset(s) to the POs calculated by the legacy PO formula to determine their NES POs, without the extended Ns values in case that N is reduced. For example, the NES PO can be k slots or symbols offset to the slot of the PO calculated by the legacy formula. To save the signalling overhead, the value of the offset can be determined by an NES UE based on the NES UE’s id and a configured number of paging groups, and the paging group of an NES UE can be determined by the NES UE’s id. In this alternative, the values of Ns can still be configured as legacy. For example, when N is reduced from T/4 to T/8 and the number of POs should be increased from 2 to 4 to keep the paging capacity, the Ns value can still be configured as 2. That is to say, the 2 POs determined by the legacy formula can be used by the legacy UEs (and also the NES UEs), and the additional 2 POs can be derived by NES UEs by the offsets to the first PO which is determined by legacy formula. The number of the paging group can be configured as 4, and then the NES UEs are distributed in the 4 POs in one PF. In this alternative, since only the number of the paging groups is configured additionally and the firstPDCCH-MonitoringOccasionOfPO parameter does not need to be configured for every PO, the RRC signalling overhead is lower compared to the first alternative to extend the Ns values. Moreover, legacy UEs can use the legacy paging configuration and the legacy PO formula to determine their POs without being impacted. Therefore, we propose RAN2 to also discuss this alternative.

Proposal 5: RAN2 to discuss the following alternatives for b):

- NES PO is configured to use offset(s) to the legacy PO.

- Extend values of IE ns in PCCH-Config.

Legacy UE impact

In RAN2#125bis meeting, RAN2 agreed to discuss the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

In Option 1, the NES cell with paging adaptation may be deployed as hotspots and normal cells should be deployed providing the overlapped coverage with the NES cell. Then the legacy UEs can find a normal cell to camp on. In addition, the NES UEs may need to ignore the legacy barring e.g., in MIB and consider other indication(s) in the NES cell to determine whether the NES cell is suitable.

In Option 2, the legacy UEs are also allowed to camp on the cell with paging adaptation, so the network capacity or coverage would be better compared with Option 1. For the legacy/non-NES UEs, the legacy PCCH-config is used for the determination of PF/PO. For the NES UEs, a new paging configuration may be needed to confine the paging occasions in the time domain.

From network deployment and capacity perspective, we prefer Option 2.

Proposal 6: RAN2 adopts separate paging resources for legacy UEs and Rel-19 NES UEs.

Conclusion

In this contribution, we have some discussions about the adaptation of common signals or channels, and the following proposals are made:

Proposal 1: RAN2 confirms it is possible to support adaptation of SSB in both RRC_CONNECTED mode and RRC_IDLE/RRC_INACTIVE mode:

    - Rel-19 NES UEs can be provided assumed SSB with a periodicity larger than 20ms.

    - Legacy UEs can be prevented from accessing the NES cell.

Proposal 2: RAN2 to discuss whether the following type of random access should be supported for additional PRACH resource for NES UEs.

    - Option1: Support both 4-step RA and 2-step RA.

    - Option2: Support only 4-step RA.

Proposal 3: RAN2 to discuss whether the following contention type should be supported for additional PRACH resource for NES UEs.

    - Option1: Support both CBRA and CFRA.

    - Option2: Support only CBRA.

Proposal 4: For the adaptation of paging occasions in time domain, b) is adopted, i.e., extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

Proposal 5: RAN2 to discuss the following alternativesfor b):

- NES PO is configured to use offset(s) to the legacy PO.

- Extend values of IE ns in PCCH-Config.

Proposal 6: RAN2 adopts separate paging resources for legacy UEs and Rel-19 NES UEs.

References

- RP-234065, New WID: Enhancements of network energy savings for NR, Ericsson (Moderator), RAN#102 meeting.

- RAN1#116bis chair’s note

- RAN2#125bis chair's note