3GPP TSG-RAN WG2 Meeting #127bis	R2-2408150

Hefei, China, October 14th –18th, 2024

Agenda item:	8.6.3

Source: 	Fujitsu

Title: 	Discussions on event triggered L1 measurement reporting

Document for:	Discussion and decision

1. Introduction

The following objective of the work item aims at specifying measurement related enhancements for New WID [1] on NR mobility enhancements:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

This contribution will provide more considerations on the measurement reporting procedure, including:

- Configuration signalling design

- Event evaluation and triggering

- MAC CE for measurement report

2. Discussion

2.1 Configuration signalling design

In Rel-18, L1 measurement reporting via UCI is supported. For this, the network configures both measurement resources and reporting resources via RRC signalling.

Figure 1: Rel-18 configuration of L1 measurement and reporting

As shown in Figure 1, the IE LTM-CSI-ReportConfig is used to configure report on the cell in which the LTM-CSI-ReportConfig is included while the IE LTM-CSI-ResourceConfig (included in LTM-Config) defines a group of one or more CSI resources for one or more LTM candidate configurations (SSB and corresponding LTM candidate ID).

For event/reporting configuration for event triggered L1 measurement reporting, RAN2 has agreed that event triggered reporting configuration is provided in serving cell config. Considering information for reporting configuration needs more discussion, RAN2 should further discuss the details of reporting configuration after RAN2 has conclusions on e.g. possible conditions to trigger L1 measurement reporting, and how to avoid frequent reporting or Ping-pong.

RAN2 has agreed that LTM measurement resource configuration is provided in LTM-config and both SSB and CSI-RS are supported. We think that Rel-18 signalling design can be reused as baseline and CSI-RS is supported as the CSI resources in addition to SSB. One possibility is to introduce Rel-19 LTM CSI resource set including two sets, one for CSI resources choosing from SSB index and CSI-RS resource ID and the other for LTM candidate ID, as shown in Figure 2.

Figure 2: Example of resource configuration for event-triggered L1 measurement reporting

Proposal 1: For LTM measurement resource configuration, a new Rel-19 LTM CSI resource set is provided in LTM-config, including:

- A set of CSI resources, choosing from SSB index and CSI-RS resource ID

- A set of LTM candidate IDs, each corresponding to the CSI resource

In Rel-18, SSB configuration and CSI-RS configuration can be provided for each LTM candidate and both configurations can be reused for Rel-19. In this way, duplicated configuration is avoided. It means SSB index refers to LTM-SSB Config and CSI-RS resource ID refers to NZP-CSI-RS-Resource for candidate cell corresponding to the LTM-CandidateId.

In addition, as shown in Figure 1, the ID for RS configuration, i.e. LTM-CSI-ResourceConfigId is included in each reporting configuration. For event triggered L1 measurement reporting, RAN2 has agreed that L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in the LTM configuration via RRC signalling. There can be different options for signalling design to implement this. For example, when RS(s) for L1 measurement are provided explicitly, a similar mechanism as in Rel-18 can be reused, i.e. RS resource ID (e.g. LTM-CSI-ResourceConfigId) is included in the event configuration. If RS for L1 measurement is not provided explicitly by RRC signalling, the UE performs event evaluation using RS associated with TCI state (s) and thus TCI state ID(s) is included in the event configuration for association.

Alternatively, L3 measurement-like configuration can be used for L1 measurement. In this mechanism, RS resource ID is used to identify the L1 measurement RS and a reporting configuration ID is introduced to define the event configuration or reporting configuration. An additional L1 measurement ID is used to associate the RS resource ID and the reporting configuration ID.

Proposal 1bis: RAN2 to discuss the following options to associate event/reporting configuration with resource configuration:

    - Option 1: RS resource ID is included in the event/reporting configuration

    - Option 2: L3 measurement style

2.2 Event evaluation and triggering

Beam(s) for event evaluation

According to RAN2 agreement reached at last meeting, RS associated with indicated beam for the serving cell is used for event evaluation. However, if the indicated beam is associated with PCI of non-serving cell, LTM cell switch based on event-triggered L1 measurement report cannot be supported. In case of inter-cell beam management, the UE is using the beam from the neighbouring cell associated to the serving cell and thus the RS from the neighbouring cell is used for event evaluation. Event e.g. LTM2/3/5 cannot be satisfied since the quality of beam of serving cell used for event evaluation, i.e. the RS from the neighbouring cell, is good.

To support LTM cell switch after ICBM, a set of RS(s) can be configured for the serving cell for L1 measurement. Alternatively, RS associated with activated TCI states of the serving cell can be used for event evaluation.

Proposal 2: For event evaluations, RAN2 to discuss if configured RS or RS associated with activated TCI states of the serving cell can be used.

In addition, RAN2 assumes that the same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. If beam of LTM candidate used for event evaluation is an SSB, while beam of the serving cell used for event evaluation is a CSI-RS, the SSB which is QCLed with the CSI-RS should be used instead of the CSI-RS.

Proposal 2bis: The SSB which is QCLed with the CSI-RS should be used so that the same RS type are used for serving and neighbouring cell when event LTM3 and event LTM5 are evaluated.

Support of other conditions to trigger measurement report

For L3 measurement reporting, entering condition and leaving condition are defined and measurement reporting on leaving condition can be configured by the gNB. With this, the network can make decision on LTM cell switch or early synchronization based on the newly received L1 measurement results. So, ping-pong or too early synchronization can be mitigated. We propose that a similar mechanism can be applied for event triggered L1 measurement reporting.

Proposal 3: L1 Measurement reporting on leaving condition can be supported.

For L3 measurement reporting, to address transmission failure of the L3 measurement report, a timer T312 can be configured. The timer T312 can be started if the L3 measurement report is triggered during T310 is running. Failure reporting or connection re-establishment procedure is initiated when the timer expires.

For L1 measurement reporting, RAN2 has agreed that the event triggered L1 measurement reporting should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization

- Select the target beam/cell and trigger LTM cell switch procedure

A similar mechanism can be used for the L1 measurement reporting triggered by the events defined for LTM decision. With this, the UE can take some actions when the transmission of the event triggered L1 measurement report fails.

Proposal 4: If the event is configured for LTM decision, RAN2 considers T312-like mechanism.

These LTMx events can also be the configured to select the candidate beam/cell to trigger early synchronization. Then, if the quality of candidate cell changes, the activated TCI states could be deactivated. Depending on the quality of the candidate cell, the network may or may not transmit the PDCCH order to trigger early TA acquisition again, e.g., when the TA becomes invalid. So, multiple or periodic reporting after the event-triggered L1 measurement reporting needs to be considered.

Proposal 5: If the event is configured for early synchronization decision, multiple or periodic reporting after the event-triggered reporting is considered.

The trigger of L1 measurement reporting

RAN2 has already agreed that TTT is applied at least for the entering condition of LTM events. Then the issue is how to implement the TTT mechanism for LTM events. As Rel-19 MIMO, the UE can trigger the L1 measurement reporting by counting the number of indication instances from lower layers and the indication is received by the MAC entity when the entering condition of LTM event is satisfied. Alternatively, L3 measurement like TTT mechanism is applied. For example, a timer is started when the entering condition of LTM event is satisfied, e.g., the indication is received from the lower layer; the L1 measurement reporting is triggered when the timer expires.

Proposal 6: RAN2 to discuss the following options to implement TTT-like mechanism for the entering condition of LTM Event X:

    - Option 1: counting the number of indication instances, which can be received when the entering condition is satisfied

    - Option 2: introducing a timer to control the triggering of L1 measurement reporting

Then the next issue is the applied level of TTT. The TTT can be applied at beam level or CSI resource set level. If it is applied for CSI resource set, the L1 measurement reporting can be triggered even if the beam meeting the entering condition of LTM event changes during the TTT. In this case, if the RS set is configured for L1 measurement, the UE performs event evaluation per RS set. Otherwise, the UE performs event evaluation per candidate cell.

Proposal 7: TTT-like mechanism is applied to each RS set or each candidate cell/serving cell.

Proposal 7bis: For each RS set or each candidate cell, the MAC entity performs event evaluation to determine if the LTM event is satisfied or not.

2.3 MAC CE for measurement report (MR MAC CE)

Information of MR MAC CE

Regarding the content of the MAC CE, from RAN2 point of view, some information related to the event which triggers the L1 measurement reporting needs to be included in the measurement report. With the information, the network can associate the measurement results with event configuration, e.g. event type, related threshold(s) and offset and to make decision including LTM cell switch. RS information and L1 measurement result of the beam which meets the entering condition of LTM event should be included in the MR MAC CE, which may help the gNB to make decision.

Proposal 8: From RAN2 perspective, event information, RS information and corresponding L1 measurement result of the beam(s) which trigger the LTM event are included in the MR MAC CE.

UE procedure to transmit MR MAC CE

UE procedure to transmit MR MAC CE is shown in the figure below. When the L1 measurement reporting is triggered, if there is UL grant, the MR MAC CE is generated; otherwise, the SR is triggered.

Figure 3: UE procedure to transmit MR MAC CE

Proposal 9: Similar to legacy MAC CE, when the L1 measurement reporting is triggered, if there is UL grant, the MR MAC CE is generated; otherwise, the SR is triggered.

3. Conclusion

In this contribution, we have some discussions on potential measurement enhancements for LTM and made the following proposals:

Configuration signalling design

Proposal 1: For LTM measurement resource configuration, a new Rel-19 LTM CSI resource set is provided in LTM-config, including:

- A set of CSI resources, choosing from SSB index and CSI-RS resource ID

- A set of LTM candidate IDs, each corresponding to the CSI resource

Proposal 1bis: RAN2 to discuss the following options to associate event/reporting configuration with resource configuration:

    - Option 1: RS resource ID is included in the event/reporting configuration

    - Option 2: L3 measurement style

Beam(s) for event evaluation

Proposal 2: For event evaluations, RAN2 to discuss if configured RS or RS associated with activated TCI states of the serving cell can be used.

Proposal 2bis: The SSB which is QCLed with the CSI-RS should be used so that the same RS type are used for serving and neighbouring cell when event LTM3 and event LTM5 are evaluated.

Support of other conditions to trigger measurement report

Proposal 3: L1 Measurement reporting on leaving condition can be supported.

Proposal 4: If the event is configured for LTM decision, RAN2 considers T312-like mechanism.

Proposal 5: If the event is configured for early synchronization decision, multiple or periodic reporting after the event-triggered reporting is considered.

The trigger of L1 measurement reporting

Proposal 6: RAN2 to discuss the following options to implement TTT-like mechanism for the entering condition of LTM Event X:

    - Option 1: counting the number of indication instances, which can be received when the entering condition is satisfied

    - Option 2: introducing a timer to control the triggering of L1 measurement reporting

Proposal 7: TTT-like mechanism is applied to each RS set or each candidate cell/serving cell.

Proposal 7bis: For each RS set or each candidate cell, the MAC entity performs event evaluation to determine if the LTM event is satisfied or not.

Information of MR MAC CE

Proposal 8: From RAN2 perspective, event information, RS information and corresponding L1 measurement result of the beam(s) which trigger the LTM event are included in the MR MAC CE.

UE procedure to transmit MR MAC CE

Proposal 9: Similar to legacy MAC CE, when the L1 measurement reporting is triggered, if there is UL grant, the MR MAC CE is generated; otherwise, the SR is triggered.

Reference

- RP-242356, “Revised Work Item: NR mobility enhancements Phase 4”, RAN#105, Sep. 9-12, 2024