3GPP TSG-RAN WG2 Meeting #127bis	R2-2408154

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.7.5

Source: 	Fujitsu

Title: 	Discussions on RLC enhancements

Document for:	Discussion and decision

Introduction

On RLC retransmission enhancement in XR, for ensuring timely RLC retransmissions, there are following agreements in RAN2#126 meeting [1]:

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

For avoiding unnecessary retransmissions, RAN2 has made the following agreements in RAN2#127 meeting [2]:

Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first. In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer. In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

In this contribution, we would like to discuss the potential enhancements to achieve timely RLC retransmissions and to avoid unnecessary RLC retransmissions for XR.

Discussion

Ensuring timely RLC retransmission(s) for XR

As agreed by RAN2, the following options will be considered:

- Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered)

- Retransmission based on enhanced status report

- Retransmission based on enhanced polling

Among these options, autonomous retransmission can achieve high reliability for XR traffics in the small PDB. In the last meeting, some companies think that the overhead of autonomous retransmission is large, and they have concern on the system capacity. From our perspective, autonomous retransmission can be applied in certain cases to save the radio resource occupation. For example, the trigger of autonomous retransmission can be e.g., when the remaining time of the data becomes lower than a threshold, or when the network configures the UE to apply autonomous retransmission. When PDB is reached, the autonomous retransmission in RLC should be stopped. Therefore, the impact of autonomous retransmission to the system capacity can be acceptable.

On the other hand, the options of retransmission based on enhanced status report or enhanced polling also have overhead, since the UE would transmit polling and/or the network would transmit the status reports more frequently with configured small value of pollPDU or pollByte or t_reassembly timer. The transmission of polling and status reports would delay the transmission of data, which also has impact on the system capacity.

Observation 1: Autonomous retransmission has acceptable impact on system capacity with proper triggers.

In the options of retransmission based on enhanced status report or enhanced polling, the round-trip-time of an RLC retransmission may still be long since there may be HARQ retransmissions of the status report. From this point of view, retransmission based on enhanced status report or enhanced polling may not be so fast to achieve timely RLC retransmission within PDB.

Observation 2: Retransmission based on enhanced status report or enhanced polling may not achieve timely RLC retransmission within PDB.

Based on the above observations, autonomous retransmission may achieve better performance than retransmission based on enhanced status report or enhanced polling. So, we propose to adopt autonomous retransmission to achieve timely RLC retransmission, i.e., the transmitting side of an AM RLC entity can perform retransmission even if it does not receive the negative acknowledgement. By this way, the number of RLC retransmission(s) is increased in the PDB compared with the current scheme, and thus the reliability of the XR traffic can be ensured.

Proposal 1: RAN2 adopts autonomous retransmissions to achieve timely RLC retransmission in RLC AM.

Proposal 1a: Autonomous retransmission can be activated/deactivated by the network or can be triggered when the remaining time of the data becomes lower than a threshold.

The autonomous RLC retransmission(s) can occur in the consecutive slots or can have an interval. If positive acknowledgement is received by the transmitting side of the AM RLC entity, these autonomous RLC retransmission(s) can be stopped. If negative acknowledgement is received by the transmitting side of the AM RLC entity, the transmitting side of the AM RLC entity can continue the autonomous RLC retransmissions.

The number of the autonomous RLC retransmissions can be configured by the network or can be pre-defined in RLC. In our opinion, these autonomous RLC retransmissions should not be counted into RETX_COUNT, so that RETX_COUNT would not easily reach the maximum retransmission number (maxRetxThreshold) and RLF would not be initiated.

Proposal 2: The number of the autonomous RLC retransmissions can be configured by the network or can be pre-defined in RLC.

Proposal 3: The autonomous RLC retransmissions are not counted into RETX_COUNT.

Avoiding unnecessary retransmissions

In RAN2#126 meeting and RAN2#127 meeting, RAN2 have extensive discussions on the following approaches for avoiding unnecessary RLC AM retransmissions:

Approach 1. Rx initiated approach

Approach 2. Tx initiated approach

Approach 3. combined Rx and Tx approach

For Approach 1 (Rx initiated approach), RAN2 agrees that the Rx side abandons missing SDUs like already done by PDCP, i.e. based on a timer. For the uplink XR service, the Rx is at the network side and this timer need not to be specified in the standard. On the other hand, for the downlink XR service, the Rx is at the UE side, so the definition of the timer and the UE behaviours related to the timer should be specified. In our understanding, this timer should be configured by the network based on the PDB of the XR service, like the PDCP discard timer. How to configure the value of the timer may depend on the network implementation. For an RLC SDU the associated timer can be started when another RLC SDU with a higher SN is received in the Rx side. If the RLC SDU is received, the timer associated with it can be stopped if the timer is running. When the timer expires, Rx side can consider the associated RLC SDU as abandoned.

Observation 3: In the Rx initiated approach, the timer in the Rx side can be configured by the network.

For Approach 2 (Tx initiated approach), as discussed and proposed in the previous meetings, a new RLC SN gap report should be transmitted from the Tx side to the Rx side. In the RLC SN gap report, the obsolete RLC SDUs will be indicated, so that Rx side can advance its receiving window. In the last meeting, it is discussed whether the Tx side follows the legacy behaviour to advance the Tx window based on the RLC status PDU from the Rx side or the Tx side can advance its window first e.g. based on PDCP discard indication or a timer. In our understanding, the RLC Tx side can advance the Tx window upon transmitting the RLC SDU gap report to the Rx side, so that window sync between the Tx side and the Rx side can be achieved.

Observation 4: In the Tx initiated approach, the Tx side can advance the Tx window first e.g. upon transmitting the RLC SDU gap report to the Rx side.

For Approach 3 (combined Rx and Tx approach), the Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach, and the Rx side stops to receive an obsolete SDU based on a local timer as for Rx initiated approach. In this approach, the signalling overhead is saved without introducing RLC SDU gap report or new trigger of RLC status report. However, since there is not signalling exchange between the Tx side and the Rx side, how to ensure the window sync between the two sides should be further discussed or can be up to the good network implementation.

Observation 5: In the combined Rx and Tx approach, signalling overhead is saved, but it is unclear how to ensure window sync between the Tx side and the Rx side.

From our perspective, Rx initiated approach seems simpler and can ensure window sync between the Tx side and the Rx side. In addition, the signalling overhead due to the new trigger of RLC status report is acceptable, since the unnecessary RLC retransmissions would not happen frequently by proper network configuration of the maximum number of RLC retransmissions. Therefore, we prefer Rx initiated approach.

Proposal 5: Rx initiated approach is adopted for avoiding unnecessary retransmissions.

Conclusion

In this contribution, we have discussed the RLC enhancements to achieve timely RLC retransmission and to avoid unnecessary retransmissions. We have the following observations and proposals:

Observation 1: Autonomous retransmission has acceptable impact on system capacity with proper triggers.

Observation 2: Retransmission based on enhanced status report or enhanced polling may not achieve timely RLC retransmission within PDB.

Proposal 1: RAN2 adopts autonomous retransmissions to achieve timely RLC retransmission in RLC AM.

Proposal 1a: Autonomous retransmission can be activated/deactivated by the network or can be triggered when the remaining time of the data becomes lower than a threshold.

Proposal 2: The number of the autonomous RLC retransmissions can be configured by the network or can be pre-defined in RLC.

Proposal 3: The autonomous RLC retransmissions are not counted into RETX_COUNT.

Observation 3: In the Rx initiated approach, the timer in the Rx side can be configured by the network.

Observation 4: In the Tx initiated approach, the Tx side can advance the Tx window first e.g. upon transmitting the RLC SDU gap report to the Rx side.

Observation 5: In the combined Rx and Tx approach, signalling overhead is saved, but it is unclear how to ensure window sync between the Tx side and the Rx side.

Proposal 5: Rx initiated approach is adopted for avoiding unnecessary retransmissions.

References

- R2-2405706, Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei), RAN2#126

- R2-2407575 Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei), RAN2#127