3GPP TSG-RAN WG2 Meeting #127bis	R2-2408166

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.2.2

Source:	Spreadtrum Communications

Title:	Discussion on the functionalities required for Ambient IoT

Document for:	Discussion and Decision

1 Introduction

In RAN2#127 meeting, RAN2 discussed the functionalities required for A-IoT and achieved the following agreements [1]:

Agreements on functionality Send an LS to RAN1 regarding TBS size. Ask SA2 on the max and typical upper layer packet size. RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages. FFS details on signaling RAN2 will study the need and use of a simple message “size” reporting to the reader At least the following information are considered useful to be visible to the reader from CN 1) The service type of A-IoT (e.g. inventory, command). FFS if more information on command type (e.g. read/write/disable) is useful 2) targeted for one or more than one devices; 3) approximate number of target devices (if available). FFS on mandatory/optional RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the A-IoT interface as upper layer data.

In this contribution, we will continue discussion the functionalities required for A-IoT.

2 Discussion

## 2.1 The protocol stack of A-IoT

In RAN2 meeting, it agreed that SDAP, PDCP and RLC layer are not needed for A-IoT. RAN2 assumes that RRC layer is not necessary between the reader and the device, RAN2 later discusses whether we will have a new AS protocol on top of A-IoT MAC layer or A-IoT MAC.

In our view, to meet the compact protocol stack system design objective for A-IoT, a new AS protocol on the top of A-IoT MAC is not needed. The necessary function A-IoT device required may done in MAC layer, we should define a new MAC layer for A-IoT.

Proposal 1: No a new AS protocol on top of A-IoT MAC layer.

Only MAC and PHY protocol layers are needed in the protocol stack for A-IoT, the compact protocol stack for A-IoT as following Figure 1.

Figure 1 A-IoT protocol stack for topology 1

Proposal 2: For A-IoT topology 1, RAN2 agrees the protocol stack in Figure 1.

## 2.2 Functionalities required for A-IoT devices in MAC

Resource allocation

The wireless resource (e.g. time-domain, frequency-domain) used in air interface for A-IoT devices should be allocated. In topology 1, the resource of A-IoT device D2R transmission is allocated by network. Specifically, gNB (reader) can allocate RACH related resource (e.g. time domain resource Q value) by MAC CE to device. R2D transmission resource are selected by gNB itself. This is implementation of gNB and has no effect on A-IoT device.

In topology 2, it has agreed that NW is in control of resources to be used by A-IoT air interface [1]. In other works, gNB allocates R2D and D2R transmission resource to intermediate UE. The above R2D and D2R resource can be either dynamically allocated or semi-persistent scheduling.

Proposal 3: For topology 1, gNB allocates the D2R transmission resource (Q value) to A-IoT device by MAC CE.

Segmentation/reassembly

The segmentation and reassembly are functions of RLC layer, the segmentation function requires extra L2 buffer in device. The extra L2 buffer will cause extra memory. Due to the limited memory of the A-IoT device, this function is not expected to be supported.

From RAN2 perspective, to simplify A-IoT design, there is no need to support segmentation for A-IoT data transmission. According the design target for inventory and command use cases in TR 38.848, the maximum message size is approximately 1000 bits to be received by the Ambient IoT device, and approximately 1000 bits to be transmitted from the Ambient IoT device.

Whether segmentation is supported or not depends on the TB size of RAN1 and the upper layer packet size of SA2. In last RAN2 meeting, RAN2 sent a LS to RAN1 and SA2 [2]. Asking RAN1 to indicate what maximum and minimum TB sizes are expected to be specified in PHY, in both D2R and R2D directions and the limitations/conditions under which TBs of different sizes can be transmitted. And asking SA2 to indicate the maximum data block size delivered from upper layers to the AIoT AS layers for inventory and command cases, in both D2R and R2D directions.

From PHY layer perspective, if it is able to provide a TB size of 1000 bits, then segmentation is not needed. If not, segmentation in AS layer is also not needed since upper layer can do segmentation.

Proposal 4: Segmentation is not needed.

Higher layer repetition

RAN1 is studying physical layer repetition to improve the reliability of D2R transmission, and they note that discussions regarding higher-layer repetitions are up to RAN2 [3]. From RAN2 perspective, repetition refers to retransmission.

For the reliability of R2D transmission, it is by reader implementation. If no response is received within the time window from the device after reader R2D transmission, the reader can retransmission R2D message. For the reliability of D2R transmission, there are several ways. Firstly, if reader failed to receive D2R message, it may feedback NACK and allocate retransmission resource to device. Or Device can be (pre)configured by reader to do blind retransmission. Besides, D2R retransmission can be up to device implementation (e.g. the amount of battery left on the device) and the resource for retransmission can be pre-configured.

Proposal 5: R2D repetition can be up to reader implementation.

Proposal 6: D2R repetition can be triggered by reader indication or up to device implementation.

QoS handling

RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL) in last meeting. And there also no SDAP layer, QoS flow to DRB mapping is not required. The LCP will not be needed. Thus, QoS handling in MAC layer is not needed.

Proposal 7: QoS handling in MAC layer is not needed.

Message size indication

As agreed in RAN2 last meeting, legacy NR BSR/SR is not needed for A-IoT communication. For further study whether further indication of device message size/status is needed. In order to make reader better schedule the next D2R transmission, the reader needs to know the message size of next D2R transmission. One possible way is that Device can carry a message size indication during this D2R transmission, indicating gNB that the next D2R message size.

Proposal 8: The message size indication is needed for A-IoT.

Energy status report

Due to the small capacitance of the device, the energy stored at the device side is not enough for the follow up messages transmission or reception during an inventory service. Energy status information of A-IoT device are useful on the reader side. If device is low energy state, in this case, it would report energy status to reader, reader would wait until the device has sufficient energy, then proceed to schedule this device.

Proposal 9: The energy status report from A-IoT device to reader is needed.

3 Conclusion

In this contribution we discuss the functionalities required for A-IoT, with the following proposals:

Proposal 1: No a new AS protocol on top of A-IoT MAC layer.

Proposal 2: For A-IoT topology 1, RAN2 agrees the protocol stack in Figure 1.

Proposal 3: For topology 1, gNB allocates the D2R transmission resource (Q value) to A-IoT device by MAC CE.

Proposal 4: Segmentation is not needed.

Proposal 5: R2D repetition can be up to reader implementation.

Proposal 6: D2R repetition can be triggered by reader indication or up to device implementation.

Proposal 7: QoS handling in MAC layer is not needed.

Proposal 8: The message size indication is needed for A-IoT.

Proposal 9: The energy status report from A-IoT device to reader is needed.

- References

- Draft report of 3GPP TSG RAN WG2 #127, RAN2 chair (interDigtal), Maastricht, Netherlands, Aug 19th – 23rd, 2024.

- R2-2407831, LS on data block sizes for Ambient IoT, RAN2, Maastricht, Netherlands, Aug 19th – 23rd, 2024.

- Draft report of 3GPP TSG RAN WG1 #116, MCC Support, Athens, Greece February 26th - March 1st, 2024.