3GPP TSG-RAN WG2 Meeting #127bis	R2-2408167

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.2.4

Source:	Spreadtrum Communications

Title:	Discussion on random access of Ambient IoT

Document for:	Discussion and Decision

1 Introduction

In RAN2#127 meeting, RAN2 discussed the random access for A-IoT and achieved the following agreements [1]:

Agreements on “3 step” CBRA For 3-step CBRA Support fixed random ID size is 16 bit. The ID is randomly generated. Failure/success indication of D2R will be studied. FFS if it would be implicit or explicit and for which use case it is needed. FFS whether it is applied only to some cases. RAN2 assumes that Msg3 transmission resource can explicitly be indicated in Msg2 (based on RAN1 agreements/design). Wait for RAN1 for further details. Agreements on 2 step CBRA For 2-step CBRA, RAN2 design will support msg2. Whether it is needed it is up to the reader. FFS when it is needed. For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2. FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices. Agreement on CFRA In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2). FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes. FFS if reader assigns the AS ID for scheduling purposes.

In this contribution, we will continue discuss the random access of Ambient IoT.

2 Discussion

## 2.1 Contention-based random access for A-IoT

3-step CBRA:

For initial inventory to identify multiple A-IoT devices, when a response is expected from multiple devices, an A-IoT contention-based access (CBRA) procedure initiated by the reader is used. For A-IoT contention-based access procedure, slotted-ALOHA based access is baseline. 3-step CBRA random access for A-IoT is shown in Figure 1.

Msg0: Access trigger. The reader sends signal to trigger A-IoT device to perform access. The trigger signal may be paging or other R2D message. The resource for device subsequent transmission may also include in the message.

Figure 1 3-step CBRA random access for A-IoT

Msg1: The device sends a random ID to the reader. During the inventory, reader sends trigger message which contains a Q value, A-IoT device randomly selects a value from 0 to 2Q-1 as the initial count value, and when the count value is 0 or it is reduced to 0, A-IoT device can send a random ID to reader. If the count value is not equal to 0, device transmits nothing and waits for the next trigger signal indicating the end of the previous slot and the start of the next slot.

Msg2: The reader echos the ID received in Msg1. If reader successful reception the RN16, it sends an ACK which at least carries the same RN16 from the A-IoT device. The resource for device next transmission may also include in the ACK.

Msg3: Device sends Device ID and any other upper layer data (depending on upper layer request). If the A-IoT device receives the ACK with the same RN16 that it sent in Msg1, A-IoT device sends its unique device identity (e.g. like EPC in RFID) to Reader. A-IoT device may also send the device type or capability in the message to reader. If ACK is not received or a different RN16 is received, A-IoT device will not transmit the device ID and waits for the next trigger signal.

For 3-step CBRA, one remaining issue is whether need failure/success indication for D2R transmission (Msg3 and any following D2R transmission for data). In our view, if reader has subsequent R2D transmission with device, the success indication of D2R transmission may not be needed. The subsequent transmission implicitly indicates that the D2R are successful transmission. Otherwise, the reader will send success indication with ACK (e.g. contains Random ID). Upon receiving the ACK, device considers the access successful. If reader has not received the D2R successfully, the reader will send failure indication with NACK, then the device will retransmit D2R.

Proposal 1: Success indication of D2R transmission is not needed if there is a subsequent R2D transmission.

Proposal 2: Success indication of D2R transmission is needed if there is no subsequent R2D transmission.

Proposal 3: Failure indication of D2R transmission is needed.

2-step CBRA:

For some cases (e.g. Low collision probability of access), such as a small number of devices are triggered, Msg1 and Msg2 in 3-step CBRA may not be needed.  For 2-step CBRA, RAN2 meeting has agreed that A-IoT device can transmit device ID and any other upper layer data directly in Msg1 when receives an initial trigger message. And when Mgs2 is needed, the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.

Figure 2 2-step CBRA random access for A-IoT

Msg 0: Access trigger. The reader sends trigger signal to trigger A-IoT device to perform access. The trigger signal may be paging or other R2D message and resource for RACH are include in.

Msg 1: A-IoT device sends device ID and any other upper layer data to reader. If the A-IoT device correctly reception the trigger signal and find the device ID matches, it sends device ID and as response.

Msg 2: Reader echos Msg1 with random ID. If reader successful reception Msg1, it sends Msg2 to device for contention resolution. If device does not receive the correct Msg2, it represents a random access failure.

In 2-step CBRA, if the device has command service after inventory, it needs a random ID to communication with the reader. For further optimization the signaling overhead, Msg1 only includes Device ID and any other upper layer data. After reader receiving Msg1, reader generates a random ID for the device based on the device ID. The random ID generation rules are also known to the device side. Then, the random ID is sent to the device in Msg2.

Proposal 4: Msg2 is needed at least for the contention resolution purpose.

Proposal 5: Reader generates a random ID based on device ID and echoes Msg1 with the random ID in Msg2.

## 2.2 Contention resolution failure in CBRA

Device only supports time-domain multiple access in RFID, multiple devices may collide in the same access occasion since device is randomly selecting the access occasion. Last RAN2 meeting has agreed that the device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. In other words, device considers the contention resolution failure if the same ID is not received in the response from reader within a certain time, after transmitting the random access ID. There is no handling for failed access in RFID, the devices that experience collisions do not get a chance to access and should wait until the next inventory round to access again upon receiving the R2D trigger message from reader.

Proposal 6: If contention resolution fails due to collision, A-IoT device will perform access again in the next access round upon receiving the corresponding R2D trigger message.

## 2.3 Contention-free random access for A-IoT

RAN2 agreed that for contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion). Chair notes indicate that we will start by discussing single device case for contention-free access.

For dedicated inventory or command for a identified A-IoT device, the specific A-IoT device ID will included in trigger signal in Msg0. Only the device ID of A-IoT device match can initiate access, so it can be a contention-free random access (CFRA). CFRA based access procedure is shown in Figure 3.

Figure 3 CFRA random access for A-IoT

Msg 0: Access trigger. The reader sends trigger signal to trigger A-IoT device to perform access. The trigger signal may be paging/query or command (read/write etc.) message which the device ID and resource for RACH are include in.

Msg 1: A-IoT device sends device ID and any other upper layer data to reader. If the A-IoT device correctly reception the trigger signal and find the device ID matches, it sends device ID as response. A-IoT device may also send the device type or capability or any other upper layer data within the message to reader.

Msg2 (reader echos Msg1 with random ID) is not needed in CFRA in any cases as the following analysis.

- Case 1: Msg2 is not needed when Msg1 failure. (Reader can retransmit trigger message which include device ID)

- Case 2: Msg2 is not needed for the correctly reception of Msg1 for the case that there is no subsequent access trigger message

- Case 3: Msg2 is not needed if Msg1 is correctly received and there is the subsequent trigger message

Proposal 7: Msg2 is not needed in CFRA.

Proposal 8: The CFRA random access procedure for A-IoT as follows can be supported in R19.

- Msg1: A-IoT device sends device ID and any other upper layer data to reader.

## 2.4 Determine CBRA or CFRA

It has been agreed that random access is triggered by the reader. Therefore, it would be reasonable to leave the reader to decide which type of access procedures to apply. Furthermore, the reader can indicate the appropriate random access type for the device in the trigger message.

Proposal 9: The reader indicates the random access type (2/3 step CBRA or CFRA) for device.

## 2.5 Efficiency enhancement on random access

Slotted-ALOHA is a contention resolution based on TDMA in backscatter wireless system. It is often referred as a probabilistic anti-collision algorithm, only slots that do not collide can be decoded successfully, with an efficiency of 36.8%. The efficiency here is defined as the ratio of the number of devices successfully inventoried to the number of time slots used.

And there indeed is another anti-collision algorithm candidate that is conflict resolution diversity slotted-ALOHA (CRDSA). The CRDSA key idea is to transmit two replicas of the same packet at random two slots within the same frame with a little extra signalling to point to the replica packet location. In case of successful packet reception at network this extra signalling information allows to locate the replica packet within the frame and to accurately cancel it in addition to the one successfully decoded. The effect of the other replica packet is eliminated by iterative self-interference cancellation (SIC) technology. The efficiency of CRDSA is 52%.

Figure 4 Conflict resolution diversity slotted-ALOHA (CRDSA)

Figure 5 Efficiency comparison of slotted-ALOHA and CRDSA

| Random access algorithm | Efficiency |
| --- | --- |
| Slotted-ALOHA | 36.8% |
| CRDSA | 52% |

Figure 6 The efficiency of random access algorithm

If there are chances to discuss enhancement for slotted-ALOHA, and the CRDSA access procedure should be considered.

Proposal 10: RAN2 to study CRDSA based access as a candidate for random access.

3 Conclusion

In this contribution we discuss the random access procedure of A-IoT device, with the following proposals:

Proposal 1: Success indication of D2R transmission is not needed if there is a subsequent R2D transmission.

Proposal 2: Success indication of D2R transmission is needed if there is no subsequent R2D transmission.

Proposal 3: Failure indication of D2R transmission is needed.

Proposal 4: Msg2 is needed at least for the contention resolution purpose.

Proposal 5: Reader generates a random ID based on device ID and echoes Msg1 with the random ID in Msg2.

Proposal 6: If contention resolution fails due to collision, A-IoT device will perform access again in the next access round upon receiving the corresponding R2D trigger message.

Proposal 7: Msg2 is not needed in CFRA.

Proposal 8: The CFRA random access procedure for A-IoT as follows can be supported in R19.

- Msg1: A-IoT device sends device ID and any other upper layer data to reader.

Proposal 9: The reader indicates the random access type (2/3 step CBRA or CFRA) for device.

Proposal 10: RAN2 to study CRDSA based access as a candidate for random access.

- References

- Draft report of 3GPP TSG RAN WG2 #127, RAN2 chair (interDigtal), Maastricht, Netherlands, Aug 19th – 23rd, 2024.