3GPP TSG-RAN WG2 Meeting #127bis	R2-2408168

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.4.2

Source:	Spreadtrum Communications

Title:	Discussion on LP-WUS operation in IDLE/INACTIVE mode

Document for:	Discussion and Decision

1 Introduction

In RAN2#127 meeting, RAN2 discussed the procedure and configuration of LP-WUS in RRC_IDLE INACTIVE and achieved the following agreements [1]:

LP-WUS configuration Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE. Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. LP-WUS entry/exit condition Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

In this contribution, we will continue discussing the procedure and configuration of LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring.

2 Discussion

## 2.1 LP-WUS configuration

In last RAN2 meeting, it agreed that the LP-WUS related configuration at least includes LP-SS configuration, LP-WUS configuration and entry/exit condition for LP-WUS monitoring. And the LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed.

Similar to PEI, the LP-WUS configuration is at a cell level for idle/inactive UE, and all the idle/inactive UEs in the cell can use the configuration in the SIB. LP-WUS configuration via dedicated signalling for idle/inactive UE is not needed, it will increase the signalling overhead.

Proposal 1: The dedicated LP-WUS related configuration for IDLE/INACTIVE state is not needed.

## 2.2 Entry/exit conditions for LP-WUS monitoring

LP-WUS has been configured in the cell by SIB, both UE and gNB could apply LP-WUS. Then it need to further determine the entry and exit conditions which UE monitoring LP-WUS. In last RAN2 meeting,  RAN2 made a working assumption that if the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

If gNB configures both MR-based thresholds and LR-based thresholds, there is a problem that UE how to use the thresholds to start LP-WUS monitoring. In our view, if gNB configures same threshold for both LP-WUS monitoring and serving cell RRM measurement offloading. Before UE starting LP-WUS monitoring, UE MR is wake-up and monitors legacy PO, MR also measures the serving cell, so UE use the serving cell measurement result of MR (e.g. serving cell measurement result based on MR is above MR-based threshold) to start LP-WUS monitoring is more suitable due to the measurement accuracy of MR is higher than that of LR.

If gNB configures different thresholds, and the threshold for serving cell RRM measurement offloading is stricter than the threshold for LP-WUS monitoring, UE has offloaded the serving cell measurement form MR to LR before LP-WUS monitoring. Then the condition for entry LP-WUS monitoring can be the measurement result of LR (e.g. serving cell measurement result based on LR is above the LR-based threshold).

Proposal 2: The serving cell measurement result of MR is above MR-based threshold as entry condition for LP-WUS monitoring, if MR has been measuring the serving cell before LP-WUS monitoring.

Proposal 3: The serving cell measurement result of LR is above LR-based threshold as entry condition for LP-WUS monitoring, if the serving cell measurement has offloaded form MR to LR before LP-WUS monitoring.

## 2.3 Subgrouping methods for LP-WUS

It agreed that the PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

In Rel-17, subgroup indication up to 8 subgroups are carried by PEI for an associated PO. In LP-WUS, RAN1 discussed and agreed that the maximum number of subgroups per PO is X, where the value of X is one of 8, 16, 32, 64, 128 and 256 [2]. Specifically, this is related to the specific content (e.g. bitmap or codepoint) carried in the LP-WUS. It has been discussed in RAN1, but has not concluded yet.

Agreement For the maximum number of subgroups per PO (X), down-select from the following options in RAN1#118bis: - Option 1: X = 8 - Option 2: X = 16 - Option 3: X = 32 - Option 4: X = 64 - Option 5: X = 128 - Option 6: X = 256

From RAN2 perspective, large subgroup in LP-WUS means more payload and will result in more overhead. To achieve the trade-off between the payload size and power saving gain for a UE, the number of subgroups can be carried in LP-WUS is 8, same as PEI.

Proposal 4: The number of subgroups can be carried in LP-WUS is 8.

RAN1 discussed the association relation between PO and LO [3]. The association relationship between PO and LO is equivalent to the relationship between the subgroup of PEI and LP-WUS in RAN2. According to RAN1 agreements, the subgroup of LP-WUS can be same as the PEI, or the subgroup of PEI can be a subset of LP-WUS subgroup and vice versa. Cases with or without intersection between PEI and LP-WUS subgroup are excluded.

Agreement It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options: • Option 1: UEs monitoring the same PO monitor the same LO • Option 2: UEs corresponding to different POs monitor the same LO • Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. Combinations of the above options can be considered.

In case LP-WUS and PEI are used together for one UE, it is expected that UE could achieve more power saving gain via two levels of subgroup for both LP-WUS and PEI. UE first monitors LP-WUS, then monitors PEI. The subgroup of PEI should be a subset of LP-WUS subgroup, or the subgroup of PEI and the subgroup of LP-WUS have an intersection, as shown in figure 1a and 1b. In this way, it is expected that some UEs will be filtered out to further monitor PO.

However, the latter way has been excluded by RAN1. The subgroup of PEI is a subset of LP-WUS subgroup, it is easy to implementation. The formula of PEI subgroup can be reused, just make the subgroupsNum (bitmap) shorter.

1a

1b

Figure 1 relationship of LP-WUS subgroup and PEI subgroup

Proposal 5: For monitoring PEI after monitoring LP-WUS, the subgroup of PEI should be a subset of LP-WUS subgroup.

3 Conclusion

In this contribution we discuss the LP-WUS operation in IDLE/INACTIVE modes, with the following proposals:

Proposal 1: The dedicated LP-WUS related configuration for IDLE/INACTIVE state is not needed.

Proposal 2: The serving cell measurement result of MR is above MR-based threshold as entry condition for LP-WUS monitoring, if MR has been measuring the serving cell before LP-WUS monitoring.

Proposal 3: The serving cell measurement result of LR is above LR-based threshold as entry condition for LP-WUS monitoring, if the serving cell measurement has offloaded form MR to LR before LP-WUS monitoring.

Proposal 4: The number of subgroups can be carried in LP-WUS is 8.

Proposal 5: For monitoring PEI after monitoring LP-WUS, the subgroup of PEI should be a subset of LP-WUS subgroup.

- References

- Draft report of 3GPP TSG RAN WG2 #127, RAN2 chair (interDigtal), Maastricht, Holland, Aug 19th – 23rd, 2024.

- Draft report of 3GPP TSG RAN WG1 #118, RAN1 chair’s notes, Maastricht, the Netherlands, Aug 19th – 23rd, 2024.

- Draft report of 3GPP TSG RAN WG1 #117, RAN1 chair’s notes, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.