3GPP TSG-RAN WG2 Meeting #127-bis 	R2- 2408172

Hefei, China, 14th – 18th Oct, 2024

Agenda Item:	8.5.2

Source:	Spreadtrum Communications

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion and Decision

Introduction

Based on the outcome of RAN#102 meeting, the following objectives were agreed in the WID (RP-234065):

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, SCell activation/deactivation signaling) Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we will discuss the issues related to the on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. And in our contribution, we will discuss the issues related to the activation/deactivation of on-demand SSB operations, L3 measurement and provide our opinions.

On-demand SSB SCell operation

On-demand SSB transmission MAC CE

For the scenarios on OD SSB, the following agreements were made [1].

RAN2 #127 Agreement: Scenarios on OD-SSB: 22. RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

And for OD SSB transmission indication, RAN2 #127 agreed that RRC based OD SSB transmission indication is used to indicate at least initial activation/deactivation state and new MEC CE is for dynamic activation/deactivation OD SSB for scenario 2 and 2A. For the design of RRC configuration signalling, RAN2 can wait for further progress of RAN1 to specify signalling.

Proposal 1: RAN2 can wait for further RAN1 progress to design RRC configuration signalling to indicate OD SSB transmission.

And RAN1 last meeting, there is a LS to RAN2 for on-demand SSB Scell operation [2]. In this LS, RAN1 requests us to design MAC CE based signaling to indicated on-demand SSB transmission for Scenarios #2/2A. And RAN1 also agreed that the applocable value can be indicated by MAC CE.

RAN2 #127 Agreement: OD-SSB transmission indication: 23. RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. 24. New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A. Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

So on-demand SSB transmission indication MAC CE not only needs to indicate the activation/deactivation of OD-demand SSB transmission but also needs to indicate the periodicity of upcoming OD-SSB transmission to support switching OD SSB periodicity by NW. According to RAN1 discussion, the periodicity value of OD-SSB will most likely reuse the legacy SSB’s configuration. The legacy SSB has 6 periodicity value, including: 5ms, 10ms, 20ms, 40ms, 80ms, 160ms. So three bit is enough to indicate the periodicity of OD SSB. So Figure 1 below shows a possible design of MAC CE.

Figure 1 the design of OD-SSB transmission indication MAC CE

The new MAC CE has three field

- The Si field is to indicates the activation/deactivation status of OD-SSB transmission in the SCell with SCellIndex i. The Si field is set to 1 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be activated. The Si field is set to 0 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be deactivated.

- The Pi field is to indicate the applicable value of OD SSB periodicity. There are two options to indicate periodicity. One option is directly indicates an index to Table 1 via Pi field. Option 2 is that network configures a list of OD-SSB periodicity (e.g., ssb-periodicityServingCell) by RRC signalling and Pi can be the index of SSB periodicity with ssb-periodicityServingCell (reusing legacy IE, it can be updated if defined for OD-SSB configuration). The length of the Pi field is 3 bits.

- The R field is the reserved bit, set to 0.

Table 1: values for OD-SSB periodicity

| index | Periodicity value (ms) |
| --- | --- |
| 0 | 5 |
| 1 | 10 |
| 2 | 20 |
| 3 | 40 |
| 4 | 80 |
| 5 | 160 |

Proposal 2: The following new OD-SSB MAC-CE design with three fields can be considered:

  - The Si field is to indicate the activation/deactivation status of OD-SSB transmission.

  - The Pi field is to indicate the applicable value of OD SSB periodicity.

  - The R field is the reserved bit, set to 0.

Proposal 3: The Si field is to indicates the activation/deactivation status of OD-SSB transmission in the SCell with SCellIndex i. The Si field is set to 1 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be activated. The Si field is set to 0 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be deactivated.

Proposal 4: There are two options to indicate OD-SSB periodicity by MAC CE:

- Option 1: The Pi field directly indicates an index to Table 1.

- Option 2: Network configures a list of OD-SSB periodicity (e.g., ssb-periodicityServingCell) by RRC signalling and Pi can be the index of SSB periodicity with ssb-periodicityServingCell (reusing legacy IE, it can be updated if defined for OD-SSB configuration).

- 2.3 L3 measurement based on on-demand SSB

In the legacy NR RRM based on SSB, the network will configure SMTC to measure SSB. The SMTC is configured by SCellconfig or MeasObjectNR of Scell for SSB measurement as following configuration. And UE only performs L3 measurements based on SSB within the SMTC window according to current spec [3]. In last RAN2 meeting, we have agreed to consider measurement based on OD-SSB in both case1 and case2. And for case #1, the UE only expects to measure SSB when on-demand SSB transmission is activated. RAN2 will study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon      OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig            OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                      OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}           OPTIONAL,   -- Cond SCellAddSync

Omit unrelated part

]]

}

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                   OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

absThreshSS-BlocksConsolidation     ThresholdNR                                                     OPTIONAL,   -- Need R

absThreshCSI-RS-Consolidation       ThresholdNR                                                     OPTIONAL,   -- Need R

omit unrelated part

}

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

SSB-ConfigMobility::=               SEQUENCE {

ssb-ToMeasure                       SetupRelease { SSB-ToMeasure }                                  OPTIONAL,   -- Need M

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                             OPTIONAL,   -- Need M

...,

omit unrelated part

}

For supporting L3 measurement based on on-demand SSB, a new measurement object for OD-SSB can be configured for SCell. Or a new measurement object with short periodicity to cover both always-on SSB and OD-SSB can be configured.

According to RAN1 #118 agreement, the periodicity of on-demand SSB can be changed by MAC CE. The legacy SMTC configuration of SSB may not work because RRC signalling is slower that MAC CE. And RRM based on on-demand SSB may failure. So for the solution of configureing a new MO for OD-SSB, the new MO may be activated when OD-SSB is activated by MAC CE.

Proposal 5: For supporting L3 measurement based on OD-SSB, a new measurement object only for OD-SSB can be configured for SCell or a new measurement object with short periodicity covering both always-on SSB and OD-SSB can be configured.

Conclusions

In this contribution, we discuss the Rel-19 on-demand SSB SCell operation issues. And the proposals are given as follows:

Proposal 1: RAN2 can wait for further RAN1 progress to design RRC configuration signalling to indicate OD SSB transmission.

Proposal 2: The following new OD-SSB MAC-CE design with three fields can be considered:

  - The Si field is to indicate the activation/deactivation status of OD-SSB transmission.

  - The Pi field is to indicate the applicable value of OD SSB periodicity.

  - The R field is the reserved bit, set to 0.

Proposal 3: The Si field is to indicates the activation/deactivation status of OD-SSB transmission in the SCell with SCellIndex i. The Si field is set to 1 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be activated. The Si field is set to 0 to indicate that OD-SSB transmission in the SCell with SCellIndex i shall be deactivated.

Proposal 4: There are two options to indicate OD-SSB periodicity by MAC CE:

- Option 1: The Pi field directly indicates an index to Table 1.

- Option 2: Network configures a list of OD-SSB periodicity (e.g., ssb-periodicityServingCell) by RRC signalling and Pi can be the index of SSB periodicity with ssb-periodicityServingCell (reusing legacy IE, it can be updated if defined for OD-SSB configuration).

Proposal 5: For supporting L3 measurement based on OD-SSB, a new measurement object only for OD-SSB can be configured for SCell or a new measurement object with short periodicity covering both always-on SSB and OD-SSB can be configured.

References

[1] RAN2 #127 meeting chair notes.

[2] R1-2407438, LS to RAN2 for on-demand SSB SCell operation.

[3] TS 38.331 v18.0.0, RRC protocol specification.