3GPP TSG-RAN WG2 Meeting #127bis                                                               R2-2408182

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	LP-WUS in RRC_IDLE/INACTIVE

Agenda Item:	8.4.2

Document for:	Discussion and Decision

Introduction

In RAN2#127 meeting, some agreements on LP-WUS entry/exit condition were achieved.

Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

In this contribution, some further considerations are provided, including the carrier/frequency of LR, further considerations on entry/exit condition, subgrouping and configurations for derivation of serving cell from LR.

Discussion

Band/carrier of LR

In RAN1#118 meeting, RAN1 discussed the issue of LR and MR on different carriers/frequencies. RAN1 sent the LS [1] to RAN2/RAN4 as follows:

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

In this LS, RAN1 understand UE may not support LP-WUS reception on all the bands supported by the UE. That means LR and MR may be operated in different bands/carriers.

Currently, serving cell measurement result based on LR is considered for exit condition for LP-WUS monitoring. If LR and MR can be operated in different bands/carriers, the coverage of LR may be different with that of MR. Then RRM measurements based on LR may not be applicable to evaluate the channel condition on MR. Hence, serving cell measurement result based on MR needs to be considered for exit condition for LP-WUS monitoring as well.

Then, the exit condition for LP-WUS monitoring turns to: if the serving cell measurement result based on LR is below a threshold (if configured) or the serving cell measurement result based on MR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

In RAN2#127 meeting, agreed conditions for serving cell measurement offloading are similar to entry/exit conditions for LP-WUS monitoring.

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

If LR and MR can be operated in different bands/carriers, RRM measurements based on LR cannot be used to evaluate the channel condition on MR. Thus, RRC measurements based on MR are needed when LR and MR are operated in different bands/carriers. If so, serving cell measurement offloading may not be achieved in this case. Furthermore, it is questionable if serving cell measurement based on MR is allowed to be relaxed, as LR and MR are operated in different bands/carriers. The power saving gain of LP-WUS is reduced.

In addition, if LR and MR can be operated in different bands/carriers, additional UE capabilities are required for LP-WUS operation.

Based on above we suggest RAN2 confirm the above observations and send LS reply to RAN1.

Proposal 1: RAN2 confirm and include the following observations in the LS reply to RAN1:

When LR and MR are operated in different bands/carriers,

- Not only serving cell measurement result based on LR but also serving cell measurement result based on MR needs to be considered for the exit condition of LP-WUS monitoring, e.g. if the serving cell measurement result based on LR is below a threshold (if configured) or the serving cell measurement result based on MR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

- Serving cell measurement offloading is not allowed. Furthermore, it is questionable if serving cell measurement based on MR is allowed to be relaxed. These reduce the power saving gain of LP-WUS.

- Additional UE capabilities may be required for LP-WUS operation.

Entry/exit condition

RAN2 agreed separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. But OFDM based LP-WUS may have the same coverage of MR. In PEI mechanism, there is no entry/exit condition for PEI monitoring as PEI and paging have the same coverage. Similar to PEI mechanism, the threshold(s) for entry/exit conditions for LP-WUS monitoring is not needed if LP-WUS and MR have the same coverage. And it is up to UE implementation whether/when to start/stop LP-WUS monitoring when threshold(s) for entry/exit conditions for LP-WUS is not configured.

Proposal 2: Thresholds for entry/exit conditions of LP-WUS monitoring are optionally configured by the gNB. If threshold(s) is not configured, it is up to UE implementation whether/when to start/stop LP-WUS monitoring.

Subgrouping

In RAN2#126 meeting, we agreed:

- RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. - RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

RAN1 has agreed it is up to UE implementation whether to monitor PEI or not after the UE receives LP-WUS indicating wake-up. In order to reduce paging false alarm rate when LP-WUS monitoring is used together with PEI monitoring, UEs belonging to the same LP-WUS subgrouping are expected to be assigned to different PEI subgroupings as much as possible.

For CN-based subgrouping, it is up to CN to assign the UE ID. RAN2 could send LS to e.g., SA2, to inform the principle on subgrouping, i.e. UEs belonging to the same LP-WUS subgrouping are expected to be assigned to different PEI subgroupings as much as possible. Let other WGs consider this principle when design the CN-based subgrouping.

Proposal 3: In order to reduce paging false alarm rate when LP-WUS monitoring is used together with PEI monitoring, UEs belonging to the same LP-WUS subgrouping are expected to be assigned to different PEI subgroupings as much as possible. Send LS to SA2 to ask them take this into account when design the CN-based subgrouping.

For UE_ID based PEI subgrouping, the gNB and UE can determine the subgroup ID based on the UE ID and the total number of subgroups for UE ID based subgrouping in the cell. The following is the formula for UE_ID based PEI subgrouping in TS 38.304 [2]:

SubgroupID = (floor(UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID), where: N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1 Ns: number of paging occasions for a PF UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192 subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information

It is straightforward to reuse the similar formula as PEI as possible for UE-ID based subgrouping. However, the case is a little more complex for UE-ID-based subgrouping. Take DRX case as one example. The part of UE ID which determines the PO is the up to 10 LSBs (least significant bits), and part of UE ID used for PEI is the up to 3 LSBs next to the ones used to determine the PO. That is to say, up to 13 LSBs is used if a UE is configured with PEI only. For CN-originated paging the RAN is provided with the entire UE ID, and for RAN-originated paging RAN node is provided with an extended IE of 16 LSBs [3].

#### 9.3.3.52 Extended UE Identity Index Value This IE is used by the NG-RAN node to calculate the Paging Frame and Paging Occasion as specified in TS 36.304 [29], the Paging Frame and Paging Occasion for eDRX and the UE_ID based subgroup ID as specified in TS 38.304 [12]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Extended UE Identity Index Value; M; BIT STRING (SIZE(16)).

Hence, only 3 bits are left for LP-WUS subgrouping. For LP-WUS can be used with eDRX, the issue is more severe since only 1 bit is left for LP-WUS subgrouping. The LP-WUS subgrouping number is still under discussion in RAN1. If the LP-WUS subgrouping number exceeds 8 for DRX case (which requires 4bits) or LP-WUS can be configured together with eDRX (for which only 1 bit left), how to design the UE-ID-based subgrouping can be further discussed by RAN2 and perhaps brings impact to other WGs.

Observation 1: The remaining bits of UE ID may be not enough for LP-WUS subgrouping if LP-WUS subgrouping number exceeds 8 for DRX case or LP-WUS can be configured together with eDRX.

Therefore, for UE-ID based solution, the baseline is that RAN2 reuses the formula of PEI as much as possible. But details can be further discussed until some progresses are made in RAN1, e.g. the maximum LP-WUS subgroup number.

Hence, we propose:

Proposal 4: For UE_ID based subgrouping for LP-WUS, similar formula for PEI subgrouping is reused, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns)) mod subgroupsNumForUEID_LP) + (subgroupsNumPerPO_LP – subgroupsNumForUEID_LP), where

UE_ID is related to 5G-S-TMSI, detail depending on RAN1 progress, e.g. LP-WUS subgrouping number,

N is the number of total paging frames in DRX cycle,

Ns is the number of the PO for a PF,

subgroupsNumForUEID_LP and subgroupsNumPerPO_LP are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Configurations for derivation of serving cell from LR

In RAN2#126 meeting, it was agreed:

- The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

According to the agreements for entry/exit condition for LP-WUS monitoring, serving cell quality from LR needs to be defined. And multi-beam operation is supported for LP-WUR. Therefore, RAN2 needs to discuss how to derive serving cell measurement quantity from LR.

For cell reselection, how to derive a cell measurement quantity of MR in multi-beam operations is captured in TS 38.304 [2] as follows:

For cell selection in multi-beam operations, measurement quantity of a cell is up to UE implementation. For cell reselection in multi-beam operations, including inter-RAT reselection from E-UTRA to NR, the measurement quantity of this cell is derived amongst the beams corresponding to the same cell based on SS/PBCH block as follows: - if nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or - if absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or - if the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA): - derive a cell measurement quantity as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [11]. - else: - derive a cell measurement quantity as the linear average of the power values of up to nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) of highest beam measurement quantity values above absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA).

The derivation of cell measurement quantity from MR with multi-beam operation can be reused for LR, e.g. derivation of cell measurement quality from LR as the linear average of the power values of up to N highest beam measurement quantity values above a threshold. And similar to derivation of cell measurement quantity from MR, parameters for derivation of serving cell from LR, e.g. the threshold for beams used for derivation of cell measurement quantity and the number of beams (e.g. N ) used for derivation of cell measurement quantity, need to be included in SIB for LP-WUS monitoring in RRC_IDLE/INACTIVE.

Proposal 5: Cell measurement quality from LR is derived from the linear average of the power values of up to N highest beam measurement quantity values above a threshold. Parameters for derivation of serving cell from LR in SIB for LP-WUS monitoring in RRC_IDLE/INACTIVE, including:

- the threshold for beams used for derivation of cell measurement quantity;

- N, the number of beams to average for cell measurement derivation.

Conclusion

According to the analysis in section 2, we propose:

Band/carrier of LR:

Proposal 1: RAN2 confirm and include the following observations in the LS reply to RAN1:

When LR and MR are operated in different bands/carriers,

- Not only serving cell measurement result based on LR but also serving cell measurement result based on MR needs to be considered for the exit condition of LP-WUS monitoring, e.g. if the serving cell measurement result based on LR is below a threshold (if configured) or the serving cell measurement result based on MR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

- Serving cell measurement offloading is not allowed. Furthermore, it is questionable if serving cell measurement based on MR is allowed to be relaxed. These reduce the power saving gain of LP-WUS.

- Additional UE capabilities may be required for LP-WUS operation.

Entry/exit condition for LP-WUS monitoring:

Proposal 2: Thresholds for entry/exit conditions of LP-WUS monitoring are optionally configured by the gNB. If threshold(s) is not configured, it is up to UE implementation whether/when to start/stop LP-WUS monitoring.

Subgrouping:

Proposal 3: In order to reduce paging false alarm rate when LP-WUS monitoring is used together with PEI monitoring, UEs belonging to the same LP-WUS subgrouping are expected to be assigned to different PEI subgroupings as much as possible. Send LS to SA2 to ask them take this into account when design the CN-based subgrouping.

Observation 1: The remaining bits of UE ID may be not enough for LP-WUS subgrouping if LP-WUS subgrouping number exceeds 8 for DRX case or LP-WUS can be configured together with eDRX.

Proposal 4: For UE_ID based subgrouping for LP-WUS, similar formula for PEI subgrouping is reused, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns)) mod subgroupsNumForUEID_LP) + (subgroupsNumPerPO_LP – subgroupsNumForUEID_LP), where

UE_ID is related to 5G-S-TMSI, detail depending on RAN1 progress, e.g. LP-WUS subgrouping number,

N is the number of total paging frames in DRX cycle,

Ns is the number of the PO for a PF,

subgroupsNumForUEID_LP and subgroupsNumPerPO_LP are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Configurations for derivation of serving cell from LR:

Proposal 5: Cell measurement quality from LR is derived from the linear average of the power values of up to N highest beam measurement quantity values above a threshold. Parameters for derivation of serving cell from LR in SIB for LP-WUS monitoring in RRC_IDLE/INACTIVE, including:

- the threshold for beams used for derivation of cell measurement quantity;

- N, the number of beams to average for cell measurement derivation.

Reference

- R2-2407921, LS on LP-WUS operation in IDLE/INACTIVE mode

- 3GPP TS 38.304 V18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state;

- 3GPP TS 38.413 V18.2.0, NG-RAN, NG Application Protocol (NGAP).