3GPP TSG-RAN WG2 Meeting #127bis	R2-2408184

Hefei, China, October 14th – 18th, 2024

Source:	CATT

Title:	Analysis on LP-WUS for RRC_CONNECTED Mode

Agenda Item:	8.4.4

Document for:	Discussion and Decision

Introduction

In RAN1#118 meeting [1] and RAN2#127 meeting [2], great progress was made for LP-WUS in RRC_CONNECTED mode. Based on the agreements, we will continue to analyze the LP-WUS monitoring procedure in RRC_CONNECTED mode and our observations and proposals are provided.

Discussion

2.1	Common issues for Option 1-1 and Option 1-2

Regarding the options for LP-WUS monitoring in RRC_CONNECTED mode, RAN1 has made the following assumption:

Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

According to the agreement, it can be seen that Option 1-1 and Option 1-2 were assumed to be supported by RAN1 and RAN2 can revisit the assumption if necessary.

From RAN2’s perspective, Option 1-1 can be supported considering it has power saving gain with minimum spec impacts. Regarding option 1-2, it brings benefits on latency reduction compared with option 1-1 considering the UE can be waked up when necessary. Therefore, it is proposed that:

Proposal 1: Option 1-1 can be supported for PDCCH monitoring triggered by LP-WUS in RRC CONNECTED.

Proposal 2: Option 1-2 can be supported for PDCCH monitoring triggered by LP-WUS in RRC CONNECTED.

It is still FFS on whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE based on RAN1 agreement. In our understanding, the UE behaviours are complex if both Option 1-1 and Option 1-2 are configured simultaneously. For Option 1-1, the UE will start drx-onDurationTimer when it is waked up by LP-WUS, while for Option 1-2, the UE will be triggered by LP-WUS to start a timer duration which UE monitors PDCCH. If these two options are combined, one possible procedure is that the UE performs LP-WUS monitoring outside at least legacy C-DRX Active time according to the LP-WUS. And the UE also monitors LP-WUS which is located before drx-onDurationTimer. When this LP-WUS indicates the UE to start drx-onDurationTimer, the drx-onDurationTimer will be started like Option 1-1. Otherwise, the UE continues to monitor LP-WUS during the legacy drx-onDurationTimer. Apparently, when both Option 1-1 and Option 1-2 simultaneously configured, the procedure will be quite complex and no further benefit compared with separate Option 1-1 or Option 1-2. Therefore, it is proposed that:

Proposal 3: It is NOT supported to configure both Option 1-1 and Option 1-2 simultaneously for the same UE.

2.2	Option 1-1

In RAN1#116bis, option 1-1 was agreed as one option for LP-WUS monitoring:

- Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality

And in RAN2#126 meeting, the agreements for option 1-1 were made as following:

For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

Along with the progress made in RAN1 and RAN2, it can be seen that solutions/operations introduced for DCP mechanism is taken as baseline. In the following, we continue to analyze other issues for option 1-1.

    - Dual DRX group

In order to reduce the power consumption when both FR1 and FR2 cells are configured via Carrier Aggregation, a secondary DRX group was introduced with a separate drx-InactivityTimer and drx-onDurationTimer configured for the FR2 cells enabling FR2 to go to sleep more quickly. However, in order to reduce the impacts to RAN1 and keep the simplicity of DRX group mechanism, dual DRX group is not configured simutanously with DCP.

As RAN2 has assumed that the solutions/ operations introduced for DCP mechanism is taken as baseline, the LP-WUS should not configured simultaneously with dual DRX group to reduce the specification complexity.

Proposal 4: In option 1-1, LP-WUS should not be configured with dual DRX groups simultaneously.

    - Short DRX cycle

For DCP, if short DRX cycle is supported, it may require a huge overhead in the DL which may reduce the power saving gain. In RAN1 replied LS [3], it indicated RAN1 assumed that DCP was not applicable for the short DRX cycle if both long and short DRX cycles are configured for the UE. Finally, it was agreed that DCP was only applicable to long DRX cycle.

For LP-WUS, it can simply follow the same mechanism of DCP as above.

Proposal 5: Short DRX cycle is not applicable for LP-WUS.

    - Miss-detection

In Rel-16, DCP is located at one offset before drx-onDurationTimer. It may happen that the UE does not detect DCP and can’t figure out whether to start drx-onDurationTimer. Hence, one flag ps-WakeUp was defined to indicate whether the UE to wake-up if DCP is not detected outside active time.

DCP mechanism[4]

ps-WakeUp Indicates the UE to wake-up if DCI format 2-6 is not detected outside active time (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not wake-up if DCI format 2-6 is not detected outside active time.

As RAN2 has agreed that the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer, the similar miss-detection issue may also happen. From RAN2 perspective, we think this can follow the mechanism of DCP.

Proposal 6: Define a flag to indicate whether the UE to wake-up to monitor PDCCH if miss-detection happens for LP-WUS in Option 1-1.

2.3	Option 1-2

Regarding Option 1-2, RAN1 and RAN2 made the following agreements:

RAN1#118 meeting: Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18 - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration - LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

RAN2#127 meeting: For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

Based on the agreements above, we continue to discuss the remaining issues for option 1-2.

- 2.3.1	Timer triggered by LP-WUS to monitor PDCCH

It has been agreed that the UE starts one timer after LP-WUS triggers the UE to perform PDCCH monitoring. But it is still FFS on whether the timer is newly defined or reusing legacy timer. Based on current MAC spec, the DRX Active Time which is related to DRX timers consists of drx-onDurationTimer, drx-InactivityTime and drx-RetransmissionTimerDL/drx-RetransmissionTimerUL.

We first analyze whether drx-RetransmissionTimerDL/drx-RetransmissionTimerUL can be reused. For drx-RetransmissionTimerDL/drx-RetransmissionTimerUL, they are per HARQ process maintained and used to define the minimum duration before a DL assignment for HARQ retransmission/ a UL HARQ retransmission grant is expected by the MAC entity respectively. Hence, they are not suitable for the timer triggered by LP-WUS to monitor PDCCH.

Observation 1: DRX retransmission timers including drx-RetransmissionTimerDL and drx-RetransmissionTimerUL are not suitable for the timer triggered by LP-WUS to monitor PDCCH considering they are used to monitor PDCCH for retransmission and per HARQ process maintained.

In the subsequent sections we compare the following aspects including procedure and value rage for onDurationTimer, drx-InactivityTime and new defined timer and figure out which timer is suitable in LP-WUS case.

2.3.1.1	Procedure analysis

Regarding drx-onDurationTimer, it can be started periodically based on the long/short DRX cycle. When it is running, the UE monitors PDCCH. And drx-onDurationTimer is not restarted, considering it is used to monitor potential PDCCH. And it is per MAC entity maintained.

For drx-InacitivityTimer, it is used to define the duration after the PDCCH occasion in which a PDCCH indicates a new UL, DL or SL transmission for the MAC entity. When drx-InacitivityTimer is running, the UE remains in DRX Active Time. When drx-InacitivityTimer is started or restarted, UE Active Time is extended.

For new timer defined for LP-WUS, it can be started after one offset after LP-WUS indication. When the new timer is running, the UE monitors PDCCH which can be defined as DRX Active Time. Then, aligning with RAN1 agreements for option 1-2 that the UE PDCCH monitoring behaviors related to drx-InactivityTimer is not affected, drx-InactivityTimer can be started or restarted during the running time of the new timer. According to current spec, both drx-onDurationTimer and drx-InactivityTimer will be stopped when a DRX command MAC CE or a long DRX command MAC CE is received. We think, this principle can also be applied to the new timer. That is when the UE receives a DRX command MAC CE or a long DRX command MAC CE, the new timer is stoppoed.

We summarize the differences for drx-onDurationTimer, drx-InactivityTimer and new timer for LP-WUS in Table 1:

Table 1: Comparison among drx-onDurationTimer, drx-InactivityTimer and new timer for LP-WUS

| Timers | drx-onDurationTimer | Drx-inactivityTimer | New timer for LP-WUS |
| --- | --- | --- | --- |
| The condition to start the timer | Based on the DRX formula considering short/long DRX cycle | When a PDCCH indicates a new UL, DL or SL transmission for the MAC entity and when the UE is in DRX Active Timer | When LP-WUS indicating the UE to wake up |
| Wake up opportunity | Periodically | Occasionally according to PDCCH indicating new transmission | Occasionally according to LP-WUS indication |
| UE Behavior when the timer is running | The UE monitors PDCCH and the UE is in DRX Active Time. |  |  |
| Whether can be restarted | Not allowed | Can be restarted | Not needed, considering drx-InactivityTimer can be started/restarted during DRX Active Time |
| Whether can be terminated | Yes, by (long)DRX command MAC CE |  | Yes, but needs confirmation |

According to Table 1, it can be seen that the timer defined for LP-WUS triggered PDCCH monitoring has different procedure as legacy DRX timers. If legacy DRX timers are used for PDCCH monitoring triggered by LP-WUS, it may bring unclear impacts to the legacy DRX procedure.

Observation 2: The new timer defined for PDCCH monitoring triggered by LP-WUS has different procedure as legacy DRX timers, i.e., drx-onDurationTimer and drx-InactivityTimer as illustrated in Table 1.

2.3.1.2	Value configuration

The original usage of drx-onDurationTimer is to wake up the UE periodically to monitor occasionally scheduled PDCCH. Therefore, drx-onDurationTimer can be started with specific pattern which is configured by the network. Generally, it is configured with relative small value in case there is no transmission to be performed within the drx-onDurationTimer, i.e., the UE stops monitoring PDCCH when drx-onDurationTimer expires and no PDCCH is received to avoid extra power consumption. However, the timer triggered by LP-WUS is to wake up the UE to monitor PDCCH since the actual transmission will be started later on. Hence, the value of drx-onDurationTimer is not suitable for LP-WUS case.

Regarding drx-InactivityTimer, it may be configured with large values to facilitate the UE to monitor subsequent PDCCH. However, in order to guarantee the configuration flexibility, the value for drx-InactivityTimer and PDCCH monitoring triggered by LP-WUS may be configured separately. Thus, the value of drx-InactivityTimer is not suitable for PDCCH monitoring triggered by LP-WUS case either.

Observation 3: The values configured for drx-onDurationTimer may not be suitable for PDCCH monitoring triggered by LP-WUS considering drx-onDurationTimer is mainly used for potential PDCCH monitoring which can be set with relative small value.

Observation 4: In order to keep the flexibility, the value for drx-InactivityTimer and PDCCH monitoring triggered by LP-WUS should be configured separately.

2.3.1.3	Summary of the analysis of timers

Based on the Observation 1-4 in section 2.3.1.1-2.3.1.2, it is proposed that:

Proposal 7: Define a new timer for PDCCH monitoring triggered by LP-WUS, i.e. after LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts a new timer. When the new timer is running, the UE monitors PDCCH, i.e. the UE is in DRX Active Time.

Proposal 8: When the UE receives PDCCH schedules new transmission for DL/UL, the UE starts/restarts drx-InactivityTimer.

Proposal 9: When the UE receives (long) DRX Command MAC CE, the UE stops the new timer.

And the procedure of the new timer is illustrated in the Figure 1:

Figure 1: PDCCH monitoring in Option 1-2

- 2.3.2	drx-onDurationTimer

According to the current spec, the periodic CSI/L1-RSRP report is configured independently on DRX pattern.

And it has been agreed in RAN1 for Option 1-2 that:

- For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

Based on the agreement, the UE behavior can be clarified as following.

- Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up

For this branch, the UE is configured periodic CSI/L1-RSRP reporting but not enabled to perform periodic CSI/L1-RSRP reporting, since the periodic CSI/L1-RSRP is not reported if the UE is not indicated to wake-up. The periodic CSI/L1-RSRP only can be reported when the UE is indicated to wake-up. It brings benefits on power saving. On the other hand, the channel status may not be updated promptly especially when the UE keeps monitoring LP-WUS.

- Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

For this branch, the UE will report CSI/L1-RSRP periodically not matter whether it is indicated to wake-up or not. Even though there is no traffic, the UE has to wake up periodically to report Periodic CSI/L1-RSRP according to the periodic CSI/L1-RSRP report configuration. It is not friendly to power saving. But it guarantees that the network is awareness of the updated channel status.

Based on the agreements from RAN1, the UE can report periodic CSI/L1-RSRP based on RRC configuration.

Then, it can be concluded that whether periodic CSI/L1-RSRP can be reported is independent of the drx-onDurationTimer. Considering it has been agreed for Option 1-2 that UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS, we think the simplest way to handle legacy drx-onDurationTimer is that the UE does not start drx-onDurationTimer when the UE is configured with option 1-2, which is clear and simple. Therefore it is proposed that:

Proposal 10: The drx-onDurationTimer is not started in Option 1-2.

Conclusion

In this document, we analyse procedure and activation/deactivation of LP-WUS in RRC_CONNECTED state, and we have the following observations and proposls:

Common issues for Option 1-1 and Option 1-2:

Proposal 1: Option 1-1 can be supported for PDCCH monitoring triggered by LP-WUS in RRC CONNECTED.

Proposal 2: Option 1-2 can be supported for PDCCH monitoring triggered by LP-WUS in RRC CONNECTED.

Proposal 3: It is NOT supported to configure both Option 1-1 and Option 1-2 simultaneously for the same UE.

Option 1-1:

Proposal 4: In option 1-1, LP-WUS should not be configured with dual DRX groups simultaneously.

Proposal 5: Short DRX cycle is not applicable for LP-WUS.

Proposal 6: Define a flag to indicate whether the UE to wake-up to monitor PDCCH if miss-detection happens for LP-WUS in Option 1-1.

Option 1-2:

Observation 1: DRX retransmission timers including drx-RetransmissionTimerDL and drx-RetransmissionTimerUL are not suitable for the timer triggered by LP-WUS to monitor PDCCH considering they are used to monitor PDCCH for retransmission and per HARQ process maintained.

Observation 2: The new timer defined for PDCCH monitoring triggered by LP-WUS has different procedure as legacy DRX timers, i.e., drx-onDurationTimer and drx-InactivityTimer as illustrated in Table 1.

Observation 3: The values configured for drx-onDurationTimer may not be suitable for PDCCH monitoring triggered by LP-WUS considering drx-onDurationTimer is mainly used for potential PDCCH monitoring which can be set with relative small value.

Observation 4: In order to keep the flexibility, the value for drx-InactivityTimer and PDCCH monitoring triggered by LP-WUS should be configured separately.

Proposal 7: Define a new timer for PDCCH monitoring triggered by LP-WUS, i.e. after LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts a new timer. When the new timer is running, the UE monitors PDCCH, i.e. the UE is in DRX Active Time.

Proposal 8: When the UE receives PDCCH schedules new transmission for DL/UL, the UE starts/restarts drx-InactivityTimer.

Proposal 9: When the UE receives (long) DRX Command MAC CE, the UE stops the new timer.

Proposal 10: The drx-onDurationTimer is not started in Option 1-2.

Reference

- R1-2407587, RAN1#118 chairman notes;

- R2-2407902, RAN2#127 chairman notes;

- R1-1911475, LS on PDCCH-based Power Saving Signal/Channel carrying indication of UE wakeup before DRX ON, CATT;

- TS 38.331, V18.2.0, Radio Resource Control (RRC) protocol specification, Release 18.