3GPP TSG-RAN WG2 Meeting #127bis	R2-2408188

Hefei, China, October 14th – 18th, 2024

Agenda Item:   8.2.5

Source:        Xiaomi

Title:           Considerations on TP2 related aspects between BS and UE reader

Document for:  Discussion and Decision

- Introduction

During the last meeting, RAN2 achieved the following agreements about the TP2 related aspects between BS and UE reader.[1]

Agreements

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

- A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e., topology is transparent to the A-IoT device and there is no impact on A-IoT device.

- RAN2 assumes that intermediate UE authorization is performed by upper layers.  RAN2 impact can be studied after further SA/RAN3 progress.

- RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e., no new AS layer architecture options will be studied)

- NW is in control of resources to be used by AIoT air interface.

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

In this contribution, we provide our further considerations on the TP2 related aspects between BS and UE reader.

- Discussion

  - Resource allocation for the AIoT air interface

In last meeting, for the resource allocation for the AIoT air interface, we agreed that the radio resource used by the AIoT air interface should be in control of the network, while there is no consensus on the details of the resource allocation for R2D and D2R, and the proposed options are quoted below. [2]

- Option1: resource allocation via the gNB directly.

- Option2: resource allocation via the intermediate node based on the resource pool allocated by the gNB.

For option1, we think it is difficult for the gNB to perform the resource allocation directly. In more details, if the resource allocation is done in a dynamic way, the UE reader needs to indicate the AIoT operation related information before sending each R2D command over the A-IoT interface This will result in significant signaling overhead on the Uu interface and introduce additional latency due to the resource allocation process. If the resource allocation is done in a semi-static way, e.g., similar to CG in the NR , as configuration only supports periodic resource allocation with fixed time/frequency domain resources, it is difficult to efficiently allocate resources to accommodate the variable data transmission sizes in each AIoT operation for the UE reader, and it also limits the UE reader's behaviour to the available periodic resources.

Compared to option 1, we prefer to adopt option 2, as it reduces the resource allocation signaling between the network and the UE reader, and also provides more flexibility for the UE reader. In more detail, once a UE is selected as a reader by the network, the network can provide one or more resource pools to the UE reader. Then, when the UE reader performs R2D transmission, it can select resources from the configured resource pool for the R2D transmission, and also for the D2R transmission if any. Regarding the configured resource pool, it can be shared among multiple UEs or dedicated to a single UE. If the resource pool is shared among multiple UEs, whether or how to avoid the resource collision among these UEs needs to be further discussed.

Observation 1: Dynamic resource allocation via the gNB directly will introduce significant signalling overhead and latency.

Observation 2: Configured grant-based resource allocation via the gNB directly only supports periodic and fixed time/frequency domain resources, which is inefficient for the variable sizes of AIoT transmissions and may restrict the behaviour of UE reader.

Proposal 1: Resource allocation for the AIoT air interface is performed by the UE reader based on the resource pool allocated by the gNB.

  - Device data aggregated on the UE reader side

During the previous discussion, RAN2 agreed to support paging multiple devices for response in one A-IoT procedure. For the procedure involving multiple devices, upon receiving the response from the device, there are two options for the reader to forward it to the network:

- Option 1: Forward the device response one by one immediately upon receiving it. Take inventory as an example, as shown below:

Fig.1 Non-aggregated data forwarding on reader side

- Option 2: Aggregate multiple responses and forward them together in one message. Take inventory as an example, as shown below:

Fig.2 Aggregated data forwarding on reader side

For the options above, we can note that option1 could result in a signalling storm between the reader and the A-IoT CN node, while option2 would have a significantly smaller signalling overhead at the expense of response latency.

RAN3 discussed this issue for TP1 last meeting and achieved the following agreement. [3]

In topology1, it is FFS whether in response to a single Inventory Request, AIoT RAS provides only a single Inventory Report message or there can be also multiple Inventory Report messages provided.

As the options above have impacts on different interfaces in TP1 and TP2, we should discuss in RAN3 and RAN2 separately. Also, for the signaling storm introduced in Option 1, it has much more serious impact in the TP2 compared to TP1, as usually we care more about the radio resource consumption in the Uu interface.

From this view, Option 2 is a more suitable and efficient choice for TP2. Furtherly, considering the increased response latency, Option 2 can be applied to the A-IoT procedure, which is not sensitive to latency, such as the inventory use case. As for the command use case, we can analyze it further or leave it to the UE implementation.

Observation 3: UE forwarding the D2R data transmissions one by one may result in the signalling storm in the Uu interface.

Proposal 2: In TP2, RAN2 to discuss whether the UE reader can aggregate the received D2R data transmissions over Uu interface.

- Option1: UE reader forwarding the received D2R data transmission one by one.

- Option2: UE reader aggregating the received D2R data transmission over Uu interface and forwarding to BS together, at least for the inventory use case.

  - RRC state of the UE reader

In TP2, another issue is whether a UE needs to remain in the RRC_CONNECTED during the whole AIoT procedure once it is authorized as the reader.

To ensure real-time communication with the network, the UE can serve as a reader in the RRC_CONNECTED during the AIoT procedure as a baseline. However, since maintaining the RRC_CONNECTED is not power efficient, a UE that is authorized as the reader and has received the service request can also be transitioned to the RRC_INACTIVE to perform the AIoT operation. In this case, data transmission can be carried out via the SDT procedure while in the RRC_INACTIVE.

Regarding the RRC_IDLE, if device data can be aggregated on the UE side, such as during inventory procedures, this option can also be considered. However, since CN will release the context of the AIoT service for the UE reader in the RRC_IDLE, this could lead to the CN not recognizing the UE reader when it reports later. As such, whether/how to support the UE reader in the RRC_IDLE during the AIoT procedure needs to be further discussed.

Proposal 3: RAN2 assumes the UE reader can at least be in RRC_CONNECTED or RRC_INACTIVE during AIoT procedure, which is decided by the gNB. FFS if RRC_IDLE is supported.

  - AIoT procedure handling in case UE reader out of connection

In last meeting, we discussed whether the AIoT scenarios the UE reader is in/out of connection or in/out of coverage can be considered in Rel-19, and achieved the following agreements:

Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

For the case of UE reader being “temporarily” out of coverage, based on the discussion in the RAN2 WG and the RAN plenary, we think it is the common understanding that this scenario is out of scope and not considered in Rel-19. As for the case of UE reader being “temporarily” out of connection but still in the coverage, we can further discuss it.

During an on-going AIoT procedure, if the UE reader is temporarily disconnected on the Uu interface, the handling of the on the AIoT air interface needs to be clarified, including the management of AIoT configurations and data during recovery. There are three options which can be further discussed in RAN2 as follows:

- Option1: Continue the on-going AIoT procedure.

In detail, if the UE reader is out of connection, it stores the AIoT-related configuration and any untransmitted data, and continues to perform the on-going AIoT procedure based on the stored AIoT related configuration at least within the cell where UE receive the AIoT related configuration.

Regarding the AIoT-related configuration used by the UE reader during recovery, as a baseline, it is the configuration that was used while the UE reader was connected. Alternatively, it could also be a separate AIoT configuration used only in the case of out of connection, this option can be finally decided by the network vendor and we have no strong view on this.

- Option2: Suspend the on-going AIoT procedure.

In detail, if the UE reader is out of connection, it stores the AIoT-related configuration and any untransmitted data, but suspends the on-going AIoT procedure. If the UE reader successfully recovers within the same cell or service area (if any), it can resume the AIoT procedure and transmit the collected device data to the network. Or, the UE reader can follow the network control to resume, re-initiate or stop the AIoT procedure.

- Option3: Stop the on-going AIoT procedure.

In detail, if the UE reader is out of connection, it releases the AIoT related configuration and any untransmitted data, and stops the on-going AIoT procedure. If the UE reader successfully recovers, it can follow the network control to re-initiate the AIoT procedure.

Proposal 4: RAN2 confirms not to consider the case of UE reader “temporarily” out of coverage in Rel-19.

Proposal 5: If the UE reader is out of connection within the coverage area, the following options can be considered for handling the on-going AIoT procedure:

- Option1: Store the AIoT related configuration and/or data, and continue the on-going AIoT procedure at least within the cell proving the configuration.

- Option2: Store the AIoT related configuration and/or data, and suspend the on-going AIoT procedure.

- Option3: Release the AIoT related configuration and/or data, and stop the on-going AIoT procedure.

  - Service continuity during the UE reader mobility

According to the SID, the AIoT device does not support mobility. However, in TP2, the UE serving as the intermediate node can be a normal UE with the AIoT functionalities so that it needs to support the legacy mobility as in the NR system, including the cell selection/reselection and the handover.

However, during the mobility of the UE reader, it may affect the AIoT procedure on the AIoT air interface. For example, if the UE moves to the edge of the BS coverage or to a BS that does not support the AIoT procedure, the AIoT procedure may be interrupted, which could cause data loss.

As such, it is necessary to consider how to ensure service continuity and reduce data loss during the mobility of the UE reader. For instance, the UE should prioritize moving to a cell that supports at least the ongoing AIoT procedure during cell selection, reselection, or handover procedures. Moreover, if the UE is moving to the edge of a BS's coverage area or to a BS that does not support the AIoT procedure, the UE reader can transmit the collected device data to the network as soon as possible to minimize data loss.

Observation 4: The UE reader can be a normal UE with AIoT functionalities and needs to support the mobility including the cell selection/reselection and handover.

Observation 5: Service interruption or data loss may occur if the UE reader moves to the edge of a BS or to a BS that does not support A-IoT functionalities.

Proposal 6: RAN2 considers to support the Ambient IoT service continuity during the UE reader mobility.

  - UE BSR for the device data forwarding

In the existing mechanism, there is pre-emptive Buffer Status Report (BSR) mechanism which is introduced for the IAB-node to reduce the UL scheduling latency through signalling of a Pre-emptive BSR to its parent node.

Similarly, in the A-IoT system, the UE reader could also consider a BSR mechanism similar to the pre-emptive BSR. In detail, the UE reader may be able to predict the volume of data it will receive from devices. This prediction could be based on factors such as the estimated number of device responses during an inventory procedure or the expected size of the data to be read from the device. With this predicted data volume, the UE reader could initiate a pre-emptive BSR to reduce data transmission delays. This is particularly beneficial for delay-sensitive processes, such as command use case.

Observation 6: UE reader can predict the data volume from the devices based on the estimated number of device and/or the expected data size per device, which can be used to initiate the pre-emptive BSR for the latency reduction.

Proposal 7: The pre-emptive BSR can be supported by the UE reader for the data transmission latency reduction.

  - Impacts on the UE power control

In the existing NR system, UE power control only considers the power consumption on the Uu signalling interface. However, in the Ambient IoT system, if a UE serves as an intermediate node or a CW node, there is additional power consumption on the R2D and/or CW transmissions, in addition to the existing NR Uu signaling transmission. Therefore, to achieve accurate power control for a UE acting as an intermediate node or CW node in the Ambient IoT system, the network should also account for this additional power consumption during R2D and/or CW transmissions. Otherwise, it may lead to inaccurate power control for the UE reader.

To achieve accurate power control on the UE serving as intermediate node or CW node in the Ambient IoT system, following options can be considered:

- Option 1: The UE determines the power consumption on R2D and/or CW transmissions and indicates it to the network for power control, for example, by using a new PHR type.

- Option 2: The network performs a hard split of the power consumption on the R2D and/or CW transmissions, and NR Uu interface transmissions, and indicates this split to the UE.

- Option 3: The calculation of the PCmax takes into account the power consumption on the R2D and/or CW transmissions, which can be further discussed and determined in RAN4.

Observation 7: In NR system, only the UE power consumption on the Uu signaling transmission is considered for UE power control.

Proposal 8: In the AIoT system, if the UE serves as an intermediate node or CW node, the power consumption on the R2D and/or CW transmissions should also be considered for the UE power control.

  - Active time control of PDRCH reception

In the current spec, if the UE is configured with DRX, it does not need to continuously monitor PDCCH for power saving. For a UE serving as an intermediate node in the A-IoT system, it also needs to monitor the PDRCH in the A-IoT interface as well as the PDCCH, and the impacts of the UE DRX on the PDRCH need to be considered.

If there are separate reception module for the PDCCH and PDRCH, the control of the PDCCH and PDRCH monitoring is separate, there is no need for the coordination of the both. However, the additional reception module for PDRCH monitoring on the UE side will increase costs to some extent.

If there is a shared reception module for PDCCH and PDRCH, the DRX configured for the UE applies to both PDCCH and PDRCH monitoring. During the inactivity period for PDCCH monitoring, the UE will also not monitor the PDRCH. To avoid missing PDRCH, the UE reader should remain active for PDRCH reception during A-IoT operation, and the impacts on the UE DRX need to be considered, such as not to configure UE reader with the DRX to always enable PDRCH monitoring, or to configure the UE DRX on-duration time with PDRCH reception taken into account, etc.

Observation 8: In case the reception module for PDCCH and PDRCH is shared, the active time of the UE DRX will also impact on the PDRCH reception.

Proposal 9: If the PDCCH and PDRCH share the same reception module on the UE reader side, the UE DRX configuration should take into account the PDRCH monitoring time to avoid missing the PDRCH.

  - Co-existence of the A-IoT function and CA

Carrier aggregation (CA) is used to increase data rates and improve network capacity by utilizing multiple frequency bands simultaneously. However, due to the limited capabilities of A-IoT devices, such as the small size of data packets they handle, there is no need for UE reader to support CA, as it would only add unnecessary complexity to both the UE and the network. Therefore, we propose that a UE serving as an intermediate node in the A-IoT system should not be required to operate in CA mode. Even if the UE can be configured with CA, to reduce the complexity and the impact on the spec, we propose that only the radio resource of the PCell can be configured to UE for the A-IoT operation towards the device.

Proposal 10: For the co-existence of the A-IoT function and CA, the following options can be considered.

- Option1: Not to consider the UE reader operating with CA.

- Option2: For a UE reader operating with CA, only the radio resource of the PCell can be configured for the A-IoT operation in the A-IoT interface.

- Conclusions

According to the analysis given above, we have the following observations and proposals:

Resource allocation for the AIoT air interface

Observation 1: Dynamic resource allocation via the gNB directly will introduce significant signalling overhead and latency.

Observation 2: Configured grant-based resource allocation via the gNB directly only supports periodic and fixed time/frequency domain resources, which is inefficient for the variable sizes of AIoT transmissions and may restrict the behaviour of UE reader.

Proposal 1: Resource allocation for the AIoT air interface is performed by the UE reader based on the resource pool allocated by the gNB.

Device data aggregated on the UE reader side

Observation 3: UE forwarding the D2R data transmissions one by one may result in the signalling storm in the Uu interface.

Proposal 2: In TP2, RAN2 to discuss whether the UE reader can aggregate the received D2R data transmissions over Uu interface.

- Option1: UE reader forwarding the received D2R data transmission one by one.

- Option2: UE reader aggregating the received D2R data transmission over Uu interface and forwarding to BS together, at least for the inventory use case.

RRC state of the UE reader

Proposal 3: RAN2 assumes the UE reader can at least be in RRC_CONNECTED or RRC_INACTIVE during AIoT procedure, which is decided by the gNB. FFS if RRC_IDLE is supported.

AIoT procedure handling in case UE reader out of connection

Proposal 4: RAN2 confirms not to consider the case of UE reader “temporarily” out of coverage in Rel-19.

Proposal 5: If the UE reader is out of connection within the coverage area, the following options can be considered for handling the on-going AIoT procedure:

- Option1: Store the AIoT related configuration and/or data, and continue the on-going AIoT procedure at least within the cell proving the configuration.

- Option2: Store the AIoT related configuration and/or data, and suspend the on-going AIoT procedure.

- Option3: Release the AIoT related configuration and/or data, and stop the on-going AIoT procedure.

Service continuity during the UE reader mobility

Observation 4: The UE reader can be a normal UE with AIoT functionalities and needs to support the mobility including the cell selection/reselection and handover.

Observation 5: Service interruption or data loss may occur if the UE reader moves to the edge of a BS or to a BS that does not support A-IoT functionalities.

Proposal 6: RAN2 considers to support the Ambient IoT service continuity during the UE reader mobility.

UE BSR for the device data forwarding

Observation 6: UE reader can predict the data volume from the devices based on the estimated number of device and/or the expected data size per device, which can be used to initiate the pre-emptive BSR for the latency reduction.

Proposal 7: The pre-emptive BSR can be supported by the UE reader for the data transmission latency reduction.

Impacts on the UE power control

Observation 7: In NR system, only the UE power consumption on the Uu signaling transmission is considered for UE power control.

Proposal 8: In the AIoT system, if the UE serves as an intermediate node or CW node, the power consumption on the R2D and/or CW transmissions should also be considered for the UE power control.

Active time control of PDRCH reception

Observation 8: In case the reception module for PDCCH and PDRCH is shared, the active time of the UE DRX will also impact on the PDRCH reception.

Proposal 9: If the PDCCH and PDRCH share the same reception module on the UE reader side, the UE DRX configuration should take into account the PDRCH monitoring time to avoid missing the PDRCH.

Co-existence of the AIOT function and CA

Proposal 10: For the co-existence of the A-IoT function and CA, the following options can be considered.

- Option1: Not to consider the UE reader operating with CA.

- Option2: For a UE reader operating with CA, only the radio resource of the PCell can be configured for the A-IoT operation in the A-IoT interface.

- References

- R2_127_ChairNotes_24-08-23_13-30

- R2-2406610 Discussion on A-IoT Topology 2, Samsung, Rel-19

- RAN3 #124 Meeting Report Fukuoka, Japan, 20/05/2024 - 24/05/2024