3GPP TSG-RAN WG2 Meeting #127bis	R2-2408193

Hefei, China, Oct 14th – 18th, 2024

Source: 	Xiaomi

Title:  	Discussion on inter-CU LTM

Agenda Item:	8.6.2	Inter-CU LTM

Document for:	Discussion and Decision

Introduction

A Revised WID on NR mobility enhancements Phase 4 [1] has been approved. In the WID, one of the objectives is to specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM), as shown below.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In this contribution, we provide our understandings on inter-CU LTM.

Discussion

  - Inter-CU LTM when DC is not configured

Early sync phase

RAN2 has discussed the early sync phase in previous meetings, and early TA acquisition has been supported for inter-CU LTM. Whether to support UE based TA measurement for inter-CU cases has been discussed. To support UE based TA measurement may need to meet some synchronization requirement. Although, RAN4 already defined UE requirement for UE based TA measurement for intra-CU LTM, whether the same requirements are applied for Rel-19 inter-CU LTM needs to be confirmed by RAN4. And from RAN2 perspective, supporting UE based TA measurement is beneficial for inter-CU LTM, which can reduce the cell switch interruption when the UE doesn’t perform early TA acquisition. Hence, from RAN2 perspective, UE-based TA measurement for inter-CU LTM could be considered, but it should be finally decided by RAN4.

Proposal 1: From RAN2 perspective, supporting UE-based TA measurement is beneficial for inter-CU LTM, but it should be finally decided by RAN4.

Inter-CU LTM for failure recovery
The Rel-18 intra-CU LTM supports RLF and LTM cell switch failure recovery and when UE selects a suitable cell and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after RLF or LTM cell switch failure, then the UE attempts RACH-based LTM execution once, otherwise re-establishment is performed.

For Rel-18 intra-CU LTM, security keys are maintained upon an LTM cell switch. However, the key update is needed upon the inter-CU LTM cell switch. How to update the security key upon the execution of MAC CE triggered inter-CU LTM is still one open issue. For inter-CU LTM based failure recovery, it may also be necessary to update the security key.

RAN2 supports the configuration with a mixture of intra-CU and inter-CU candidate LTM cells. In this case, at least intra-CU LTM based recovery can be considered. RAN2 could postpone the discussion for inter-CU LTM based failure recovery and wait for the progress of SA3 on the security issue.

Proposal 2: For the LTM configuration with a mixture of intra-CU and inter-CU candidate LTM cells, at least intra-CU LTM based failure recovery can be considered.

Proposal 3: Whether to support the inter-CU LTM based failure recovery can be discussed after receiving SA3’s response on the security solution of inter-CU LTM.

  - Inter-CU LTM when DC is configured

In the WID, two secondary priority cases need to be discussed for the Inter-CU LTM when DC is configured, as show below:

- When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

In the RAN2#127 meeting, RAN2 has discussed above two secondary cases and reached the following agreements:

Inter-CU SCG LTM: 10. Inter-CU SCG LTM preparation can be initiated by source SN. 11. The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. 12. For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. 13. RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. 14. Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19. Inter-CU MCG LTM: 15. SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). 16. Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied). 17. For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

Inter-CU LTM where CU is acting as SN and MN is unchanged

The signalling flows of inter-CU SCG LTM without MN change

According to the above agreements, only SN-initiated inter-SN LTM is supported in Rel-19. For SN-initiated inter-SN LTM, more than one candidate SNs and MN will be involved in the mobility flow. As described in the WID “Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM”, subsequent inter-SN LTM shall be supported.

In Rel-18 subsequent CPAC, SN initiated inter-SN subsequent CPAC has been specified in TS 37.340, and the signalling flow for the SN initiated inter-SN subsequent CPAC can be taken as baseline. The difference between SCPAC and inter-CU SCG LTM is in how to trigger mobility, so the network interaction for inter-CU SCG LTM can follow the SN initiated inter-SN subsequent CPAC and the signalling flows between UE and SN-CU can follow Rel-18 intra-CU LTM. Hence, we provide the signalling procedure for Rel-19 inter-CU SCG LTM illustrated in Figure 1 and step 11 to 15 are the difference from SCPAC.

Proposal 4: For Rel-19 inter-CU SCG LTM, the network interaction for inter-CU SCG LTM can follow the SN initiated inter-SN subsequent CPAC and the signalling procedure for inter-CU SCG LTM illustrated in Figure 1 can be considered.

Figure 1. Signalling procedure for inter-CU SCG LTM

the mixture of intra-SN and inter-SN SCG LTM

For inter-CU LTM in non-DC case, RAN2 has agreed “Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch and UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells”. For inter-CU SCG LTM, the mixture of intra-CU and inter-CU SCG LTM can also be supported.

Proposal 5: The UE can be configured with a mixture of intra-CU and inter-CU SCG LTM.

How to configure the UE with a mixture of intra-CU and inter-CU SCG LTM shall be discussed. According to the current agreements and specifications, the Rel-18 intra-CU SCG LTM is configured to UE in SN format, but the inter-CU SCG LTM configuration is included in the MN RRC configuration message, i.e., provided in MN format. Since the subsequent SCG LTM is supported, if UE is configured with a mixture of intra-CU and inter-CU SCG LTM, when the inter-CU SCG LTM cell switch is executed firstly, the intra-CU SCG LTM needs to be taken as an inter-CU SCG LTM for the subsequent SCG LTM. Hence, a simple way to allow the change from intra-CU to inter-CU is to use a unified format to configure both intra-CU and inter-CU SCG LTM configuration for the mixture case.

SCPAC faces a similar issue, and in SCPAC, for one UE, only either MN format or SN format (only intra-SN case is possible) is used, and MN format is supported for intra-SN case (in addition to SN format). It seems feasible to use the same method for the mixture of intra-CU and inter-CU SCG LTM.

Proposal 6: MN format is supported for intra-CU SCG LTM (in addition to SN format). And the mixture configuration of intra-CU and inter-CU SCG LTM can only be provided to UE in MN format.

For the mixture configuration of intra-CU and inter-CU SCG LTM, we need to discuss the L2 handling for SCG LTM. If P6 is agreed, the similar structure will be used to configure SCG LTM and SCPAC.

For the mixture of intra-SN and inter-SN SCPAC configured in MN format, if a new sk-Counter value has been selected for subsequent CPAC execution, the UE is required to perform re-establishment of RLC and PDCP at SN side. Otherwise, i.e., if no new sk-counter has been selected and the SN key update is not needed, PDCP re-establishment in SN side is not needed. The similar L2 handling procedure for SCPAC can be used for SCG LTM.

Proposal 7: For the mixture configuration of intra-CU and inter-CU SCG LTM, SCPAC L2 handling procedure is taken as a baseline.

Inter-CU LTM where CU is acting as MN and SN is unchanged or SN is released

For the inter-CU MCG LTM in DC case, there are two sub-cases:

- Sub-Case 1: Inter-CU LTM where CU is acting as MN and SN is unchanged

- Sub-Case 2: Inter-CU LTM where CU is acting as MN and SN is released

And RAN2 has agreed that SCG configuration can be changed in inter-CU MCG LTM and leave how to handle SCG part up to NW implementation (e.g., release or reconfiguration). Both sub-case 1 and 2 have been captured in the WID. RAN2 can discuss whether the sub-case 1 and sub-case 2 can be configured to UE at the same time. For inter-CU MCG LTM with DC, when some candidate configurations with the SCG configuration but other candidate configurations with SN release are configured to UE simultaneously, if UE performed the MCG LTM cell switch with SCG release, how to perform the subsequent LTM for the candidate configuration with SCG configuration need to be discussed.

Proposal 8: RAN2 to discuss whether both inter-CU MCG LTM with SN un-change and inter-CU MCG LTM with SN release can be configured to UE simultaneously.

The coexistence of intra-MN/inter-MN MCG LTM and inter-SN/intra-SN SCG LTM

In Rel-18 and Rel-19 LTM, MCG LTM and SCG LTM in DC has been considered, as follows:

- MCG LTM including both intra-MN MCG LTM and inter-MN MCG LTM

- SCG LTM including both intra-SN SCG LTM and inter-SN SCG LTM

For above cases in DC, the coexistence between MCG LTM and SCG LTM should be discussed, including the following combination cases

- Case 1: The coexistence of intra-MN MCG LTM and intra-SN SCG LTM

- Case 2: The coexistence of intra-MN MCG LTM and inter-SN SCG LTM

- Case 3: The coexistence of inter-MN MCG LTM and intra-SN SCG LTM

- Case 4: The coexistence of inter-MN MCG LTM and inter-SN SCG LTM

Case 1 has been supported in Rel-18 and the intra-SN SCG LTM is configured in SN format. The WID clarifies that inter-CU LTM can be configured either in MN or in SN but not both at the same time, hence, Case 4 is not included in the scope of the Rel-19 WID. For a flexible configuration, Case 2 and Case 3 can be considered.

Observation 1: The coexistence of intra-MN MCG LTM and intra-SN SCG LTM has been supported in Rel-18.

Observation 2: The coexistence of inter-MN MCG LTM and inter-SN SCG LTM is not included in the scope of the WID.

Proposal 9: The coexistence of intra-MN MCG LTM and inter-SN SCG LTM can be supported

Proposal 10: The coexistence of inter-MN MCG LTM and intra-SN SCG LTM can be supported.

Conclusions

According to the analysis given above, we have the following observations and proposals:

2.1	Inter-CU LTM when DC is not configured

Proposal 1: From RAN2 perspective, supporting UE-based TA measurement is beneficial for inter-CU LTM, but it should be finally decided by RAN4.

Proposal 2: For the LTM configuration with a mixture of intra-CU and inter-CU candidate LTM cells, at least intra-CU LTM based failure recovery can be considered.

Proposal 3: Whether to support the inter-CU LTM based failure recovery can be discussed after receiving SA3’s response on the security solution of inter-CU LTM.

2.2	Inter-CU LTM when DC is configured

Proposal 4: For Rel-19 inter-CU SCG LTM, the network interaction for inter-CU SCG LTM can follow the SN initiated inter-SN subsequent CPAC and the signalling procedure for inter-CU SCG LTM illustrated in Figure 1 can be considered.

Proposal 5: The UE can be configured with a mixture of intra-CU and inter-CU SCG LTM.

Proposal 6: MN format is supported for intra-CU SCG LTM (in addition to SN format). And the mixture configuration of intra-CU and inter-CU SCG LTM can only be provided to UE in MN format.

Proposal 7: For the mixture configuration of intra-CU and inter-CU SCG LTM, SCPAC L2 handling procedure is taken as a baseline.

Proposal 8: RAN2 to discuss whether both inter-CU MCG LTM with SN un-change and inter-CU MCG LTM with SN release can be configured to UE simultaneously.

Observation 1: The coexistence of intra-MN MCG LTM and intra-SN SCG LTM has been supported in Rel-18.

Observation 2: The coexistence of inter-MN MCG LTM and inter-SN SCG LTM is not included in the scope of the WID.

Proposal 9: The coexistence of intra-MN MCG LTM and inter-SN SCG LTM can be supported

Proposal 10: The coexistence of inter-MN MCG LTM and intra-SN SCG LTM can be supported.

Reference

[1] RP-242356, “Revised WID: NR mobility enhancements Phase 4”, Apple Inc, China Telecom