3GPP TSG RAN WG2 #127bis                                                                            R2-2408198

Hefei, China, 14th – 18th Oct, 2024

Source:	CATT

Title:	Consideration on leftovers from Rel-18 SONMDT

Agenda Item:	8.10.5

Document for:	Discussion and Decision

Introduction

In last RAN2#127 meeting [1], for R18 leftover issues, RAN2 has achieved the agreements as below:

For SCG act/deact info: It is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state. Send an LS to RAN3 to ask if a NW solution is good enough to achieve the agreement above, or if a UE based solution is needed. [Post127][603][SONMDT] SCG act/deact info (CMCC) Scope: - Produce approvable LS Intended outcome: - Approved LS in R2-2407664 (CMCC) Deadline: - Short For MR-DC SCG failure: To support MRO for SCG failure in EN-DC, enhance SCGFailureInformationNR message to include previousPSCellId, failedPSCellId, timeSCGFailure. For SDT: Do not add logging of sdt-RSRP-Threshold, since already agreed by RAN3 to not support it. UE logs and reports the failure cause for SDT to the network. FFS the details, e.g. if we down select some of the failure causes.

In this contribution, we will continue to discuss the R18 leftover issues and provide our views.

Discussion

MHI for SCG Deactivation/Activation

The Mobility History Information (MHI) for PCell has been supported in Rel-16. In Rel-17, the enhancement for Dual Connectivity is introduced, where the mobility history information of most recently visited PSCell and time spent in the PSCell across all the primary cells is included in the VisitedCellInfoList, the list includes PSCells visited by UE in RRC connected states for NR or E-UTRA.

MHI for PSCell was introduced in R17 SON/MDT. Meanwhile, the SCG activation/deactivation was introduced in R17 MR DC/CA WI [2], in which the SCG could be configured with activation/deactivation state to enable reasonable UE battery consumption while having fast usage of SCG. According to below MHI procedure to log PSCell information in TS 38.331, it can be seen that MHI information will be logged for PSCell upon addition, change, or release of a PSCell etc., more details could be seen in the spec.

TS 38.331 5.7.9.2 Initiation If the UE supports storage of mobility history information, the UE shall: 1> If the UE supports PSCell mobility history information and upon addition of a PSCell: <skip> 2> for the included entry: 3> set the field timeSpent of the entry according to following: 4> if this is the first PSCell entry for the current PCell since entering the current PCell in RRC_CONNECTED: 5> include the entry as the time spent with no PSCell since entering the current PCell in RRC_CONNECTED; 4> else: 5> include the time spent with no PSCell since last PSCell release since entering the current PCell in RRC_CONNECTED; 1> If the UE supports PSCell mobility history information and upon change, or release of a PSCell while being connected to the current PCell: <skip> 2> for the included entry: 3> if the global cell identity of the previous PSCell is available: 4> include the global cell identity of that cell in the field visitedCellId of the entry; 3> else: 4> include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry; 3> set the field timeSpent of the entry as the time spent in the previous PSCell while being connected to the current PCell;

For above legacy procedure, RAN2 needs to clarify SCG could be in activated or deactivated status when MHI is performed upon addition, change of PSCell. In our view, it is natural to think that UE will log MHI for PSCell upon addition, change of a PSCell no matter SCG is configured as deactivated or not.

Observation 1: In current spec, UE will log MHI for PSCell upon addition, or change of a PSCell no matter SCG is configured as activated or deactivated.

Based on the current spec, when SCG is activated, UE needs to resume radio link monitoring on the SCG and indicate the lower layers to resume beam failure detection on the PSCell. When SCG is activated, UE will stay under the coverage of SCG and it is correct that UE logs this PSCell in MHI. However, when SCG is deactivated, UE may stop radio link monitoring on the SCG and stop the beam failure detection on the PSCell. Then it is possible that UE may move out of the coverage of current PSCell during SCG deactivation. But UE will still log this PSCell and the spent time into the MHI.

For example, Network configures UE to activate PSCell1 for data transmission at T1 and deactivate the SCG1 at T2 because of traffic is finished. If the data is arriving again at T3, the network could activate the SCG. However, since UE is moving out of SgNB1 and in the coverage of SgNB2, the network will configure the change of PSCell and the SCG2 of SgNB2 with activation.

Figure 1 UE moving and SCG configuration in DC scenario

In above procedure, the time duration between T2 and T3 will be associated to PSCell1 by legacy MHI procedure. If the network receives this MHI information, network may believe that UE is still under the coverage of PSCell1 and available for data transmission, but the UE is out of the PSCell1 coverage and into PSCell2 coverage in fact.

The MHI is used for network to estimate the UE mobility state and then optimize parameters for handover. For example, for fast UEs, the network can choose a macro cell as a target cell which can reduce the signalling overhead for mobility and reduce the mobility failure. Besides, the network can set optimum parameters (e.g. timeToTrigger) for handover measurements. The misunderstanding of the PSCell UE staying and being able to transmit data by network will cause the incorrect handover optimization.

Observation 2: Based on the UE behavior in current spec, SCG deactivation option may result in biased MHI recording and incorrect handover optimization.

Proposal 1: When UE records the MHI of PSCell, UE should record the time duration of SCG deactivation as least to avoid the network estimating the UE’s mobility state incorrectly.

Furthermore, if UE move out of the coverage of PSCell1 during SCG deactivation, the TA can be expired or RLF/beam failure is detected during SCG deactivation, which means UL/DL is un-sync between UE and PSCell1. In this scenario, some extra assistant information, such as whether TAT is expired or whether RLF/beam failure is detected during SCG deactivation, even the RRM measurement results, may be needed to help network realize the real situation when PSCell1 is in deactivated status and avoid the incorrect handover optimization.

Proposal 2: When UE records the MHI of PSCell, if SCG is deactivated, UE can record some extra information to help network understand the real situation, such as whether TAT is expired or whether RLF/beam failure is detected or not.

SDT

Background

SDT is consisted of R17 MO-SDT and R18 MT-SDT. In SDT initiation procedure, the CG-SDT has a high priority than RA based SDT. For MO-SDT, when MO-SDT data is arriving at UE side, UE will initiate the MO-SDT procedure with RA partitioning resource if CG-SDT initiation is not selected or failed. For MT-SDT, if the paging with MT-SDT indication is received by UE, UE will initiate the MT-SDT procedure with the normal RA resource if CG-SDT initiation is not selected or failed. Besides, MT-SDT based RA procedure could have MT-SDT specific RSRP threshold, MO-SDT specific RSRP threshold will be applied if MT-SDT specific RSRP threshold is not configured for MT-SDT based RA procedure.

RA report for MO-SDT

According the RAN2#126 meeting agreement for SDT as below, RSRP/data volume related information at the time when the UE evaluates if it should perform SDT will be logged in RA report. However, it is not clear in which case to log RSRP value and buffered uplink data volume. Obviously, if the SDT based RA procedure is successful, RSRP value and buffered uplink data volume could be included in RA report. If SDT based RA procedure is failed, such as T319a, the RSRP and data volume is better to be reported to network to see if the parameters associated to these two values are configured reasonable.

- RAN2#126 meeting Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT.

Proposal 3: The RSRP and data volume at the time when the UE evaluates if it should perform SDT should be included in RA report in both successful and failed SDT based RA procedure.

According to legacy procedure, the parameters including DL RSRP and pending UL data volume size are also used by CG-SDT procedure, in which the UE specific UL grant is configured for initial CG-SDT data transmission. So, it is better to make network realize the thresholds for RA based SDT is common to CG-SDT or not(i.e, CG-SDT is not configured) for a history RA procedure, then the network will consider the impact to CG-SDT or not when adjusting the thresholds. For example, if the indication of CG-SDT not configured is included in RA report for a history RA procedure, but now the CG-SDT is configured by network, then the network will adjust the parameters more carefully to consider the impact to CG resource.

Proposal 4: Whether the UE has performed the CG-SDT selection prior to RA-SDT needs to be logged in RA report.

Similarly to SI request failure and beam recovery failure, SDT failure indication has been recorded in R18 RA report for RA-partitioning. In RAN2#127 meeting, RAN2 agreed that UE logs and reports the failure cause for SDT to the network. Based on the company’s view and the specification on SDT procedure, the SDT failure could happen in below cases:

- Case 1: upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing.

- Case 2: upon random access problem indication is received from MCG MAC while SDT procedure is ongoing.

- Case 3: upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing.

- Case 4: upon integrity check failure indication is received from lower layers while SDT procedure is ongoing.

- Case 5: upon T319a expires.

- Case 6: upon network send UE to RRC_IDLE in response to the resume procedure initiated for SDT.

- Case 7: upon network send UE to RRC_INACTIVE in response to the resume procedure initiated for SDT.

- Case 8: upon unsuccessfully completed upon cell re-selection

For above cases, case 5 has been agreed in RAN2#126 meeting, the case1 and 2 are aligned to the legacy failure report cause and could be supported. Case 3 is associated to CG procedure, not to RA procedure, but it will be also beneficial to help network adjust the Timer, it could be supported as well. Furthermore, it can be seen that most of above SDT failure case are not be differentiated as the CG-SDT or RA-SDT in legacy specification, it is suggested to introduce an indication to identify it is CG-SDT or RA-SDT, which could help network to know the failure is caused by CG-SDT or RA-SDT and determine which parameters needs to be adjusted. For case 4/6/7, it is not sure what the benefits could be if supported. For case 8, the UE will move to RRC_IDLE mode and SDT is failed because of poor cell channel quality in camping cell, the cause reporting for case 8 could be supported and aligned to the rule to indicate the channel status in legacy RLF report.

Proposal 5: The causes for following cases could be logged for failed SDT:

- upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing;

- upon random access problem indication is received from MCG MAC while SDT procedure is ongoing;

- upon T319a expires;

- upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing;

- upon unsuccessfully completed upon cell re-selection.

Proposal 6: It is suggested to introduce an indication to indicate the SDT failure is CG-SDT or RA-SDT, which could help network to know the failure is happened in CG-SDT or RA-SDT procedure and determine which parameters needs to be adjusted.

RA report for MT-SDT

In R18, RA report has been enhanced for RA partitioning which includes the RA procedure triggered by R17 MO-SDT or its combination with other features, some remaining issues, such as whether to optimize the threshold for channel quality or data volume size, are left in the scope of R19 SON/MDT. However, the MT-SDT was introduced from R18, its initiation to RA-SDT is different from the MO-SDT and will apply the RA configuration for common RA procedure rather than the RA resource of MT-SDT, hence, the RA enhancement for MO-SDT in R18 could not be reused for MT-SDT, the network could also not identify the RA report is for MT-SDT or legacy RA procedure. Thus, it is suggested to consider MT-SDT in RACH optimization for SDT. For MT-SDT, the legacy RA report for normal RA procedure (rather than RA partitioning) could be reused, but an additional RA purpose of MT-SDT should be included in the RA report, then the network could identify the RA report is for MT-SDT and adjust the RA resource for MT-SDT.

Based on above analysis, the basic enhancement of RA report for MT-SDT is just to introduce a new RA purpose for MT-SDT. So, it is suggested to consider the RACH optimization for MT-SDT.

Proposal 7: It is suggested to consider the RACH optimization for MT-SDT, and introduce a new RA purpose of MT-SDT in RA report.

MRO for MR-DC SCG failure

It is agreed in last meeting to support MRO for SCG failure in EN-DC, enhance SCGFailureInformationNR message to include previousPSCellId, failedPSCellId, timeSCGFailure, and RA-info is not supported.

And RAN3 agreed the reported parameters could also be extended to support in NGEN-DC scenario.

Basically, the IE of the fields used for SCG failure in NR-DC scenario could be directly copied into the SCGFailureInformationNR message for (NG)EN-DC scenario. So the possible IEs are listed in the table as below:

Table 1 Possible (NG)EN-DC SCG failure report content

| Parameter | IE/value & condition |
| --- | --- |
| failedPSCellId and previousPSCellId | Frequency and PCI |
| timeSCGFailure | 0-1023 |
| failureType | Reuse current failure types |

For failedPSCellId and previousPSCellId

The frequency and the PCI of the PSCell could be introduced for these two fields similar as NR-DC scenario. No CGI is needed since the SCGFailureInformationNR message will be sent to the network immediately after the SCG failure occurred and there will be no addressing error.

For timeSCGFailure

The value range 0-1023 defined in NR-DC scenario can be introduced.

For failureType

The legacy failure types can be reused.

Proposal 8: The parameters reported for SCG failure report in (NG)EN-DC scenario could be:

- For failedPSCellId and previousPSCellId: frequency and the PCI of the PSCell;

- For timeSCGFailure: value range 0-1023;

- For failureType: Reuse the legacy field.

Conclusion

According to the analysis in section 2, the observations and proposals are summarized as follows:

MHI for SCG Deactivation/Activation

Observation 1: In current spec, UE will log MHI for PSCell upon addition, or change of a PSCell no matter SCG is configured as activated or deactivated.

Observation 2: Based on the UE behavior in current spec, SCG deactivation option may result in biased MHI recording and incorrect handover optimization.

Proposal 1: When UE records the MHI of PSCell, UE should record the time duration of SCG deactivation as least to avoid the network estimating the UE’s mobility state incorrectly.

Proposal 2: When UE records the MHI of PSCell, if SCG is deactivated, UE can record some extra information to help network understand the real situation, such as whether TAT is expired or whether RLF/beam failure is detected or not.

RACH optimization for SDT

Proposal 3: The RSRP and data volume at the time when the UE evaluates if it should perform SDT should be included in RA report in both successful and failed SDT based RA procedure.

Proposal 4: Whether the UE has performed the CG-SDT selection prior to RA-SDT needs to be logged in RA report.

Proposal 5: The causes for following cases could be logged for failed SDT:

- upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing;

- upon random access problem indication is received from MCG MAC while SDT procedure is ongoing;

- upon T319a expires;

- upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing;

- upon unsuccessfully completed upon cell re-selection.

Proposal 6: It is suggested to introduce an indication to indicate the SDT failure is CG-SDT or RA-SDT, which could help network to know the failure is happened in CG-SDT or RA-SDT procedure and determine which parameters needs to be adjusted.

Proposal 7: It is suggested to consider the RACH optimization for MT-SDT, and introduce a new RA purpose of MT-SDT in RA report.

MRO for MR-DC SCG failure

Proposal 8: The parameters reported for SCG failure report in (NG)EN-DC scenario could be:

- For failedPSCellId and previousPSCellId: frequency and the PCI of the PSCell;

- For timeSCGFailure: value range 0-1023;

- For failureType: Reuse the legacy field.

Reference

- Chairman’s Notes RAN2#127, Maastricht, Netherlands, Aug 19th – 23rd, 2024

- RP-201040 Revised WID on Further Multi-RAT Dual-Connectivity enhancements