3GPP TSG RAN WG2 #127bis                                                                           R2-2408205

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Discussion on the number of SDT UEs in RRC_INACTIVE state (LS S5-245138)

Agenda Item:	8.0

Document for:	Discussion and Decision

Introduction

SA5 sent an LS to RAN2 [1], asking whether there is a possibility to specify the number of UEs in RRC_INACTIVE state with data transmission:

The intention of the SDT (Small Data Transmission) mechanism aims to allow UEs to transmit data in the RRC_INACTIVE state. SA5 understands the number of SDT UEs in RRC_INACTIVE state can help operators to evaluate the load status of a specific cell. SA5 would like to ask RAN2 to evaluate whether there is a possibility to specify the number of UEs in RRC_INACTIVE state with data transmission.

In this contribution, we discuss this issue and provide our views.

Discussion

Monitoring the number of UEs in RRC_CONNECTED state and RRC_INACTIVE state can help to estimate load/cell congestion status and better configure SDT threshold parameter. To enhance the statistic of number of INACTIVE UEs, SA5 discussed whether to collect the number of UEs related statistics for both CONNECTED and INACTIVE UEs, but there was no consensus.

From RAN2 point of view, it is only possible to record the number of UEs in RRC_INACTIVE state within an RNA, but the gNB is not capable of knowing the number of INACTIVE UEs of a specific cell, as an inactive UE is free to move across cells within a RNA without reporting to the host gNB.

Observation 1: gNB is aware of the number of UEs in RRC_INACTIVE state within an RNA, but it doesn’t know the number of UEs in RRC_INACTIVE state within an specific cell.

For INACTIVE UEs with data transmission, a gNB could be aware of all the initiated SDT procedures, and the UE number of all the past SDT procedures can be recorded. But the gNB doesn’t know if these UEs may initiate another SDT procedure in the same cell, e.g., over RACH, since a UE may move to other cells after a SDT procedure without reporting to the host gNB. Therefore, the gNB cannot exactly know the number of SDT UEs in RRC_INACTIVE state with data transmission for a specific cell.

Observation 2: gNB cannot exactly know the number of SDT UEs in RRC_INACTIVE state with data transmission for a specific cell, since a UE may move to another cell within a RNA after a SDT procedure without reporting to network.

Proposal: in the reply LS to SA5, RAN2 indicates the following understandings:

Understanding 1: gNB is aware of the number of UEs in RRC_INACTIVE state within an RNA, but it doesn’t know the number of UEs in RRC_INACTIVE state within an specific cell.

Understanding 2: gNB cannot exactly know the number of SDT UEs in RRC_INACTIVE state with data transmission for a specific cell, since a UE may move to another cell within a RNA after a SDT procedure without reporting to network.

Conclusion

According to the analysis in section 2, the observations and proposals are summarized as follows:

Observation 1: gNB is aware of the number of UEs in RRC_INACTIVE state within an RNA, but it doesn’t know the number of UEs in RRC_INACTIVE state within an specific cell.

Observation 2: gNB cannot exactly know the number of SDT UEs in RRC_INACTIVE state with data transmission for a specific cell, since a UE may move to another cell within a RNA after a SDT procedure without reporting to network.

Proposal: in the reply LS to SA5, RAN2 indicates the following understandings:

Understanding 1: gNB is aware of the number of UEs in RRC_INACTIVE state within an RNA, but it doesn’t know the number of UEs in RRC_INACTIVE state within an specific cell.

Understanding 2: gNB cannot exactly know the number of SDT UEs in RRC_INACTIVE state with data transmission for a specific cell, since a UE may move to another cell within a RNA after a SDT procedure without reporting to network.

Reference

- S5-245138, LS on Number of UEs in RRC_INACTIVE state with data transmission, SA5, LS in, Rel-19

Annex for draft reply LS

1. Overall Description:

RAN2 thanks SA5 for the LS on Number of UEs in RRC_INACTIVE state with data transmission. Regarding the reply to the SA5 question, RAN2 has achieved the following consensus:

Understanding 1: gNB could be aware of the number of UEs in RRC_INACTIVE state within an RNA, but it doesn’t know the number of UEs in RRC_INACTIVE state within an specific cell.

Understanding 2: gNB cannot exactly know the number of SDT UEs in RRC_INACTIVE state with data transmission for a specific cell.

2. Actions:

To SA5 group.

ACTION: 	RAN2 kindly asks SA5 to take the above feedback into consideration.