3GPP TSG RAN WG2 #127bis                                                                           R2-2408207

Hefei, China, Oct 14th – Oct 18th, 2024

Source:	CATT, Turkcell

Title:	Discussion on Measurement Event Prediction

Agenda Item:	8.3.3

Document for:	Discussion and Decision

- Introduction

In RAN2#125bis meeting, RAN2 discussed the measurement event prediction use case, some agreements were achieved as follow [1].

Agreements:

- At least measurement event evaluation based on RRM measurement prediction result will be studied.   Direct measurement event prediction are is also allowed.

- Clarifications on what is being as input should be provided with results

- Start with A3 as a baseline.

- Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

Since during the latest meetings, some progress on RRM measurement prediction has been made, e.g., simulation assumptions and evaluation methodology, the study on measurement event prediction use case can start.

In this contribution, we investigate and study the potential sub use cases of the measurement event prediction, and discuss the simulation assumptions and evaluation methodology and performance metrics/KPIs.

- Discussion

  - Potential sub use case

AI/ML based measurement event prediction was agreed as a use case for AI/ML based mobility optimization in Rel-19. In RAN2#125bis meeting, measurement event prediction use case was discussed and some agreements were achieved. But the discussion of potential sub use cases still hasn't reached a consensus yet. In previous meeting, some sub use cases were proposed, e.g., measurement event reporting trigger prediction, ping-pong/short of stay prediction, and measurement reduction based on event prediction (if no event fulfilled based on prediction, no measurement is needed) etc..

Among these proposed sub use cases, from our perspective, measurement event reporting trigger prediction can be the representative sub use case of measurement event prediction. The AI/ML algorithm can be used to predict whether/when the entering condition of the A3 event can stably be met during the period (i.e., TTT) configured for the event, which means the measurement reporting will be triggered due to the fulfilled event and then the UE could receive the handover command with high success rate.  In legacy, the measurement results reporting and receiving the handover command by UE will take some time. In this time duration, the signal quality of source cell could become worse which will cause the UE cannot report the measurement results or cannot receiving the handover command from network successfully, and the radio link failure may occur as well. If the UE can predict the measurement event reporting trigger, such failure can be avoided if the UE triggers the measurement reporting earlier to network to assist the network’s handover decision, e.g., the network can send the HO command to UE in advance based on UE reported predicted measurement results to avoid too late handover.

Observation 1: Measurement event reporting trigger prediction can reduce the risk of RLF due to NW may not receive the UE report or UEmay not receive handover command from NW.

In short, measurement event reporting trigger prediction will bring significant gains to mobility optimization, thus, we suggest prioritizing the discussion for measurement event reporting trigger prediction in measurement event prediction use case.

Proposal 1: For measurement event prediction use case, to predict whether/when a measurement event reporting may occur is selected as one representative sub use case.

  - Simulation assumptions and evaluation methodology

- Simulation methodology

Based on the current specification, the UE performs RRM measurement and reporting based on network configuration. For event triggered reporting, taking event A3 as an example, UE needs to evaluate whether the entering condition of the event A3 is stably met during TTT. If UE evaluates the entering condition of the event A3 is stably met during TTT, UE triggers measurement reporting. When receiving the measurement results from UE, network can send the handover command to UE to indicate UE to perform handover procedure. This procedure is illustrated in Figure 1 below.

Figure 1: Conventional event triggered measurement results reporting procedure

For measurement event reporting trigger prediction sub use case, the AI/ML model is used to predict the event trigger reporting in advance by predicting whether/when the event A3 can be fulfilled, e.g., the prediction window size can be set to the same size of TTT. When the A3 event entering condition is met, the UE can further predict whether the A3 event can stably met during TTT by AI/M model, then , at this time point, the UE can send the predicted measurement results or event fulfilled situation (e.g., which measId will be met for measurement reporting) to assist the network handover decision. This procedure is illustrated in Figure 2 below.

Figure 2: AI/ML based event triggered measurement results reporting procedure

Proposal 2: RAN2 to take the Figure 2 as baseline simulation methodology to study the measurement event reporting trigger prediction.

- Simulation assumptions

For RRM measurement prediction use case, some simulation assumptions were discussed and captured in TR. Considering the measurement event prediction is based on the RRM measurement, especially for indirect measurement event prediction (i.e., based on RRM measurement prediction result), these simulation assumptions for RRM measurement prediction (as below extracted from TR) can be reused for measurement event prediction.

Table 5.2.1-2

| L3 filtering parameter | value |
| --- | --- |
| FR1 FilterCoefficient | 4 |
| FR2 FilterCoefficient | 4 |

Table 5.2.1-3

| Measurement period | value |
| --- | --- |
| FR1 to FR1 intra-frequency without gap | 200ms |
| FR1 to FR1 inter-frequency with gap | 200ms |
| FR2 to FR2 intra-frequency without gap | 400ms |

Table 5.2.1-4

| Consolidation parameter | value |
| --- | --- |
| nrofSS-BlocksToAverage for FR1 | 1 |
| nrofSS-BlocksToAverage for FR2 | 3 |
| absThreshSS-BlocksConsolidation for FR1 | -110dbm |
| absThreshSS-BlocksConsolidation for FR2 | -110dbm |

Proposal 3: Simulation assumptions for RRM measurement prediction can be reused for measurement event prediction.

For measurement event prediction use case, the offset (e.g., a3-Offset) and the timeToTrigger parameters need to be considered in simulation assumptions. Based on TR36.839, the following parameter sets can be considered.

Table 1: Parameter sets for measurement event prediction

| Parameter | Set 1 | Set 2 | Set 3 | Set 4 | Set 5 |
| --- | --- | --- | --- | --- | --- |
| TTT [ms] | 480 | 160 | 160 | 80 | 40 |
| A3 offset [dB] | 3 | 3 | 2 | 1 | -1 |

Proposal 4: RAN2 to discuss the parameter sets (a3-Offset and timeToTrigger) in simulation assumptions.

In RRM measurement prediction use case, the following three cases were selected as high prioritization cases.

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

For indirect measurement event prediction, the measurement event evaluation is based on RRM measurement prediction results. An issue is which RRM measurement prediction case(s) above is used for measurement event prediction. Considering that the measurement event prediction is temporal domain prediction, case 3 is not applicable as it is frequency domain prediction. For case 2 and case 4, we prefer case 4 as it is for enhance handover performance.

Proposal 5: RRM measurement prediction case 4 is used for measurement event prediction.

  - Performance metrics/KPIs

In last meeting, KPIs for RLF/HO failure prediction use case were discussed and achieved some agreements, details as follow.

Agreements on KPI

For indirect

-	SINR difference, missed RLF detection, false RLF detection, F1 score, time difference of true time RLF and predicted RLF, true RLF prediction.

Direct

-	missed RLF detection, false RLF detection, F1 score.

For measurement event prediction use case, the similar KPIs can be considered, i.e.,

For indirect, the RSRP difference, missed event detection, false event detection, F1 score, time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction can be the KPIs for measurement event prediction.

For direct, missed event detection, false event detection, F1 score can be the KPIs for measurement event prediction.

Proposal 6: RAN2 to adopt the following KPIs for measurement event prediction use case:

- for indirect: the RSRP difference, missed event detection, false event detection, F1 score, time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction;

- for direct: missed event detection, false event detection, F1 score.

- Conclusion

In this contribution, we provide our views on the measurement event prediction use case of AI/ML for mobility and the observation and proposals are summarized as follows:

Potential sub use case

Observation 1: Measurement event reporting trigger prediction can reduce the risk of RLF due to NW may not receive the UE report or UE may not receive handover command from NW.

Proposal 1: For measurement event prediction use case, to predict whether/when a measurement event reporting may occur is selected as one representative sub use case.

Simulation assumptions and evaluation methodology

Proposal 2: RAN2 to take the Figure 2 as baseline simulation methodology to study the measurement event reporting trigger prediction.

Proposal 3: Simulation assumptions for RRM measurement prediction can be reused for measurement event prediction.

Proposal 4: RAN2 to discuss the parameter sets (a3-Offset and timeToTrigger) in simulation assumptions.

Proposal 5: RRM measurement prediction case 4 is used for measurement event prediction.

Performance metrics/KPIs

Proposal 6: RAN2 to adopt the following KPIs for measurement event prediction use case:

- for indirect: the RSRP difference, missed event detection, false event detection, F1 score, time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction;

- for direct: missed event detection, false event detection, F1 score.

- References

- R2-2404102 Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China