3GPP TSG RAN WG2 #127bis                                                                            R2-2408208

Hefei, China, Oct 14th – Oct 18th, 2024

Source:	CATT

Title:	Discussion on LCM for UE-sided model for BM use case

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

- Introduction

In RAN2#127 meeting, RAN2 discussed the LCM for UE-sided model for beam management use case and achieved the agreements as follows [1].

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

After RAN2#127 meeting, RAN2 sent an LS to RAN1 to provide RAN2’s agreements and asked specific questions that RAN2 needs to enable further progress. In this contribution, we will focus on the issues that are not dependent on RAN1’s reply and issues we haven’t yet discussed (e.g. necessary signalling/protocols to configure the UE for training, etc).

- Discussion

  - Non-applicable functionalities reporting

After RAN2#126 meeting, during the email discussion [2], an issue was discussed that whether to define available functionalities separately from applicable functionalities, but it is still FFS. In [3], the related issue was discussed that whether UE can include non-applicable functionalities in Step 4 (i.e., along with reporting applicable functionalities), which means that UE can report the functionalities for which a model is available in device, but not applicable. From our perspective, the intention of reporting the non-applicable functionalities is not clear, e.g., whether the non-applicable functionalities are reported in step 4 is for the network to provide the configuration (e.g., the NW-side additional condition) to make the non-applicable functionalities change to applicable functionalities. If it is for the network to provide the configuration to change the non-applicable functionalities to applicable functionalities, RAN2 needs to check whether it is feasible.

Proposal 1: RAN2 to discuss whether it is useful that UE reports the non-applicable functionalities with available model to network, e.g., for network to update NW-side additional condition.

In addition, if the model is unavailable in device for a functionality, whether the UE can also report the functionality without an available model to network for e.g., model transfer needs to be discussed, which is also related to model transfer discussion.

Proposal 2: RAN2 to discuss whether it is useful that UE reports the non-applicable functionalities without available model to network, e.g., for model transfer.

A related issue is when the UE should report the non-applicable functionalities (if supported), e.g., should it provide the non-applicable functionalities when it has no any applicable functionality can be reported upon change of applicable functionality or upon receiving the network request applicable functionalities, or the UE reports the non-applicable functionalities as long as there are non-applicable functionalities. This can be discussed after Proposal 1 and/or Proposal 2 are agreed.

Another related issue is what the UE behavior is when it has no any applicable functionality can be reported upon change of applicable functionality or upon receiving the network request applicable functionalities, e.g., whether it should report the empty content or whether some other assistant information (e.g., non-applicable functionalities) should be sent to network for network to transfer model to UE or provide NW-side additional condition to UE.

Proposal 3: RAN2 to discuss UE behavior when no applicable functionality can be reported upon change of applicable functionality or upon receiving the network request, e.g., report empty content or reporting other assistant information (e.g., non-applicable functionalities).

  - Neighbouring cell’s applicable functionality reporting

In [3], an issue was proposed by companies that whether neighbouring cell’s applicability needs to be considered in applicable functionality reporting to serving cell. In the current stage, the intention of neighbouring cell’s applicable functionality reporting is not clear, based on the agreements in last meeting, it seems that the NW-side additional condition needs to be provided to UE from network for determining the applicable functionalities in UE side, it can be e.g., based on associated ID to indicate the NW-side additional condition, relevant issues have been added in LS to RAN1 and we have to wait for RAN1’s reply.

In addition, in RAN1#118 meeting, the agreement was achieved as follow.

Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Based on the highlighted in yellow part, the associated ID is in cell level, and ffs whether it can applicable for other cells, RAN1 will continue to discuss the ffs issue which has the impact on neighbouring cell’s applicable functionality reporting.

Considering that the NW-side additional condition related issue is under RAN1 discussion, it is too early to discuss the neighbouring cell’s applicable functionality reporting issue. Therefore, we suggest that RAN2 to postpone the discussion on neighbouring cell’s applicable functionality reporting after the NW-side additional condition is clear.

Proposal 4: RAN2 to postpone the discussion on neighbouring cell’s applicable functionality reporting until the NW-side additional condition is clear.

  - Configuration for training data collection

In RAN1#116bis meeting, RAN1 discussed and agreed the following agreement for UE-side model(s) developed (e.g., trained, updated) at UE side.

Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

Based on the description highlighted in yellow, Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a solution for resolving the consistency for functionality. In step A, i.e., training data collection configuration procedure, network needs to signal the data collection related configuration(s) and associated ID(s) to UE.

Proposal 5: For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID can be included in training data collection configuration.

For the data collection related configuration(s), e.g., measurement resources configuration etc., we understand it is similar as the inference configuration except for it does not include the inference results reporting configuration. The detail contents in the data collection related configuration(s) can wait for RAN1’s input for inference configuration.

Proposal 6: For BM use case for UE-side model, the details contents of data collection related configuration(s) is left to RAN1.

- Conclusion

According to the discussion in section 2, we propose:

Non-applicable functionalities reporting

Proposal 1: RAN2 to discuss whether it is useful that UE reports the non-applicable functionalities with available model to network, e.g., for network to update NW-side additional condition.

Proposal 2: RAN2 to discuss whether it is useful that UE reports the non-applicable functionalities without available model to network, e.g., for model transfer.

Proposal 3: RAN2 to discuss UE behavior when no applicable functionality can be reported upon change of applicable functionality or upon receiving the network request, e.g., report empty content or reporting other assistant information (e.g., non-applicable functionalities).

Neighbouring cell’s applicable functionality reporting

Proposal 4: RAN2 to postpone the discussion on neighbouring cell’s applicable functionality reporting until the NW-side additional condition is clear.

Configuration for training data collection

Proposal 5: For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID can be included in training data collection configuration.

Proposal 6: For BM use case for UE-side model, the details contents of data collection related configuration(s) is left to RAN1.

- References

- Draft_RAN2_127_Meeting_Report_v1

- [POST126][032][AIML PHY] LCM (Intel, Samsung) – phase1

- [POST126][032][AIML PHY] LCM (Intel, Samsung) – phase2