3GPP TSG RAN WG2 #127bis                                                                           R2-2408209

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Consideration on LCM for Positioning use case

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

- Introduction

In RAN#105 meeting, it is approved to focus on the first priority issues until end of 2024 [1]:

Offline proposal 5 Conclusion - For second priority issues of positioning - RAN1 will not discuss any proposals targeting second priority issues in Q4, 2024.

The first priority issues include 1 UE-sided model case and 2 NW-sided model cases:

  - Direct AI/ML positioning:

    - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

Since aspects related to data collection should be covered in other section, in this contribution, we discuss the LCM procedures related to inference and monitoring on the first prioritized Pos cases.

- Case 1

  - Inference related

For case 1, the UE-side model is deployed at UE side. There is no any agreement about the inference input/output of PoS case 1 in RAN1 discussion until RAN1#118 meeting.

The basic assumption is that UE-side model is trained by UE side (including by OTT server), and thus the type or format of the measurement corresponding to AI/ML model input is naturally known at UE side. If the AI/ML model is trained at other side such as NW side, assistance information on input type or format may be needed. But model transfer/delivery itself is still under study without clear standardization support. Therefore, when UE uses an UE-side AI/ML model for inference, the UE can generate the measurement for AI/ML model inference using the same type or format of the measurement as training phase, by e.g. UE implementation or assistance information.

Proposal 1: For case 1, UE generates the measurement for AI/ML model inference using the same type or format of the measurement as training phase.

For case 1, the model output is the location of UE. Therefore the legacy LPP message of ProvideLocationInformation can be directly reused. We do not foresee specification impact based on the model output on its own, but we think the LMF needs to understand whether the UE’s location is based on prediction or based on measurement. So an explicit indicator or a new field of predicted UE location may need to be introduced.

Proposal 2: For case 1, RAN2 to discuss whether an explicit indicator or a new field of predicted UE location is needed to assist the LMF to identify the predicted/measured result.

For the reference signaling, RAN1 has the agreement for training data collection:

Agreement For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

The RS should be the same for training and inference for consistency. Since current LPP could already support the Options above, no RS related specification impact is foreseen.

Observation 1: For case 1, no RS related specification impact is foreseen for model inference.

  - Monitoring related

For case 1, RAN1 has listed multiple options with regard to how to generate information on ground truth label:

- Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF.

Until RAN1#118 meeting, it is only concluded that Option A-4 could be realized by implementation and no further discussion is needed on Option A-4. All the other options are not discussed. Therefore, no discussion is needed on case 1 performance monitoring in RAN2 until triggered by RAN1 input.

Observation 2: All the specification impact related options regard to ground truth label generation for case 1 performance monitoring have not been discussed in RAN1 yet.

Proposal 3: For case 1, no discussion is needed for performance monitoring in RAN2 until triggered by RAN1 input.

- Case 3a

  - Inference related

For case 3a, the gNB-side model is deployed at gNB side. The basic assumption is that the gNB-side model is trained at gNB/TRP since the gNB/TRP has higher capability of computing and storage capacity to train an AI/ML model, or trained by NW-side server. Anyway, the type or format of the measurement corresponding to AI/ML model input is naturally known at gNB/TRP.

Proposal 4: For case 3a, gNB/TRP generates the measurement for AI/ML model inference. The input type or format is up to gNB/TRP implementation.

And for Case 3a measurement, it was agreed in RAN1#118 that the existing procedures can be reused in terms of SRS configuration, but parameter values for SRS configuration can be further discussed. Therefore, we do not think RAN2 could start the specification impact related discussion now. More RAN1 input is needed.

Proposal 5: For case 3a, no inference related discussion is needed in RAN2 until triggered by RAN1 input about SRS configuration parameter.

  - Monitoring related

For case 3a, RAN1 provided 2 options for model performance monitoring metric calculation in label-based model monitoring:

Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

Observation 3: For case 3a, both the NG-RAN and LMF node could perform monitoring metric calculation in label-based monitoring.

The performance monitoring metric could be calculated based on the inference output generated in gNB and the label generated in LMF for label-based monitoring. Therefore, Option A and Option B only have impact in RAN3, and no specification impact is foreseen in RAN2.

Observation 4: For case 3a, no performance monitoring related specification impact is foreseen in RAN2.

- Case 3b

  - Inference related

For case 3b, the model input is the gNB channel measurements which may include the timing information and the power information:

For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information.

Both the timing information and the power information have been defined in RAN1#116 and RAN1#118 meetings, respectively. The signaling impact should be discussed by RAN3, e.g. to use the legacy NRPPa messages or to introduce to transmit the gNB channel measurements to LMF.

From RAN2 point of will, both case 3a and case 3b are measured base on SRS in gNB, so the existing procedures can also be reused in terms of SRS configuration of case 3b. Since the parameter values for SRS configuration can be further discussed, we do not think RAN2 could start the specification impact related discussion now. More RAN1 input is needed.

Proposal 6: For case 3b, no inference related discussion is needed in RAN2 until triggered by RAN1 input about SRS configuration parameter.

  - Monitoring related

There is no any agreement about performance monitoring of PoS case 3b in RAN1 discussion until RAN1#118 meeting. But for case 3b, it is reasonable that LMF generates UE’s location as ground truth label based on some measurement (e.g. legacy measurement RTOA) transmitted by gNB/TRP and TRP’s location coordinate.

The performance monitoring metric could be calculated based on the inference output generated in LMF and the label generated in LMF for label-based monitoring. Therefore, no specification impact is foreseen in RAN2.

Observation 5: For case 3b, no performance monitoring related specification impact is foreseen in RAN2.

- Conclusion

In this contribution, we provide our views on the LCM of first priority cases, and the observation and proposals are summarized as follows:

For Case 1

Proposal 1: For case 1, UE generates the measurement for AI/ML model inference using the same type or format of the measurement as training phase.

Proposal 2: For case 1, RAN2 to discuss whether an explicit indicator or a new field of predicted UE location is needed to assist the LMF to identify the predicted/measured result.

Observation 1: For case 1, no RS related specification impact is foreseen for model inference.

Observation 2: All the specification impact related options regard to ground truth label generation for case 1 performance monitoring have not been discussed in RAN1 yet.

Proposal 3: For case 1, no discussion is needed for performance monitoring in RAN2 until triggered by RAN1 input.

For Case 3a

Proposal 4: For case 3a, gNB/TRP generates the measurement for AI/ML model inference. The input type or format is up to gNB/TRP implementation.

Proposal 5: For case 3a, no inference related discussion is needed in RAN2 until triggered by RAN1 input about SRS configuration parameter.

Observation 3: For case 3a, both the NG-RAN and LMF node could perform monitoring metric calculation in label-based monitoring.

Observation 4: For case 3a, no performance monitoring related specification impact is foreseen in RAN2.

For Case 3b

Proposal 6: For case 3b, no inference related discussion is needed in RAN2 until triggered by RAN1 input about SRS configuration parameter.

Observation 5: For case 3b, no performance monitoring related specification impact is foreseen in RAN2.

- References

- RP-242387	Moderator's summary on offline discussion about non-RAN2 aspects on REL-19 WI NR_AIML_Air	RAN1 vice-chair (CMCC)