3GPP TSG-RAN WG2 Meeting #127bis	R2-2408211

Hefei, China, October 14th – 18th, 2024

Source:	CATT

Title:	AI/ML model transfer/delivery

Agenda Item:	8.1.5

Document for:	Discussion and Decision

Introduction

For AI/ML model transfer/delivery, RAN1 and RAN2 have studied from different aspects. In this document, we continue to study the model transfer/deliver based on RAN1 progress and the options studied by RAN2 in the SI. Our observations and proposals are provided.

Discussion

2.1	RAN1 progress

In Rel-18 SI phase, RAN1 studied different Model transfer/delivery cases and options. And the following Table was captured in TR 38.843[1].

Table 4.3-1: Model delivery/transfer cases

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |
| Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. |  |  |  |

In Rel-19 SI phase, RAN1 continued to study AI/ML model transfer/delivery and some cases including z2, z3 and z5 were deprioritized.

RAN1#116 Conclusion From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19.

RAN1#116bis Conclusion From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons: - Risk of proprietary design disclosure - Burden of offline cross-vendor collaboration Conclusion From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y): - No much benefit compared to Case y - Risk of proprietary design disclosure - Large burden of offline cross-vendor collaboration - Additional burden on model storage within in 3GPP network

And based on RAN1 chairman guidance, z1 will not be discussed. Additionally, Case y may not need to be studied since it is transparent to 3GPP signaling. Thus, until RAN1#118, the situation of model transfer/delivery cases is summarized in the following table.

Table 1 Situation of model transfer/delivery cases.

| Case | UE-sided | Two-sided |
| --- | --- | --- |
| y | No need to discuss | No need to discuss |
| z1 | (No more discussion in this agenda) | (No more discussion in this agenda) |
| z2 | Deprioritized | Open |
| z3 | Deprioritized | Deprioritized |
| z4 | Open | Open |
| z5 | Deprioritized | Deprioritized |

Thus RAN1 is only focusing on case z4.

For case z4, the following agreements were made in RAN1 [2]-[3]:

RAN1#117 Agreement From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits): - Alt. A - Step A-1: UE reports the supported known model structure(s) to network - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1 - FFS: whether some additional step(s), and/or whether other information is needed - Alt. B - Step B-0: UE reports to NW its support of model transfer/delivery case z4 - Note: Step B-0 may be before or after Step B-1, or not necessary - Step B-1: NW indicates to UE the candidate known model structure(s) - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2 - FFS: whether some additional step(s), and/or whether other information is needed - Note: Other alternative(s) is not precluded - Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion.

RAN1#118 Agreement From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects - Model type/backbone (e.g., Transformer, CNN and so on) - In case model type is a neural network - Number of layers - Layer types/structure (e.g., full connected, activation layer and so on) - Layer size (e.g., the number of parameters of a layer) - Connection between different layers - model input/output related information

According to the agreements in RAN1, it can be seen that the steps of Model transfer/delivery consist of the indication of know model structure(s) and transfer of the parameters for one or more of the supported known model structures.

2.2	Impacts of RAN1 agreements on RAN2

Based on the agreements made in RAN1, it can be seen that the procedure in z4 requires signaling interactions between UE and the NW. Some examples for Alt.A and Alt.B are illustrated in Figure 1 and 2.

Figure 1: Example procedure for Alt. A

Figure 2: Example procedure for Alt. B

It can be seen that the UE needs to report which known model structure(s) that it supports (step A-1 in Alt.A and Step B-2 in Alt.B). And the NW needs to transfers to UE the parameters for part of the supported known model structure(s) (Step A-2 in Alt.A and Step B-3 in Alt.B). From UE’s perspective, the common part of model transfer/delivery procedure consists of:

- reporting known model structure(s) to the network;

- the network needs to send parameters for the supported known model structures to the UE.

Observation 1: In Case z4, model transfer/delivery procedure at least consists of at least:

- reporting to NW the known model structure(s) from the UE;

- delivery of parameters for the supported known model structures from the NW to the UE.

Based on the Observation 1, it is proposed that:

Proposal 1: RAN2 studies the impacts of model transfer/delivery in Case z4 which consists of at least:

- reporting to NW the known model structures from the UE;

- delivery of parameters for the supported known model structures from the NW to the UE.

RAN1 has made progress on the “known model structures” of the model transfer/delivery Case z4 at least include known information on the following aspects:

- Model type/backbone (e.g., Transformer, CNN and so on)

- In case model type is a neural network

- model input/output related information

Nevertheless, regardless Alt.A or Alt.B, there are some FFSs on how procedure z4 works. For example it is still not clear on how to align the understanding of supported known model structure(s) between UE and NW which is the key of feasibility study. RAN1 will continue the discussion on model transfer/deliver. Hence, RAN2 can wait for more progress from RAN1.

Based on the discussion above, it is proposed that:

Proposal 2: RAN2 waits for more progress from RAN1 to study the details of procedure in Case z4.

2.3	RAN2 progress

In Rel-18 SI, RAN2 studied Model transfer/delivery according to the transmission entity and the data type (e.g., RRC signaling, NAS signaling or UP)

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP). - Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

RAN2 also studied the above solutions via the following aspects:

- A1: Large, no upper limit model/model parameter size, - A2: Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs), - A3: Network controllability on model transfer/delivery (e.g., management decision at gNB), - A4: Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB).

These options and evaluation on the different model transfer/delivery solutions were also captured in the TR 38.843[1].

2.3	Analysis on RAN2 solutions

- 2.2.1	Solution 1a/2a/3a

According to the progress in RAN2, Solution 1a, 2a and 3a mainly focus on the CP solution. And these options are categorized based on the network entity which performs AI/ML model transfer/delivery and which signaling is used to transfer/delivery AI/ML model.

Taking Observation 1 into account, the network entity which is responsible of AI/ML model transfer/delivery in Option 1a, 2a and 3a should be the entity which performs reception of the report from the UE for the known model structure(s) and delivers parameters for the supported known model structures to the UE. And the signaling corresponding to the ones in each option, i.e. RRC, NAS and LPP, should be used to carry the known model structure(s) and the parameters for one or more of supported known model structure(s).

Observation 2: The network entity which is responsible of AI/ML model transfer/delivery in Option 1a, 2a and 3a should be the entity which performs reception of the report from the UE for the known model structure(s) and delivers parameters for the supported known model structures to the UE with corresponding signaling. That is:

- Solution 1a: gNB can receive the known model structures from UE and send parameters for one or more of supported known model structure(s) to UE via RRC signaling;

- Solution 2a: Core Network (except LMF) can receive the known model structures from UE and send parameters for one or more of supported known model structure(s) to UE via NAS signaling;

- Solution 3a: LMF can receive the known model structures from UE and send parameters for one or more of supported known model structure(s) to UE via LPP signaling.

- 2.2.2	Solution 1b

Regarding Solution 1b in which gNB can transfer/deliver AI/ML model(s) to UE via UP data, there are still some issues. Considering the UE can transfer known model structure(s) to the NW, it is FFS on which layer is responsible for construction of the uplink transmission for the known model structure(s). Unlike traditional UP, the UE can process the data delivered by upper layers, e.g. IP layer. In Option 1b, the data is not delivered from upper layer. Currently, no suitable AS layer can be applied to construct the packet for the uplink transmission for Solution 1b.

For downlink, the model transfer/delivery highly depends on the known model structure(s) between the UE and the NW and the parameters or one or more of supported known model structure(s) which is sent from the NW to the UE. The UE has to derive the model based the know model structure(s) and these parameters. There is no terminated layer for the UE to process the model structures and the parameters in Solution 1b.

Observation 3: In Solution 1b, the tradition UP protocol is not applicable for AI/ML model transfer/delivery since there is no suitable layer to generate the uplink message/data and process the known model structure(s) together with received parameters for one or more of the supported known model structure(s) based on current protocol stack.

- 2.2.3	Solution 2b/3b/4a

It has been identified that some potential spec impacts for Solution 2b, 3b and 4a for NW controllability on model transfer/delivery and management at gNB which is copied from TR 38.843 and shown in the following table.

Table 7.3.1.4-4 Analysis of current status and gaps, and potential RAN specification impact for Solutions 2b and 3b Discussion Area; Current status and Gaps; Potential RAN specification impact. A1. Large, no upper limit model/model parameter size; No model size limitation; - No RAN impact - Note: The detail procedure of model transfer from Core Network/LMF to UE is out of RAN scope. A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs); - For Solution 2b, supported - For Solution 3b, depends on Rel-18 CT1 solution LPP message over a user plane connection between UE and LMF; Note: supporting service continuity across LMF is out of RAN scope. A3. Network controllability on model transfer/delivery and management at gNB; gNB cannot perform model management directly; Requires management and model transfer interaction between Core Network/LMF and gNB when model management at gNB. Management and interaction between UE and gNB is not supported; Requires management and interaction between UE and gNB (e.g., model identification, model transfer completion, etc.) when model management at gNB. A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB); - Procedure latency depends on model size, QoS requirement and DRB priority - Other latency includes forwarding data from Core Network to gNB; Note: The detail QoS requirement on Core Network for model transfer/delivery is out of RAN scope.

Table 7.3.1.4-5 Analysis of current status and gaps, and potential RAN specification impact for Solutions 4a Discussion Area; Current status and Gaps; Potential RAN specification impact. A1. Large, no upper limit model/model parameter size; No model size limitation; No RAN impact. A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs); - If model transfer/delivery from OTT server via Core Network, supported - If model transfer/delivery from OTT server via LMF, depends on Rel-18 CT1 solution LPP message over a User Plane connection between UE and LMF; Note: supporting service continuity across LMF is out of RAN scope. A3. Network controllability on model transfer/delivery and management at gNB; Model transfer/delivery is transparent to RAN; - Requires management and model transfer interaction between OTT server and gNB when model management at gNB - Note: it is unclear whether this is within RAN scope - Requires interaction between UE and gNB for the network controllability of the model transfer/delivery (e.g., model identification, model transfer completion, etc.) if management is in gNB. A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB); - Procedure latency depends on model size, QoS requirement and DRB priority - Other latency includes forwarding data from OTT server to gNB; Note: The detail QoS requirement for model transfer/delivery of solution 4a is out of RAN scope.

For aspect of network controllability on model transfer/delivery and management at gNB in Solution 2b, 3b and 4a, it requires interaction between UE and gNB for the network controllability of the model transfer/delivery (e.g., model identification, model transfer completion, etc.) if management is in gNB. This aspect depends on RAN1’s progress, e.g., model transfer/delivery. However, based on the analysis in Section 2.1, the details of the z4 procedure are still under RAN1 discussion.

Observation 4: Spec impacts in Solution 2b, 3b and 4a highly depends on the procedure, e.g., model transfer/delivery with details are still under discussion in RAN1.

- 2.3.4	Solution 4b

For Solution 4b, it is split into 2 sub-solutions: over Control Plane (CP) signalling and Over, e.g., IP. And the most of potential RAN specification impact are out of RAN scope except for aspect of large, no upper limit model/model parameter size, Solution 4b over CP may need RRC segmentation if OAM does not do segmentation for model transfer/delivery. However, this also depends on the model/model parameter size which is still unclear at this stage.

Observation 5: The potential RAN specification impact of Solution 4b is only for over CP signalling sub-solution which may need RRC segmentation if OAM does not do segmentation for model transfer/delivery and the model/model parameter size is larger the upper limitation of the signalling.

- 2.3.5	Short summary

Based on the Observation 2-5, RAN2 could not make big progress for option 1b, 2b, 3b, 4a and 4b and can firstly focus on Solution 1a, 2a and 3a. Then, it is proposed that:

Proposal 3: Solution 1a, 2a and 3a are prioritized for further study for Model transfer/delivery.

Conclusion

In this document, the progress in both RAN1 and RAN2 are provided for information, and we study the RAN2 impacts brought the current RAN1 agreements and the solutions identified in TR in Rel-18. And the following obserations and proposals are provided:

RAN1 progress and impacts:

Observation 1: In Case z4, model transfer/delivery procedure at least consists of at least:

- reporting to NW the known model structure(s) from the UE;

- delivery of parameters for the supported known model structures from the NW to the UE.

Proposal 1: RAN2 studies the impacts of model transfer/delivery in Case z4 which consists of at least:

- reporting to NW the known model structures from the UE;

- delivery of parameters for the supported known model structures from the NW to the UE.

Proposal 2: RAN2 waits for more progress from RAN1 to study the z4 procedure details of procedure in Case z4.

Analysis on Solutions in TR:

Observation 2: The network entity which is responsible of AI/ML model transfer/delivery in Option 1a, 2a and 3a should be the entity which performs reception of the report from the UE for the known model structure(s) and delivers parameters for the supported known model structures to the UE with corresponding signaling. That is:

- Solution 1a: gNB can receive the known model structures from UE and send parameters for one or more of supported known model structure(s) to UE via RRC signaling;

- Solution 2a: Core Network (except LMF) can receive the known model structures from UE and send parameters for one or more of supported known model structure(s) to UE via NAS signaling;

- Solution 3a: LMF can receive the known model structures from UE and send parameters for one or more of supported known model structure(s) to UE via LPP signaling.

Observation 3: In Solution 1b, the tradition UP protocol is not applicable for AI/ML model transfer/delivery since there is no suitable layer to generate the uplink message/data and process the known model structure(s) together with received parameters for one or more of the supported known model structure(s) based on current protocol stack.

Observation 4: Spec impacts in Solution 2b, 3b and 4a highly depends on the procedure, e.g., model transfer/delivery with details are still under discussion in RAN1.

Observation 5: The potential RAN specification impact of Solution 4b is only for over CP signalling sub-solution which may need RRC segmentation if OAM does not do segmentation for model transfer/delivery and the model/model parameter size is larger the upper limitation of the signalling.

Proposal 3: Solution 1a, 2a and 3a are prioritized for model transfer/delivery for further  study for Model transfer/delivery.

Reference

- TR 38.843 V18.0.0, Study on Artificial Intelligence (AI)/Machine Learning (ML);

- RAN1#117 chairman notes;

- RAN1#118 chairman notes;