3GPP TSG-RAN WG2 Meeting #127bis                                               R2-2408212

Hefei, China, Oct 14th – 18th, 2024

Source:     CATT

Title:	Discussion on early implementation of UE capability on measurements gap enhancement (LS R4-2413936)

Agenda Item:	5.1.3.2

Document for:	Discussion and Decision

Introduction

According to RAN4 LS [1], RAN4 asks RAN2 to design early implementation of the Rel-18 features nr-NeedForInterruptionReport-r18 and interRAT-NeedForInterruptionNR-r18 starting from Rel-17. In this paper, we give our view on how to address this issue, and the corresponding TPs are also provided in Annex.

Discussion

According to RAN4 LS [1], an early implementation of the Rel-18 features nr-NeedForInterruptionReport-r18 and interRAT-NeedForInterruptionNR-r18 starting from Rel-17 is required. The content of RAN4 LS [1] is as follows:

In Rel-18, a new capability interRAT-NeedForInterruptionNR-r18 was introduced by RAN2 to indicate whether interruption is needed when UE reports “no gap” and the following is agreed by RAN4 regarding NR measurements with NeedForGapsInfoNR-r16 and interFrequencyMeas-Nogap-r16: - LTE – NR inter-RAT measurement: - interRAT-NeedForGapsNR-r16=FALSE means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of interRAT-NeedForInterruptionNR-r18: - Optional since Rel-17 - NR measurements without gaps - “no-gap” as part of NeedForGapsInfoNR-r16 means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of nr-NeedForInterruptionReport-r18: - Optional since Rel-17. Considering the discussion, RAN4 believes that early implementation of nr-NeedForInterruptionReport-r18 and interRAT-NeedForInterruptionNR-r18 from Rel-17 is needed. For this reason, RAN4 would like to respectfully ask RAN2 to consider early implementation of the Rel-18 features nr-NeedForInterruptionReport-r18 and interRAT-NeedForInterruptionNR-r18 starting from Rel-17.

For R18 feature of measurement with no gap and w/o interruption, RAN2 defined a new UE capability in LTE and NR specification respectively, and introduced the reporting procedure of needForInterruptionInfoNR in TS 38.331. More details could be seen in RP-233915 and RP-233940. Based on this NR procedure, the UE will report the indication on whether to support the measurement w/o interruption in needForInterruptionInfoNR only if network knows UE supporting this capability and requests UE to report via the controlling flag needForInterruptionConfigNR in RRCReconfiguration/RRCResume message. For LTE, only a new UE capability is defined to indicate whether measurement with interruption is needed for a band or a band combination.

Since RAN4’s intention is to support early implementation of this R18 feature, RAN2 needs to analyse whether there is an inter-operability issue. In our view, if only the R17 network is implemented supporting this feature and the UE is not, since UE will not report the assoicated capability, then the network will not enable this feature for the UE and not request UE to report its requirement on interruption, so no interoperability problems are foreseen. If only the UE is implemented supporting this feature and the network is not,  if UE report this capability to network, the network will not recoganize this capability and not request UE reporting its requirement on interruption, so no interoperability problems are foreseen as well. Therefore, it can be seen that there is no interoperability problem if it is early implemented from R17. As the magic sentence is usually added in the CR cover sheet, but the corresponding CRs had been approved before and there is no chance to make change in their cover sheets, we just propose to make this clarification in RAN2 chair note.

Proposal 1: RAN2 to confirm that the early implementation of Rel-18 feature nr-NeedForInterruptionReport-r18 or interRAT-NeedForInterruptionNR-r18 from Rel-17 will not cause interoperability issues.

Furthermore, from the CR early implementation perspective, the associated CR RP-233915/RP-233940 should be added in the annex G/C of TS 36.331/38.331. The corresponding TPs are shown in Annex.

Proposal 2: To support RAN4 requirement, the associated CR RP-233915/RP-233940 is added in the annex G/C of TS 36.331/38.331.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: RAN2 to confirm that the early implementation of Rel-18 feature nr-NeedForInterruptionReport-r18 or interRAT-NeedForInterruptionNR-r18 from Rel-17 will not cause interoperability issues.

Proposal 2: To support RAN4 requirement, the associated CR RP-233915/RP-233940 is added in the annex G/C of TS 36.331/38.331.

Reference

- R4-2413936, LS on measurements without gap

Annex

36.331 TP

Annex G (normative):	List of CRs Containing Early Implementable Features and Corrections

This annex lists the Change Requests (CRs) whose changes may be implemented by a UE of an earlier release than which the CR was approved in (i.e. CRs that contain on their coversheets the sentence "Implementation of this CR from Rel-N will not cause interoperability issues").

Table G-1: List of CRs Containing Early Implementable Features and Corrections

| TDoc Number (RP-xxxxxx): CR Title | CR Number(s) | CR Revision Number(s) | Earliest Implementable Release | Additional Information |
| --- | --- | --- | --- | --- |
| RP-181233: Successful acknowledgement of RRCConnectionRelease for BL and CE UE | 3324 | 1 | Release 13 | RRCConnectionRelease message, for which the poll bit is not set, can be considered succesfully acknowledged when UE has sent HARQ ACK feedback. |
| RP-182674: CR for T312 on LTE HetNet mobility | 3506 | 5 | Release 12 | Remove T312 in leaving condition for event trigger. |
| RP-182671: Corrections on paging monitoring and SI acquisition in RRC_CONNECTED for BL UEs and UEs in CE | 3647 | 2 | Release 13 |  |
| RP-190548: Update description of ack-NACK-NumRepetitions | 3899 | 2 | Release 13 |  |
| RP-190548: Corrections of NB-IoT Access Barring | 3900 | 2 | Release 13 |  |
| RP-191382: SI update notification and access barring in NB-IoT | 4020 | 2 | Release 13 |  |
| RP-192195 : Correction on handling of SCell(s) during Make Before Break handover | 3986 | 3 | Release 14 |  |
| RP-192940: Stop using redirectedCarrierOffsetDedicated after reselection to another frequency | 4144 | 1 | Release 14 |  |
| RP-200338: Corrections to T312 and Discovery Signals measurement | 4198 | 1 | Release 12 |  |
| RP-200367: Correction on H1 and H2 events | 4103 | 2 | Release 15 |  |
| RP-201166: Allowing PDCP version change without handover | 4262 | 2 | Release 15 |  |
| RP-201166: upperLayerIndication enhancements | 4266 | 3 | Release 15 |  |
| RP-201192: Relaxed serving cell measurement for UEs using WUS | 4344 | - | Release 15 |  |
| RP-202780: Corrections to the field descriptions for TDD/FDD capability differentiation, and to nMaxResource value range | 4389 | 5 | Release 12 | The CR corrects multiple UE capability field descriptions introduced in various releases, the changes are early implementable back to the release in which the corresponding capability was introduced. |
| RP-202789: Correction on uac-AC1-SelectAssistInfo | 4488 | 2 | Release 15 |  |
| RP-211481: Clarification on the initiation of RNA update | 4651 | 1 | Release 15 |  |
| RP-212596: Distinguishing support of extended band n77 | 4723 | 2 | Release 15 |  |
| RP-220472: Introduction of carrier specific NRSRP thresholds for NPRACH resource selection | 4777 | 1 | Release 14 |  |
| RP-221738: Distinguishing support of band n77 restrictions in Canada | 4799 | 2 | Release 15 |  |
| RP-232570: Addition of extended number range for NS value | 4917 | 6 | Release 16 |  |
| RP-233884: Correction to flightPathInfoAvailable when connected to 5GC | 4959 | 2 | Release 15 |  |
| RP-233883: Protection against improper reselection to GERAN/UTRAN [RESELECTION_TO GSM_AND_UTRAN] | 4971 | 1 | Release 15 |  |
| RP-233915: Introduction of measurements without gap with interruption | 4929 | 6 | Release 17 | Early implementation part is referring to the aspect covered by: - R2-2314066: Introduction of measurements without gap with interruption |
| NOTE 1: In case a CR has mirror CR(s), the mirror CR(s) are not listed. NOTE 2: The Additional Information column briefly describes the content of a CR in cases where the CR title may not be descriptive enough. If the CR title is descriptive enough, then the Additional Information column may be left blank. |  |  |  |  |

38.331 TP

Annex C (normative):	List of CRs Containing Early Implementable Features and Corrections

This annex lists the Change Requests (CRs) whose changes may be implemented by a UE of an earlier release than which the CR was approved in (i.e. CRs that contain on their coversheets the sentence "Implementation of this CR from Rel-N will not cause interoperability issues").

Table C-1: List of CRs Containing Early Implementable Features and Corrections

| TDoc Number (RP-xxxxxx): CR Title | CR Number(s) | CR Revision Number(s) | Earliest Implementable Release | Additional Information |
| --- | --- | --- | --- | --- |
| RP-200335: Correction on usage of access category 2 for UAC for RNA update | 1141 | 2 | Release 15 |  |
| RP-201185: Introduction of signalling for high-speed train scenarios | 1464 | 5 | Release 15 |  |
| RP-201216: Release-16 UE capabilities based on RAN1, RAN4 feature lists and RAN2 | 1665 | 2 | Release 15 | Early implementation part is referring to the aspect covered by - R2-2006203: Extension of CSI-RS capabilities per codebook type - R2-2006360: Intraband EN_DC power class expansion for 29 dBm |
| RP-202768: UE behaviour when UL 7.5KHz shift is not supported | 2107 | 2 | Release 15 |  |
| RP-202790: Correction on uac-AccessCategory1-SelectionAssistanceInfo | 2130 | 1 | Release 15 |  |
| RP-211483: Clarification on the initiation of RNA update | 2581 | 1 | Release 15 |  |
| RP-201190: Introduction of eCall over IMS for NR | 1670 | - | Release 15 |  |
| RP-212598: Distinguishing support of extended band n77 | 2810 | 2 | Release 15 |  |
| RP-213342: Duty cycle signalling for power class 1.5 | 2817 | 1 | Release 15 |  |
| RP-213345: CR on 38.331 for introducing UE capability of txDiversity | 2859 | 1 | Release 15 |  |
| RP-220497: Introduction of function for RRM enhancements for Rel-17 NR FR1 HST | 2898 | 2 | Release 16 |  |
| RP-220838: Release-17 UE capabilities based on R1 and R4 feature lists (TS38.331) | 2901 | 1 | Release 15 | Early implementation part is referring to the aspect covered by: - R2-2203898: Introduction of BCS4 and BCS5 - R2-2203836: Introducing UE capability for power class 5 for FR2 FWA |
| RP-221721: CR on the CBM/IBM reporting-38331 | 2916 | 2 | Release 16 |  |
| RP-221736: Distinguishing support of band n77 restrictions in Canada [n77 Canada] | 3078 | 2 | Release 15 |  |
| RP-222527: Correction to additionalSpectrumEmission for UL CA in n77 for the US | 3476 | - | Release 15 |  |
| RP-222527: Correction to additionalSpectrumEmission for UL CA in n77 for Canada | 3478 | - | Release 15 |  |
| RP-232570: Addition of extended number range for NS value | 3900 | 6 | Release 16 |  |
| RP-233888: Introduction of FR2 FBG2 CA BW classes | 2867 | 6 | Release 15 |  |
| RP-233882: Enhancing SCell A2 event reporting | 4375 | 2 | Release 15 |  |
| RP-233890: PTM retransmission reception for multicast DRX with HARQ feedback disabled [PTM_ReTx_Mcast_HARQ_Disb] | 4504 | - | Release 17 |  |
| RP-240667: UE capability for Enhanced channel raster | 4445 | 3 | Release 16 |  |
| RP-240658: Introduction of TxDiversity for 2Tx capability | 4639 | 2 | Release 17 |  |
| RP-241543: Introduction of barring exemption for RedCap UEs for emergency calls | 4570 | 4 | Release 17 |  |
| RP-241543: Correction on Redcap 1 Rx and 2 Rx barring | 4632 | 5 | Release 17 |  |
| RP-241544: Correction on TRS in idle and inactive | 4755 | 3 | Release 17 |  |
| RP-241544: Clarification RLM/BFD relaxation and short DRX | 4771 | 2 | Release 17 |  |
| RP-233940: Introduction of Rel-18 UE capabilities | 4510 | - | Release 17 | Early implementation part is referring to the aspect covered by: - R2-2311897: Introduction of UE capabilities for further measurement gap enhancements |
| RP-233915: Introduction of further measurement gap enhancements | 4063 | 6 | Release 17 | Early implementation part is referring to the aspect covered by change<1>, change<2> and change<3> in R2-2313625. |
| RP-233940: Introduction of Rel-18 UE capabilities | 4510 | - | Release 17 | Early implementation part is referring to the aspect covered by: - R2-2311897: Introduction of UE capabilities for further measurement gap enhancements |

CHANGE END