3GPP TSG-RAN WG2 Meeting #127-bis                                     R2-2408219

Hefei, China, Oct 14th – 18th, 2024

Source:        ZTE Corporation

Title:           Discussion on random access procedure in SBFD

Agenda item:   8.11.2

Document for:  Discussion and Decision

- Introduction

The WID [1] of Rel-19 SBFD related to random access is given as below:

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

This contribution provides our view on random access procedures in SBFD technique.

- Discussion

Supported RA events in SBFD RO

There is a list of RA triggering events (specified in TS38.300 [2]) summarized in the below table:

| RA scenario | RRC state | CBRA/CFRA | initiator |
| --- | --- | --- | --- |
| Initial access from RRC_IDLE | IDLE | CBRA | UE |
| RRC Connection Resume procedure from RRC_INACTIVE | INACTIVE | CBRA | UE |
| Request for Other SI | IDLE/INACTIVE/CONNECTED | CBRA（Msg3-based）/CFRA (Msg1-based) | UE |
| SDT in RRC_INACTIVE | INACTIVE | CBRA | UE |
| Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning | INACTIVE | CBRA | UE |
| RRC Connection Re-establishment procedure | CONNECTED | CBRA | UE |
| DL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronisation status is "non-synchronised" | CONNECTED | CBRA/CFRA(PDCCH order) | gNB |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronisation status is "non-synchronised" | CONNECTED | CBRA | UE |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; | CONNECTED | CBRA | UE |
| Handover, except for when RACH-less HO is configured; | CONNECTED | CFRA | gNB |
| SR failure | CONNECTED | CBRA | UE |
| Explicit request by RRC upon synchronous reconfiguration; | CONNECTED | CFRA | gNB |
| Beam failure recovery | CONNECTED | CBRA/CFRA | UE |
| To establish time alignment for a primary or a secondary TAG | CONNECTED | CFRA (PDCCH order) | gNB |
| Consistent UL LBT failure on SpCell | CONNECTED | CBRA | UE |
| Early UL synchronization with an LTM candidate cell | CONNECTED | CFRA(PDCCH order) | gNB |
| RACH-based LTM cell switch | CONNECTED | CBRA(RRC based)/CFRA(MAC CE based, RRC based) | UE |

In addition, the WID indicates SBFD-aware UE should be able to perform RA in SBFD resources under all of RRC states (RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode). Also in RAN1#116bis meeting [3], RAN1 has agreed to support both CBRA and CFRA for SBFD-aware UE to perform RA:

Agreement Confirm the working assumption: Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.

So from the RAN1’s agreement and RAN2’s trigger events, technically it is feasible to support all kinds of RA in SBFD resources for SBFD UE.

From motivation perspective, the benefit of supporting RA in SBFD resources can be:

- Reduce latency of RA procedure: supporting RA in SBFD resources provides more RA chances or earlier RA chances for SBFD-aware UE, since the number of D slot/symbol is usually larger than U slot/symbol in a TDD system, more ROs in legacy D slot/symbol can be well utilized by SBFD-aware UE;

- Increase UL coverage: Separate RACH configuration may provide a whole different RACH parameters for SBFD UE, e.g., longer preamble format. This helps to increase the UL coverage of SBFD-aware UE.

We do not observe any RA scenarios that does not need to reduce latency or increase UL coverage, for example, SBFD-aware UE wants to perform BFR, or SBFD-aware UE wants to perform re-synchronization. Therefore, there should be no restriction on the supported RA scenarios for RA in SBFD resource.

Therefore, we suggest to confirm the following WA made during RAN2#127 meeting:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Proposal 1: RAN2 to confirm the working assumption that, random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

SBFD RACH configuration in RRC layer

RAN2#127 [4] has made the following agreements to discuss the RACH configuration in RRC signaling:

RAN2#127 Agreement RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS.

Wherein the SBFD RACH configuration in the SIB is for RRC_INACTIVE/IDLE UE to perform CBRA (or CFRA for SI request), and in dedicated RRC signaling is for RRC_CONNECTED UE to perform CFRA (or CBRA when fallback).

The following section 2.2.1 and section 2.2.2 provides the specification impact of SBFD RACH configuration for CBRA and CFRA, respectively.

SBFD RACH configuration for CBRA

CBRA RACH resource is provided in SIB1 -> BWP-UplinkCommon for RRC_IDLE/INACTIVE UEs, or in RRCReconfiguration -> BWP-UplinkCommon for RRC_CONNECTED UEs. This section provides our thoughts on the RRC RACH configuration for SBFD-aware UEs in CBRA.

RACH configuration for option 1

In RAN1#118[5] meeting, the further agreement of RACH configuration option 1 is achieved as follows:

Agreement For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following: - For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration.

This means, both SBFD RO and non-SBFD ROs will be the valid RO type for option 1.

Observation 1: RACH configuration Option 1 can provide both valid SBFD ROs and valid non-SBFD ROs.

Single RACH configuration means SBFD-aware UE reuse the legacy RACH configuration to determine RO location/SSB mapping/power control/etc., but the valid RO of SBFD-aware UE may be different from the valid RO of legacy UE. So, at least gNB should indicate whether the provided RACH configuration in BWP-UplinkCommon can contain SBFD RO or not.

Further re-interpretation of some RACH parameters for option 1 should wait for RAN1’s further discussion.

Proposal 2: For CBRA, gNB should indicate whether the provided RACH configuration in BWP-UplinkCommon can contain SBFD RO or not, i.e., whether option 1 is available.

RACH configuration for option 2

In RAN1#118 meeting, the further agreement of RACH configuration option 2 is achieved as follows:

Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs).

This means, for option 2, only SBFD RO will be the valid RO type.

Observation 2: RACH configuration Option 2 can only provide valid SBFD ROs.

Separate RACH configuration provides independent basic RACH parameters compared with the legacy RACH configuration. So the valid RO of derived from separate RACH configuration and legacy RACH configuration can be totally in different pattern to increase the UL coverage, reduce the latency, etc.

For option 2, a new RACH configuration dedicated for SBFD-aware UE will be needed. There are two kinds of new RACH configuration in RRC signaling, one is to add the SBFD RACH configuration parallel to RACH-ConfigCommon in BWP-UplinkCommon:

Figure 1. RACH configuration option 2, first way of RRC configuration

This way is to configure option 2 using similar principle of introducing 2-step RACH configuration, i.e., the new SBFD RACH configuration is also contained in each element in AdditionalRACH-ConfigList.

The other possible way is to introduce a list in BWP-UplinCommon, the list contains one or more SBFD RACH configurations:

Figure 2. RACH configuration option 2, second way of RRC configuration

This way is to configure option 2 purely independent with legacy, i.e., introduce a new list containing one or more SBFD RACH configurations.

The above RRC configuration has different impact to the set selection procedure in MAC layer, e.g., which RACH resources can be contained within one selected set. RAN2 should take these two kinds of option 2 configuration into consideration and make down-selection in the future meetings.

Proposal 3: For CBRA, RAN2 considers the following RRC configurations of the RACH configuration option 2:

- Add a SBFD RACH configuration parallel to RACH-ConfigCommon in BWP-UplinkCommon, and add the SBFD RACH configuration in each AdditionalRACH-Config;

- Add a list in BWP-UplinCommon, the list contains one or more SBFD RACH configurations.

Configuration of both option 1 and option 2

RAN1#117 meeting [6] has made the following agreement:

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

For option 1 or option 2 of SBFD RACH configuration, RAN1’s agreement indicates ‘UE is not required to support both options’, which means from UE capability perspective, UE can support only option 1, or only option 2, or support both. So in CBRA where gNB does not know UE’s identity and capability, gNB is better to provide both single SBFD RACH configuration and separate SBFD RACH configuration to suit different UE’s capabilities. Then, UE can be allowed to choose the configuration or choose RO type based on rules, UE capabilities, etc.

Therefore, RAN2 should specify both options for CBRA. CBRA RACH resource is configured in SIB1 -> BWP-UplinkCommon, or RRCReconfiguration -> BWP-UplinkCommon.

However this should not be a strict configuration restriction for gNB, since gNB will always provide legacy basic RACH configuration for all UEs to initiate RA.

Proposal 4：For CBRA, gNB should be allowed to send legacy RACH configuration (only containing non-SBFD RO), single RACH configuration (i.e., option 1, containing SBFD RO and non-SBFD RO) and separate RACH configuration (i.e., option 2, only containing SBFD RO) in BWP-UplinkCommon.

SBFD RACH configuration for CFRA

RAN1#117 has achieved the following agreements:

Agreement Confirm the working assumption: Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.

That is to say, RAN2 should specify SBFD RACH configuration in the CFRA resource.

Observation 3: Based on RAN1’s agreement, RAN2 should specify SBFD RACH configuration in the CFRA resource.

gNB configures different CFRA RACH resource for different CFRA scenarios. To be specific:

- For UE performing CFRA for SI request, the legacy RACH configuration is provided in SIB1 - SI-SchedulingInfo - SI-RequestConfig;

- For UE performing CFRA for BFR, the legacy RACH configuration is provided in BWP-UplinkDedicated - BeamFailureRecoveryConfig;

- For UE performing CFRA for ReconfigurationWithSync, the legacy RACH configuration is provided in ReconfigurationWithSync - rach-ConfigDedicated;

- For UE performing CFRA for LTM, the legacy RACH configuration is provided in RRCReconfiguration - LTM-Config -  LTM-Candidate - ltm-EarlyUL-SyncConfig - EarlyUL-SyncConfig;

Since the CFRA RACH resource is provided to UE via dedicated RRC signaling, gNB will know the UE capability before allocating the CFRA RACH resource for specific SBFD-aware UE. So we think in CFRA, there is no need for UE to make further selection on the RACH configuration type and/or RO type, instead, gNB should explicitly indicate the RACH configuration type and/or RO type in the CFRA resource.

Each SBFD RACH configuration mentioned above is associated with a RACH-ConfigGeneric, the RACH-ConfigGeneric can configure SBFD RO or non-SBFD RO. So, one simple way is to add an indication in each CFRA RACH configuration to say that, whether UE should only consider SBFD ROs provided by the corresponding RACH-ConfigGeneric to be valid, or UE should only consider non-SBFD ROs provided by the corresponding RACH-ConfigGeneric to be valid.

Proposal 5: For CFRA, gNB should explicitly indicate the SBFD RACH configuration type (option 1 or option 2) and/or RO type (SBFD RO or non-SBFD RO) in each provided CFRA RACH configuration.

SBFD RACH configuration in NUL carrier

Legacy RACH configuration can be provided within UL BWP configuration, where the UL BWP can be allocated on NUL carrier or SUL carrier. For legacy RACH configuration in NUL, ROs in DL slot/symbol is considered as invalid; however, SUL carrier can always work even in D slot/symbol, i.e., UE can perform RA in SUL regardless of cell-level TDD pattern.

For SBFD, UL subband should only be configured within DL carrier, and SBFD symbol should only be legacy D or F slot/symbol. If UE performs random access in SBFD symbols, the ROs should occupy or overlap with both SBFD symbol (time domain) and UL subband (frequency domain). As the fact that SUL usually has non-overlapping lower frequency than NUL DL carrier to provide larger UL coverage, in order to occupy or overlap with resources in the UL subband, ROs should be configured within NUL carrier, not SUL carrier.

Figure 3. Example relationship between UL subband, DL carrier, NUL carrier in a TDD system

Proposal 6: RAN2 confirms that RACH configuration for SBFD-aware UE should only be configured in NUL carrier, no matter the SBFD RACH configuration is in SIB or in dedicated signaling.

SBFD RA procedure in MAC layer

RACH configuration selection

Based on the analysis in section 2.2.2, for CFRA, gNB doesn’t need to provide more than one RACH configuration to a specific UE.

Based on the analysis in section 2.2.1.3, for CBRA, gNB should be allowed to provide legacy RACH configuration (only containing non-SBFD RO), single RACH configuration (i.e., option 1, containing SBFD RO and non-SBFD RO) and separate RACH configuration (i.e., option 2, only containing SBFD RO) in BWP-UplinkCommon.

When processing RA in MAC layer, the order is: UE selects set -> UE determines the RACH configuration and corresponding RACH parameters -> UE selects RA type (2-step RACH or 4-step RACH). So if all of these three kinds of RACH configurations are provided in SIB, after selecting set, SBFD-aware UE should know which RACH configuration to use.

In addition, this is also proved by the following RAN1 agreement. In the agreement, selecting between legacy-RO and additional-RO in option 2 is actually selecting legacy RACH configuration and separate RACH configuration.

Agreement in RAN1#118 For SBFD-aware UEs and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs

There are several kinds of RACH configuration selection rule proposed by companies, to be specific:

- SBFD-aware UE selects the RACH configuration based on UE’s implementation;

- SBFD-aware UE selects the RACH configuration that associates the highest configured priority;

- SBFD-aware UE firstly selects RACH configuration if SBFD RO is provided.

In summary, RAN2 should firstly acknowledge the issue that SBFD-aware UE should select one RACH configuration, and this UE behavior should be specified in MAC spec. The detailed rule of selection can be further discussed by RAN2/RAN1.

Proposal 7: For SBFD-aware UE, MAC spec should specify UE behavior to select between legacy RACH configuration (only containing non-SBFD RO), single RACH configuration (i.e., option 1, containing SBFD RO and non-SBFD RO) and separate RACH configuration (i.e., option 2, only containing SBFD RO), if all of them are provided in CBRA.

RO type selection

After the RACH configuration selection in section 2.3.1,

- When SBFD-aware UE selects legacy RACH configuration, the UE should initiate RA procedure using non-SBFD RO;

- When SBFD-aware UE selects separate RACH configuration, the UE should initiate RA procedure using SBFD-RO;

- When SBFD-aware UE selects single RACH configuration, it means UE should further selects SBFD RO or non-SBFD RO to initiate RA procedure.

In addition, it can be seen from the following RAN1 agreement:

Agreement in RAN1#118 For SBFD-aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for initial PRACH transmission attempt in one random access procedure: - Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations - Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs

Observation 4: Since single RACH configuration can provide valid SBFD RO and valid non-SBFD RO, if single RACH configuration is selected, UE should further selects SBFD RO or non-SBFD RO to initiate RA procedure.

Further, according to RAN1’s design, the UE has to determine a RO type before initiating preamble transmission, and once UE chooses one type of RO, UE should use this type of RO for the first one or more Msg1/MsgA attempt, since different RO type may have different power control parameters due to different CLI interference.

Currently UE selects nearest RO after SSB/CSI-RS is selected, as specified in TS38.321 [7] below:

5.1.2 Random Access Resource selection If the selected RA_TYPE is set to 4-stepRA, the MAC entity shall: <omitted> 1> else if an SSB is selected above: 2> if the set of Random Access resources associated with Msg1 repetition is selected for this Random Access procedure: 3> determine the next available set of PRACH occasions (as specified in TS 38.213 [6]) for the Msg1 repetition number applicable for this Random Access procedure corresponding to the selected SSB (the MAC entity shall select a set of PRACH occasions randomly with equal probability amongst sets of PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB and selected Msg1 repetition number for this Random Access procedure; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available set of PRACH occasions corresponding to the selected SSB). 2> else: 3> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, or ssb-SharedRO-MaskIndex if configured, or indicated by PDCCH, or indicated by the LTM Cell Switch Command MAC CE (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected SSB).

When a gNB configures SBFD resource and lets UE initiating RA on the SBFD resource, it aims to provide more ROs and reduce the RA latency, so it is likely that the system has more SBFD ROs than non-SBFD ROs configured within one RACH configuration. Based on this, if a SBFD-aware UE selects a nearest non-SBFD RO according to current spec for the first several Msg1/MsgA attempts, the whole RA time will very likely be longer than choosing SBFD ROs in the beginning, since the subsequent SBFD ROs are pretty intense even if SBFD RO is not nearest RO type. In addition, this is not good for load balancing, the non-SBFD ROs may suffer from multiple UE’s access. Therefore, always selecting the nearest RO may not be a good way when SBFD RO is introduced.

Observation 5: Always selecting the nearest RO may not be a good way when SBFD RO is introduced.

RAN2 should firstly acknowledge the issue that SBFD-aware UE should select between SBFD RO and non-SBFD RO when RACH configuration option 1 is adopted, and this UE behavior should be specified in MAC spec. The detailed rule of selection can be further discussed by RAN2/RAN1.

Proposal 8: For SBFD-aware UE, MAC spec should specify UE behavior to select between SBFD RO and non-SBFD RO, if RACH configuration option 1 is selected.

MAC order of RACH resource selection and RO type selection

Based on the analysis above, for CFRA, UE can select RACH configuration and RO type as gNB explicated indicated; for CBRA, UE can select RACH configuration and RO type based on some criteria. RAN2 needs to consider that how does such selection affect the current RA procedure order in UE’s MAC layer.

The current RA procedure in MAC is as follows:

Figure 4. Current MAC order of initiating RA procedure

The RACH configuration selection/RO type selection can happen before resource set selection, or after resource set selection. The main difference is that, the definition of set will be different in these two ways. When before, the selected set of RACH resource the UE selected is per RO type; when after, the selected set of RACH resource is per UE. RAN2 should analysis the MAC order of RACH configuration selection/ RO type selection.

Proposal 9: RAN2 to further study the RA procedure order of RACH configuration selection/ RO type selection in the MAC spec, e.g., before set selection, or after set selection.

Fallback from SBFD RO to non-SBFD RO

RAN1#118’s agreement has touched the fallback mechanism for option 1 and option 2:

Agreement For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure Agreement For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure: - Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure - Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure - Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

When UE use SBFD ROs as the earlier PRACH transmission for a certain number of times and does not receive gNB’s correct response, the reason may be the current CLI is large. So it is reasonable to let UE to fall back to non-SBFD ROs to try the rest time of preamble attempts. In addition, we do not observe any benefit/usecase for UE to fallback from non-SBFD RO to SBFD RO.

For RACH configuration option 1, if fallback from SBFD RO to non-SBFD RO is allowed, the fallback is within one RACH configuration;

For RACH configuration option 2, if fallback from SBFD RO to non-SBFD RO is allowed, the fallback is from SBFD RACH configuration to legacy RACH configuration. This is similar like 2-step RACH configuration fallback to 4-step RACH configuration.

Proposal 10: Support SBFD-aware UE to fallback from SBFD RACH configuration to legacy RACH configuration, and support SBFD-aware UE to fallback from SBFD RO to non-SBFD RO. FFS the detailed fallback procedure of RACH configuration option 1 and RACH configuration option 2.

Early identification of SBFD-aware UE

RAN1#116 [8] agreed the following:

Agreement For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects: Msg2[/Msg4 PDSCH] reception in DL subband(s) Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping Msg3 repetition Msg3 PUSCH[/Msg4 HARQ-ACK PUCCH] power control FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE Note: Strive to make progress in accordance to the discussion in AI 9.3.1.

Legacy Msg3 should only be scheduled in U slots if RA is initiated in NUL. This agreement indicates whether Msg3 can be scheduled/transmitted in SBFD resources needs further discussion. From RAN2 perspective, using SBFD resource to transmit Msg3 can increase the UL transmission chance compared to only using legacy U symbol, more importantly, it can also reduce latency of the whole RA procedure, which is also the motivation of sending Msg1 in SBFD resources. Therefore, sending Msg3 in SBFD resource should be supported.

Observation 6: Using SBFD resource to transmit Msg3 can reduce latency and increase the UL transmission chance compared to only using legacy U symbol, more importantly, it can also reduce latency of the whole RA procedure.

Proposal 11: RAN2 supports that SBFD-aware UE is allowed to send Msg3 in SBFD resource.

The system consists of non-SBFD-aware UEs and SBFD-aware UEs. When in CBRA, both kinds of UEs will send Msg1 according to the broadcast RACH configuration. If gNB wants to send the Msg2 RAR containing the Msg3 UL grant on SBFD resources to SBFD-aware UE, gNB needs to recognize SBFD-aware UE in the first place, i.e., during Msg1 transmission.

Figure 5. SBFD-aware UE early identification in CBRA

Observation 7: It is beneficial for gNB to recognize SBFD-aware UE and non-SBFD-aware UE during Msg1, since gNB can schedule SBFD-aware UE to transmit Msg3 on the following SBFD ROs to reduce RA latency.

If UE initiates RA in SBFD ROs, gNB will directly recognize SBFD-aware UE;

If UE initiates RA in non-SBFD ROs, gNB will not recognize SBFD-aware UE and non-SBFD-aware UE.  Therefore, only receiving preamble on the non-SBFD ROs does gNB need to recognize SBFD-aware UE and non SBFD-aware UE.

Observation 8: gNB will need to recognize SBFD-aware UE and non-SBFD-aware UE only in non-SBFD ROs.

The above early identification can be realized via several ways in the current specification, e.g., RACH partition, or explicitly configure separate preamble indexes, etc. RAN2 should further discuss the method to achieve early identification on non-SBFD ROs.

Proposal 12: RAN2 to support early identification of SBFD-aware UE during Msg1 transmission on non-SBFD ROs.

Msg1 repetition in SBFD ROs

RAN1#118 has made the following agreement regarding Msg1 repetition:

Agreement For SBFD aware UEs, at least support the following: - PRACH transmission with preamble repetitions within additional-Ros (not across additional-Ros and legacy Ros) Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-Ros and legacy Ros) as in legacy releases.

It can be seen that Msg1 repetition in SBFD ROs is supported in RAN1, so RAN2 should discuss solutions and specification impacts on this aspect.

The potential discussion point includes whether to support Msg1 repetition in SBFD ROs in CFRA and/or in CBRA , how to determine the Msg1 repetition number in SBFD ROs in CFRA and/or CBRA, whether/how to support Msg1 repetition number fallback in SBFD ROs in CBRA, etc.

Proposal 13: RAN2 to further discuss the solutions related to Msg1 repetition in SBFD ROs, including:

- Whether to support Msg1 repetition in SBFD ROs in CFRA and/or in CBRA;

- How to determine the Msg1 repetition number in SBFD ROs in CFRA or in CBRA;

- Whether/how to support Msg1 repetition number fallback in SBFD ROs in CBRA.

- Conclusion

In this contribution, we propose the following observation and proposals:

Supported RA events in SBFD RO:

Proposal 1: RAN2 to confirm the working assumption that, random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

SBFD RACH configuration in RRC layer:

Observation 1: RACH configuration Option 1 can provide both valid SBFD ROs and valid non-SBFD ROs.

Observation 2: RACH configuration Option 2 can only provide valid SBFD ROs.

Observation 3: Based on RAN1’s agreement, RAN2 should specify SBFD RACH configuration in the CFRA resource.

Proposal 2: For CBRA, gNB should indicate whether the provided RACH configuration in BWP-UplinkCommon can contain SBFD RO or not, i.e., whether option 1 is available.

Proposal 3: For CBRA, RAN2 considers the following RRC configurations of the RACH configuration option 2:

- Add a SBFD RACH configuration parallel to RACH-ConfigCommon in BWP-UplinkCommon, and add the SBFD RACH configuration in each AdditionalRACH-Config;

- Add a list in BWP-UplinCommon, the list contains one or more SBFD RACH configurations.

Proposal 4：For CBRA, gNB should be allowed to send legacy RACH configuration (only containing non-SBFD RO), single RACH configuration (i.e., option 1, containing SBFD RO and non-SBFD RO) and separate RACH configuration (i.e., option 2, only containing SBFD RO) in BWP-UplinkCommon.

Proposal 5: For CFRA, gNB should explicitly indicate the SBFD RACH configuration type (option 1 or option 2) and/or RO type (SBFD RO or non-SBFD RO) in each provided CFRA RACH configuration.

Proposal 6: RAN2 confirms that RACH configuration for SBFD-aware UE should only be configured in NUL carrier, no matter the SBFD RACH configuration is in SIB or in dedicated signaling.

SBFD RA procedure in MAC layer:

Observation 4: Since single RACH configuration can provide valid SBFD RO and valid non-SBFD RO, if single RACH configuration is selected, UE should further selects SBFD RO or non-SBFD RO to initiate RA procedure.

Observation 5: Always selecting the nearest RO may not be a good way when SBFD RO is introduced.

Proposal 7: For SBFD-aware UE, MAC spec should specify UE behavior to select between legacy RACH configuration (only containing non-SBFD RO), single RACH configuration (i.e., option 1, containing SBFD RO and non-SBFD RO) and separate RACH configuration (i.e., option 2, only containing SBFD RO), if all of them are provided in CBRA.

Proposal 8: For SBFD-aware UE, MAC spec should specify UE behavior to select between SBFD RO and non-SBFD RO, if RACH configuration option 1 is selected.

Proposal 9: RAN2 to further study the RA procedure order of RACH configuration selection/ RO type selection in the MAC spec, e.g., before set selection, or after set selection.

Proposal 10: Support SBFD-aware UE to fallback from SBFD RACH configuration to legacy RACH configuration, and support SBFD-aware UE to fallback from SBFD RO to non-SBFD RO. FFS the detailed fallback procedure of RACH configuration option 1 and RACH configuration option 2.

Early identification of SBFD-aware UE:

Observation 6: Using SBFD resource to transmit Msg3 can reduce latency and increase the UL transmission chance compared to only using legacy U symbol, more importantly, it can also reduce latency of the whole RA procedure.

Observation 7: It is beneficial for gNB to recognize SBFD-aware UE and non-SBFD-aware UE during Msg1, since gNB can schedule SBFD-aware UE to transmit Msg3 on the following SBFD ROs to reduce RA latency.

Observation 8: gNB will need to recognize SBFD-aware UE and non-SBFD-aware UE only in non-SBFD ROs.

Proposal 11: RAN2 supports that SBFD-aware UE is allowed to send Msg3 in SBFD resource.

Proposal 12: RAN2 to support early identification of SBFD-aware UE during Msg1 transmission on non-SBFD ROs.

Msg1 repetition in SBFD ROs:

Proposal 13: RAN2 to further discuss the solutions related to Msg1 repetition in SBFD ROs, including:

- Whether to support Msg1 repetition in SBFD ROs in CFRA and/or in CBRA;

- How to determine the Msg1 repetition number in SBFD ROs in CFRA or in CBRA;

- Whether/how to support Msg1 repetition number fallback in SBFD ROs in CBRA.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, 2024-06

- TS38.300-i20, ‘NR and NG-RAN Overall Description; Stage 2’, Release 18, 2024-06

- R1-2403789, Final Summary on SBFD random access operation, RAN1#116-bis, CMCC, 2024-04

- R2-2407572, Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD, RAN2#127, CATT, 2024-08

- 1 118 Final Summary on SBFD random access operation, RAN1#118, CMCC, 2024-08

- R1-2405656, Final Summary on SBFD random access operation, RAN1#117, CMCC, 2024-05

- TS38.321-i30, ‘Medium Access Control (MAC) protocol specification’, Release 18, 2024-09

- R1-2401678, Final Summary on SBFD random access operation, RAN1#116, CMCC, 2024-02