3GPP TSG-RAN WG2 Meeting #127bis	R2-2408223

Hefei, China, 14th – 18th Oct 2024

Source:	vivo

Title:	Discussion on functionality LCM for AI/ML enhanced positioning

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

- Introduction

RAN2’s already progresses on functionality-based LCM of UE-sided model for beam management are as follow.

Agreements on definitions - Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) - Applicable functionalities refer to functionalities that the UE is ready to apply for inference - Activated functionalities refers to functionalities already enabled for performing inference Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can be considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality

In this paper, we mainly focus on the overall LCM procedure for UE-sided model of AI/ML positioning, aiming to align with the achievements in LCM for beam management with best effort from the following perspectives:

- Supported functionality reporting

- Applicable functionality reporting

- NW Activation/deactivation of functionality

Besides, the impact on positioning method is also discussed.

- Discussion

## Case 1: UE-sided model for direct AI/ML positioning

Recurring the agreed functionality LCM signaling procedure of UE-sided beam management case in Figure 1, we find there are mainly three parts to constitute the whole identified procedure. To be specific, Step 1/2 are related to UE supported AI/ML functionality transfer, Step 3/4/5 related to UE flexible reporting applicable AI/ML functionalities and NW provides corresponding configuration for such applicable functionalities, and Step 6 is related to following functionality management. In the aim of achieving a common design for beam management and positioning accuracy, we would like to consider whether we can reuse these partitions on signaling in positioning accuracy enhancement case as well.

Figure 1: signaling procedure for LCM of UE-sided model for Beam Management

### Supported functionality reporting

Principally, positioning is in general taken charged by LMF in the normal positioning session, where it receives LCS request from a LCS client or AMF containing the positioning request of the target UE; or it receives LCS request from UE itself containing the request of location estimation or just the positioning assistance data [2]. As the charging role in the positioning session, LMF has to interact with UE and gNB to achieve positioning procedure.

For the case of UE-sided model, there can be two realizations of AI/ML enhanced positioning procedure. The first one is that UE only requires LMF with DL positioning assistance data for autonomous self-location, and UE can perform functionality LCM itself. In this case, AI/ML would expose no signaling specification impact other than possibly newly-introduced positioning assistance data as displayed in Figure 2(a).

(a) UE autonomous self-location                     (b) UE-based positioning

Figure 2: Two approaches for Case 1

Figure 2(b) shows that LMF decides UE-based positioning is utilized for the positioning session. If AI/ML is in use, it should also be LMF that decides to enable and manage the LCM. For instance, in use case 1, LMF should be informed that UE has model to support. In this way, LMF is able to configure UE with positioning assistance data aiming to assist UE-sided model for direct AI/ML positioning and to control of functionality-based LCM.

- For UE autonomous self-location in Case 1, target UE can decide to perform direct AI/ML positioning without NW awareness, i.e., no specification impact from RAN2 perspective.

- For LMF decision on UE-based positioning in Case 1, LMF should be informed of UE supported functionalities to decide whether direct AI/ML positioning to be performed.

As illustrated in Figure 1, the supported functionality of beam management case is indicated in RRC capability. The supported functionality for positioning case can also be indicated in LPP capability.

- LMF requests about UE supported functionality via LPP Request Capability; UE indicates its supported functionalities of AI/ML direct positioning for UE-based positioning via LPP Provide Capability.

### Applicable functionality reporting

After supported functionality interaction, it comes to the further selection on applicable functionalities considering the current conditions. Since AI/ML model locates at UE side, it is more appropriate to allow UE with responsibility for applicability decision.

In this understanding, the network should inform its additional conditions to UE to ensure consistency between training and inference. Due to the unawareness of NW-sided additional condition at the current stage, we itemize two options for NW-additional conditions transfer.

- Option 1: If NW-side additional condition is associated to legacy positioning assistance data existed in LPP, e.g., TRP/ARP information, PRS configurations, the NW-side additional condition can be provided along/through the assistance data.

- Option 2: Reuse LPP Request Capability to contain NW-side additional condition as the trigger of the succeeding applicable functionality reporting in LPP Provide Capability.

Detailed NW-side applicable condition is to be identified by RAN1.

- To assist UE deciding the applicable functionalities, LMF may provide NW-side additional condition (to be identified by RAN1) to UE via two candidate LPP signaling:

- Option 1: LPP Provide Assistance data;

- Option 2: LPP Request Capability.

After informed of NW-side additional condition, UE can decide on its applicable functionality and can further inform LMF. Unlike UAI in RRC, there is no individual signaling in LPP to indicate NW about UE’s preference. However, the essence of LPP capability is relatively dynamic, which can change and transmit to LMF in any stage of positioning. In such understanding, it is appropriate to report applicable functionality still via LPP Provide Capability to better integrate with the current LPP messages, i.e., no new LPP message is introduced. The detailed stage 3 design can be two different IEs of supported and applicable functionalities referred to the same fields.

- UE indicates applicable functionalities via LPP Provide Capability.

Like Step 5 in functionality LCM for beam management, the further inference configuration related to applicable functionalities, if any, can be provided to UE. Since there is no agreed content of inference configuration, we also discuss two signaling options for inference configuration.

- Option 1: If the inference configuration are related to the requirement of inference, e.g., inference periodicity (which may finally equal reporting periodicity), LPP Request Location Information can be considered as a trigger of inference and positioning measurement.

- Option 2: If the configurations have relationship with other assistance data, such as TRP, ARP or PRS configuration, LPP Provide Assistance Data should be appropriate to include NW configurations.

- LMF may provide inference configuration (if the content is identified by RAN1) to UE after applicable functionality reporting via two candidate LPP signaling:

- Option 1: via LPP Provide Assistance data;

- Option 2: via LPP Request Location Information.

Apart from that, since UE owns AI/ML model/functionality, it has the knowledge of configurations during training stage. When it comes to the stage of inference, to ensure the consistency of training and inference, UE may request for suitable PRS configurations via UE-initiated on-demand PRS scheme. In this way, when assigning inference configurations, LMF can forward such preference to gNB, and the consistency can be ensured.

Furthermore, for some scenarios, LMF does not provide NW-side additional condition to UE during applicable functionality reporting. However, UE may provide its knowledge about NW-side additional condition at the training stage via the applicable functionality reporting. That is, LMF can recognize the PRS configurations for model training and further recommend to gNB accordingly. LMF-initiated on-demand PRS scheme can therefore be performed to ensure consistency.

- On-demand PRS scheme can be utilized to ensure consistency between training and inference.

### Activation/deactivation of functionality

Summarized from the RAN1 agreement, there can be three options of management for positioning use case 1.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

- Option 1: UE to decide whether to enable functionality, UE to perform performance monitoring

With no LMF perception of AI/ML enabling, UE autonomously decides to enable AI/ML based positioning, UE reports AI/ML output of location prediction as the legacy location information.

However, since RAN2 has agreed that “’UE-autonomous, UE’s decision is not reported to the network’ is not considered for Rel-19” and the merits of NR positioning system, the first situation is excluded from work scope.

- Option 2: LMF to decide whether to enable functionality, UE to perform performance monitoring

Based on RAN1 input, LMF may provide some assistance information to UE to achieve performance monitoring at UE, e.g., ground truth label of the target UE, position calculation assistance data, PRU measurement or PRU location. UE can also obtain label from PRU in 3GPP-transparent way, which has no specification impact and is outside RAN2 scope.

- UE performing performance monitoring requires LMF to provide ground truth label, position calculation assistance data, PRU measurement or PRU location.

Logically speaking, if LMF configures UE to perform performance monitoring, or UE itself has intention to perform performance monitoring, the information can be provided via LPP Assistance Data Transfer procedure in both proactive/reactive way.

- Option 3: LMF to decide whether to enable functionality, LMF to perform performance monitoring.

Based on RAN1 input, to achieve performance monitoring at LMF, the model output is needed from UE. Additionally, UE also needs PRU measurement from LMF.

Since the model output is location estimate, which is already the location information request by LMF as in the legacy positioning session, this can directly reuse LPP Provide Location Information to transmit. And for the PRU measurement part, as mentioned above, LPP Provide Assistance Data can be reused.

- LMF performing performance monitoring requires UE to provide model inference output; which may additionally require LMF to provide UE with PRU measurement.

- To achieve performance monitoring at UE side, target UE obtains assistance data (to be concluded by RAN1) from LMF via LPP Request/Provide Assistance Data in proactive or reactive way.

- To achieve performance monitoring at LMF side, LMF obtains model inference output from UE via LPP Request/Provide Location Information in proactive/reactive way.

On the basis of the above analysis, LMF or UE is able to perform performance monitoring. LMF can be of long-term decision while UE can make real-time to decide whether AI/ML positioning accuracy is trustworthy. However, RAN1 has been parallelly discussing entities to perform performance monitoring, we would like RAN2 to wait for RAN1 progress on whether both options should be introduced in specification.

- RAN2 to wait further RAN1 decision on whether UE and/or LMF to perform performance monitoring.

Regarding the functionality management, whether there is explicit signaling to indicate (de)activation should be dependent to RAN1 Reply LS aiming at the unified design for all the use cases.

- Wait for RAN1 Reply LS to decide whether there is explicit functionality activation/deactivation.

    - Overall Signaling for LMF control of UE-sided model

Based on our understanding for positioning use case 1, we illustrate the logical procedure among UE, gNB and LMF in Figure 3 to enable direct AI/ML positioning with UE-sided model.

Figure 3: Signaling procedure of functionality-level LCM for case1 in positioning accuracy enhancement

- LMF may request and acquire TRP information from gNB. - LMF may request and acquire UE supported functionality of direct AI/ML positioning functionality via LPP capability transfer from UE. - LMF determines to enable AI enhanced positioning accuracy with UE-based positioning with model possessed by UE. - LMF and gNB negotiate about DL-PRS configuration. - LMF provides UE with NW additional conditions (if identified) and other assistance data. - With or without request from LMF, UE may provide its applicable functionality to LMF via LPP Capability transfer. - LMF may further provide UE with inference configuration considering applicability. - LMF requests UE to perform DL measurement and request about location estimation. - UE performs DL-PRS measurement. - Based on DL-PRS measurement, UE performs model inference and outputs predicted location. - UE reports predicted location to LMF. - LMF/UE performs performance monitoring. - If LMF performs performance monitoring, LMF should indicate UE about performance metrics or whether current model is applicable to help UE with model-level LCM. Besides, NW can also realize functionality activation/deactivation via assistance data provision.

    - Impact on positioning method

In consideration to the specialty of positioning protocol organization, the reporting and request are organized in a per positioning method way. That is, except for some commonly used IEs which are defined outside the scope of any method but referred in each method, other IEs are designed method specifically.

With regards to introduction of AI/ML based positioning, some newly added IEs are being discussed by RAN1 currently. What RAN2 should focus on is that how to integrate AI/ML enhancement with legacy positioning protocol design.

- RAN2 to decide how to integrate AI/ML positioning with the current positioning methods:

- Option 1: Introduce AI/ML positioning as new positioning method;

- Option 2: Merge AI/ML specific information in the existing related positioning method(s).

## Case 3a/3b: NW-sided model

Due to the parallel discussion with other WGs (e.g., RAN3, SA2) on the interaction procedure for positioning use cases 3a and 3b, we suggest to wait for more inputs from other WGs before RAN2’s further investigation.

- RAN2 to wait other WGs for further progress on positioning use cases 3a/3b.

- Conclusion

In summary, we make the following proposals on functionality-based LCM for AI/ML enhanced positioning:

Supported functionality reporting

- For UE autonomous self-location in Case 1, target UE can decide to perform direct AI/ML positioning without NW awareness, i.e., no specification impact from RAN2 perspective.

- For LMF decision on UE-based positioning in Case 1, LMF should be informed of UE supported functionalities to decide whether direct AI/ML positioning to be performed.

- LMF requests about UE supported functionality via LPP Request Capability; UE indicates its supported functionalities of AI/ML direct positioning for UE-based positioning via LPP Provide Capability.

Applicable functionality reporting

- To assist UE deciding the applicable functionalities, LMF may provide NW-side additional condition (to be identified by RAN1) to UE via two candidate LPP signaling:

- Option 1: LPP Provide Assistance data;

- Option 2: LPP Request Capability.

- UE indicates applicable functionalities via LPP Provide Capability.

- LMF may provide inference configuration (if the content is identified by RAN1) to UE after applicable functionality reporting via two candidate LPP signaling:

- Option 1: via LPP Provide Assistance data;

- Option 2: via LPP Request Location Information.

- On-demand PRS scheme can be utilized to ensure consistency between training and inference.

NW Activation/deactivation of functionality

- UE performing performance monitoring requires LMF to provide ground truth label, position calculation assistance data, PRU measurement or PRU location.

- LMF performing performance monitoring requires UE to provide model inference output; which may additionally require LMF to provide UE with PRU measurement.

- To achieve performance monitoring at UE side, target UE obtains assistance data (to be concluded by RAN1) from LMF via LPP Request/Provide Assistance Data in proactive or reactive way.

- To achieve performance monitoring at LMF side, LMF obtains model inference output from UE via LPP Request/Provide Location Information in proactive/reactive way.

- RAN2 to wait further RAN1 decision on whether UE and/or LMF to perform performance monitoring.

- Wait for RAN1 Reply LS to decide whether there is explicit functionality activation/deactivation.

Impact on positioning method

- RAN2 to decide how to integrate AI/ML positioning with the current positioning methods:

- Option 1: Introduce AI/ML positioning as new positioning method;

- Option 2: Merge AI/ML specific information in the existing related positioning method(s).

Case 3a/3b

- RAN2 to wait other WGs for further progress on positioning use cases 3a/3b.

- Reference

- RAN2#127 meeting Chair’s Notes

- TS 23.273	5G System (5GS) Location Services (LCS)