3GPP TSG-RAN WG2 Meeting #127bis		R2-2408224

Hefei, China, 14th – 18th Aug 2024

Source:	vivo

Title:          Discussion on NW side data collection framework

Agenda Item:	8.1.3

Document for:	Discussion and Decision

# Introduction

At the RAN2#126 and RAN2#127 meeting, on NW side data collection for beam management, RAN2 made the following agreements, respectively:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network.

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

In this contribution, we will further discuss some remaining issues on data collection frameworks for NW-side model training for beam management.

# Discussion

Enhancements of immediate MDT may require UE to log a certain amount of measurement data before reporting them. Consequently, the reporting enhancement may also be frequent enough to adapt the collection requirement. Thus, it would impose some complexity on UE in terms of memory constraint, signaling overhead, etc. Considering how to reduce/minimize UE power consumption and signaling overhead, UE may be allowed its preference on enhanced MDT data logging/reporting, therefore:

- For the enhancement of immediate MDT to support data logging, the following aspects should be taken into consideration:

- UE memory constraint: i.e., logging of data should be minimized to avoid excessive memory utilization.

- Signaling complexity, i.e., for both gNB-centric and OAM-centric, like current MDT procedure, UE is not expected to be configured with different data collection processes for gNB and OAM.

During measurements for data collection purposes and logging based, UE may run out of power or UE may identify that its power state is low to some critical level. In such cases, UE may autonomously stop measurement and data logging. In addition, the UE should report the status to the network to inform that the data collection has been stopped and the UE is not expected to receive new data collection configuration from Netwok.

Therefore, we propose:

- Based on UE power state UE can autonomously stop data measurement and logging and reporting to network.

In case multiple configurations are provided to the UE, if dynamic activation/deactivation is support, should be discussed and decided by RAN1. Thus, from RAN2 perspective, there is no need for dynamic activation/deactivation requirement.

- From RAN2 perspective there is no need to support dynamic activation/deactivation for multiple data collection configurations.

RAN2 has already agreed to enhance the immediate MDT framework to support periodical reporting. On whether and what event-based reporting is supported and FFS on network request, as NW can configure UE with periodical reporting, there is no need to consider event-based reporting, e.g., AS buffer event-based reporting.	When AS buffer is full, UE can indicate it to network, and then the network can request for the logged data.

- There is no need to support event-based reporting of enhanced MDT report, e.g., AS buffer event-based reporting.

- When AS buffer is full, UE can indicate to Network that its buffer is full.

When configured with periodical reporting, to facilitate data filtering and reduce data logging UE can perform event triggered logging, if any, within the reporting period. However, event-triggered logging does not necessarily mean event-based reporting. That is, event-triggered logged data can be reported in the periodical Report.

- To facilitate data filtering and reduce data logging, event-triggered data logging should be supported. And the event-triggered data logging can be sent by periodical reporting.

Based on RAN2 previous progress and the above discussion, a procedure of data logging and reporting can be considered as in below Proposal 7:

- Baseline procedure of enhanced immediate MDT for NW-sided data collection:

- Step 1: gNB configures enhanced immediate MDT (instances of logged L1 measurement) to a UE in RRC_CONNECTED state via a RRC message for data logging and event-based logging:

- Step 2: Upon reception of enhanced immediate MDT configuration, the UE stores the configuration, starts performing instances of logged L1 measurements.

- Step 3: The UE determines that reporting of measurements is triggered, including periodically, or requested by gNB, i.e.:

  - Periodic reporting (including of event triggered logged data)

  - Requested by NW based on UE indication of the stored enhanced immediate MDT data availability.

- Step 4: The UE reports the stored instances of logged L1 meas to NW via RRC message.

# Conclusion

This contribution concludes with the following:

Data logging and UE power state

- For the enhancement of immediate MDT to support data logging, the following aspects should be taken into consideration:

- UE memory constraint: i.e., logging of data should be minimized to avoid excessive memory utilization.

- Signaling complexity, i.e., for both gNB-centric and OAM-centric, like current MDT procedure, UE is not expected to be configured with different data collection processes for gNB and OAM.

- Based on UE power state UE can autonomously stop data measurement and logging and reporting to network.

Dynamic activation/deactivatione

- From RAN2 perspective there is no need to support dynamic activation/deactivation for multiple data collection configurations.

Event-based logging/reporting

- There is no need to support event-based reporting of enhanced MDT report, e.g., AS buffer event-based reporting.

- When AS buffer is full, UE can indicate to Network that its buffer is full.

- To facilitate data filtering and reduce data logging, event-triggered data logging should be supported. And the event-triggered data logging can be sent by periodical reporting.

Baseline procedure of enhanced immediate MDT

- Baseline procedure of enhanced immediate MDT for NW-sided data collection:

- Step 1: gNB configures enhanced immediate MDT (instances of logged L1 measurement) to a UE in RRC_CONNECTED state via a RRC message for data logging and event-based logging:

- Step 2: Upon reception of enhanced immediate MDT configuration, the UE stores the configuration, starts performing instances of logged L1 measurements.

- Step 3: The UE determines that reporting of measurements is triggered, including periodically, or requested by gNB, i.e.:

  - Periodic reporting (including of event triggered logged data)

  - Requested by NW based on UE indication of the stored enhanced immediate MDT data availability.

- Step 4: The UE reports the stored instances of logged L1 meas to NW via RRC message.

# Reference

[1] R2-2306906	LS out on Data Collection Requirements and Assumptions

[2] R2-2309435	Reply LS on Data Collection Requirements and Assumptions

[3] R1-2310681	Reply LS on Data Collection Requirements and Assumptions