3GPP TSG-RAN WG2 Meeting #127bis	R2-2408225

Hefei, China, Oct 14th – 18th, 2024

Source:	vivo

Title:	Further analysis of model transfer/delivery

Agenda Item:	8.1.5

Document for:	Discussion and Decision

- Introduction

During the SI phase in Rel-18, RAN2 discussed the following solutions of model transfer/delivery and captured the analysis in section 7.2.1.4 of TR 38.843 [1].

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP). - Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

At the RANP#105 meeting, the study of model transfer/delivery was agreed to be extended till the end of Rel-19.

In this contribution, we will provide further analysis of the solutions of model transfer/delivery.

- Discussion

## Model delivery/transfer solutions

### 2.1.1 Further clarification on solution 4a

Considering different combinations of model format and functionality location, the following model delivery/transfer cases were introduced by RAN1 in TR 38.843.

Table 4.3-1: Model delivery/transfer cases Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top.; Outside 3GPP Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format.; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format.; 3GPP Network; NW-side. z3; model transfer in open format.; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support.; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known.; 3GPP Network; NW-side. Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization..

From the above table, it can be observed that the model can be stored either within the 3GPP network or outside the 3GPP network. For case y, the model is stored outside the 3GPP network, and the corresponding solution is model delivery Solution 4a. In this case, model delivery is transparent to 3GPP.

Furthermore, the definitions of model transfer and model delivery are as follows, according to the definitions, when the model is sent from an entity outside the 3GPP network, it cannot be classified as model transfer.

AI/ML model delivery: A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. AI/ML model transfer: Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

Therefore, it is recommended to clarify the definition of Solution 4a as follow:

Proposal 1: Refine the definition of solution 4a as: OTT server can deliver AI/ML model(s) to UE transparently to 3GPP, i.e., solution 4a has no specification impact during model delivery.

The analysis of the current status and gaps, and potential RAN specification impact for Solutions 4a is listed as follows:

Table 7.3.1.4-5 Analysis of current status and gaps, and potential RAN specification impact for Solutions 4a Discussion Area; Current status and Gaps; Potential RAN specification impact. A1. Large, no upper limit model/model parameter size; No model size limitation; No RAN impact. A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs); - If model transfer/delivery from OTT server via Core Network, supported - If model transfer/delivery from OTT server via LMF, depends on Rel-18 CT1 solution LPP message over a User Plane connection between UE and LMF; Note: supporting service continuity across LMF is out of RAN scope. A3. Network controllability on model transfer/delivery and management at gNB; Model transfer/delivery is transparent to RAN; - Requires management and model transfer interaction between OTT server and gNB when model management at gNB - Note: it is unclear whether this is within RAN scope - Requires interaction between UE and gNB for the network controllability of the model transfer/delivery (e.g., model identification, model transfer completion, etc.) if management is in gNB. A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB); - Procedure latency depends on model size, QoS requirement and DRB priority - Other latency includes forwarding data from OTT server to gNB; Note: The detail QoS requirement for model transfer/delivery of solution 4a is out of RAN scope.

Regarding aspect A2, if the model is firstly delivered from the OTT server to the LMF, and then transferred from LMF to UE, this scenario should be classified as 3b, thus this approach should be removed in solution 4a.

As to aspect A3, currently, the procedure of applicable functionality reporting from UE to gNB has been discussed. With this procedure, there is no need for interaction between the OTT server and the gNB to achieve model management. Furthermore, for solution 4a, the gNB also does not need to control model transfer/delivery, and whether/how to obtain the model is based on the implementation of the UE and OTT server.

Based on the above analysis, it is recommended to update the table 7.3.1.4-5 for Solution 4a:

Proposal 2: Refine the potential RAN specification impact in the analysis table 7.3.1.4-5 for solution 4a:

- Remove the approach that the model is firstly delivered from the OTT server to the LMF, and then transferred from LMF to UE;

- Remove the interaction between gNB and UE/OTT server for controllability of the model delivery.

| Discussion Area | Current status and Gaps | Potential RAN specification impact |
| --- | --- | --- |
| A1. Large, no upper limit model/model parameter size | No model size limitation | No RAN impact |
| A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs) | - If model transfer/delivery from OTT server via Core Network, supported - If model transfer/delivery from OTT server via LMF, depends on Rel-18 CT1 solution LPP message over a User Plane connection between UE and LMF | No RAN impact Note: supporting service continuity across LMF is out of RAN scope |
| A3. Network controllability on model transfer/ delivery and management at gNB | Model transfer/delivery is transparent to RAN | No RAN impact - Requires management and model transfer interaction between OTT server and gNB when model management at gNB - Note: it is unclear whether this is within RAN scope - Requires interaction between UE and gNB for the network controllability of the model transfer/delivery (e.g., model identification, model transfer completion, etc.) if management is in gNB |
| A4. Model transfer/ delivery QoS (for DRB) (including latency, etc.) and priority (for SRB) | - Procedure latency depends on model size, QoS requirement and DRB priority - Other latency includes forwarding data from OTT server to gNB | Note: The detail QoS requirement for model transfer/delivery of solution 4a is out of RAN scope |

### 2.1.2 Further study on the solutions in RAN2 scope

At the previous meetings, RAN2 discussed the mechanisms of data collection for network-side and UE-side model training. Currently, the data collection mechanisms for model training on the NW side have been relatively stable. Specifically, for beam management use cases, the MDT framework can be considered for OAM-centric data collection and model training. For positioning, LPP can be reused for data collection from the UE to LMF.

From our understanding, the model transfer/delivery from NW to UE can be seen as a reverse transfer of network-side data collection. With this understanding, further analysis in RAN2 may start with CP-based solutions related to gNB and LMF

During the SI phase, it was suggested that both reactive and proactive approaches for initiating a model transfer/delivery can be considered in the normative phase.

A reactive and a proactive approach for initiating a model transfer/delivery can be considered in a normative phase. For the reactive approach, an AI/ML model is transferred/delivered (i.e., downloaded) to the UE when needed. This could typically happen due to changes in scenarios, configurations, sites, etc. While for the proactive model transfer/delivery approach, an AI/ML model is pre-download to the UE, and a model switch can typically be performed due to changes in scenarios, configurations, sites, etc.

Based on the typical scenarios, for proactive model transfer/delivery, the AI/ML model can be pre-download to UE in a non-time critical way, i.e., there is no latency requirement for proactive model transfer/delivery; For reactive model transfer/delivery, the AI/ML model can be sent to UE when needed (e.g., upon handover) in a near real-time way, i.e., there is a latency requirement for reactive model transfer/delivery.

Proposal 3: For proactive model transfer/delivery, the AI/ML model can be pre-downloaded to UE in a non-time critical way, i.e., there is no latency requirement for proactive model transfer/delivery.

Proposal 4: For reactive model transfer/delivery, the AI/ML model can be sent to UE when needed in a near real-time way, i.e., there is a latency requirement for reactive model transfer/delivery.

From our understanding, For the candidate Solutions:

- Solution 1a and Solution 3a (CP-based solution) can fulfill the above latency requirements. The biggest concern is the model size and input from RAN1 on the typical model size is needed.

- Solutions 2a/2b/3b/4b are not in the scope of RAN2, or at least related discussion should not first be considered in RAN2, and should be further discussed by SA WGs;

- Solution 1b is quite similar to solution 1a with a new SRB with configurable priority. Considering that massive specification effort is needed to support PDU session termination at gNB, Solution 1b should be deprioritized from RAN2 perspective.

Proposal 5a: Model transfer/delivery CP-based Solution 1a and Solution 3a are in the scope of RAN2 and should be prioritized for further study.

Proposal 5b: LS to RAN1 to ask about the typical model size.

Proposal 6: Model transfer/delivery Solution 1b is deprioritized from RAN2 perspective.

Proposal 7a: Model transfer/delivery solutions 2a/2b/3b/4b are not in the scope of RAN2 and input from SA WGs is needed.

Proposal 7b: LS to SA WGs to ask them to provide analysis on the feasibility of solutions 2a/2b/3b/4b for RAN2 to make the final decision. LS includes the latency requirements from RAN2 perspective.

## Potential procedure for model transfer/delivery case z4

During the Rel-19, RAN1 further discussed the cases, and cases z3/z5 are deprioritized, case z2 is deprioritized for UE-sided model.

R1#116 Conclusion: From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19. R1#116 Conclusion From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons: - Risk of proprietary design disclosure - Burden of offline cross-vendor collaboration Conclusion From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y): - No much benefit compared to Case y - Risk of proprietary design disclosure - Large burden of offline cross-vendor collaboration - Additional burden on model storage within in 3GPP network R1#117 Agreement From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits): - Alt. A - Step A-1: UE reports the supported known model structure(s) to network - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1 - FFS: whether some additional step(s), and/or whether other information is needed - Alt. B - Step B-0: UE reports to NW its support of model transfer/delivery case z4 - Note: Step B-0 may be before or after Step B-1, or not necessary - Step B-1: NW indicates to UE the candidate known model structure(s) - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2 - FFS: whether some additional step(s), and/or whether other information is needed - Note: Other alternative(s) is not precluded - Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion. R1#118 Agreement From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects - Model type/backbone (e.g., Transformer, CNN and so on) - In case model type is a neural network - Number of layers - Layer types/structure (e.g., full connected, activation layer and so on) - Layer size (e.g., the number of parameters of a layer) - Connection between different layers - model input/output related information Conclusion From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported)

For the rest cases, RAN1 provided 2 alternatives of procedures for Model delivery/transfer case z4. Besides, the model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported). To make some progress, RAN2 may discuss the potential procedure/signaling impact on Model delivery/transfer case z4 for CSI compression as a starting point.

Proposal 8: RAN2 may discuss the potential procedure/signaling impact on Model delivery/transfer case z4 for CSI compression as a starting point.

For the two alternatives provided by RAN1, the information in the first step can be provided from UE to gNB via UE capability. The difference is the granularity of information in the UE capability:

- In Alt.A, the information is the detailed supported known model structure(s);

- In Alt.B, the information is an indication on whether support model transfer/delivery case z4. The whole procedure of Alt.B can follow the procedure of applicability funcitionality reporting, i.e, UAI framework can be reused.

Proposal 9: For Alt.A model delivery/transfer Case z4, the supported known model structure(s) can be reported to NW via UE capability.

Proposal 10: For Alt.B model delivery/transfer Case z4:

- The support of model transfer/delivery case z4 can be reported to NW via UE capability.

- UAI framework can be used to exchange the candidate and supported known model structure(s).

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Further clarification on solution 4a

Proposal 1: Refine the definition of solution 4a as: OTT server can deliver AI/ML model(s) to UE transparently to 3GPP, i.e., solution 4a has no specification impact during model delivery.

Proposal 2: Refine the potential RAN specification impact in the analysis table 7.3.1.4-5 for solution 4a:

- Remove the approach that the model is firstly delivered from the OTT server to the LMF, and then transferred from LMF to UE;

- Remove the interaction between gNB and UE/OTT server for controllability of the model delivery.

Further study on the solutions in RAN2 scope

Proposal 3: For proactive model transfer/delivery, the AI/ML model can be pre-downloaded to UE in a non-time critical way, i.e., there is no latency requirement for proactive model transfer/delivery.

Proposal 4: For reactive model transfer/delivery, the AI/ML model can be sent to UE when needed in a near real-time way, i.e., there is a latency requirement for reactive model transfer/delivery.

Proposal 5a: Model transfer/delivery CP-based Solution 1a and Solution 3a are in the scope of RAN2 and should be prioritized for further study.

Proposal 5b: LS to RAN1 to ask about the typical model size.

Proposal 6: Model transfer/delivery Solution 1b is deprioritized from RAN2 perspective.

Proposal 7a: Model transfer/delivery solutions 2a/2b/3b/4b are not in the scope of RAN2 and input from SA WGs is needed.

Proposal 7b: LS to SA WGs to ask them to provide analysis on the feasibility of solutions 2a/2b/3b/4b for RAN2 to make the final decision. LS includes the latency requirements from RAN2 perspective.

Potential procedure for model transfer/delivery case z4

Proposal 8: RAN2 may discuss the potential procedure/signaling impact on Model delivery/transfer case z4 for CSI compression as a starting point.

Proposal 9: For Alt.A model delivery/transfer Case z4, the supported known model structure(s) can be reported to NW via UE capability.

Proposal 10: For Alt.B model delivery/transfer Case z4:

- The support of model transfer/delivery case z4 can be reported to NW via UE capability.

- UAI framework can be used to exchange the candidate and supported known model structure(s).

- Reference

- TR 38.843	Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface