3GPP TSG-RAN WG2 Meeting #127bis	R2-2408226

Hefei, China, Oct 14th – 18th, 2024

Source:	vivo

Title:	Discussion on functionality aspects for Ambient IoT

Agenda Item:	8.2.2

Document for:	Discussion and Decision

# Introduction

In the previous RAN2#126 and RAN2#127 meetings [1] [2], the following agreements have been reached for functionality aspects:

RAN2#126 Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed.

4	AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed).

RAN2#127 Agreements on functionality

1	Send an LS to RAN1 regarding TBS size.  Ask SA2 on the max and typical upper layer packet size..

2	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

3	RAN2 will study the need and use of a simple message “size” reporting to the reader.

4	At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

5	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

In this contribution, we will further discuss the remaining issues based on the above agreements, and provide our views respectively.

# Discussion

## Device AS Short ID Management

In the post email discussion on random access [3], the usage of device AS Short ID is still under discussing. According to our analysis, an AS short ID, e.g. 16-bit, is necessary for both CBRA and CFRA at least for purpose of subsequent scheduling. Moreover, it is suggested to consider reader control of AS Short ID allocation for all RA types as following:

- the AS short ID is randomly selected by device and confirmed by reader (in CBRA case); or

- the AS short ID is allocated by reader directly (in CFRA case), which is unique in the range of a reader.

Moreover, the AS short ID is somewhat similar with C-RNTI for legacy RRC_CONNECTED UE. The difference is that legacy UE has RRC connection management procedure to manage C-RNTI, e.g. release it when back to IDLE, but AIoT device has no RRC state and connection concept. It should be addressed how to release the AS short ID after this ID is no longer used. Explicit release indication from reader is the most straightforward solution, e.g. upon reader reception of ending indication from CN. Furthermore, the AS short ID can also be released based on a pre-defined/configured timer expiry. The timer will be restarted upon each R2D/D2R transmission and separate timers can be maintained at both device and reader side. When timer expires, the AS short ID is released respectively at device and reader side.

- The Device AS short ID allocation and uniqueness is under reader control, i.e., the AS short ID is randomly selected by device and confirmed by reader (in CBRA case), or is allocated by reader directly (in CFRA case).

- The Device AS short ID can be released on a timer expiry or explicit indication from reader.

## Device type/capability reporting

According to RAN1#116 agreement, three device type has been agreed as below.

Agreement For the purpose of the study, RAN1 uses the following terminologies: - Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device.

In RAN2#126 meeting, there was initial discussion on the assumptions on the need of device type and device capability reporting but with no consensus. Although some companies think that it may be too early to conclude the necessity of device type and device capability reporting, we believe that some guideline on these aspects is still beneficial to facilitate RAN2 future study. Moreover, since our study strives for harmonized air interface design for all device types, any functionality if required is better to be designed commonly for all device types as much as possible. Hence, to achieve the harmonized design, it is proposed that RAN2 study should aim not to support device type/capability information reporting procedure as baseline.

- RAN2 study should aim at solutions that do not require device type and device radio capability information reporting from device to reader as baseline.

## Segmentation

In RAN2#127 [2], there were the following agreements regarding segmentation:

Agreements

-	Send an LS to RAN1 regarding TBS size.  Ask SA2 on the max and typical upper layer packet size.

According to the above agreements, a LS was sent to RAN1 and SA2 to ask for input to help RAN2 determine the necessity of segmentation procedure for R2D and D2R transmission. During the period until the response LS coming from SA2/RAN1, RAN2 can discuss the candidate way-forwards for segmentations so that RAN2 can identify a feasible segmentation scheme once segmentation is determined to be necessary according to the response LS(s) from SA2/RAN1.

From the contributions in the previous meetings, there are two types of candidate way-forwards:

- MAC based segmentation: rely on AIoT MAC to perform segmentation according to the message size and the radio capacity of the AIoT air interface, wherein the radio capacity of the AIoT radio interface depends on the radio quality and the radio resource availability etc. The example flow chart for MAC based segmentation is illustrated in Figure 1, wherein the brief procedure is provided below:

  - Step 1: the CN transmits the service request for transmitting a R2D/D2R message to the reader;

  - Step 2: Upon reception of the service request, the reader pages the target AIoT device for radio link setup via an inventory procedure;

  - Step 3: The reader transmits the service request to the target device; either via the message size from the service request or the message status information reported by the AIoT device, the reader can schedule the AIoT device to transmit the R2D/D2R message piece via respective scheduling information:

    - 3-1. The reader transmits the service request and schedules the AIoT device to transmit/receive the 1st D2R/R2D message segment;

    - 3-2. The reader schedules the AIoT device to transmit/receive the 2nd D2R/R2D message segment;

    - …

    - 3-M. The reader schedules the AIoT device to transmit/receive the M-th (M<=N, N is the required number of segments in CN based segmentation) D2R/R2D message segment;

Note: due to that radio condition, resource availability etc are well considered in segment size determination in MAC segmentation procedure, optimal segment sizes will be adapted to and the required number of segments for transmission of the same message may be probably smaller than that in CN based segmentation.

  - Step 4: Once the operation for the transmission of the n-th piece of the R2D/D2R message is finished, the reader transmits the associated response message to the CN.

Figure 1 Flow chart example of MAC based segmentation

- CN based segmentation: rely on CN or app server to perform segmentations if the R2D/D2R message size is too big to be transmitted in one shot in the AIoT air interface, for which the example procedure is illustrated in Figure 2. In this figure, it is assumed that the R2D/D2R message is to be transmitted in N pieces and N respective service procedures are carried out to transmit these N pieces, wherein each service procedure is for transmitting one piece. The n-th service procedure is described below as an example:

  - Service procedure n: transmit the n-th (1≤n≤N) piece of the R2D/D2R message

    - Step n-a: A service request is generated to transmit the first piece of the R2D/D2R message and transmit the service request to the reader;

    - Step n-b: Upon reception of the service request, the reader pages the target AioT device for radio link setup via an inventory procedure;

    - Step n-c: via the radio link which has been setup via the inventory procedure, the reader transmits the service request to the target device via the radio link; and the reader and the target AioT device perform the transmission of the n-th piece of the R2D/D2R message according to the service request.

    - Step n-d: once the operation for the transmission of the n-th piece of the R2D/D2R message is finished, the reader transmits the associated response message to the CN.

Figure 2 Flow chart example of CN based segmentation

- RAN2 to consider the following two candidate way-forwards for segmentation:

  - CN based segmentation: rely on CN or app server to perform segmentations if the message size is too big to be transmitted in one shot;

  - MAC based segmentation: relay on AIoT MAC to perform segmentation according to the message size, radio condition, radio resource availability etc.

For MAC based segmentation, there can be various options for scheduling transmissions (Step 3-a~M) of segments.  two candidate schemes are provided below:

- Automatic segmentation: When the message size is larger than the TB size, the transmitter (reader or device) performs segmentation automatically and uses a segment index associated with the segment to indicate that a segment is transmitted. The receiver identifies the segment according to segment index. Assuming the segments are always sequentially transmitted, since there is no disorder caused by HARQ, single bit to indicate the end segment or non-segment can be enough for receiver (device or reader) to recover the full msg.

- Scheduled segmentation: When the reader determines that segmentation is required, the reader includes a segment indication (e.g. a memory address offset from the start of the message storage) in the scheduling information to configure the AIoT device to receive or transmit the message piece by piece. After transmission of one message segment, the reader shifts the memory address offset and include the new memory address offset for scheduling the transmission of the next message segment, until the full message is transmitted.

- RAN2 considers the following two candidate schemes for MAC segmentation:

  - Automatic segmentation: the transmitter (reader/device) performs segmentation when the message size is larger than the TB size, and includes the segment indication (e.g.  end or non-end segment) in the transmission.

  - Scheduled segmentation: the reader schedules the AIoT device to transmit/receive one msg piece by piece with segment indication (e.g. a memory address offset) in the scheduling information.

According to the respective example flow charts illustrated in Figure 1 and Figure 2 and the above discussions, the CN based segmentation scheme and the MAC based segmentation scheme are compared in different aspects and the results are summarized in Table 1 below.

Table 1 Comparison between CN based and MAC based segmentation for one AIoT service

|  | CN based segmentation | MAC based segmentation |
| --- | --- | --- |
| Number of service procedure | Multiple respective service procedures wherein each R2D/D2R segment transmission requires one individual service procedure | One |
| Number of paging/inventory procedure | Multiple respective paging/inventory procedures wherein each segment transmission requires one individual paging/inventory procedure | one |
| Link adaptation considering radio condition and resource availability | No or pool due to that CN does not own timely radio condition or resource availability information of AIoT air interface. The CN may have to always determine small segment size in order to get through with the poor radio condition in AioT air interface. | Good. The reader can determine the optimal segment size according to the radio condition and resource availability in AIoT air interface, thanks to which MAC based segmentation may adopt a relatively smaller number of segments. |
| Service delay | Large. | much smaller than CN based segmentation. |
| CN impact | Yes. CN needs to be equipped with segmentation procedure performs the segmentation for both R2D and D2R transmission. | No |
| RAN impact | No. Probably transparent to AIoT RAN. | Yes. Segmentation is implemented in AIoT MAC |
| Buffer/memory for D2R direction | NO. | NO. For cyphering process, the transmitter processes the data in bitstream, regardless of whether there is segmentation. For integrity protection, the device can carry the related process based on the raw message in the storage upon the MAC PDU construction of the end segment, for which the required buffer is not larger than that without segmentation. |

Based on Figure 1 and Figure 2 flow charts, and Table 1, there are the following observations:

- CN based segmentation requires one service procedure per segment, which comprising one service request, one inventory procedure, one reader-AIoT device data transmission procedure and one response message to CN.

- MAC based segmentation requires only one service procedure for the all segments of one upper layer message, which comprises: one service request, one inventory procedure, multiple respective scheduled transmissions for all segments in AIoT air interface, and one response message to CN.

- MAC based segmentation can use optimal segment size based on radio condition and resource availability while CN based segmentation has to use conservative segment size due to lack of timely radio condition and radio resource availability information.

- CN based segmentation is expected to have much larger service delay and overhead than MAC based segmentation due to CN based segmentation requires more signalling procedures.

- With MAC segmentation for D2R transmission, the required buffer size at the AIoT device for cyphering and integrity protection related processes is not larger than that without MAC segmentation.

Based on the above discussion and observations, we can conclude that MAC based segmentation scheme outperforms the CN based segmentation scheme:

- MAC based segmentation should be considered as a work assumption.

## Service request visibility to reader

The visibility to reader regarding service request was discussed in RAN2#127[2], and there were the following agreements:

Agreements

- At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

2	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

According to above agreements, service type information (inventory or command) visibility can be helpful for the reader while whether command type information (e.g. read/write/disable) is also helpful is still an open issue. From our understanding, different command types correspond to different AS procedures, for instance:

- The corresponding AS procedure for read command: the reader should schedule the D2R transmission with adequate radio resources;

- The corresponding AS procedure for write command: the AS procedure should schedule the R2D transmission with adequate radio resources and certain radio resource may be allocated for an associated D2R response from the AIoT device to the reader;

- The corresponding AS procedure for disable command: more robust transmission in AIoT interface will be scheduled to avoid uncertainty due to command transmission failure, and the read may need to allocate radio resource for the AIoT device to respond to the reader/CN regarding the disable command execution results.

If the above understanding is reasonable, then the command type (e.g. read/write/disable) is also helpful for the reader to determine the corresponding AS procedure and the radio resources to execute the AS procedure.

Except for the visibility of the service/command type and AIoT device population mentioned in the above agreements, the following awareness in relation to the service request from CN by the reader can be considered:

- If the reader can know the D2R data/message size from the service request for reading operation, the reader can determine the amount of radio resources that is required for the D2R data/message transmission. For instance, if a service request is to read certain message of fixed length, using the message size information from the service request, the reader can allocate the exact amount of radio resources to transmit the D2R message, while the case of insufficient/overallocated radio resource allocation can be avoided.

- If the reader can know the periodicity for executing the received service request (e.g. periodically read the temperature from an AIoT device), the reader can periodically execute the read procedure according to the periodicity. Without such mechanism, for periodic AIoT service, the CN has to initiate one service request to reader for each period, which causes signaling overhead.

- In addition, the reader may also need to derive the QoS requirement from the service request from CN so that the reader can execute the related AS procedure. Typically, the e2e latency fulfillment can be considered. Furthermore, for inventory service for certain population of AIoT devices in the reader’s coverage, certain target ratio for successful inventory of the target in-coverage devices can help the reader to determine the R2D/D2R radio parameters e.g. coding rate, transmit power level etc, to fulfill the target inventory ratio of devices in reader’s coverage.

Based on the above discussion, we propose:

- The reader executes the AS procedure in response to the service request from CN based on the necessary information derived from the received service request, wherein the necessary information may comprise:

  - command type information, e.g. read/write/disable.

  - D2R data/message size.

  - periodicity to execute the service request.

  - QoS requirement (e.g. e2e latency and target successful inventory ratio)

## MAC PDU structure

The MAC PDU would be used to configure the AS actions of the AIoT device, provide the associated parameters to AIoT device (if there is) to take these actions and transmit the related associated data/message (if there is) for the AIoT device to take these actions.

For one instance, when the reader transmits data to the target AIoT device(s) a R2D MAC PDU, the R2D MAC PDU should carry the temporary ID of the target AIoT device so that the target AIoT device can identify that it is the target for the R2D MAC PDU. In the reverse direction, the target AIoT device can generate a D2R MAC PDU in response with its temporary ID included so that the target reader can know it is the response message from the target AIoT device.

For another instance, during an inventory procedure, when the reader pages the target AIoT devices, the reader can construct an R2D MAC PDU comprising the paging indication and the Q value.

To be more generic, a message type field can be included in R2D MAC PDU, wherein the message type field indicates the AS procedure corresponding to inventory/command service and the MAC PDU content/format. For each message type, there could be associated parameters to configure the AIoT device to execute the corresponding AS procedure. Some of the parameters could be conditionally present parameters and/or there could be associated upper layer message/data in the R2D MAC PDU, wherein complementary fields to indicate the presence of these optional fields could be necessary.

Based on the above discussion, the following R2D MAC PDU structure is proposed:

- Design an R2D MAC PDU format comprising:

  - message type, which indicates the MAC PDU format/content,

  - associated parameters (e.g. temporary ID, Q-value, R2D/D2R resource allocation) to facilitate AIoT device action if any, and

  - associated upper layer signalling message/service data if any.

When a R2D MAC PDU is received and the AIoT device determines taking the related actions, the AIoT device may also generate a D2R MAC PDU for subsequent response.

For one instance, the AIoT device may generate a D2R MAC PDU comprising of certain ID (e.g. RN16) and sends it to the reader. As the D2R MAC PDU is in response to the R2D MAC PDU, the format/content of the D2R MAC PDU can be fully determined based on this R2D MAC PDU content. In such sense, the message type field is not mandatory for D2R MAC PDU when its transmission is triggered a received R2D MAC PDU.

- When a D2R MAC PDU is in response to an R2D MAC PDU, the content/format of the D2R MAC PDU can be identified by the reader based on the R2D MAC PDU content.

- Design D2R MAC PDU format according to the corresponding R2D MAC PDU, wherein the D2R MAC PDU’s format/content is determined based on the corresponding R2D MAC PDU.

In radio transmission protocol, there are the following purposes for sequence number in a service packet:

- identify the packet loss so that related upper layer retransmission in the transmitter can be triggered.

- identify the duplicated packets and remove the duplicated ones.

- to enable in-order-delivery to upper layer in the receiver side.

As PDCP/RLC is not to be supported for AIoT, there is no upper layer (MAC and above) retransmission which requires duplicated packet removal. Furthermore, in case of multiple repetitions of one MAC PDU, the physical layer always stops decoding the subsequent repetitions once the TB is correctly decoded and there is no duplicated TB delivery from physical layer to MAC layer. With these assumptions, it can be derived that if there are multiple service data packets for one service, the service data packets can be sequentially received and delivered to upper layer, i.e. in-order-delivery to upper layer can be automatically ensured. This means that none of the above purposes for sequence number is still valid for AIoT service. Hence, we propose:

- No MAC layer sequence number is needed for service data transmitted in MAC PDU.

## Multiplexing

It was agreed that multiple AIoT logical channel will not be supported. This means that there will no multiple con-current services, wherein each of the AIoT services owns one AIoT logical channel. However, it is not determined whether there is only single or multiple service data packet(s) per service in one MAC PDU per AIoT service. From our understanding, even if there can be multiple service data packets per service for one AIoT service, the reader can schedule multiple individual transmissions respectively, wherein each transmission can transmit one individual service data packet. In this way, the fixed overhead of the MAC sub-header (e.g. sub-MAC PDU type, the start/length of a sub-MAC PDU) to facilitate packet multiplexing of multiple sub-MAC PDUs in one MAC PDU can be avoided. This can be very essential from the spectrum efficiency perspective considering that the data rate of the AIoT air interface is lower than that of NB-IoT.

- If there are multiple data packets for one AIoT service, the packets can be respectively transmitted in separate transmissions.

Hence,

- RAN2 assumes no multiplexing of multiple service data packets in one MAC PDU.

## BSR

In RAN#126[1], there were the following conclusions regarding the SR/BSR procedure for AIoT:

Agreements

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

In RAN#127[2], there were the following conclusions regarding the SR/BSR procedure for AIoT:

Agreements

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

-	RAN2 will study the need and use of a simple message “size” reporting to the reader

For D2R transmission of upper layer data/msg of variable size for some service, there can be no exact data/msg size information in the service request from the CN. However, to allocate the proper amount of radio resources for data transmission from the AIoT device, the reader still needs to know the data size of the AIoT device. From this perspective, certain BSR mechanisms are still necessary for AIoT devices.

- In case that CN cannot indicate the exact D2R message size, BSR is required to facilitate the D2R radio resource allocation by the reader.

- BSR for the remaining message bits should be supported for AIoT device.

If certain BSR procedure is supported, corresponding triggering procedure should be considered. With the BSR, the reader can determine the amount of D2R resource to be allocated for D2R transmissions. Typically, the BSR should be triggered on whether there are remaining bits after the first transmission of an upper layer message.

- If BSR is supported, BSR is triggered upon the first transmission of the upper layer message for remaining message bits after the first transmission.

In previous meetings, single bit SR/BSR was proposed to indicate there were data left in the AIoT device after the current D2R transmission. However, upon reception of single-bit SR/BSR, the reader can only know there is data left in the UE side but no information regarding the exact number of remaining bits. The reader cannot determine the amount of resource that is required to transmit the remaining bits. If less radio resource than required is allocated, more iterations of UL transmissions are required, which results large overhead due to more associated scheduling messages. If more radio resources are allocated than the required radio resource for the transmission of the remaining bits, there is radio resource waste.

- With single bit BSR, the reader cannot derive the amount of radio resource required by the remaining message bits, which result in low D2R radio resource efficiency.

There will be no concurrent multiple services for an AIoT device as multiple AIoT logical channel will not be supported. A very simplified BSR with single BS field to indicate the total buffer size of the AIoT device could be considered. For instance, the BSR format with only single-byte BS field (i.e. no LCG/LCH indication in BSR) can be adopted. For simplicity, a Linear BS table can be considered.

- If BSR is supported, RAN2 to support single-byte BSR using to indicate the total number of the remaining message bits.

## Energy status report

In RAN2#127[2], there were the following agreements regarding the energy status of AIoT devices:

Agreements

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

-	RAN2 will study the need and use of a simple message “size” reporting to the reader

From our understanding, there can be situations where the AIoT device may lack sufficient energy to complete a service request corresponding AS procedure. For one instance, the AS procedures for one service request may comprise of an inventory procedure plus a subsequent read procedure. After the inventory procedure, if the device determines that it does not have sufficient energy to complete the read procedure, it can report to the reader of its energy shortage. For another instance, when a device is scheduled with a long-duration D2R transmission while the device does not have sufficient energy to complete the whole transmission, instead of initiating the scheduled D2R transmission, the device can report an energy shortage indication to the reader. Based on the energy shortage indication, the reader can determine to stop/suspend transmission with the AIoT device.

- If the AIoT device determines that its remaining energy cannot complete the received service request corresponding AS procedure, energy status report should be triggered to indicate the energy shortage.

Then there is an issue on how an AIoT device to determine the remaining energy level and whether the remaining energy is enough for carrying the AS procedures corresponding to a service request or for carrying a transmission scheduled by the reader.  How to determine the energy level and how to determine energy shortage may be AIoT device hardware dependent and may not be easy to be standardized, thus can be left up to implementation for AIoT devices.

- How to determine the remaining energy level and how to determine whether the remaining energy level for AS procedure completion is left to AIoT device implementation in this release.

After receiving the energy shortage report from an AIoT device, it is expected that the reader will stop/suspend the service until the AIoT device has been recharged. In order to facilitate the reader to determine when the AIoT device will be charged with sufficient power to execute AS procedure(s) of the service request, certain assistant information can be reported in the energy shortage report. For instance, the AIoT device may report the estimated time to be ready to execute the service request. Based on the assistance information, the reader can determine when to page the AIoT device for executing the service request accordingly.

- The energy status report from an AIoT device to indicate energy shortage should at least contain the information to facilitate the reader to determine when to page the AIoT device.

# Conclusion

This contribution mainly discussed the remaining issues from functionality aspects for Ambient IoT. The contribution concludes with:

- CN based segmentation requires one service procedure per segment, which comprising one service request, one inventory procedure, one reader-AIoT device data transmission procedure and one response message to CN.

- MAC based segmentation requires only one service procedure for the all segments of one upper layer message, which comprises: one service request, one inventory procedure, multiple respective scheduled transmissions for all segments in AIoT air interface, and one response message to CN.

- MAC based segmentation can use optimal segment size based on radio condition and resource availability while CN based segmentation has to use conservative segment size due to lack of timely radio condition and radio resource availability information.

- CN based segmentation is expected to have much larger service delay and overhead than MAC based segmentation due to CN based segmentation requires more signalling procedures.

- With MAC segmentation for D2R transmission, the required buffer size at the AIoT device for cyphering and integrity protection related processes is not larger than that without MAC segmentation.

- When a D2R MAC PDU is in response to an R2D MAC PDU, the content/format of the D2R MAC PDU can be identified by the reader based on the R2D MAC PDU content.

- If there are multiple data packets for one AIoT service, the packets can be respectively transmitted in separate transmissions.

- In case that CN cannot give the exact D2R message size, BSR is required to facilitate the D2R radio resource allocation by the reader.

- With single bit BSR, the reader cannot derive the amount of radio resource required by the remaining message bits, which result in low D2R radio resource efficiency.

Device AS Short ID Management

- The Device AS short ID allocation and uniqueness is under reader control, i.e., the AS short ID is randomly selected by device and confirmed by reader (in CBRA case), or is allocated by reader directly (in CFRA case).

- The Device AS short ID can be released on a timer expiry or explicit indication from reader.

Device type/capability reporting

- RAN2 study should aim at solutions that do not require device type and device radio capability information reporting from device to reader as baseline.

Segmentation

- RAN2 to consider the following two candidate way-forwards for segmentation:

  - CN based segmentation: rely on CN or app server to perform segmentations if the message size is too big to be transmitted in one shot;

  - MAC based segmentation: relay on AIoT MAC to perform segmentation according to the message size, radio condition, radio resource availability etc.

- RAN2 considers the following two candidate schemes for MAC segmentation:

  - Automatic segmentation: the transmitter (reader/device) performs segmentation when the message size is larger than the TB size, and includes the segment indication (e.g.  end or non-end segment) in the transmission.

  - Scheduled segmentation: the reader schedules the AIoT device to transmit/receive one msg piece by piece with segment indication (e.g. a memory address offset) in the scheduling information.

- MAC based segmentation should be considered as a work assumption.

Service request visibility to reader

- The reader executes the AS procedure in response to the service request from CN based on the necessary information derived from the received service request, wherein the necessary information may comprise:

  - command type information, e.g. read/write/disable.

  - D2R data/message size.

  - periodicity to execute the service request.

  - QoS requirement (e.g. e2e latency and target successful inventory ratio)

MAC PDU structure

- Design an R2D MAC PDU format comprising:

  - message type, which indicates the MAC PDU format/content,

  - associated parameters (e.g. temporary ID, Q-value, R2D/D2R resource allocation) to facilitate AIoT device action if any, and

  - associated upper layer signalling message/service data if any.

- Design D2R MAC PDU format according to the corresponding R2D MAC PDU, wherein the D2R MAC PDU’s format/content is determined based on the corresponding R2D MAC PDU.

- No MAC layer sequence number is needed for service data transmitted in MAC PDU.

Multiplexing

- RAN2 assumes no multiplexing of multiple service data packets in one MAC PDU.

BSR

- BSR for the remaining message bits should be supported for AIoT device.

- If BSR is supported, BSR is triggered upon the first transmission of the upper layer message for remaining message bits after the first transmission.

- If BSR is supported, RAN2 to support single-byte BSR using to indicate the total number of the remaining message bits.

Energy status report

- If the AIoT device determines that its remaining energy cannot complete the received service request corresponding AS procedure, energy status report should be triggered to indicate the energy shortage.

- How to determine the remaining energy level and how to determine whether the remaining energy level for AS procedure completion is left to AIoT device implementation in this release

- The energy status report from an AIoT device to indicate energy shortage should at least contain the information to facilitate the reader to determine when to page the AIoT device.

# References

- RAN2#126 Meeting Chair Notes.

- RAN2#127 Meeting Chair Notes.

- R2-2407985, Report of [POST127][033][AIoT] Random Access.