3GPP TSG-RAN WG2 Meeting #127bis	R2-2408227

Hefei, China, 14th – 18th Oct 2024

Source:	vivo

Title:	Discussion on AIoT Paging

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

Several agreements have been reached on the functionality of paging in the context of AIoT [1][2][3]:

R2#125 Agreements

- Baseline procedure:

Step A: Based on the service request, the reader sends the Initial Trigger Message indicating device(s) that need to respond; Details FFS

FFS if Initial Trigger Message can also include “command”.

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

- RAN2 assumes that the device will not support tracking/RAN area update procedure.

- For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message.    FFS pending the details from SA2

R2#126 Agreements

1	RAN2 will study the following cases for AIoT paging message:

•	a message containing an ID of a single AIoT device.

•	a message containing a group ID that maps to multiple AIoT devices.

•	a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message.

•	a message containing multiple IDs of AIoT devices. Need to confirm the need for this use case based on SA2 discussion.

What device ID and group ID and scenarios is depending on SA2 discussion.

2	AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

3	From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

R2#127 agreements

1	Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

In this contribution, we further discuss AIoT paging from the following perspectives:

- Content related to random access

- Content to avoid repeated access

- Discussion

## Content related to random access

For support of inventory and inventory followed by command services, AIoT paging message is responsible to notify the AIoT device(s) to respond. As agreed, AIoT device identification is contained in the paging message, with the exact format decided by SA2.

Figure 2.1-1: inventory service optionally followed by command service from CN

At reception of the AIoT paging message, it is significant for AIoT device(s) to know how to access the network. Since it is agreed that slotted aloha is used as a baseline for random access procedure, the reader will allocate RA parameters, e.g., a Q value to distribute devices in time domain to several access period i.e., “slots” within the available shared resources.

- For contention-based access, random access parameter to indicate the number of R2D triggers for access period in one access round is included in AIoT paging message.

Further referring to RAN1 agreement [4], several multiple access methods are considered.

Agreement Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Agreement Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS. Agreement Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS.

If different multiple accesses are actually introduced by RAN1, the indication on how to access in either domain should also be included within the allowed “slot” for AIoT communication. However, whether the access parameters within the slot (i.e., parameters to determine the number of access occasions in one access period) should be included in paging message in a fixed manner, or can be indicated through R2D (access) trigger message variably, is within RAN1 scope. It is proposed to wait further progress on the format and carrier message of these parameters.

- RAN2 to wait for RAN1 progress on whether/how the random access parameters to indicate the number of access occasions in one access period is carried in AIoT paging message or another R2D trigger message.

What is more, when 2-step and 3-step are both allowed for one device, how does the reader inform devices about which type of CBRA is in use? From our point of view, there can be two options for AIoT devices to differentiate in between. Since there is a natural differentiation in the packet size of the following D2R Msg1 transmission with reporting 16-bit random ID and optionally EPC for 3/2-step procedures, the indication can be deduced by a device via the size of allocated resources or scheduling parameters implicitly. However, an explicit flag of CBRA type is also feasible in AIoT paging message.

- If both 3&2-step CBRA are supported, RAN2 to discuss how to indicate the type of CBRA in AIoT paging message.

Apart from that, the discussion about contention-free access procedure is also in parallel. That is, from the device perspective, it may obtain dedicated access occasions/resources from the reader. To achieve this through AIoT paging, there are also two cases to discuss. It is possible to indicate Q=0 to realize contention free access for the case that only one AIoT device is paged. In this sense, the device still uses the shared resources but no backoff is needed. Another case can be applicable for more than one device ID, that the paging message should indicate the dedicated resource allocation in each domain for each paged AIoT device ID respectively. However, for this scheme, the identification of device(s) should be realized by device ID, where no device ID included in paging message is not feasible to indicate paged device(s) with dedicated resources. Still, whether there is a case for multiple device ID should be determined by SA2, and whether/how CFRA is supported for group ID case or non-ID case should be determined by RAN1. In our point of view, the unsolved issues also have relationship with the assistance information exposed to reader from CN. In general, for CFRA, dedicated access resources should be indicated to each paged device.

- CFRA does not work for the case of no identifier included in the paging message, since no dedicated resources can be uniquely assigned to a specific device.

- For contention-free access, dedicated access resource configuration corresponding to each paged device is included in the AIoT paging message.

Similar to the discussion on the optionality of 2-step CBRA for A-IoT device, whether CFRA is optional or mandatory or not is also worth a discussion. From our point of view, CFRA should be a mandatory feature supported for AIoT communication. The main reason is that no collision would be caused in such scheme and thus could reduce the service latency to some extent, which seems necessary for timing-sensitive AIoT service.

Moreover, due to the large number and the low capability of device, the explicit indication seems avoidable for AIoT device to know CBRA or CFRA is allowed. A simple way is achieved through the content of AIoT paging message. For instance, if RAN2 agrees that CFRA is only applicable for the case of one paged device with setting random access parameter as 0, device can recognize the access type by the exact number of this random-access parameter. On the other side, if the case of multiple device IDs is also workable, device can realize it should perform CFRA with receiving dedicated scheduling rather than random access parameter. Furthermore, with reference to the reception of R2D message, AIoT device can determine its D2R transmission by the reference time/frequency point with the time/frequency shift from the reference, which is contained in the R2D/AIoT paging message. The detailed design of resource indication should be determined in Stage 3 discussion on the basis of RAN1 progress.

- If both CBRA and CFRA are supported, RAN2 to discuss how AIoT device determines the access type via AIoT paging message.

On top of that, we see little benefit from inclusion of paging cause in the AIoT paging message. Paging cause is introduced in NR to assist a device to determine how to handle a received paging message, e.g. response the paging message immediately or ignore the paging message. For example, for the dual-USIM UE, if a USIM is paged with a low priority cause value while the other USIM has a high priority service ongoing, the UE may ignore the paging triggered by low priority service, assuming the two USIMs cannot work simultaneously. On the contrary, due to the low capability of passive AIoT device, paging is what it must respond to once it is paged. Besides, since paging message is designed to serve all types of the AIoT service, to finish any service, AIoT device is triggered to respond with no differentiation in the service type. Thus, there is no need for device to be informed about the cause it is being paged for.

- NR paging cause helps UE to decide on service priority and whether to respond; while AIoT device needs to respond AIoT paging with no differentiation on AIoT service type.

- AIoT paging cause to differentiate AIoT service type is not included in AIoT paging message.

## Content to avoid repeated access

According to SA2 design [5], one AIoT service may be passed to multiple AIoT readers to be processed parallelly with a same upper layer session ID and increase the service success rate. For example, inventory towards the same bunch of devices may be passed to readers within a local range. From the device side, it is of no use to repeatedly respond to all the paging procedures induced by the same service.

- Session ID in AIoT upper layer can be introduced to identify the same AIoT service among multiple AIoT readers.

Apart from that, owing to the possibility of the access failure, the reader may trigger multiple rounds of paging corresponding to the same service in the AS level. In this sense, the paged devices need to know whether it has successfully been paged and found. That is, if the device receives the paging identified by the same identifier as it responded to before, it will know that the current paging is another round of the one it has responded and accessed; and there is no need to respond again. Thus, from the perspective of the lower layer i.e., AIoT MAC layer, session ID is also necessary to resolve such power consumption issue caused by repeated access attempts from the device point of view. To include a session ID indicating that paging is brought by a specific service, device can respond at will. Whether the lower layer session ID reuses the higher layer one can be discussed accordingly. From our point of view, what SA2 has agreed is enough to reuse to solve two problems in a win-win way.

- Session ID is beneficial for AIoT MAC layer to distinguish between different rounds of paging corresponding to the same AIoT service under the AIoT reader.

- Session ID (details up to SA2) is included in AIoT paging message and employed to avoid device’s duplicated response for the same service request. LS to SA2 (Cc RAN3) to inform RAN2 decision on Session ID if agreed.

Apart from that, RAN3 is exploring the proximity issue for A-IoT device and has come the following conclusion.

AIoT device location information may be used for the following purposes: (a) improving the AIoT operation itself, e.g. by AIoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the AIoT service. Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

- RAN3 has agreed the case that CN may send a Command to multiple readers; and Reader ID is employed in locating AIoT device to distinguish different AIoT readers.

As we mentioned above, paging across different readers may carry the same session ID corresponding to the same A-IoT service. In this sense, the device may only response once for the sake of energy saving. However, if the service is of the location purpose, the device is expected to response to different readers. One way to solve the issue is to include the reader ID in paging message for device to identify the paging messages with the same purpose are from the different reader. Then device may perform access to different readers accordingly. However, device has no insight about location service even with revealing reader ID. It is the reader that can be informed of location purpose from CN. In this understanding, another way is that the reader has an indication about whether response to a paging message with the same session ID is compulsive. Besides, it is easier to count on NW implementation by assigning different session IDs to different readers but for a same proximity service.

- RAN2 to discuss the following ways in which the AIoT device is allowed to respond AIoT paging messages from different readers associated with the same request for proximity service:

- Option 1: including reader ID in AIoT paging message;

- Option 2: indication on whether respond is allowed when receiving AIoT paging messages with the same session ID;

- Option 3: relying on NW implementation to by assigning different session IDs to different readers but for a same service.

- Conclusion

On a basis of above analysis, we make the following proposals regarding AIoT paging:

Content related to random access

- For contention-based access, random access parameter to indicate the number of R2D triggers for access period in one access round is included in AIoT paging message.

- RAN2 to wait for RAN1 progress on whether/how the random access parameters to indicate the number of access occasions in one access period is carried in AIoT paging message or another R2D trigger message.

- If both 3&2-step CBRA are supported, RAN2 to discuss how to indicate the type of CBRA in AIoT paging message.

- CFRA does not work for the case of no identifier included in the paging message, since no dedicated resources can be uniquely assigned to a specific device.

- For contention-free access, dedicated access resource configuration corresponding to each paged device is included in the AIoT paging message.

- If both CBRA and CFRA are supported, RAN2 to discuss how AIoT device determines the access type via AIoT paging message.

- NR paging cause helps UE to decide on service priority and whether to respond; while AIoT device needs to respond AIoT paging with no differentiation on AIoT service type.

- AIoT paging cause to differentiate AIoT service type is not included in AIoT paging message.

Content related to random access

- Session ID in AIoT upper layer can be introduced to identify the same AIoT service among multiple AIoT readers.

- Session ID is beneficial for AIoT MAC layer to distinguish between different rounds of paging corresponding to the same AIoT service under the AIoT reader.

- Session ID (details up to SA2) is included in AIoT paging message and employed to avoid device’s duplicated response for the same service request. LS to SA2 (Cc RAN3) to inform RAN2 decision on Session ID if agreed.

- RAN3 has agreed the case that CN may send a Command to multiple readers; and Reader ID is employed in locating AIoT device to distinguish different AIoT readers.

- RAN2 to discuss the following ways in which the AIoT device is allowed to respond AIoT paging messages from different readers associated with the same request for proximity service:

- Option 1: including reader ID in AIoT paging message;

- Option 2: indication on whether respond is allowed when receiving AIoT paging messages with the same session ID;

- Option 3: relying on NW implementation to by assigning different session IDs to different readers but for a same service.

- Reference

- Report of 3GPP TSG RAN WG2 meeting #127, Maastrict, NL

- Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

- Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China

- Report of 3GPP TSG RAN WG1 meeting #116bis, Changsha, China

- TR 23.700-13	Study on Architecture support of Ambient power-enabled Internet of Things

- R3-244030	(BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR

- TR 38.769	Study on solutions for ambient IoT (Internet of Things)