3GPP TSG-RAN WG2 Meeting #127bis		R2-2408228

Hefei, China, Oct 14th – 18th, 2024

Source:	vivo

Title:	Random Access Procedure for A-IoT Device

Agenda Item:	8.2.4

Document for:	Discussion

# Introduction

In the last RAN2 meeting, random access procedure of A-IoT device was discussed and some agreements were achieved as follows [1]:

Agreements

-	For 3-step CBRA Support fixed random ID size is 16 bit.   The ID is randomly generated.

-	Failure/success indication of D2R will be studied.   FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

Agreements

-	for 2step CBRA, RAN2 design will support msg2.  Whether it is needed it is up to the reader.  FFS when it is needed.  For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.   FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

-	In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).   FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes.

-	FFS if reader assigns the AS ID for scheduling purposes

In this contribution, we continue to discuss A-IoT random access procedure, which includes terminology and modelling for RACH, further details about 3-step CBRA, 2-step CBRA and CFRA, AS short ID management, Msg2 multiplexing and power ramping for RACH.

# Discussion

## 2.1 Terminology and modelling

In the following figure, we give our understanding of terminology and modelling for AIoT RACH procedures.

Access occasion: An opportunity of time/frequency resource for A-IoT device to perform access (e.g. transmitting the A-IoT Msg1).

Access period: One access period consists of a certain amount of access occasions for difference devices, e.g. 2M in time domain and 2N in frequency domain, which are assigned via one R2D message (e.g. R2D Access Round Trigger and FFS for R2D Rep Trigger) by the reader.

Access round: One access round consists of one or multiple access periods, e.g. 2Q, which is initiated by a specific R2D trigger message carrying with the A-IoT paging message or paging session ID and full access parameters (e.g. R2D Access Round Trigger). One service request may associate with multiple access rounds.

Each R2D Rep Trigger is used to mark the start of the next access period. One access round consists of one or multiple access periods, e.g. 2Q, and one access period consists of a certain amount of access occasion for different devices, e.g. 2M in time domain and 2N in frequency domain. The value of Q, M and N is assigned via the first R2D Access Round Trigger. The power of 2 may be used for ease of calculation and signaling overhead reduction. Detailed numbers are also available and can be decided in stage3. Hence, the total number of access occasions in one access round can be:

Number of access occasions in one access round = 2Q * 2M * 2N

Observation 1: 	The total number of access occasions in one access round can be determined by Q value (e.g. number of access period in one access round), M value (e.g. number of access occasions in time domain for each access trigger) and N value (e.g. number of access occasions in frequency domain for each access trigger).

In the above modelling, separate paging rounds may be not needed since paging message can be merged into the first R2D Access Round Trigger of each access round. Furthermore, in the very beginning round, full paging message is carried, and in the next repeat round, only paging session ID is enough to indicate the same paging purpose and avoid redundant access of success devices. Full paging message with the same session ID can be also carried in the following repeated round for new arrival devices or devices missing the previous paging.

- Paging message can be merged with the first R2D access trigger of each access round, which carries full paging message or paging session ID, and full access parameters, e.g. the number of Access Periods and the numbers of access occasions in time domain and/or frequency domain.

## 2.2  3-step CBRA

The basic 3-step CBRA procedures have been concluded in the previous RAN2 meetings. The left issue is whether and how to indicate failure/success of Msg3. From our understanding, the success of Msg3 can indicate the success of inventory from RAN side. The probability of successful Msg2 and Msg3 transmission is generally affected by link quality and transmission parameters. In a typical Uu interface transmission and common evaluation assumptions, the probability of success of a single transmission is about 90%. In other words, without indication of Msg3, the reliability of the inventory is about 90% which means at least 10% of devices with normal link quality participate in the inventory but fail. However, the 90% success rate of inventory is obviously not enough to meet all the requirements. The indication and recovery of Msg3 is necessary.

There are three common options for the indication of Msg3:

- NACK-only based indication,

- ACK-only based indication

- ACK/NACK based indication.

The information and overhead of these three options are all 1-bit. The main differences are the frequency and reliability of feedback. ACK/NACK based indication will have the highest overhead and once the feedback fails, it can not be interpreted. ACK-only based indication has medium but almost 90% peaked overhead and once it fails, redundant re-access may be triggered and may cause a waste of resources. NACK-only based indication has the lowest and almost 10% peaked overhead. Furthermore, NACK-only based indication can be also compatible with different cases, e.g. with subsequent command or not, or followed by scheduling of other devices etc. Absence of NACK indication will be directly translated into success of Msg3. Hence, NACK-only based indication has the best performance.

- A NACK-only based indication for Msg 3 should be supported.

For detailed NACK-only based indication, one option is a NACK with RN16 after Msg3 from reader to device. When the device has received Msg2 successfully, it can interpret the NACK as the failure of Msg3. Then there are two feasible device behaviors now: re-access or retransmission of Msg3 with previous resource allocation in Msg2. These two behaviors will cause difficulty for the reader to reserve resources:

- Re-access may be more preferable. On the other hand, if the device has not received Msg2 successfully, it can only interpret the NACK as the failure of contention resolution or Msg1. Re-access is the only feasible behaviour for the device. To sum up, a NACK with RN16 after Msg3 is used to indicate the failure of Msg2/3 and trigger device re-access. This option may be a baseline.

- The other option is Msg2 retry. When reader cannot receive Msg3 successfully, it can retry Msg2, e.g. confirm RN16 in Msg1 and optionally carry resources for Msg3. When the device receives the second Msg2, it will transmit Msg3 again. This option enables quick Msg3 retransmission and recovery, which may have better access efficiency than re-access but not be easy to succeed during link deterioration. This option is an optimization somehow.

These two options can be further studied and evaluated or left to reader implementation.

- The following NACK-only based indication options can be further studied:

- Opiton 1: A NACK with RN16 for Msg3, e.g. to trigger re-access;

- Option 2: Msg2 retry, e.g. to trigger transmission of Msg3 again.

If Msg1 or Msg3 failure is indicated by reader, re-access will be triggered by device. Re-access immediately may still fail with some probability when device is in link deterioration period. Furthermore, re-access immediately may cause additional complexity and delay for other devices. The overall access procedure design will also be more complex. Hence, we prefer to trigger re-access in the next access round.

- If access failure is indicated, device should trigger re-access in the next access round.

## 2.3 2-step CBRA

In the last RAN2 meeting, it had been agreed that the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2 for 2-step CBRA. In our understanding, Msg2 is mandatory for the case that there is subsequent scheduling requirement, e.g. other command, but seems optional for one-time transmission, e.g. inventory only. Our concern is whether the reader can determine precisely there is any further data or not in the phase of Msg1, e.g. CN confirmation for inventory or subsequent command(s). Such precision comes in the expense of overhead, e.g. enough assistance information from CN in advance, and loss of flexibility. Furthermore, the absence of Msg2 can reduce the reliability of Msg1, e.g. the probability of inventory success is only about 90% or less. To sum up, we propose Msg2 shall be always sent. And the absence of Msg2 means Msg1 failure and device will re-access in the next access round.

- Msg2 for 2-step CBRA is always sent.

- The absence of Msg2 means Msg1 failure and device will re-access in the next access round for 2-step CBRA.

There is also an open issue about if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.  In NR Uu, 4-step RACH is baseline from R15 and 2-step RACH is an optional feature introduced in R16. For AIoT scenarios, 3-step CBRA is more adaptable than 2-step CBRA since the efficiency of 2-step CBRA is very low in the case of high collision probability. And 2-step CBRA seems to need extra device capability requirement to combine Msg1 and Msg3. It is not reasonable to support only 2-step RA and to limit reader implementation. From the perspective of device capability, maintaining a single device capability will simplify the system design, e.g. avoid capability reporting and differentiate treatment.

- Both 3-step and 2-step RA are baseline supported by all types of devices, i.e. no need for optimization of 2-step only device.

## 2.4 CFRA

In CFRA case, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message with its dedicated resources. A short AS ID seems not need to transmit from device to reader since there is no need for contention resolution purpose. However, if there are subsequent scheduling requirements, e.g. further command or upper layer data, an AS ID is also necessary. It is a feasible solution that reader assign a device with a specific AS ID in its dedicated resource, e.g. specified response for the Msg1. Another unified solution is RN16 may be carried in Msg1 and echoed in Msg2 like CBRA. The latter will waste some amount of D2R resources. Hence, we slightly prefer the former, i.e. AS ID assignment by reader.

- For CFRA, reader assigns a device specific AS ID, e.g. 16-bit, in Msg2.

## 2.5 Msg2 Multiplexing

RAN1 had discussed Msg1 TDMA and FDMA scenarios. If Msg1 transmission uses TDMA and/or FDMA scheme, it should be further studied how to deliver Msg2 efficiently. RAN1 [2] had considered different solutions for Msg2 transmission as followings:

In Alt-1, Msg2 transmission is per device. Msg2 and Msg3 of device 1 are transmitted first, and then Msg2 and Msg3 of device 2 are transmitted and so on. TDMA or FDMA scheme of Msg3 can not be used.

In Alt-2, Msg2 transmission is also per device. The difference with Alt-1 is all Msg2 of success devices are continuously transmitted before Msg3 transmission. Then all Msg3 of success devices can use TDMA or FDMA scheme. The transmission efficiency of Msg3 will be increased greatly. However, the distance between each pair of Msg2 and Msg3 varies, and the resource indication may be complicated.

In Alt-3, Msg2 multiplexing mechanism is introduced to replace separate Msg2 transmission in Alt-2. Alt-3 seems to have more streamline processes and greater efficiency than other alternatives, e.g. beneficial for access latency reduction and efficiency improvement.

Observation 2: 	Msg2 multiplexing seems to be beneficial for access latency reduction and efficiency improvement in the cases of Msg1 TDMA and/or FDMA.

Of course, the detailed solution should be evaluated and concluded in RAN1 first.  However, format and multiplexing of Msg2 are in RAN2 scope. RAN2 can concurrently evaluate the feasibility of Msg2 multiplexing and with RAN1 together perform the final selection of Msg2 transmission mechansim.

- RAN2 to evaluate the feasibility of Msg2 multiplexing.

## 2.7 Power ramping for RACH

Legacy Uu RACH procedure also supports power ramping for RACH preambles repetition. If UE fails to receive a Msg2 in RAR window, UE will increase TX power and transmits a RACH preamble again. For A-IoT devices, the feasible of power ramping is questionable since the limited energy may not be enough to support this feature except that RAN1 can provide evaluation results to confirm that. From RAN2 point of view, re-access of the failed device should be better to start in the next access round. Whether to support power ramping for Msg1 transmission does not affect the RAN2 access scheme design.

- Whether to support power ramping for Msg1 transmission is up to RAN1 conclusion and does not affect the RAN2 access scheme design.

# Conclusion

Based on the discussion, we have the following observations and proposals:

Observation 1: 	The total number of access occasions in one access round can be determined by Q value (e.g. number of access period in one access round), M value (e.g. number of access occasions in time domain for each access trigger) and N value (e.g. number of access occasions in frequency domain for each access trigger).

Observation 2: 	Msg2 multiplexing seems to be beneficial for access latency reduction and efficiency improvement in the cases of Msg1 TDMA and/or FDMA.

Hence, we propose:

Paging message int R2D access trigger

- Paging message can be merged with the first R2D access trigger of each access round, which carries full paging message or paging session ID, and full access parameters, e.g. the number of Access Periods and the numbers of access occasions in time domain and/or frequency domain.

3-step CBRA

- A NACK-only based indication for Msg 3 should be supported.

- The following NACK-only based indication options can be further studied:

- Opiton 1: A NACK with RN16 for Msg3, e.g. to trigger re-access;

- Option 2: Msg2 retry, e.g. to trigger transmission of Msg3 again.

- If access failure is indicated, device should trigger re-access in the next access round.

2-step CBRA

- Msg2 for 2-step CBRA is always sent.

- The absence of Msg2 means Msg1 failure and device will re-access in the next access round for 2-step CBRA.

- Both 3-step and 2-step RA are baseline supported by all types of devices, i.e. no need for optimization of 2-step only device.

CFRA

- For CFRA, reader assigns a device specific AS ID, e.g. 16-bit, in Msg2.

Msg2 Multiplexing

- RAN2 to evaluate the feasibility of Msg2 multiplexing.

Msg2 Multiplexing

- Whether to support power ramping for Msg1 transmission is up to RAN1 conclusion and does not affect the RAN2 access scheme design.

# References

- RAN2#127, Chair’s meeting report;

- RAN1#118, Chair’s meeting report;