3GPP TSG-RAN WG2 Meeting #127bis	R2-2408237

Hefei, China, 14 - 18 Oct, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on on-demand SSB SCell operation for NES

Agenda Item:	8.5.2

Document for:	Discussion and decision

Introduction

In RAN2#127, the following agreements on on-demand SSB were made:

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

In this contribution, we analyse RAN2 impacts of on-demand SSB SCell operation.

Discussion

2.1 MAC CE design

Both RAN1 and RAN2 have agreed to introduce a new MAC CE for indication OD-SSB transmission. RAN1 has further agreed to include SSB periodicity in the MAC CE.

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB. - FFS: Any other relevant parameters.

There is an open issue in RAN2 on whether the new MAC CE can also be used for SCell activation/deactivation, or the NW sends both new MAC CE and legacy SCell activation/deactivation MAC CE in Scenario 2A. From our perspective, the former option is preferred, based on the following reasons:

- The new MAC CE is used in both Scenario #2 and Scenario #2A, thus a common design is preferred;

- When the OD-SSB is for (fast) synchronization during SCell activation (Scenario #2A), it is up to NW implementation to transmit two MAC CEs jointly in one PDSCH. Or the NW can send the new MAC CE (OD-SSB transmission) first and then send the legacy MAC CE (SCell activation) in a separate PDSCH, some may consider this implementation falls into Scenario #2 but this does not make much difference in terms of SCell activation process, as RAN1 has already agreed that UE expects OD-SSB to be transmitted from time instance A (which means the SSB is not transmitted directly after receiving the MAC CE) and the NW can send legacy MAC CE before time instance A. Even in the case where NW sends legacy MAC CE first and new MAC CE later, it is up to RAN4 to define an uncertainty period during which the UE waits for the OD-SSB to be transmitted, and it is common practice in RAN4 to define such uncertainty period as they did for other features.

- From signalling perspective, if the new MAC CE also serves for SCell activation/deactivation, additional signalling overhead is caused because the new MAC CE needs to include all the SCells, some of which may not even support OD-SSB. Considering RAN1 has agreed to indicate additional information (e.g. SSB periodicity), the signalling overhead is even larger.

Proposal 1: The new OD-SSB transmission MAC CE is not used to indicate SCell activation/deactivation. Legacy SCell activation/deactivation MAC CE is used jointly with the new MAC CE in Scenario #2A.

On MAC CE design, there are several issues to be discussed:

Issue 1: Which SCells to be included? Only the SCells supporting OD-SSB or all the SCells?

Issue 2: How to identify an SCell, using explicit SCell indexes or the SCells are listed in an implicit order?

Issue 3: How to indicate SSB periodicity?

To save signalling overhead, we think only the SCells supporting OD-SSB should be indicated in the new MAC CE for OD-SSB transmission. The Issue 2 above implies two different ways of MAC CE design, the illustration will be provided in the following discussion. On Issue 3, the existing SSB periodicity contains 6 values (5ms, 10ms, 20ms, 40ms, 80ms, 160ms), and 3 bits can be used to indicate the SSB periodicity.

In the MAC CE design, we include some reserved bit in case other parameters are agreed to be included in the MAC CE (e.g. gaps between bursts, time instance A and time instance B, etc)

Option 1: MAC CE includes explicit SCell index and SSB periodicities (fixed size)

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	Periodicity: This field indicates the periodicity of the on-demand SSB, this field can also indicate the on-demand SSB is deactivated. The length of the field is 3 bits;

Figure 1: On-demand SSB indication MAC CE (Option 1)

Table 1: SSB periodicity for On-demand SSB indication MAC CE (Option 1)

| Periodicity value | On-demand SSB periodicity |
| --- | --- |
| 000 | Deactivation |
| 001 | 5 ms |
| 010 | 10 ms |
| 011 | 20 ms |
| 100 | 40 ms |
| 101 | 80 ms |
| 110 | 160 ms |
| 111 | Spare |

Option 2: MAC CE includes implicit SCell index and SSB periodicities (variable size)

This option requires RRC signalling to configure an additional index for the SCells supporting OD-SSB (i.e. enabling the OD-SSB feature from UE perspective), and the assumption is that there will not be more than 8 SCells supporting OD-SSB simultaneously. To be specific, a new field od-SSB-SCellIndex can be added to SCellConfig with the value range “INTEGER (1..8)”. And the MAC CE can be designed as follows:

-	Oi: This field indicates the activation/deactivation status of the on-demand SSB transmission of the SCell with Od-SSB-SCellIndex i as specified in TS 38.331 [5]. The Oi field is set to 1 to indicate that the on-demand SSB transmission of the corresponding SCell is activated. The Oi field is set to 0 to indicate that the on-demand SSB transmission of the corresponding SCell is deactivated;

-	Periodicity: This field indicates the periodicity of the on-demand SSB of the SCell for which on-demand SSB transmission is activated. The length of the field is 3 bits. The Periodicy fields are included in ascending order based on the Oi set to 1.

Figure 2: On-demand SSB indication MAC CE (Option 2)

Table 2: SSB periodicity for On-demand SSB indication MAC CE (Option 2)

| Periodicity value | On-demand SSB periodicity |
| --- | --- |
| 000 | 5 ms |
| 001 | 10 ms |
| 010 | 20 ms |
| 011 | 40 ms |
| 100 | 80 ms |
| 101 | 160 ms |
| 110 | Spare |
| 111 | Spare |

Proposal 2: RAN2 to discuss the MAC CE design for on-demand SSB transmission:

- Option 1: MAC CE includes the OD-SSB transmission status of a single SCell, which includes explicit SCell index and SSB periodicities (fixed size MAC CE);

- Option 2: RRC indicates whether an SCell supports OD-SSB when configuring the SCell, MAC CE indicates the activation/deactivation status sequentially, and indicates SSB periodicity for the SCells with OD-SSB activated (variable size MAC CE).

For Case #1 (no always-on SSB), the above design is straightforward. For Case #2 (always-on SSB), the above design is based on the understanding that Case #2 means having a periodic SSB transmission as a baseline and using on-demand SSB for triggering additional SSB transmission, and MAC CE is only used to activate and indicate the SSB periodicity of the OD-SSB, it has nothing to do with the always-on SSB.

Proposal 3: For Case #2 (always-on SSB), the new MAC CE is only used to activate and indicate periodicity of OD-SSB, it is not used to modify/activate/deactivate the always-on SSB.

2.2 L3 measurements

In legacy mechanism, L3 measurement is based on the measConfig, which uses measIds to link measObjects (which provides SMTC for UE to monitor SSB) and reportConfigs, and UE measurement reporting is also based on measIds. For OD-SSB, at least the OD-SSB pattern of the serving cell will be informed to the UE. Therefore, RAN2 needs to determine whether the L3 measurement is still based on SMTC or it is based on the OD-SSB pattern or both.

For both Case #1 (no always-on SSB) and Case #2 (always-on SSB), as long as the OD-SSB pattern is informed to the UE, UE can perform serving cell measurements. There is no need to modify the legacy SMTC or make up a new dedicated SMTC for serving cell measurement purpose.

Proposal 4: For both Case #1 (no always-on SSB) and Case #2 (always-on SSB), the OD-SSB pattern is informed to the UE for performing L3 serving cell measurements. No need to modify the legacy SMTC or make up a dedicated SMTC for serving cell measurements.

Another question is, for Case #2 (always-on SSB), there is always-on SSB within the legacy SMTC, it is unclear whether the UE should ignore these SSBs or the UE needs to measure both always-on SSB and OD-SSB and combine them to generate measurement results, as illustrated in the below picture:

Figure 3: L3 measurements on OD-SSB

Understanding 1 is simpler: since the serving cell can use OD-SSB for measurements, the NW can make sure the OD-SSB is transmitted with enough periods/occasions to satisfy the measurement need, and UE does not need to refer to the SMTC for serving cell measurements.

Understanding 2 is more flexible: the UE can utilize both OD-SSB and always-on SSB to generate measurement results, but the complexity to combine the measurement results is increased.

Proposal 5: For Case #2 (always-on SSB), discuss whether the always-on SSB is also used to generate serving cell measurement results.

The next issue is how to obtain OD-SSB pattern for serving cell measurements. The necessary parameter includes SSB start offset, duration, periodicity, which are quite similar with the SMTC parameters. Besides, a bitmap indicating the transmitted SSB indexes (similar to ssb-ToMeasure or ssb-PositionsInBurst) is needed.

RAN1 discussion used the terminology “time instance A” and “time instance B”, which describe the whole transmission duration for OD-SSB, but it does not illustrate the individual SSB burst duration.

<Excepted from RAN1#116 agreements> - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

Time instance A can be viewed as the start offset of the OD-SSB pattern. The SSB duration can be determined by the UE after detecting the SSB index. Therefore, at least the SSB periodicity and the SSB bitmap need to be informed to the UE.

Proposal 6: The SSB periodicity and the ssb-PositionsInBurst of the OD-SSB need to be indicated to the UE. RAN2 further discuss whether to use RRC signalling or MAC CE for such information.

For neighbour cells measurements, we think on-demand SSB status of neighbour cells should be transparent to the UE (otherwise large signalling overhead in Uu interface and Xn interface are incurred), and it is up to gNB implementation how to handle the reported neighbour cell measurement results which could be degraded based on the intermittent SSB transmission status of neighbour cell.

Proposal 7: On L3 measurements towards neighbour cells, legacy measurement and reporting procedures are used, and the OD-SSB transmission status of neighbour cells is transparent to the UE.

Conclusion

In this contribution, we discussed on-demand SSB for intra/inter-band CA, and propose the following:

Proposal 1: The new OD-SSB transmission MAC CE is not used to indicate SCell activation/deactivation. Legacy SCell activation/deactivation MAC CE is used jointly with the new MAC CE in Scenario #2A.

Proposal 2: RAN2 to discuss the MAC CE design for on-demand SSB transmission:

- Option 1: MAC CE includes the OD-SSB transmission status of a single SCell, which includes explicit SCell index and SSB periodicities (fixed size MAC CE);

- Option 2: RRC indicates whether an SCell supports OD-SSB when configuring the SCell, MAC CE indicates the activation/deactivation status sequentially, and indicates SSB periodicity for the SCells with OD-SSB activated (variable size MAC CE).

Proposal 3: For Case #2 (always-on SSB), the new MAC CE is only used to activate and indicate periodicity of OD-SSB, it is not used to modify/activate/deactivate the always-on SSB.

Proposal 4: For both Case #1 (no always-on SSB) and Case #2 (always-on SSB), the OD-SSB pattern is informed to the UE for performing L3 serving cell measurements. No need to modify the legacy SMTC or make up a dedicated SMTC for serving cell measurements.

Proposal 5: For Case #2 (always-on SSB), discuss whether the always-on SSB is also used to generate serving cell measurement results.

Proposal 6: The SSB periodicity and the ssb-PositionsInBurst of the OD-SSB need to be indicated to the UE. RAN2 further discuss whether to use RRC signalling or MAC CE for such information.

Proposal 7: On L3 measurements towards neighbour cells, legacy measurement and reporting procedures are used, and the OD-SSB transmission status of neighbour cells is transparent to the UE.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson