3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408243

Hefei, China, 14th – 18th October, 2024

Agenda Item:	8.2.4

Source:	Huawei, HiSilicon

Title:	A-IoT random access procedure

Document for: Discussion and Decision

1	Introduction

In this contribution, the details of A-IoT random access procedure are further discussed.

2	Discussion

2.1	Slotted ALOHA

2.1.1	Time domain distribution

Figure 2.1.1-1 Example of slotted ALOHA procedure

RAN2 agreed the A-IoT paging message and its function to select the devices to respond by random access procedures. Then, there is one concept of “paging round”, which is the slotted ALOHA procedure in between two A-IoT paging message.

During one paging round, if the reader wants to allocate some opportunities for initial access and then allocate some more opportunities specific for re-access, then there is the concept of “access round”. For example, one access round is for the selected devices to perform initial access, while the following one or more access rounds are for the re-access of those devices, if needed.

In one access round, e.g. the initial access round, the reader needs to allocate multiple opportunities for the CBRA of multiple devices. Those opportunities are the actual physical resources for devices to send Msg1.

Some RAN1 progress on the above Msg1 resource related issues:

FL proposal in R1-2407532 for TDMA FL4 High priority Proposal 6.1.1-1b: A R2D transmission triggering random access determines X time domain resource(s) available for D2R transmission(s) for Msg1, where each D2R transmission occurs in one time domain resource. - FFS X=1 or X>=1 considering the necessity, pros and cons. RAN1 Agreement Study FDMA of D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access, including following - How the frequency domain resources are allocated for the FDMA of D2R transmissions for Msg.1 - How a device determines the frequency domain resource for the D2R transmissions for Msg.1 Note: this does not preclude discussion on TDMA for D2R transmissions for Msg.1

If we define the minimum unit for sending the Msg1 by one device as “access occasion”, i.e. a time-frequency resource block, the RAN1 discussed R2D transmission, which schedules/triggers one or more access occasions, can be considered as another concept, i.e. “access occasion set”. So, one R2D transmission can trigger one set of X*Y access occasions.

As one of the option/candidate solution being studied in RAN1, the start of the Msg1 transmission/resource can be indicated by one R2D transmission (i.e. option 1).

For the time interval between a R2D transmission and the corresponding D2R transmission following it, there are two options studied: Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D ≥ TR2D_min.

By considering all possible terms/concepts for the study and discussion, it is proposed to first define those terms and then to discuss the need of each, especially the need of access round and paging round.

Proposal 1:	The following four terms in different levels of the slotted ALOHA procedure are used for the sake of further discussion (FFS on the needed of each):

Access occasion: An opportunity of time-frequency resource for A-IoT device(s) to perform access (e.g., transmitting the A-IoT Msg1).

Access occasion set: A set of access occasion(s) for different device(s), which is scheduled via one R2D message by the reader.

Access round: One access round consists of a certain amount of access occasion sets for difference devices, whose amount are assigned via one R2D message (e.g. Round Trigger message) by the reader.

Paging round: One paging round consists of one or multiple access rounds, which is initiated by the A-IoT paging message. One service request may be associated with multiple paging rounds.

RAN2 confirmed that slotted-ALOHA is the baseline for Ambient IoT random access.

From RAN2 perspective, as to the slotted-ALOHA procedure, reader first selects many devices and then distribute those devices into many “occasions”. Based on the TR 38.848 target device density, there could be up to thousands of devices to respond the paging trying to perform the random access.

Observation 1:	From RAN2 perspective, slotted-ALOHA needs to support the distribution of many devices (value N), selected by the one A-IoT paging, into similar number of access occasions (value Q).

Observation 2:	One A-IoT paging message may select up to several hundred of devices (or possibly even more).

As to the RAN1 discussion on the access occasions in response to/assigned by one “R2D transmission triggering”, it can be X*Y access occasions, which is a limited number (e.g. 2*4 in some cases). This is because the large SFO of A-IoT device limits the value of X, and the frequency-shit capability of A-IoT device limits the value of Y.

For example, if the total number of selected device N is similar to the total number of allocated access occasion Q =600, when the (X*Y) =10, it requires R=60 access occasion sets.

Observation 3:	When reader intends to allocate many access occasions (e.g. Q=several hundred), it needs to allocate multiple (value R) blocks of X*Y access occasions, due to the limited number of X*Y (e.g. less than or about 10).

Therefore, it is necessary to support multiple “R2D transmission triggering” after one A-IoT paging, i.e. one paging round may include multiple access occasion sets.

Proposal 2:	Following one A-IoT paging message (which selects the devices), there can be multiple R2D messages, each of which schedules a set of access occasion(s).

2.1.2	Selection of the Msg1 resources

RAN1 has some conclusion on the Msg1 FDMA:

Study FDMA of D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access, including following - How the frequency domain resources are allocated for the FDMA of D2R transmissions for Msg.1 - How a device determines the frequency domain resource for the D2R transmissions for Msg.1 Note: this does not preclude discussion on TDMA for D2R transmissions for Msg.1

After the reader assigns a certain amount of time and frequency resources for Msg1 transmission, random selection by devices can distribute many devices into those assigned access occasions. This is the very fundamental principle for contention-based random access/slotted ALOHA procedure, to reduce the collision probability.

As one general description, we propose the following Msg1 resource selection principle.

Proposal 3:	From RAN2 perspective for contention-based random access, the device randomly selects one access occasion for A-IoT Msg1 (corresponding to one time and frequency resource) from all the access occasions provided/assigned by the reader.

2.2	Re-access

A device may fail to complete contention resolution if it does not receive a successful response after it sends Msg1. Referring to other communication systems, e.g., NR/LTE, re-access designs are supported to address contention resolution failure. UE can perform random access again if a random access procedure is not successfully completed.

A-IoT devices also have the contention resolution failure issue due to collision. The devices whose transmissions collided will not get the opportunity to transmit data. For example, in the inventory case, these devices will be missed if they do not have another opportunity to access. Besides, if the device fails in A-IoT Msg3 or upper layer data transmission, it also needs to access again.

It should be one common understanding to support the re-access at least in case of random access failure.

Observation 4:	After the D2R transmission, the device should perform re-access in case of the “A-IoT procedure failure”, including the contention resolution failure and D2R data transmission failure.

If the failed devices re-select a new access occasion in the same access round, the collision probability of the devices selecting the following access occasions will be larger.

Observation 5:	Re-accessing in the same access round will cause more collisions with the devices that will perform initial access.

To solve this problem, re-access in the next round after failure needs to be supported for the A-IoT contention-based random access procedure, i.e., the reader initiates the next round of access occasions for the device(s) failing to complete the access procedure in the previous round. The device(s) that previously failed to access can access again in the new/next access round. Besides, a necessary improvement is that reader can adjust/assign the new number of access occasion in the next round according to the number of collided occasions. The collided occasion can help to determine the number of devices which have not been inventoried. The efficiency is strongly related to the number of devices and occasions. The optimal efficiency can be achieved when the number of occasions equals the number of devices. So, adjusting the suitable number of access occasions can improve the inventory efficiency.

Observation 6:	If re-accessing in the next access round, the reader is able to adjust and assign the new number of access occasions in the next round, to achieve the optimized efficiency.

Indicating/triggering more sub-occasions in one access occasion or more occasions in the same round are also possible options for the collided devices to perform re-access. However, this will increase the latency of the other devices selecting the following access occasions, especially for the devices selecting the last few access occasions, they need to wait for a long time to ensure all the devices selecting earlier access occasions finished access procedure and data transmission successfully, and so they may run out of power.

Observation 7:	Re-accessing in the same access occasion or the same round will delay the access for the other devices selecting the later occasions in this round, which may run out of power due to this delay.

In addition, in some case CN may not be able to provide the total number of expected devices, in which case reader has to blindly try to assign an initial number of occasions and then do the adjustment based on the actual contention situation.

Observation 8:	When the reader has no information on the number of expected devices, reader needs to use R2D message to adjust the number of allocated access occasions based on the situation after the initial assignment.

In summary, it is much better if the first access round is for the initial access, while the following access round is for the re-access of the devices that failed the access in the previous access round.

Proposal 4a:	The device performs the re-access in the next access round, rather than in the same access round of the initial access, so that in the next round the reader is able to adjust the number of access occasions.

Proposal 4b:	Following one A-IoT paging message (which selects the devices), it is proposed to support another R2D message (e.g., Round Trigger message):

It is used to assign/modify/adjust the total number of access occasions after this message;

It does not include the paging identifier for selecting devices (it is different from paging);

Multiple access rounds in between A-IoT paging are supported.

2.3	Failure handling

2.3.1	Msg1/2 failure detection

For the time interval between a R2D transmission and the corresponding D2R transmission following it, there are two options studied: Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D ≥ TR2D_min.

In NR, RAR window and contention resolution timer are defined for random access procedure. If the RAR window ends and Random Access Response that matches the transmitted Preamble has not been received, Random Access Response reception is considered as not successful. If the contention resolution timer expires, the contention resolution is considered as not successful.

In A-IoT, accurate timing alignment cannot be supported due to large SFO. Furthermore, the concepts of RAR window and contention resolution timer are not suitable/practical for A-IoT, since they require a register with a large number of bits to count the timer. Then, as to the handling of the A-IoT Msg1/3 transmission failure and Msg2/“Msg4” reception failure, the following cases are considered.

The device can consider A-IoT Msg1 transmission as successful if A-IoT Msg2 is received (with correct content). There is a case that reader does not receive A-IoT Msg1 successfully, then the reader deems that no device initiated the A-IoT random access in this access occasion, and will send R2D message indicating the (next) access occasion. Even if the reader receives A-IoT Msg1 successfully and sends the A-IoT Msg2, there is the case of A-IoT Msg2 reception failure at the device side. In that case, if the reader does not receive A-IoT Msg3 (after a period of time TR2D_max) after sending the A-IoT Msg2 (for several attempts), it may consider that transmission failed and send R2D message indicating the (next) access occasion (without success indication). From the device perspective, after it transmits the A-IoT Msg1, it expects to receive the A-IoT Msg2 rather than R2D message indicating the (next) access occasion. In this case, the device can consider the A-IoT random access as failed, if the access response (e.g. A-IoT Msg2) is not received successfully and it has received R2D message indicating the (next) access occasion.

Proposal 5:	RAN2 assumes the Msg1 transmission and/or Msg2 reception failure can be detected at least based on the RAN1 defined timing relationship.

2.3.2	Data re-transmission

From reader side, during a R2D-to-D2R transmission procedure, there are two abnormal conditions:

- R2D failure: A R2D message is transmitted by reader but is not received successfully by device, in which case the device does not reply the corresponding D2R message;

- The corresponding D2R failure: A R2D message is transmitted by reader and received successfully by device, but the reader fails to receive the D2R message replied by the device.

The results of both the above cases are that the reader does not successfully receive the D2R message after sending a R2D message. An effective and direct way to detect/determine such a failure is to define for the reader a maximum waiting time between a R2D transmission and the corresponding D2R reception.

TR2D_max is a requirement of maximum time in which a device should process the received R2D message and reply the corresponding D2R message. In our understanding, TR2D_max is to specify the processing requirements of different A-IoT devices.

Hence, based on the RAN1 defined TR2D_max, a reader can detect/determine the failure of R2D transmission or the following D2R transmission. The timer runs once a R2D message is transmitted by reader, the reader considers the R2D/D2R failure, if it does not successfully receive the corresponding D2R message before the timer expires.

Observation 9:	Reader can detect/determine the R2D transmission failure (or the following D2R transmission failure), based on the RAN1 defined timing relationship value TR2D_max.

If the command re-transmission is implemented by AS layer (e.g. via NACK-triggered AS re-transmission), the device needs additional AS buffer to store the un-ACKed command. This should be avoided for A-IoT devices.

To improve the reliability of the service without introducing additional design complexity, upper layers (e.g., application layer and/or NAS-like layer) can directly retransmit the command or service request, i.e. we can rely on the upper layer implementation. In addition, the reader by implementation can retransmit the command buffered by the reader, if the command or corresponding D2R message is not successfully received, in order to trigger the device to resend those upper layer D2R message again. The upper layer retransmission of the command or service request can achieve high(er) reliability.

Proposal 6a:	For D2R data re-transmission of command case:

By implementation, the reader can re-send the upper layer “command” to trigger the device to re-send the upper layer “response”.

Do not support the D2R “response” retransmission based on the NACK indication from reader (otherwise, the device needs to buffer the either the previous “command” or the un-ACKed “response”, just in case retransmission is needed).

As for the failure of device ID transmission, one way to re-trigger the device to re-send Msg3 is to let the reader to re-send Msg2 again or some failure indication. At the device side, for inventory case, the device needs to re-fetch the device ID from its memory when it observes the failure based on reader feedback. This does not require any AS buffer to buffer the device ID for re-transmission.

There is some uncertainty on whether the “not receiving device ID” at reader side is caused by Msg2 failure or Msg3 transmission failure. Normally, the device will perform re-access (not sending the Msg3 at all) if there is no correct Msg2 received. However, the reader may not be able to differentiate this kind of failure and the actual Msg3 transmission failure. If the reader detects some D2R transmission but fails to decode the Msg3, sending just one NACK indication should be sufficient. If the reader detects nothing, there could be either the Msg2 failure (in which case the device will perform re-access later), or the Msg3 transmission failure. In this case, immediately sending Msg2 is the safe way to address both the Msg2 failure and Msg3 transmission, because the device will again process the Msg2 and send the Msg3 accordingly.

Observation 10:	Pure NACK/failure indication-based device ID re-transmission may not be able to address the device ID transmission failure due to Msg2 reception failure.

Proposal 6b:	For D2R data re-transmission of inventory case, RAN2 to discuss whether to support the device ID re-transmission triggered by the Msg2 re-transmission or failure indication from reader (aiming for no AS buffer of the device ID).

2.3.3	Failure/success indication of D2R

In the email discussion [POST127][033][AIoT] Random Access, there are two major directions to handle the AS reliability of D2R data as to the failure/success indication:

- Direction 1: The device needs to know the result of D2R failure or success, so that the device can decide whether to response the subsequent paging/access occasion (to avoid redundant response).

- Direction 2: When the device is not sure about the D2R failure or success, the device considers successful D2R transmission by default and will not response the subsequent paging/access occasion (even if the D2R data actually fails).

In case the reader may schedule some further access occasion for re-access to handle the devices that failed the access due to contention, it is efficient to also immediately handle the failure due to D2R data transmission, rather than waiting for the CN to re-initiate a new service.

In addition, the CN may not be able to identify the missed device ID reporting, since the CN may not have the full list of devices which are supposed to respond in the inventory case.

Therefore, we see highly beneficial and efficient to let the reader to handle the transmission reliability by AS layer solution. Note that the additional efforts is just one 1bit indication, which can be piggyback in any R2D message, while the re-access solution will be supported due to other cases anyway.

Proposal 7:	The piggyback failure/success indication is supported for D2R data transmission, in case the reader intends to support the AS reliability. FFS on the details and its presence.

2.4	Contention free access

After the trigger by A-IoT paging message, the very first D2R message for contention-based access and contention-free access is different. Thus, how to determine whether the service is triggered for contention-based access or contention-free access needs to be considered. In the A-IoT paging message, the reader can indicate whether the current random access is for contention-free access and allocate the associated device specific access resources for contention-free access case.

As to the detailed signalling format in A-IoT paging for device specific resource provision, it can be discussed in stage-3. For example, each access occasion resource is associated with one device ID in the A-IoT paging.

Proposal 8a:	The reader indicates whether the current random access is contention-free access type.

Proposal 8b:	The reader provides the device specific access occasion resource in contention-free case. The mapping mechanism from one device into the specific access occasion resource is FFS as stage-3 signalling design.

2.5	Way forward on 2step vs. 3step RA

Compared to 3-step RA, 2-step RA requires a multiplexing processing in Msg1, i.e., multiplexing the random ID (an AS payload) and the data (upper-layer payload, e.g., NAS-like or application-layer payload). It needs a cross-layer processing when generating Msg1, causing additional processing complexity.

Observation 11:	The D2R multiplexing in Msg1 (including both device ID and random ID) will cause some processing complexity.

Though 2-step CBRA can save one step compared to 3-step RA (Msg4 does not need to be always sent in 4-step RA), the efficiency will not be better than 3-step RA. As described in Sec 2.4.3 (re-access), once Msg1 collides, Msg1 needs to be retransmitted in the later access occasion. In 2-step CBRA, Device ID, e.g., 96 bits, needs to be retransmitted if Msg1 collides. In 3-step RA, a random ID, e.g., 16 bits, needs to be retransmitted if Msg1 collides. By comparison, in 2-step CBRA, retransmitting a larger Msg1 in case of Msg1 collision will result in a larger latency.

On the other hand, allocating more access occasions can reduce Msg1 collision probability, therefore improving the efficiency. But this causes more idle/empty occasions, which leads to wasted time and increases the total inventory latency.

Furthermore, to compare the performance of 2-step CBRA and 3-step RA procedure, we calculate and compare the total latency of identifying the same amounts of devices by these two methods, and get the following conclusions. The evaluation results are shown in Appendix.

Observation 12:	The 2-step CBRA can save one step of data transmission (to reduce the latency) in case of low probability of Msg1 collision. But, once we reduce the collision probability by using a large number of access occasions, the unused access occasions will then increase the latency.

Observation 13:	The 2-step CBRA has low efficiency in case of relatively high probability of Msg1 collision.

Observation 14:	Based on the simulation/calculation in the Appendix, the total latency of 2-step CBRA is larger than 3-step RA.

Observation 15:	The 3-step RA has universal applicability, while 2-step RA is only beneficial in very few scenarios.

Based on the above analyses, it is proposed to at least first support the 3-step RA and the support of the 2-step RA is FFS.

Proposal 9a:	The solution of “A-IoT Msg1 without data” is anyway supported in A-IoT random access procedure.

To further consider the need of 2-step RA, it will be good to have the commonality with 3-step RA and less requirement on supporting the Msg1 multiplexing with the upper layer data. A new solution could be up to device implementation to decide whether to multiplex the upper layer data in Msg1, while the Msg2 handling will be same for any solution. This can reduce the complexity caused by device capability coordination or by reader to indicate the RA type.

Proposal 9b:	Add Solution 3 in the TR for further down-selection, i.e. Msg1 optionally with data.

A-IoT Msg1: random ID is mandatory, and FFS on how the device determines whether to also include the optional upper layer data;

A-IoT Msg2 echoes the random ID for contention resolution.

# 3	Conclusion

This contribution makes the following observations and proposals:

Observation 1:	From RAN2 perspective, slotted-ALOHA needs to support the distribution of many devices (value N), selected by the one A-IoT paging, into similar number of access occasions (value Q).

Observation 2:	One A-IoT paging message may select up to several hundred of devices (or possibly even more).

Observation 3:	When reader intends to allocate many access occasions (e.g. Q=several hundred), it needs to allocate multiple (value R) blocks of X*Y access occasions, due to the limited number of X*Y (e.g. less than or about 10).

Observation 4:	After the D2R transmission, the device should perform re-access in case of the “A-IoT procedure failure”, including the contention resolution failure and D2R data transmission failure.

Observation 5:	Re-accessing in the same access round will cause more collisions with the devices that will perform initial access.

Observation 6:	If re-accessing in the next access round, the reader is able to adjust and assign the new number of access occasions in the next round, to achieve the optimized efficiency.

Observation 7:	Re-accessing in the same access occasion or the same round will delay the access for the other devices selecting the later occasions in this round, which may run out of power due to this delay.

Observation 8:	When the reader has no information on the number of expected devices, reader needs to use R2D message to adjust the number of allocated access occasions based on the situation after the initial assignment.

Observation 9:	Reader can detect/determine the R2D transmission failure (or the following D2R transmission failure), based on the RAN1 defined timing relationship value TR2D_max.

Observation 10:	Pure NACK/failure indication-based device ID re-transmission may not be able to address the device ID transmission failure due to Msg2 reception failure.

Observation 11:	The D2R multiplexing in Msg1 (including both device ID and random ID) will cause some processing complexity.

Observation 12:	The 2-step CBRA can save one step of data transmission (to reduce the latency) in case of low probability of Msg1 collision. But, once we reduce the collision probability by using a large number of access occasions, the unused access occasions will then increase the latency.

Observation 13:	The 2-step CBRA has low efficiency in case of relatively high probability of Msg1 collision.

Observation 14:	Based on the simulation/calculation in the Appendix, the total latency of 2-step CBRA is larger than 3-step RA.

Observation 15:	The 3-step RA has universal applicability, while 2-step RA is only beneficial in very few scenarios.

Slotted ALOHA

Proposal 1:	The following four terms in different levels of the slotted ALOHA procedure are used for the sake of further discussion (FFS on the needed of each):

Access occasion: An opportunity of time-frequency resource for A-IoT device(s) to perform access (e.g., transmitting the A-IoT Msg1).

Access occasion set: A set of access occasion(s) for different device(s), which is scheduled via one R2D message by the reader.

Access round: One access round consists of a certain amount of access occasion sets for difference devices, whose amount are assigned via one R2D message (e.g. Round Trigger message) by the reader.

Paging round: One paging round consists of one or multiple access rounds, which is initiated by the A-IoT paging message. One service request may be associated with multiple paging rounds.

Proposal 2:	Following one A-IoT paging message (which selects the devices), there can be multiple R2D messages, each of which schedules a set of access occasion(s).

Proposal 3:	From RAN2 perspective for contention-based random access, the device randomly selects one access occasion for A-IoT Msg1 (corresponding to one time and frequency resource) from all the access occasions provided/assigned by the reader.

Re-access

Proposal 4a:	The device performs the re-access in the next access round, rather than in the same access round of the initial access, so that in the next round the reader is able to adjust the number of access occasions.

Proposal 4b:	Following one A-IoT paging message (which selects the devices), it is proposed to support another R2D message (e.g., Round Trigger message):

It is used to assign/modify/adjust the total number of access occasions after this message;

It does not include the paging identifier for selecting devices (it is different from paging);

Multiple access rounds in between A-IoT paging are supported.

Failure handling

Proposal 5:	RAN2 assumes the Msg1 transmission and/or Msg2 reception failure can be detected at least based on the RAN1 defined timing relationship.

Proposal 6a:	For D2R data re-transmission of command case:

By implementation, the reader can re-send the upper layer “command” to trigger the device to re-send the upper layer “response”.

Do not support the D2R “response” retransmission based on the NACK indication from reader (otherwise, the device needs to buffer the either the previous “command” or the un-ACKed “response”, just in case retransmission is needed).

Proposal 6b:	For D2R data re-transmission of inventory case, RAN2 to discuss whether to support the device ID re-transmission triggered by the Msg2 re-transmission or failure indication from reader (aiming for no AS buffer of the device ID).

Proposal 7:	The piggyback failure/success indication is supported for D2R data transmission, in case the reader intends to support the AS reliability. FFS on the details and its presence.

Contention free access

Proposal 8a:	The reader indicates whether the current random access is contention-free access type.

Proposal 8b:	The reader provides the device specific access occasion resource in contention-free case. The mapping mechanism from one device into the specific access occasion resource is FFS as stage-3 signalling design.

Way forward on 2step vs. 3step RA

Proposal 9a:	The solution of “A-IoT Msg1 without data” is anyway supported in A-IoT random access procedure.

Proposal 9b:	Add Solution 3 in the TR for further down-selection, i.e. Msg1 optionally with data.

A-IoT Msg1: random ID is mandatory, and FFS on how the device determines whether to also include the optional upper layer data;

A-IoT Msg2 echoes the random ID for contention resolution.

# 4	References

- R2-xxx Report of [POST127][033][AIoT] Random Access

- Y. Maguire and R. Pappu, “An Optimal Q-Algorithm for the ISO 18000-6C RFID Protocol,” in IEEE Transactions on Automation Science and Engineering, 2009.

5	Appendix

Considering most transmission parameters and message sizes for A-IoT are still under study, we take the related parameters captured in TR 38.848, TR 22.840 and RFID spec. According to TR 38.848, we set:

- data rate as 500 bps and,

- the size of device ID as 96 bits (in SA1 TR 22.840).

According to RFID spec, we set:

- the size of random ID as 16 bits.

We assume each packet has an 8-bit header (including both AS and upper-layer headers). According to the feature of Slotted ALOHA, we assume each device randomly select an occasion to access in each round, the collided devices can re-access in the next round.

There are three possibilities for each occasion as below, and for each we give the estimated latency by considering the assumed data rate.

- Successful occasion: one access occasion is selected by one device and the access succeeds

  - 3-step RA (Figure 5-1(a)):

    - including one access occasion trigger message (8 bits), one Msg1 (8-bit header + 16-bit random ID = 24 bits), one Msg2 (8-bit header + 16-bit random ID = 24 bits), and Msg3 (8-bit header + 96-bit device ID = 108 bits)

  - 2-step RA (Figure 5-1(b)):

    - including one access occasion trigger message (8 bits), one Msg1 with data (8-bit header + 96-bit device ID + 16-bit random ID = 120 bits) and one Msg2 (8-bit header + 16-bit random ID = 24 bits);

- Collided occasion (Figure 5-1(c)): one access occasion is selected by multiple devices and the access fails

  - including one access occasion trigger message (8 bits) and one Msg1 (24 bits)/Msg1 with data (120 bits);

- Idle occasion (Figure 5-1(b)): one access occasion is not selected by any device

  - including one access occasion trigger message (8 bits) and a minimum waiting time to at least check detect a 8-bit header;

Figure 5-1(a):	Successful occasion of using Msg1 without data and with data

Figure 5-1(b):	Collided and Idle occasion of using without data and with data

Many existing papers illustrated that the theoretical optimal efficiency of Slotted ALOHA is about 36.8% [2], and the optimal efficiency can be achieved when the number of slots equals the number of devices. Hence, we assume the total number of slots to equal the number of unidentified devices in every round.

Table 5-1 and Figure 5-2 illustrate the performance results on latency for identifying different number of devices. It is observed that the 3-step RA can achieve a lower latency, and achieves more significant improvement of efficiency as the number of devices increases.

Table 5-1	Performance results on latency of Msg1 with/without data

| The number of devices | 200 | 180 | 160 | 140 | 120 | 100 | 80 | 60 | 40 | 20 | 15 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The overall latency for Msg1 with data (seconds) | 96.9 | 87.1 | 76.5 | 67.0 | 58.4 | 48.8 | 38.6 | 28.5 | 18.6 | 9.4 | 7.3 | 4.7 |
| The overall latency for Msg1 without data (seconds) | 69.1 | 62.7 | 55.5 | 48.6 | 41.9 | 34.9 | 27.8 | 20.7 | 13.7 | 6.9 | 5.2 | 3.4 |
| Average probability of collision = 26.4%, Average probability of success = 36.8%, Average probability of idle occasions = 36.8% |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 5-2	Curves of latency changes as the number of devices change

Furthermore, we give the results of allocating more occasions, in which a lower probability of collision (about 9.2%) can be achieved, i.e., in each round, the number of slots is set to twice the number of devices. As shown in Table 5-2 and Figure 5-3, 3-step RA performs better, and achieves more significant improvement of efficiency as the number of devices increases. Though 2-step RA works better with more allocated slots, but still cannot catch up with 3-step RA. The reason is that allocating more slots can achieve a lower collision probability, but the drawback is that there are more idle slots, which leads to wasted time.

According to the above results, though 2-step RA can save one step, the efficiency cannot be greater than 3-step RA because once Msg1 (including Device ID) collide, a longer packet will be wasted and be retransmitted in the next access occasion. On the other hand, allocating much more slots can reduce Msg1 collision. However, this causes more idle/empty slots at the same time, which increases the total latency.

Table 5-2	Performance results on latency of Msg1 with/without data

NOTE: the number of occasions is set to twice the number of devices in each round (i.e. more allocated occasions)

| The number of devices | 200 | 180 | 160 | 140 | 120 | 100 | 80 | 60 | 40 | 20 | 15 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The overall latency of Msg1 with data (seconds) | 83.18 | 74.80 | 66.44 | 58.13 | 49.58 | 41.59 | 33.05 | 24.66 | 16.59 | 8.25 | 6.29 | 4.11 |
| The overall latency of Msg1 without data (seconds) | 72.79 | 65.50 | 58.23 | 50.93 | 43.56 | 36.41 | 29.03 | 21.71 | 14.54 | 7.26 | 5.48 | 3.61 |
| Average probability of collision = 9.2%, Average probability of success = 29.9%, Average probability of idle occasions = 60.9% |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 5-3	Curves of latency changes as the number of devices change (more allocated occasion)