3GPP TSG-RAN WG2 Meeting #127bis	R2-2408252

Hefei, China, Oct 14th -18th, 2024

Agenda Item:	7.15.2

Source:	NEC

Title:	TP for SL enh in TS 38.321

Document for:	Discussion, Decision

Introduction

In this contribution, we will discuss some remaining issues in current spec for SL further enhancement.

Discussion

In Rel-18 SL further enhancement, two main features have been introduced into sidelink technology, which are SL carrier aggregation and sidelink unlicenced carrier. Yet whether these two features can be co-existed into one network system has been discussed quite a lot.

According to the discussion happened in RAN2 #125bis meeting, the following agreement has been made:

Proposal 1: RAN2 agrees the revised Option 1 as follows:  [Option 1]: Keep the agreement in Table 1 made in RAN2 #124, and restrict NW implementation that a Rel-18 NW cannot (pre-)configure both SL-U and SL-CA.

It means network can only configure sidelink carrier aggregation or sidelink unlicenced carrier to all UEs within its coverage. Similarly, from UE side, one UE can only work with sidelink carrier aggregation or sidelink unlicenced carrier.

However, in current specification, it can be found that:

TS 38.321 ###### 5.22.1.4.1.2 Selection of logical channels and SL-PRS The MAC entity shall for each SCI corresponding to a new transmission: 1> if sl-BWP-DiscPoolConfig, sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X is configured according to TS 38.331 [5]; and 1> if COT sharing information has not been received from lower layers as specified in TS 37.213[18]; and 1> if the new transmission is not associated to a sidelink grant on Dedicated SL-PRS resource pool: 2> if the new transmission is associated to a sidelink grant in sl-DiscTxPoolSelected or sl-DiscTxPoolScheduling configured in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon: 3> select a Destination associated with NR sidelink discovery as specified in TS 23.304 [26], that is in the SL Active time for the SL transmission occasion if SL DRX is applied for the destination, and among the logical channels that satisfy all the following conditions for the SL grant associated to the SCI: 4> SL data for NR sidelink discovery is available for transmission; and 4> SBj > 0, in case there is any logical channel having SBj > 0; and 4> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and 4> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant. 2> else if the new transmission is associated to a sidelink grant in sl-TxPoolSelectedNormal configured in sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X: 3> select a Destination associated with BRID if sl-A2X-Service in sl-TxPoolSelectedNormal indicates brid or bridAndDAA or select a Destination associated with DAA if sl-A2X-Service in sl-TxPoolSelectedNormal indicates daa or bridAndDAA as specified in TS 23.256 [31], and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions for the SL grant associated to the SCI: 4> SL data for A2X communication is available for transmission; and 4> SBj > 0, in case there is any logical channel having SBj > 0. 2> else: 3> select a Destination associated to one of unicast, groupcast and broadcast (excluding the Destination(s) associated with NR sidelink discovery as specified in TS 23.304 [26] or A2X communication), that is in the SL Active time for the SL transmission occasion if SL DRX is applied for the destination, and having at least one of the MAC CE and the logical channel and pending SL-PRS transmission(s) with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, and SL-PRS(s), if any for the SL grant associated to the SCI: 4> SL data for NR sidelink communication is available for transmission; and 4> SBj > 0, in case there is any logical channel having SBj > 0; and 4> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and 4> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and 4> sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI. NOTE 0: A LCH is allowed in a carrier based on whether this selected carrier is within a subset of frequencies associated with all the PC5 QoS flows allowed to be mapped to this LCH based on RRC configuration. 1> else: 2> if COT sharing information has been received from lower layers as specified in TS 37.213[18]: 3> if resources used for initial transmission for the SL grant associated to the SCI are within the COT duration and MAC entity decides to use shared COT with type-2 LBT: 4> select a Destination associated to one of unicast, groupcast and broadcast that satisfy the following destination condition and CAPC condition, and having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and the MAC CE(s) that satisfies CAPC and destination requirement, if any, for the SL grant associated to the SCI: 5> if a Source Layer-1 ID and a Destination Layer-1 ID contained in the COT initiator's SCI or COT sharing information match to the 8 LSB of the corresponding Destination Layer-2 ID and the 16 LSB of a Source Layer-2 IDs relating to the transmission from the responding UE and the cast type indicator in the SCI is set to unicast; or if a Destination Layer-1 ID contained in the COT initiator's SCI or COT sharing information matches to the 16 LSB of a Destination Layer-2 ID relating to the transmission from the responding UE and the cast type indicator in the SCI is set to groupcast or broadcast; and 5> if a CAPC value of the SL data has an equal or smaller CAPC value than a CAPC value indicated in the COT sharing information; and 5> SL data is available for transmission; and 5> SBj > 0, in case there is any logical channel having SBj > 0; and 5> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and 5> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and 5> sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI. 2> else if sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon is not configured according to TS 38.331 [5]; and 2> if the new transmission is not associated to a sidelink grant on Dedicated SL-PRS resource pool: 3> select a Destination associated to one of unicast, groupcast and broadcast, that is in the SL Active time for the SL transmission occasion if SL DRX is applied for the destination, and having at least one of the MAC CE and the logical channel and pending SL-PRS transmission(s) with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, and SL-PRS(s), if any, for the SL grant associated to the SCI: 4> SL data is available for transmission; and 4> SBj > 0, in case there is any logical channel having SBj > 0; and 4> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and 4> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and 4> sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI; and 4> allowed on the carrier where the SCI is transmitted, if the carrier is configured by upper layers according to TS 38.331 [5] and TS 23.287 [19], if multiple carrier frequencies are configured and if the MAC entity has been configured with Sidelink resource allocation mode 2; and 4> having a priority whose associated sl-threshCBR-FreqReselection is no lower than the CBR of the carrier when the carrier is (re-)selected in accordance with clause 5.22.1.11, if multiple carrier frequencies are configured and if the MAC entity has been configured with Sidelink resource allocation mode 2. NOTE 1: If multiple Destinations have the logical channels satisfying all conditions above with the same highest priority or if multiple Destinations have either the MAC CE and/or the logical channels satisfying all conditions above with the same priority as the MAC CE, which Destination is selected among them is up to UE implementation. 1> else: (i.e., the sidelink grant is associated with Dedicated SL-PRS resource pool) 2> select a Destination corresponding to the cast type of the SL grant and having pending SL-PRS transmission(s) with the highest priority for the SL grant associated to the SCI. 1> select the logical channels satisfying all the following conditions among the logical channels belonging to the selected Destination when the new transmission is not associated to a sidelink grant on Dedicated SL-PRS resource pool: 2> SL data is available for transmission; and 2> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and. 2> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and 2> allowed on the carrier where the SCI is transmitted, if the carrier is configured by upper layers according to TS 38.331 [5] and TS 23.287 [19], if multiple carrier frequencies are configured and if the MAC entity has been configured with Sidelink resource allocation mode 2; and 2> having a priority whose associated sl-threshCBR-FreqReselection is no lower than the CBR of the carrier when the carrier is (re-)selected in accordance with clause 5.22.1.11, if multiple carrier frequencies are configured if the MAC entity has been configured with Sidelink resource allocation mode 2; and 2> CAPC value of the SL data has an equal or smaller CAPC value than a CAPC value indicated in the COT sharing information if MAC decides to use the shared COT; and 2> sl-HARQ-FeedbackEnabled is set to the value that satisfies the following conditions: 3> if PSFCH is configured for the sidelink grant associated to the SCI and the UE is capable of PSFCH reception: 4> sl-HARQ-FeedbackEnabled is set to enabled, if sl-HARQ-FeedbackEnabled is set to enabled for the highest priority logical channel satisfying the above conditions; or 4> sl-HARQ-FeedbackEnabled is set to disabled, if sl-HARQ-FeedbackEnabled is set to disabled for the highest priority logical channel satisfying the above conditions. 3> else: 4> sl-HARQ-FeedbackEnabled is set to disabled. NOTE 2: HARQ feedback enabled/disabled indicator is set to disabled for the transmission of a MAC PDU only carrying CSI reporting MAC CE or Sidelink DRX Command MAC CE or Sidelink Inter-UE Coordination Request MAC CE or Sidelink Inter-UE Coordination Information MAC CE.

The green highlighted condition is used for justification for sidelink carrier aggregation, and the yellow highlighted condition is used for justification for sidelink unlicenced carrier. However, in the upper hierarchy, it mentioned as in red highlighted text, that all of the following conditions shall be satisfied, including both green highlighted condition and yellow highlighted condition, where it is somehow against the previous agreement.

In TS 38.321 5.22.1.4.1.2, for logical channel selection, conditions for both sidelink CA and SL-U needs to be satisfied at the same time, where it is against previous agreement

Therefore, it is suggested to remove the yellow highlighted condition and adopt the corresponding TP1 in the reference section.

It is suggested to remove the condition for SL-U and adopt the corresponding TP1.

Another editorial issue is about the IE alignment between TS 38.321 and TS 38.331, for the referred RRC parameters mentioned in section 5.31.2, which are sl-lbt-FailureInstanceMaxCount, sl-lbt-FailureDetectionTimer and sl-LBT-RecoveryTimer, shall be correspondingly sl-LBT-FailureDetectionTimer, sl-LBT-FailureInstanceMaxCount and sl-LBT-RecoveryTimer defined in TS 38.331, where they are not accurately captured. Therefore, it is suggested to change the IE sl-lbt-FailureInstanceMaxCount and sl-lbt-FailureDetectionTimer into sl-LBT-FailureInstanceMaxCount and sl-LBT-FailureDetectionTimer to keep align with TS 38.331 and adopt the corresponding TP2.

In section 5.31.2, it is suggested to change the IE sl-lbt-FailureInstanceMaxCount, sl-lbt-FailureDetectionTimer and sl-lbt-FailureRecoveryConfig into sl-LBT-FailureInstanceMaxCount, sl-LBT-FailureDetectionTimer and SL-LBT-FailureRecoveryConfig to keep align with TS 38.331

It is suggested to adopt the corresponding TP2.

Conclusion

In this contribution, we have discussed the remaining issue in TS 38.321 for sidelink further enhancement, the brunch of observation and proposal are given in the following:

Observation 1	In TS 38.321 5.22.1.4.1.2, for logical channel selection, conditions for both sidelink CA and SL-U needs to be satisfied at the same time, where it is against previous agreement

Proposal 1	It is suggested to remove the condition for SL-U and adopt the corresponding TP1.

Proposal 2	In section 5.31.2, it is suggested to change the IE sl-lbt-FailureInstanceMaxCount and sl-lbt-FailureDetectionTimer into sl-LBT-FailureInstanceMaxCount and sl-LBT-FailureDetectionTimer to keep align with TS 38.331

Proposal 3	It is suggested to adopt the corresponding TP2.

Reference

### TP1 - 5.22.1.4.1.2 Selection of logical channels and SL-PRS The MAC entity shall for each SCI corresponding to a new transmission: 1> if sl-BWP-DiscPoolConfig, sl-BWP-DiscPoolConfigCommon, sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X is configured according to TS 38.331 [5]; and 1> if COT sharing information has not been received from lower layers as specified in TS 37.213[18]; and 1> if the new transmission is not associated to a sidelink grant on Dedicated SL-PRS resource pool: 2> if the new transmission is associated to a sidelink grant in sl-DiscTxPoolSelected or sl-DiscTxPoolScheduling configured in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon: 3> select a Destination associated with NR sidelink discovery as specified in TS 23.304 [26], that is in the SL Active time for the SL transmission occasion if SL DRX is applied for the destination, and among the logical channels that satisfy all the following conditions for the SL grant associated to the SCI: 4> SL data for NR sidelink discovery is available for transmission; and 4> SBj > 0, in case there is any logical channel having SBj > 0; and 4> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and 4> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant. 2> else if the new transmission is associated to a sidelink grant in sl-TxPoolSelectedNormal configured in sl-BWP-PoolConfigA2X or sl-BWP-PoolConfigCommonA2X: 3> select a Destination associated with BRID if sl-A2X-Service in sl-TxPoolSelectedNormal indicates brid or bridAndDAA or select a Destination associated with DAA if sl-A2X-Service in sl-TxPoolSelectedNormal indicates daa or bridAndDAA as specified in TS 23.256 [31], and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions for the SL grant associated to the SCI: 4> SL data for A2X communication is available for transmission; and 4> SBj > 0, in case there is any logical channel having SBj > 0. 2> else: 3> select a Destination associated to one of unicast, groupcast and broadcast (excluding the Destination(s) associated with NR sidelink discovery as specified in TS 23.304 [26] or A2X communication), that is in the SL Active time for the SL transmission occasion if SL DRX is applied for the destination, and having at least one of the MAC CE and the logical channel and pending SL-PRS transmission(s) with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, and SL-PRS(s), if any for the SL grant associated to the SCI: 4> SL data for NR sidelink communication is available for transmission; and 4> SBj > 0, in case there is any logical channel having SBj > 0; and 4> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and 4> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and 4> sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI. NOTE 0: A LCH is allowed in a carrier based on whether this selected carrier is within a subset of frequencies associated with all the PC5 QoS flows allowed to be mapped to this LCH based on RRC configuration. 1> else: 2> if COT sharing information has been received from lower layers as specified in TS 37.213[18]: 3> if resources used for initial transmission for the SL grant associated to the SCI are within the COT duration and MAC entity decides to use shared COT with type-2 LBT: 4> select a Destination associated to one of unicast, groupcast and broadcast that satisfy the following destination condition and CAPC condition, and having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and the MAC CE(s) that satisfies CAPC and destination requirement, if any, for the SL grant associated to the SCI: 5> if a Source Layer-1 ID and a Destination Layer-1 ID contained in the COT initiator's SCI or COT sharing information match to the 8 LSB of the corresponding Destination Layer-2 ID and the 16 LSB of a Source Layer-2 IDs relating to the transmission from the responding UE and the cast type indicator in the SCI is set to unicast; or if a Destination Layer-1 ID contained in the COT initiator's SCI or COT sharing information matches to the 16 LSB of a Destination Layer-2 ID relating to the transmission from the responding UE and the cast type indicator in the SCI is set to groupcast or broadcast; and 5> if a CAPC value of the SL data has an equal or smaller CAPC value than a CAPC value indicated in the COT sharing information; and 5> SL data is available for transmission; and 5> SBj > 0, in case there is any logical channel having SBj > 0; and 5> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and 5> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and 5> sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI. 2> else if sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon is not configured according to TS 38.331 [5]; and 2> if the new transmission is not associated to a sidelink grant on Dedicated SL-PRS resource pool: 3> select a Destination associated to one of unicast, groupcast and broadcast, that is in the SL Active time for the SL transmission occasion if SL DRX is applied for the destination, and having at least one of the MAC CE and the logical channel and pending SL-PRS transmission(s) with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, and SL-PRS(s), if any, for the SL grant associated to the SCI: 4> SL data is available for transmission; and 4> SBj > 0, in case there is any logical channel having SBj > 0; and 4> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and 4> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and 4> sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI; and 4> allowed on the carrier where the SCI is transmitted, if the carrier is configured by upper layers according to TS 38.331 [5] and TS 23.287 [19], if multiple carrier frequencies are configured and if the MAC entity has been configured with Sidelink resource allocation mode 2; and 4> having a priority whose associated sl-threshCBR-FreqReselection is no lower than the CBR of the carrier when the carrier is (re-)selected in accordance with clause 5.22.1.11, if multiple carrier frequencies are configured and if the MAC entity has been configured with Sidelink resource allocation mode 2. NOTE 1: If multiple Destinations have the logical channels satisfying all conditions above with the same highest priority or if multiple Destinations have either the MAC CE and/or the logical channels satisfying all conditions above with the same priority as the MAC CE, which Destination is selected among them is up to UE implementation. 1> else: (i.e., the sidelink grant is associated with Dedicated SL-PRS resource pool) 2> select a Destination corresponding to the cast type of the SL grant and having pending SL-PRS transmission(s) with the highest priority for the SL grant associated to the SCI. 1> select the logical channels satisfying all the following conditions among the logical channels belonging to the selected Destination when the new transmission is not associated to a sidelink grant on Dedicated SL-PRS resource pool: 2> SL data is available for transmission; and 2> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and. 2> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant; and 2> allowed on the carrier where the SCI is transmitted, if the carrier is configured by upper layers according to TS 38.331 [5] and TS 23.287 [19], if multiple carrier frequencies are configured and if the MAC entity has been configured with Sidelink resource allocation mode 2; and 2> having a priority whose associated sl-threshCBR-FreqReselection is no lower than the CBR of the carrier when the carrier is (re-)selected in accordance with clause 5.22.1.11, if multiple carrier frequencies are configured if the MAC entity has been configured with Sidelink resource allocation mode 2; and 2> sl-HARQ-FeedbackEnabled is set to the value that satisfies the following conditions: 3> if PSFCH is configured for the sidelink grant associated to the SCI and the UE is capable of PSFCH reception: 4> sl-HARQ-FeedbackEnabled is set to enabled, if sl-HARQ-FeedbackEnabled is set to enabled for the highest priority logical channel satisfying the above conditions; or 4> sl-HARQ-FeedbackEnabled is set to disabled, if sl-HARQ-FeedbackEnabled is set to disabled for the highest priority logical channel satisfying the above conditions. 3> else: 4> sl-HARQ-FeedbackEnabled is set to disabled. NOTE 2: HARQ feedback enabled/disabled indicator is set to disabled for the transmission of a MAC PDU only carrying CSI reporting MAC CE or Sidelink DRX Command MAC CE or Sidelink Inter-UE Coordination Request MAC CE or Sidelink Inter-UE Coordination Information MAC CE.

TP2 - 5.31.2 Sidelink LBT failure detection and recovery procedure The MAC entity may be configured by RRC with a Sidelink consistent LBT failure detection and recovery procedure. Sidelink consistent LBT failure is detected per RB set by counting SL LBT failure indications, for all SL transmissions, from the lower layers to the MAC entity. RRC configures the following parameters in the SL-LBT-FailureRecoveryConfig: - sl-LBT-FailureInstanceMaxCount for the Sidelink consistent LBT failure detection; - sl-LBT-FailureDetectionTimer for the Sidelink consistent LBT failure detection; - sl-LBT-RecoveryTimer for recovery of the triggered Sidelink consistent LBT failure. The following UE variable is used for the Sidelink consistent LBT failure detection procedure: - SL_LBT_COUNTER (per RB set): counter for SL LBT failure indication which is initially set to 0. For activated SL BWP configured with SL-LBT-FailureRecoveryConfig, the MAC entity shall: 1> if SL LBT failure indication has been received from lower layers for an RB set of the configured pool(s) of resources in the SL BWP: 2> start or restart the sl-LBT-FailureDetectionTimer for the RB set; 2> increment SL_LBT_COUNTER for the RB set by 1; 2> if SL_LBT_COUNTER >= sl-LBT-FailureInstanceMaxCount: 3> trigger Sidelink consistent LBT failure for the RB set in the SL BWP; 3> if consistent LBT failure has been triggered in all the RB sets of the configured pool(s) of resources in the SL BWP: 4> indicate Sidelink consistent LBT failure based Sidelink RLF detection for all destination IDs associated to unicast service to upper layers. 1> if all triggered Sidelink consistent LBT failures are cancelled in a RB set; or 1> if the sl-LBT-FailureDetectionTimer expires for a RB set: 2> set SL_LBT_COUNTER to 0 for the RB set. 1> if sl-LBT-FailureDetectionTimer or sl-LBT-FailureInstanceMaxCount is reconfigured by upper layers: 2> set SL_LBT_COUNTER to 0 for all the RB sets. The sl-LBT-RecoveryTimer is used for recovery of the triggered Sidelink consistent LBT failure, when RRC configures Sidelink resource allocation mode 2. The MAC entity shall: 1> if Sidelink consistent LBT failure has been triggered, and not cancelled, in the RB set(s), and SL LBT failure MAC CE for the triggered Sidelink consistent LBT failure has not been generated; 2> if the sl-LBT-RecoveryTimer for the triggered Sidelink consistent LBT failure is not running: 3> start the sl-LBT-RecoveryTimer. 2> if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the SL LBT failure MAC CE plus its subheader as a result of logical channel prioritization according to clause 5.4.3.1: 3> instruct the Multiplexing and Assembly procedure in clause 5.4.3 to generate the SL LBT failure MAC CE(s). 2> else: 3> trigger a Scheduling Request for SL LBT failure MAC CE. 1> if a MAC PDU is transmitted and this PDU includes the SL LBT failure MAC CE; or 2> cancel the triggered Sidelink consistent LBT failure(s) in RB set(s) for which Sidelink consistent LBT failure was indicated in the transmitted SL LBT failure MAC CE if the MAC entity has been configured with Sidelink resource allocation mode 1. 1> if the sl-LBT-RecoveryTimer for the triggered Sidelink consistent LBT failure(s) expires: 2> cancel the triggered Sidelink consistent LBT failure(s) in RB set(s) for which Sidelink consistent LBT failure was detected. 1> if SL-LBT-FailureRecoveryConfig is reconfigured by upper layers for the BWP: 2> cancel all the triggered Sidelink consistent LBT failure(s) in the SL BWP.