3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408256

# Hefei, China, 14th – 18th October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4994 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous CR for Rel-18 SL relay enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core |  |  |  |  |  | Date: |  |  | 2024.10.14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In 5.8.9.1.2, the Tx UE may generate a RRCReconfigurationSide message to release and add/modify RLC channels. Ususally, it should first process release related procedure and then addition/modification related procedure, so the release related description should be moved above addition/modification related description. - In 5.8.9.1a.4, both of L2 Relay UE and Remote UE need to derive the SRAP bearer mapping for end-to-end SRBs, so the restriction to Remote UE needs to be removed. - There are editorial changes in 5.8.9.7.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 5.8.9.1.2, for RLC channel related behaviour, the release related description is moved ahead of addition/modification related description. - In 5.8.9.1a.4, for SRAP bearer mapping related description, Relay UE behaviour is added. - Some editorial changes are made in 5.8.9.7.2. Impact analysis Impacted functionality: NR sidelink U2U relay operation Impacted architecture options: NR SA Inter-operability: - There are no inter-operability issues, considering this CR is only to clarify UE behaviour. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without the change, some errors remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.1.2, 5.8.9.1a.4, 5.8.9.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

5.8.9.1.2	Actions related to transmission of RRCReconfigurationSidelink message

The UE shall set the contents of RRCReconfigurationSidelink message as follows:

1>	for each sidelink DRB that is to be released, according to clause 5.8.9.1a.1.1, due to configuration by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR, by upper layers, or due to end-to-end sidelink DRB release:

2>	set the entry included in the slrb-ConfigToReleaseList corresponding to the sidelink DRB;

1>	for each sidelink DRB that is to be established or modified, according to clause 5.8.9.1a.2.1, due to receiving sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR:

2>	if the sidelink DRB is a per-hop sidelink DRB (i.e. the UE is performing NR sidelink communication with a peer UE without via a L2 U2U Relay UE):

3>	if a sidelink DRB is to be established:

4>	assign a new logical channel identity for the logical channel to be associated with the sidelink DRB and set sl-MAC-LogicalChannelConfigPC5 in the SLRB-Config to include the new logical channel identity;

3>	set the SLRB-Config included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig and sl-RLC-BearerConfig corresponding to the sidelink DRB;

2>	else if the sidelink DRB is an end-to-end sidelink DRB (i.e. the UE is acting as a L2 U2U Remote UE, and configure peer L2 U2U Remote UE with end-to-end SDAP and PDCP):

3>	set the SLRB-Config (excluding sl-RLC-ConfigPC5 and sl-MAC-LogicalChannelConfigPC5) included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig corresponding to the sidelink DRB;

1>	for each additional sidelink RLC bearer that is to be released, according to clause 5.8.9.1a.5.1, due to configuration by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or by upper layers:

2>	set the entry included in the sl-RLC-BearerToReleaseList corresponding to the additional sidelink RLC bearer;

1>	for each additional sidelink RLC bearer that is to be established or modified, according to clause 5.8.9.1a.6.1, due to receiving sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR:

2>	if an additional sidelink RLC bearer is to be established:

3>	assign a new logical channel identity for the logical channel to be associated with the additional sidelink RLC bearer and set sl-MAC-LogicalChannelConfigPC5 in the SL-RLC-BearerConfig to include the new logical channel identity;

2>	set the SL-RLC-BearerConfig included in the sl-RLC-BearerToAddModList, according to the received sl-RadioBearerConfig and sl-RLC-BearerConfig corresponding to the additional sidelink RLC bearer;

1>	for each carrier that is to be released, according to clause 5.8.9.1b.1.1:

2>	include the corresponding sidelink carrier in the sl-CarrierToReleaseList;

1>	for each carrier that is to be added, according to clause 5.8.9.1b.2.1:

2>	include the corresponding sidelink carrier in the sl-CarrierToAddModList;

1>	set the sl-MeasConfig as follows:

2>	If the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

3>	if UE is in RRC_CONNECTED:

4>	set the sl-MeasConfig according to stored NR sidelink measurement configuration information for this destination;

3>	if UE is in RRC_IDLE or RRC_INACTIVE:

4>	set the sl-MeasConfig according to stored NR sidelink measurement configuration received from SIB12;

2>	else:

3>	set the sl-MeasConfig according to the sl-MeasPreConfig in SidelinkPreconfigNR;

1>	set the sl-LatencyBoundIUC-Report;

1>	start timer T400 for the destination;

1>	set the sl-CSI-RS-Config;

1>	set the sl-LatencyBoundCSI-Report;

1>	set the sl-ResetConfig;

NOTE 1:	Whether/how to set the parameters included in sl-LatencyBoundIUC-Report, sl-CSI-RS-Config, sl-LatencyBoundCSI-Report and sl-ResetConfig is up to UE implementation.

1>	set the sl-DRX-ConfigUC-PC5 as follows:

2>	If the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList/sl-FreqInfoToAddModListExt in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

3>	if UE is in RRC_CONNECTED and if sl-ScheduledConfig is included in sl-ConfigDedicatedNR within RRCReconfiguration:

4>	set the sl-DRX-ConfigUC-PC5 according to stored NR sidelink DRX configuration information for this destination;

NOTE 2:	If UE is in RRC_IDLE or in RRC_INACTIVE or out of coverage, or in RRC_CONNECTED and sl-UE-SelectedConfig is included in sl-ConfigDedicatedNR within RRCReconfiguration, it is up to UE implementation to set the sl-DRX-ConfigUC-PC5.

1>	for each PC5 Relay RLC channel that is to be released due to configuration by sl-ConfigDedicatedNR:

2>	set the SL-RLC-ChannelID corresponding to the PC5 Relay RLC channel in the sl-RLC-ChannelToReleaseListPC5;

1>	for each PC5 Relay RLC channel that is to be established or modified due to receiving sl-ConfigDedicatedNR:

2>	if a PC5 Relay RLC channel is to be established:

3>	assign a new logical channel identity for the logical channel to be associated with the PC5 Relay RLC channel and set sl-MAC-LogicalChannelConfigPC5 in the SL-RLC-ChannelConfigPC5 to include the new logical channel identity;

2>	set the SL-RLC-ChannelConfigPC5 included in the sl-RLC-ChannelToAddModListPC5 according to the received SL-RLC-ChannelConfig corresponding to the PC5 Relay RLC channel, including setting sl-RLC-ChannelID-PC5 to the same value of sl-RLC-ChannelID received in SL-RLC-ChannelConfig;

1>	if the UE is operating as a L2 U2N Relay UE:

2>	if the destination UE is a L2 U2N Remote UE that requested the SFN-DFN offset in a previous RemoteUEInformationSidelink message:

3>	if the SFN-DFN offset has changed since a previous transmission of the RRCReconfigurationSidelink message, or no previous transmission of the RRCReconfigurationSidelink message has occurred since the reception of the RemoteUEInformationSidelink message:

4>	set the sl-SFN-DFN-Offset according to the relation between the SFN timeline of the PCell and the DFN timeline;

1>	if the UE is acting as L2 U2U Relay UE, and if the procedure is initiated to configure local ID pair to a connected L2 U2U Remote UE:

2>	if the local ID pair is to be assigned or modified for an end-to-end PC5 connection, and if the per-hop PC5-RRC connection with this L2 U2U Remote UE and the per-hop PC5-RRC connection with its peer L2 U2U Remote UE are successfully established:

3>	include an entry in sl-LocalID-PairToAddModList, and set the fields as below:

4>	set sl-RemoteUE-L2Identity to the source L2 ID of this L2 U2U Remote UE, and set sl-RemoteUE-LocalIdentity to include the new local UE ID assigned to this L2 U2U Remote UE, in the SL-SRAP-ConfigPC5;

4>	set sl-PeerRemoteUE-L2Identity to the destination L2 ID of the peer L2 U2U Remote UE, and set sl-PeerRemoteUE-LocalIdentity to include the new local UE ID assigned to the peer L2 U2U Remote UE, in the SL-SRAP-ConfigPC5;

1>	if the UE is acting as L2 U2U Remote UE (i.e. Tx UE) and is in RRC_IDLE or in RRC_INACTIVE or out of coverage, and the procedure is initiated to release the first hop PC5 Relay RLC channel of an end-to-end sidelink DRB to the connected L2 U2U Relay UE (i.e. Rx UE) according to clause 5.8.9.7.1; or

1>	if the UE is acting as L2 U2U Relay UE (i.e. Tx UE) and is in RRC_IDLE or in RRC_INACTIVE or out of coverage, and the procedure is initiated to release the second hop PC5 Relay RLC channel of an end-to-end sidelink DRB to the connected L2 U2U Remote UE (i.e. Rx UE) according to clause 5.8.9.7.1:

2>	set the SL-RLC-ChannelID corresponding to the PC5 Relay RLC channel in the sl-RLC-ChannelToReleaseListPC5;

1>	if the UE is acting as L2 U2U Remote UE (i.e. Tx UE) and is in RRC_IDLE or in RRC_INACTIVE or out of coverage, and if the procedure is initiated to add/modify the first hop PC5 Relay RLC channel of an end-to-end sidelink DRB to the connected L2 U2U Relay UE (i.e. Rx UE), based on configuration in SIB12 or SidelinkPreconfigNR; or

1>	if the UE is acting as L2 U2U Relay UE (i.e. Tx UE) and is in RRC_IDLE or in RRC_INACTIVE or out of coverage, and if the procedure is initiated to add/modify the second hop PC5 Relay RLC channel to the connected L2 U2U Remote UE (i.e. Rx UE) based on configuration in SIB12 or SidelinkPreconfigNR:

2>	if a PC5 Relay RLC channel is to be established:

3>	assign a new RLC channel ID and set sl-RLC-ChannelID-PC5 in the SL-RLC-ChannelConfigPC5 to include the new RLC channel ID;

3>	assign a new logical channel identity for the logical channel to be associated with the PC5 Relay RLC channel and set sl-MAC-LogicalChannelConfigPC5 in the SL-RLC-ChannelConfigPC5 to include the new logical channel identity;

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE:

3>	set the SL-RLC-ChannelConfigPC5 included in the sl-RLC-ChannelToAddModListPC5 according to the SL-RLC-BearerConfig derived based on the per-hop QoS of the end-to-end SLRB according to sl-RLC-BearerConfigList in SIB12;

2>	else if the UE is out of coverage:

3>	set the SL-RLC-ChannelConfigPC5 included in the sl-RLC-ChannelToAddModListPC5 according to the SL-RLC-BearerConfig derived based on the per-hop QoS of the end-to-end SLRB according to sl-RLC-BearerPreConfigList in SidelinkPreconfigNR.

1>	if the UE is acting as L2 U2U Remote UE (i.e. Tx UE) and is in RRC_IDLE or in RRC_INACTIVE or out of coverage, and the procedure is initiated to release the first hop PC5 Relay RLC channel of an end-to-end sidelink DRB to the connected L2 U2U Relay UE (i.e. Rx UE) according to clause 5.8.9.7.1; or

1>	if the UE is acting as L2 U2U Relay UE (i.e. Tx UE) and is in RRC_IDLE or in RRC_INACTIVE or out of coverage, and the procedure is initiated to release the second hop PC5 Relay RLC channel of an end-to-end sidelink DRB to the connected L2 U2U Remote UE (i.e. Rx UE) according to clause 5.8.9.7.1:

2>	set the SL-RLC-ChannelID corresponding to the PC5 Relay RLC channel in the sl-RLC-ChannelToReleaseListPC5.

NOTE 3:	Void.

The UE shall submit the RRCReconfigurationSidelink message to lower layers for transmission.

Next Change

5.8.9.1a.4	Sidelink SRB addition

The UE shall:

1>	if transmission of PC5-S message for a specific destination is requested by upper layers for sidelink SRB:

2>	establish PDCP entity, RLC entity and the logical channel of a sidelink SRB for PC5-S message if needed, as specified in clause 9.1.1.4;

2>	if in coverage on the frequency used for the NR sidelink communication as defined in TS 38.304 [20]:

3>	indicate the allowed carrier for the RLC bearer of the SRB before the reception of initial RRCReconfigurationCompleteSidelink message which confirms SL CA carrier(s) addition as indicated in sl-FreqInfoList, to lower layer;

2>	else:

3>	indicate the allowed carrier for the RLC bearer of the SRB before the reception of initial RRCReconfigurationCompleteSidelink message which confirms SL CA carrier(s) addition as indicated in sl-PreconfigFreqInfoList, to lower layer;

1>	if transmission of discovery message for a specific destination is requested by upper layers for sidelink SRB:

2>	establish PDCP entity, RLC entity and the logical channel of a sidelink SRB4 for discovery message, as specified in clause 9.1.1.4;

1>	if a PC5-RRC connection establishment for a specific destination is indicated by upper layers:

2>	establish PDCP entity, RLC entity and the logical channel of a sidelink SRB for PC5-RRC message of the specific destination if needed, as specified in clause 9.1.1.4;

2>	consider the PC5-RRC connection is established for the destination;

2>	if in coverage on the frequency used for the NR sidelink communication as defined in TS 38.304 [20]:

3>	indicate the allowed carrier for the RLC bearer of the SRB before the reception of initial RRCReconfigurationCompleteSidelink message which confirms SL CA carrier(s) addition, as indicated in sl-FreqInfoList, to lower layer;

2>	else:

3>	indicate the allowed carrier for the RLC bearer of the SRB before the reception of initial RRCReconfigurationCompleteSidelink message which confirms SL CA carrier(s) addition as specified in clause 5.8.9.1.9, as indicated in sl-PreconfigFreqInfoList, to lower layer;

1>	for end-to-end SRB0/1/2/3:

2>	consider the SL-U2U-RLC as specified in clause 9.1.1.4 as the egress PC5 Relay RLC channel;

2>	associate this end-to-end sidelink SRB with the SL-U2U-RLC and configure the mapping between the end-to-end sidelink SRB and the egress PC5 Relay RLC channel to SRAP.

Next Change

5.8.9.7.2	PC5 Relay RLC channel addition/modification

Upon PC5-RRC connection establishment between the L2 U2N Relay UE and L2 U2N Remote UE, the L2 U2N Relay UE shall:

1>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

1>	apply RLC specified configuration of SL-RLC0 as specified in clause 9.1.1.4:

1>	apply RLC default configuration of SL-RLC1 as defined in clause 9.2.4 if the L2 U2N Relay UE is in RRC_IDLE/INACTIVE state;

Upon PC5-RRC connection establishment between two UEs for L2 U2U relay operation, the L2 U2U Relay UE and the L2 U2U Remote UE shall:

1>	establish a SRAP entity as specified in TS 38.351 [66], if no SRAP entity has been established;

1>	apply RLC specified configuration of SL-U2U-RLC as specified in clause 9.1.1.4;

For L2 U2U Relay operation in RRC_IDLE/RRC_INACTVE or out of coverage, the PC5 Relay RLC channel addition/modification can be triggered due to the addition/modification/release of the end-to-end SL DRB(s). The source L2 U2U Remote UE and L2 U2U Relay UE derive the configuration for the corresponding PC5 Relay RLC channel based on SIB12/Preconfiguration, as follows:

-	The source L2 U2U Remote UE derives the configuration for the PC5 Relay RLC channel(s) between the source L2 U2U Remote UE and L2 U2U relay UE (i.e. the first hop PC5 Relay RLC channel(s)), by aggregating the QoS profile(s) of the QoS flow(s) with split QoS information of the first hop into a per-SLRB level QoS profile for each end-to-end Sidelink DRB as described in clause 5.8.9.11.4, and considering the SL-RLC-BearerConfig in either sl-RLC-BearerConfigList or sl-RLC-BearerPreConfigList (linked to the SL-RadioBearerConfig which matches the per-SLRB level QoS profile) as the first hop PC5 Relay RLC channel configuration.

-	The L2 U2U Relay UE derives the configuration for the PC5 Relay RLC channel(s) between L2 U2U relay UE and the target L2 U2U Remote UE (i.e. the second hop PC5 Relay RLC channel(s)), by aggregating the QoS profile(s) of the QoS flow(s) with split QoS information of the second hop into a per-SLRB level QoS profile for each end-to-end Sidelink DRB as described in clause 5.8.9.11.3, and considering the SL-RLC-BearerConfig in either sl-RLC-BearerConfigList or sl-RLC-BearerPreConfigList (linked to the SL-RadioBearerConfig which matches the per-SLRB level QoS profile) as the second hop PC5 Relay RLC channel configuration.

The UE shall:

1>	if the PC5 Relay RLC channel addition/modification was triggered due to the reception of the RRCReconfigurationSidelink message; or

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel addition/modification was triggered due to the configuration received within the sl-ConfigDedicatedNR; or

1>	after receiving the RRCReconfigurationCompleteSidelink message, if the PC5 Relay RLC channel addition/modification was triggered for an end-to-end sidelink DRB based on the configuration in SIB12 or SidelinkPreconfigNR:

2>	if the current configuration contains a PC5 Relay RLC channel with the received sl-RLC-ChannelID or sl-RLC-ChannelID-PC5; or

2>	if the configuration in SIB12 or SidelinkPreconfigNR has updated, based on which the PC5 Relay RLC channel is derived:

3>	reconfigure the sidelink RLC entity in accordance with the received sl-RLC-Config or sl-RLC-ConfigPC5;

3>	reconfigure the sidelink MAC entity with a logical channel in accordance with the received sl-MAC-LogicalChannelConfig or sl-MAC-LogicalChannelConfigPC5;

2>	else (a PC5 Relay RLC channel with the received sl-RLC-ChannelID or sl-RLC-ChannelID-PC5 was not configured before):

3>	establish a sidelink RLC entity in accordance with the received sl-RLC-Config (in sl-ConfigDedicatedNR, or SIB12, or SidelinkPreconfigNR) or sl-RLC-ConfigPC5;

3>	configure the sidelink MAC entity with a logical channel in accordance with the received sl-MAC-LogicalChannelConfig or sl-MAC-LogicalChannelConfigPC5.

End of Change