3GPP TSG-RAN WG2 Meeting #127bis	R2- 2408259

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.2

Source:	MediaTek Inc.

Title:	Further discussion on Inter-CU LTM

Document for:	Discussion, Decision

Introduction

In the last meeting, RAN2 discussed several aspects to support inter-CU LTM, including reference configuration, CSI resource and report configuration, Indication of PDCP re-establishment and so on.

This contribution further discusses the leftover issue for inter-CU SA LTM, and also the co-existence for inter-CU LTM with DC. Given that SA3 are not yet in a position to provide feedback regarding security updates, security update methods are not covered in this paper.

Discussion

Triggering order in LTM execution

Regarding the order of triggering cell switch command and interaction between the source gNB and target gNB, In Rel-18 LTM, RAN3 clarified that the serving DU may trigger the cell switch command MAC CE first, or inform the target DU before sending the cell switch command to the UE. In the reply, RAN2 agreed that the former approach (serving DU triggering LTM command first and then informing the target DU) should be the baseline to reduce potential latency for LTM execution.

Observation 1: In Rel-18 intra-CU LTM, RAN2 confirms that serving DU triggering LTM command first and then informing the target DU is the baseline.

In an intra-CU scenario, the typical latency for the F1 interface is around 10ms for round trip time. This results in approximately 10ms from the source DU informing the target DU in the same CU. The UE requires more than 10ms to perform TLTM_RRC-processing and TLTM_processing. This time frame allows the network to prepare well before the UE attaches to the target cell.

However, in an inter-CU scenario, it is worth discussing the latency for the source DU to inform the target DU for the initiation of the LTM cell switch. Typically, the latency over F1 interface is 5ms for one shot, and the latency over Xn interface is 10ms. In the last meeting, RAN2 agreed that it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell. So the signalling is send from Source DU->Source CU->Taget CU->Target DU, totally around 20ms.

For UE, the latency of UE to decode ASN.1 TLTM_RRC-processing can be saved (reduced to zero) by UE’s capability ltm-FastProcessingConfig-r18 (FG 39-6). If UE does not support this capability, the latency is 10ms. The latency for UE to perform LTM processing (TLTM_processing) is also depends on UE’s capability ltm-FastUE-Processing-r18 (FG 39-7), which can be 10~20ms for intra frequency and 20~40ms for inter-frequency cell switch.

Therefore, If the UE supports FG 39-6 and FG 39-7, the UE's total processing time (0+10ms) may be shorter than the latency from source DU to target DU (5+10+5ms), potentially leading to the target cell not being well-prepared when the UE finishes the LTM processing. This could lead to more interruptions (rather than latency) during LTM.
Observation 2: If the serving DU triggering LTM command first, and UE support fast RRC processing and fast LTM processing, the target cell may not receive the indication (5+10+5ms) when UE finish the LTM processing (0+10ms), which lead to more interruption.

Figure 1 Potential impact in Intra/inter CU network latency for LTM execution

In such cases, the network may need to inform the target DU earlier than sending LTM CSC MAC-CE to UE to align with the UE's processing time, as latency is anyway better than interruption. This behaviour can be dynamically adjusted according to UE’s capability and implemented by network.

Proposal 1: RAN2 assumes it is up to network implementation to adjust the order between sending LTM CSC MAC CE to UE and indicating target gNB for the cell switch, to always make sure that target cell is well-prepared when UE finishes the LTM processing.

Issues in inter-CU LTM with DC

Inter-CU MCG LTM with SCG configuration/release

The issue of Inter-CU MCG LTM with SCG configuration/release was raise [3] in the last meeting. The issue is that the change of MCG may lead to some reconfiguration for SCG, such as SN security update and L2 reset. This can result in cases where the SCG is released in the first LTM but reconfigured in the subsequent LTM, as shown in the following figure:

Figure 2 Potential configuration mismatch for inter-CU MCG LTM with SCG configuration

In the subsequent LTM, the SCG modification will cause configuration mismatch since the SCG was already released in the previous LTM cell switch.

During the meeting, several options were discussed:

- Option1: NW ensure that such subsequent LTM will not happen.

- Option2: UE to ignore the SCG config part in subsequent LTM.

- Option3: Extend the scope to support the configuration of this case (MCG LTM with SCG addition)

In our view, option 2 means that the UE does not follow the network configuration, which should be avoided. Option 3 to extend the scope may also not be a good choice. One possible way is for the network to avoid this configuration mismatch (option 1), or a simple way is to exclude this case and let SCG always be released.

Proposal 2: RAN2 to discuss whether to support inter-CU MCG LTM without releasing SCG:

- Option 1: Support inter-CU MCG LTM without releasing SCG. Network ensures that the UE does not switch to an LTM candidate cell with SCG configuration if the UE has released DC prior to the LTM cell switch.

- Option 2: Inter-CU MCG LTM always configured with SCG release.

Coexistence of MCG LTM and SCG LTM

In the last meeting, the issue of the coexistence between MCG LTM and SCG LTM was raised [3] but not discussed due to the time limit.

In Rel-18 LTM, it seems a common understanding that intra-MN LTM and intra-SN LTM can be configured simultaneously, but it’s up to the NW implementation to ensure they are not triggered simultaneously. However, when it comes to Rel-19 inter-CU LTM, the impact between MCG and SCG cannot be ignored. (No matter MCG or SCG becomes inter-CU)

The issue is that, if the MCG and SCG LTM (one of them is inter-CU) are configured simultaneously but not triggered simultaneously, the LTM that is triggered first will change the current configuration, leading to the subsequent LTM configurations being inconsistent with the initial intention. This can result in either incorrect configuration or behavior outside the scope of WID.

For example, if intra-MN LTM configured with inter-SN LTM, and intra-MN LTM trigger first, then the current MCG is changed, but not the MCG part in inter-SN LTM configuration. Then the subsequent inter-SN LTM will cause both MCG and SCG change, which is out of the scope of WID (This is inter-SN LTM with MCG changed).

Observation 3: If the MCG and SCG LTM (one of them is inter-CU) are configured simultaneously, the first triggered LTM will change the current configuration, leading to subsequent LTM configurations being inconsistent with the initial intention. This can result in either incorrect configuration or behavior outside the scope of WID.

However, when we look back to the WID, the intention of this topic should focus on single MN or SN inter-CU LTM, but not the scenario when they are configured simultaneously and triggered successively.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] … - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

Considering the dilemma that we faced for this scenario, we suggest excluding this “simultaneous LTM configuration” case for simplicity. If network need to configure both MCG and SCG LTM, it can first finish the inter-CU cell switch and then configured intra-MN and intra-SN LTM as Rel-18 LTM.

Proposal 3: The configuration of MCG and SCG LTM, with at least one of CG is inter-CU, is not configured simultaneously.

Conclusion

In this contribution, the following observations and proposals are made:

Triggering order in LTM execution

Observation 1: In Rel-18 intra-CU LTM, RAN2 confirms that serving DU triggering LTM command first and then informing the target DU is the baseline.

Observation 2: If the serving DU triggering LTM command first, and UE support fast RRC processing and fast LTM processing, the target cell may not receive the indication (5+10+5ms) when UE finish the LTM processing (0+10ms), which lead to more interruption.

Proposal 1: RAN2 assumes it is up to network implementation to adjust the order between sending LTM CSC MAC CE to UE and indicating target gNB for the cell switch, to always make sure that target cell is well-prepared when UE finishes the LTM processing.

Inter-CU MCG LTM with SCG configuration/release

Proposal 2: RAN2 to discuss whether to support inter-CU MCG LTM without releasing SCG:

- Option 1: Support inter-CU MCG LTM without releasing SCG. Network ensures that the UE does not switch to an LTM candidate cell with SCG configuration if the UE has released DC prior to the LTM cell switch.

- Option 2: Inter-CU MCG LTM always configured with SCG release.

Coexistence of MCG LTM and SCG LTM

Observation 3: If the MCG and SCG LTM (one of them is inter-CU) are configured simultaneously, the first triggered LTM will change the current configuration, leading to subsequent LTM configurations being inconsistent with the initial intention. This can result in either incorrect configuration or behavior outside the scope of WID.

Proposal 3: The configuration of MCG and SCG LTM, with at least one of CG is inter-CU, is not configured simultaneously.

References

- R2_127_R18 SL_MOB_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM

- RP-240299-Revised-WID-R19-Mobility-Enhancements

- R2-2406419_ZTE Corporation_Inter-CU LTM_discussion