3GPP RAN WG2 Meeting #127bis                                                     R2-2408275

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.2

Source:	HONOR

Title:	Discussions on A-IoT functionality

Document for:	Discussion and Decision

1. Introduction

In RAN2#127, the following aspects were agreed for functionality aspects [1]:

Agreements

- Send an LS to RAN1 regarding TBS size.  Ask SA2 on the max and typical upper layer packet size.

- RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signalling

- RAN2 will study the need and use of a simple message “size” reporting to the reader

- At least the following information are considered useful to be visible to the reader from CN

- The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

- targeted for one or more than one devices;

- approximate number of target devices (if available).

FFS on mandatory/optional

- RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

In this contribution, we will discuss the possible functionality for A-IoT device, including the scheduling issue and segmentation.

2 Discussion

2.1 Scheduling

RAN1 has been agreed that both TDMA and FDMA are candidate transmission mode for D2R transmission and PRDCH should provide scheduling information for PDRCH transmission, which may indicate the transmission resource used by the device. The resource could be time-domain information and/or frequency-domain information which depends on PHY design. Therefore, scheduling with resource is a necessary A-IoT functionality.

Proposal 1: Resource allocation for D2R transmission is a mandatory A-IoT functionality.

In release-19, the study of A-IoT focuses on the DT and DO-DTT traffic types. The procedure includes the following steps: reader triggers the D2R transmission via triggering message and then receives the A-IoT message through the allocated D2R resource. It is quite efficient if the D2R resource could be allocated along with the triggering message, especially for CFRA.

Proposal 2: RAN2 to take triggering message with allocated D2R resource as a baseline.

As shown in the TP, the overall AS procedure can be formulated as below [2]. For some use cases, it will experience the whole procedure with Step A, Step B, Step C1 and Step C2 which may involve multiple scheduling for the random access and/or data transmission. The device ID from upper layer is generally large which consumes more spaces for scheduling. It would be helpful if a shorter AS ID for scheduling purposes introduced.

Figure 1: Overall AS procedure between A-IoT device and reader

Proposal 3: AS ID is supported for scheduling. RAN2 to discuss when and how to assign AS ID for scheduling purposes.

For D2R data transmission, the allocated resource depends on the energy status of A-IoT device. If the energy is about to run out, the reader should stop scheduling the device until the charging completed which could avoid resource wastage.

Proposal 4:  The energy status information should be delivered from the A-IoT device to the reader.

2.2 Segmentation

Generally, the reader will allocate resource based on experience and it is possible that the allocated resource could not carry the whole message waiting to transmit. To utilize the allocated resource, segmentation should be performed to transmit the permitted segment.

Proposal 5: RAN2 to discuss the necessity of segmentation when the allocated resource is not enough to transmit the whole message.

Since no HARQ/ARQ, the reader should discard the received segments when one lost happens and treat the reception of the A-IoT message as failure. Therefore, the less segments, the higher transmission success rate.

Meanwhile, more segments imply multiple transmission which will consume more energy of A-IoT device. The transmission may also fail if the energy is not enough to support all the segments transmission.

Proposal 6: It is proposed to divide the message into at most two segments to avoid unnecessary energy consumption and segment loss.

Considering complexity, RAN2 has agreed to not support the legacy NR BSR/SR. To support accurate resource allocation for the left segment, the device should be able to report the left message size to the reader. It is proposed to include the left message size information with the permitted segment, e.g. indicated in the header of MAC Data PDU.

To unify the design, the message size could also be included for no segmentation case to indicate no message waiting in the buffer.

Proposal 7: The message size should be reported to the reader.

3 Conclusions

In this contribution, we analyze the possible A-IoT device functionalities and made the following proposals:

Proposal 1: Resource allocation for D2R transmission is a mandatory A-IoT functionality.

Proposal 2: RAN2 to take triggering message with allocated D2R resource as a baseline.

Proposal 3: AS ID is supported for scheduling. RAN2 to discuss when and how to assign AS ID for scheduling purposes.

Proposal 4:  The energy status information should be delivered from the A-IoT device to the reader.

Proposal 5: RAN2 to discuss the necessity of segmentation when the allocated resource is not enough to transmit the whole message.

Proposal 6: It is proposed to divide the message into at most two segments to avoid unnecessary energy consumption and segment loss.

Proposal 7: The message size should be reported to the reader.

4. References

[1]	R2-24xxxx Report of 3GPP TSG RAN WG2 meeting #127

[2]	TR 38.769-v1.0.0, Study on solutions for ambient IoT (Internet of Things), 2024-09