3GPP RAN WG2 Meeting #127bis                                                     R2-2408276

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.5

Source:	HONOR

Title:	Consideration on resource allocation for Topology 2

Document for:	Discussion and Decision

1. Introduction

In RAN2#127, the following aspects were agreed for Topology 2 [1]:

Agreements

1	RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

2	A-IoT air interface  in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

3	RAN2 assumes that intermediate UE authorization is performed by upper layers.  RAN2 impact can be studied after further SA/RAN3 progress.

4	RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

5	NW is in control of resources to be used by AIoT air interface.

6	Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

This contribution focuses on the resource allocation issue between gNB and the reader.

2 Discussion

It is agreed that A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, and the NW should control resources to be used by AIoT air interface. According to whether gNB knows how to allocate precise resource, there exists two resource allocation options:

- Option 1: gNB allocates resource for A-IoT air interface when CN or UE reader requests.

- Option 2: The UE reader selects the resource from the configured resource pool for A-IoT air interface.

Proposal 1: RAN2 is kindly asked to consider the following two resource allocation options for A-IoT air interface:

-	Option1: gNB allocates resource for A-IoT air interface when CN or UE reader requests.

-	Option2: The UE reader selects the resource from the configured resource pool for A-IoT air interface.

For option1, the CN or UE reader should provide information to assist resource allocation when there is A-IoT service request. RAN2 has achieved the consensus on the visible information for T1 which includes:

Agreements

1 At least the following information are considered useful to be visible to the reader from CN

- The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

- targeted for one or more than one devices;

- approximate number of target devices (if available).

FFS on mandatory/optional

2 RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

All the visible information could be helpful in calculating resource for the following D2R transmission. For example, the gNB will allocate resource according to service type or device number, e.g. more resource will be allocated for full inventory or large number of devices.

Message size is also one of the important factors to be considered when allocating resource, i.e., large message will consume more resources.

Proposal 2: The following information should be visible to the gNB for resource allocation:

-	The service type of A-IoT (e.g. inventory, command). FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one device;

-	approximate number of target devices (if available).

-       message size over A-IoT air interface

SA2 has identified three types of solutions for Topology 2. It is the UE reader who will get the visible information if upper layer control plane solution or upper layer user plane solution is adopted. The assistance information should be delivered to gNB when the UE request resource for A-IoT air interface. For RRC-based solution, it is expected that the gNB could be visible to the assistance information from CN and can allocate the resource directly, e.g., via RRC signalling.

Proposal 3: The visible information could be delivered to gNB by either UE reader or CN.

For option2, the gNB could configure the resource pool for A-IoT air interface and the UE will select the resource when there are some A-IoT transmission requirements. The solution could be applied in all three SA2 solutions. However, there may need a coordination scheme to avoid resource selection collision and interference, especially when there are multiple UE readers perform selection simultaneously. The scheme may be complicated.

Proposal 4: If UE is permitted to select resource from configured resource pool, whether and how to avoid collision and interference should be considered.

3 Conclusions

In this contribution, we analyzed the possible resource allocation options and made the following proposals:

Proposal 1: RAN2 is kindly asked to consider the following two resource allocation options for A-IoT air interface:

-	Option1: gNB allocates resource for A-IoT air interface when CN or UE reader requests.

-	Option2: The UE reader selects the resource from the configured resource pool for A-IoT air interface.

Proposal 2: The following information should be visible to the gNB for resource allocation:

-	The service type of A-IoT (e.g. inventory, command). FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one device;

-	approximate number of target devices (if available).

-       message size over A-IoT air interface

Proposal 3: The visible information could be delivered to gNB by either UE reader or CN.

Proposal 4: If UE is permitted to select resource from configured resource pool, whether and how to avoid collision and interference should be considered.

4. References

[1]	R2-24xxxx Report of 3GPP TSG RAN WG2 meeting #127