3GPP TSG-RAN WG2 Meeting #127bis	R2-2408280

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.3

Source:	HONOR

Title: 	Discussion on measurement event evaluation and report

Document for:	Discussion and decision

1	Introduction

Based on the discussion during RAN#105 meeting, the mobility enhancement WID in the Release 19[1] related to the measurement enhancements for LTM are updated as below.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

The following agreements related to measurement event evaluation and measurement report for purpose of supporting LTM were made during RAN2#127 meeting.

- MR event evaluation - Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. - MAC layer handles the event evaluation and measurement report triggering. - Measurement report - Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

As stated in the justification part of the WID, the L1 measurement enhancements are related to scenario extension, event triggered measurement reports and CSI-RS measurements. In this contribution, we further discuss the L1 measurement event evaluation and report and provide proposals accordingly.

- Discussion

For Rel-19, the applicable scenario is extended to inter-CU MCG/SCG in addition to intra-CU MCG/SCG in Rel-18. In addition, the discussion about CSI-RS based L1 measurement is re-triggered in Rel-19 while the study of Rel-18 LTM only finished the SSB-based L1 measurement and measurement report. During the previous discussion, RAN2 agreed that the event triggered enhancement is introduced for triggering L1 measurement reporting and LTM cell switch. Considering the extension of scenarios and RS type mentioned above, impacts on the configuration, performing measurements and measurement reports are not evitable. Enhancements related to need to be introduced with the balance of impacts on the spec and availability/effectiveness. Separate discussions on the three sub-objectives may lead to some overlaps. Thus, we prefer a unified solution for L1 measurement enhancement in Rel-19 which covers event triggered L1 measurement reporting and evaluation conditions for UE triggering LTM, SSB or CSI-RS based L1 measurement and both intra-CU and inter-CU scenario. That is, the UE could evaluate conditions for triggering LTM and the L1 measurement report by SSB-based L1 measurement or CSI-RS based L1 measurement in both inter-CU and intra-CU MCG/SCG scenario.

Proposal 1: RAN2 strives to have a unified event design in Rel-19 for inter/intra-CU MCG/SCG LTM with SSB/CSI-RS based L1 measurement on candidate cells for L1 measurement report and evaluation conditions for UE triggering LTM.

2.1 Event evaluation

During the previous meeting, multiple events for L1 measurement like events LTM2~4 were agreed, since reports triggered by different events could provide different kind of information. Note that the LTM is to trigger the switch to the specific beam, different event could be configured to the beam considering the characteristic of the beam. Thus, the L1 measurement event is configured per beam/RS, but not per cell. In addition, to achieve different effects via different events, multiple events may be configured to one beam. Furthermore, in CHO, two conditions associated with one event could be configure to UE and the event is considered as fulfilled only when both conditions are met, which improve the robustness for the handover. However, take the latency caused by the event evaluation into account, it still needs some discussions about whether one event could be associated with two conditions could be introduced into L1 measurement event evaluation.

Proposal 2: L1 measurement event is configured per beam/RS, but not per cell.

Proposal 3: Multiple L1 measurement events may be configured for one beam. FFS whether event with two conditions is introduced for conditional LTM.

2.2 Measurement report

In Rel-18 mobility enhancement WI, L1/L2 triggered mobility measurement (LTM) is introduced as a cell switch procedure while the gNB may trigger the procedure after receiving the L1 measurement report from UE. Enhancements on L1 measurement only realized the basic function with the support of the SSB-based L1 measurement for neighbour cells in the intra CU scenario. For the Rel-18 LTM, the legacy CSI report mechanism is reused as much as possible. For example, compared to the fixed number in the legacy report for the serving cell (and potential the neighbour cell as the additional PCI), the number of SSBRI reported in one CSI report is modified to L*M where L is the number of reported cells and M is the number of reported SSBRI/RSRP pair per cell. All SSBs from all candidate cells are numbered so that the report only needs to include the SSBRI. And the selection of the reported beam is up to UE implement.

Observation 1: In Rel-18, the CSI report only includes RS index and the corresponding RSRP for the SSB-based L1 measurement in intra-CU scenario.

For L1 measurement, RAN2 already confirmed the support for the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. The content of the L1 measurement should be discussion when both SSB and CSI-RS are configured in L1 measurement resource configuration.

For event LTM2 and event LTM4, the L1 measurement report includes the result(s) for the serving cell or candidate cell. Considering the limited size of the L1/L2 signalling, measurement results based on different RS type in one L1 measurement report may introduce more complexity for the design of the signalling format. For event LTM3 and event LTM5, the L1 measurement report includes the result(s) for one candidate cell and serving cell. The comparison needs to be done based on the same RS type. Thus, we propose that

Proposal 4:  Measurement results of multiple beams based on same RS type could be included in one L1 measurement report.

For Rel-19, RAN2 confirmed that all RSs from all candidate cells are numbered so that the report only needs to include the index of the RS. For the reporting content, index of the reported RS (e.g., SSBRI or CRI) associated with its L1 measurement result per beam needs to be included in the L1 measurement report. Besides, other associated information could also be needed to provide more information for the LTM-related determination. For example, the information of the triggered event could indicate the relation between the quality of the beam satisfying the L1 measurement event and the threshold/quality of the serving beam. For this meeting, we propose to confirm the potential contents for the event triggered L1 measurement reporting as follows:

Proposal 5: RAN2 to confirm the at least following contents for the event triggered L1 measurement report: beam information (e.g., RS index), measurement results per beam, event index.

RAN1 had listed the signalling content of the UE-initiated/event-driven beam reporting with variable or fixed number of beam(s) satisfying the event or at least one of the beams satisfying the event:

Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, the following options are provided for down-selection (other options are not precluded) in RAN1#117 - Option-1 (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax} - The N beam(s) should satisfy the condition of Event-2 - Nmax is configured by gNB - FFS: Whether the indication of payload size should be provided additionally. - Option-1a (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax} - At least one of N reported beam(s) should satisfy the condition of Event-2 - Nmax is configured by gNB - FFS: Whether the indication of payload size should be provided additionally. - FFS: Details on how value of N is determined by the UE - Option-1b: N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax} - The N beam(s) should satisfy the condition of Event-2 - Nmax is configured by gNB - Payload size does not vary as a function of N - FFS: Zero-padding can be provided if N is less than Nmax. - Option-2: Only N=1 beam is reported in the report instance - The reported beam should satisfy the condition of Event-2 - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - Other options are not precluded. - FFS: Whether the measurement results for current beam is always reported or can be enabled by RRC. - FFS: When current beam is reported, whether the current beam is counted in the N reported beams. - The selected option shall satisfy Event-2.

Similarly, it is reasonable to have these beams included in the L1 measurement report. Firstly, beam(s) of neighbour satisfying the event is the basic option since it is useful to help the NW determine the target beam. Secondly, the quality of the serving beam is needed when determining cell switch since the determination should be made based on the comparison between the serving and the neighbour. Considering the variety of L1 measurement, the result of the serving beam could be carried in the same report instance with the result of beam(s) of neighbour satisfying the event. Last but not least, beam of neighbour with at least one beam satisfying the event is another option for the signalling content. As the current L3 measurement and report mechanism, the UE measures several beams to derive the cell quality to ensure that UE could receive good service under the cell with more good beams. Even the LTM is based on beam measurement, it provides more robust if the target has multiple good beams within the cell or CU.

Proposal 6: RAN2 to considering the following beams to be included in the report: the beam(s) of neighbour satisfying the event, serving beam, beams of neighbour with at least one beam satisfying the event.

3	Conclusions

In this contribution, we discussed the L1 measurement enhancement for Rel-19 mobility WI. Based on the discussion, the following observation and proposals are concluded:

Proposal 1: RAN2 strives to have a unified event design in Rel-19 for inter/intra-CU MCG/SCG LTM with SSB/CSI-RS based L1 measurement on candidate cells for L1 measurement report and evaluation conditions for UE triggering LTM.

Proposal 2: L1 measurement event is configured per beam/RS, but not per cell.

Proposal 3: Multiple L1 measurement events may be configured for one beam. FFS whether event with two conditions is introduced for conditional LTM.

Proposal 4:  Measurement results of multiple beams based on same RS type could be included in one L1 measurement report.

Proposal 5: RAN2 to confirm the at least following contents for the event triggered L1 measurement report: beam information (e.g., RS index), measurement results per beam, event index.

Proposal 6: RAN2 to considering the following beams to be included in the report: the beam(s) of neighbour satisfying the event, serving beam, beams of neighbour with at least one beam satisfying the event.

4	References

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Rel-19,