3GPP RAN WG2 Meeting #127bis	R2-2408282

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.9.2

Source:	HONOR

Title:	Discussion on the Store and Forward satellite operation

Document for:	Discussion and Decision

1. Introduction

In Rel-19 Non-Terrestrial Networks (NTN) for Internet of Things (IoT) WID, the following objectives were achieved:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN2#127 meeting, the following agreements were made:

1. RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options) 2. RAN2 will consider both single satellite pass and multiple satellite pass scenarios 3. RAN2 will consider both MO and MT data within scope 4. UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature) 5. RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected. 7. RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In this contribution, we discuss how to support CP solution and UP solution when the satellite operates in Store &Forward (S&F) mode.

2. Discussion

According to the WID, the S&F mode is applied in regenerative architecture. For S&F mode, in the previous discussion in RAN2, it is a principle that at least full eNB onboard is required.

Regarding the architecture of store & forward, SA2 have made a conclusion that two kinds of architecture (split MME onboard, full CN onboard) shall be applied for S&F operation. In TR 23.700-29, the architectures are defined as follows:

MME functionality is split into two parts: MME-onboard - the MME part which is onboard the satellite and MME-ground - the MME part which is on the ground network with an interface out of scope of 3GPP. The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc.

For full CN onboard, we intuitively think it has small impact on RAN2 specification. Therefore, in this contribution, we will focus on the architecture with split MME onboard.

2.1 S&F indication

In the previous meeting, it was agreed that UE is informed whether its serving satellite is currently operating in S&F via system information broadcast. For a certain satellite, the two operation modes shall work at different time duration and switch between S&F mode and normal mode. Therefore, we think S&F mode indication is not just a capability indication. It is also responsible for directing changes in operation mode.

If a simple S&F indication is introduced, the operation mode can be switched based on SIB modification. It will result in extra SIB re-acquisition, which is very serious especially for NB_IoT UE with extra power consumption. In addition, this type of indication can only indicate the current state. For the legacy UE and Rel-19 UE without S&F capability work in normal mode currently, since the UE cannot predict the status of the next time point, the UE might initial RACH at the time that the feeder link is about to become unavailable. If so, such behavior is meaningless and results in extra signalling overhead and power consumption.

Considering the mentioned issues, some assistance information such as feeder link available/unavailable time info is needed for indicating mode switch.

In legacy, the field t-service is defined to indicate the time information on when a cell provided via NTN system is going to stop serving the area it is currently covering. It applies for both service link switches in NTN quasi-Earth fixed system and feeder link switches for both NTN quasi-Earth fixed and Earth moving system. From the perspective of its application, the field t-service is not appropriate for S&F operation, therefore, a new field should be defined, for instance, the time info of available service link should be defined.

Proposal 1: Assistance information (time info of service link, time info of feeder link) needs to be provided.

2.2 Access control

When the cell works on S&F operation, the legacy cellBarred and/or cellBarred-NTN bit shall be used to bar legacy UEs. A new cell barred bit (e.g., cellBarred-S&F) needs to be introduced to indicate if the cell is barred for S&F operation. The cellBarred-S&F field is ignored if the UE does not support S&F. For the UE supporting S&F, the UE ignores the legacy cellBarred and/or cellBarred-NTN bit and applies the new barring bit.

Proposal 2: Introduce a new barring bit for access control of R19 S&F capable UEs.

Proposal 3: Rel-19 S&F capable UEs shall ignore the legacy barring bit (cellBarred bit, cellBarred-NTN bit) and apply the new barring bit.

2.3 Paging

For CN-initiated paging, the MME-ground sends paging message to the satellites that are going to visit the UE’s tracking areas, when the feeder link becomes available. But for UE, before starting to monitor paging, the UE needs to decide whether there is a need to monitor paging. Since the last time the UE monitored paging, if the satellite has never connected to MME-ground, the UE can skip paging monitoring. Therefore, the information about the last time the feeder link was available should be provided for UE to decide whether to monitor paging.

Proposal 4: Provide the information about the last time the feeder link was available for UE to decide whether to monitor paging.

For single satellite pass scenario, it is easy for UE to decide whether to monitor paging based on the provided information as described in proposal 4. But for multiple satellites pass scenario, it becomes complicated. The MME-ground shall usually send paging to more than one satellite which will visit the UE’s tracking areas one by one in the near future. For the same one CN-initiated paging, since it is possible that multiple satellites contain the same tracing area where the UE located, it is necessary to avoid monitoring paging once again after monitoring paging from one satellite. Therefore, the issue how to monitor paging for multiple satellites pass scenario should be considered.

Proposal 5: The issue how to monitor paging for multiple satellites pass scenario should be considered.

2.4 Support CP solution

2.4.1 RRC Connection Setup

For the procedure of RRC connection setup in CP solution, when the eNB receives RRC connection setup request message, the eNB will request MME for acquiring UE radio capability. Based on UE radio capability, the eNB provides the configuration in differentiation via MSG4.

In TR 23700-29, for the architecture of split MME onboard, the following additional principles are applied:

MME-ground is an anchor node situated in ground network which has the UE context. MME-ground synchronizes the UE context with all the MME-onboard(s). The eNB and MME-onboard are onboard the satellite. Thus, they can provide basic connection management procedures. The MME-onboard executes all the procedures with the UE which does not need interaction with other core network nodes on the ground when the service link is available.

Since the UE radio capability information is part of the UE context. the MME-onboard will store UE radio capability information after synchronizing with MME-ground. It is slightly realized that the eNB might be able to acquire the UE radio capability information upon receiving RRC connection setup request message. However, we also find a problem that UE radio capability information stored in MME-onboard cannot be updated in time when the MME-ground has acquired the latest UE radio capability information, because the behavior of synchronizing the UE radio capability information is characterized by hysteresis due to unavailable feeder link. That is, if the UE initials an RRC connection setup towards a new satellite, which is different from the satellite where the UE performs UE radio capability information report, the eNB cannot fetch the latest UE radio capability information from the associated MME-onboard.

However, we believe that the mentioned problem may not be serious based on the following consideration. On the one hand, the UE radio capability information, in addition to the information related to resource allocation, it also contains other information (e.g., paging related information), so that it is reasonable to assumed that only a very limited part of the UE radio capability information may have an impact on eNB resource allocation. This part information is usually associated with UE itself, therefore, it does not change frequently. On the other hand, due to sparse IoT business, all MME-onboard might complete to update UE radio capability information before the UE initials the next RRC connection setup. Therefore, we think it is feasible to support RRC connection setup directly without any adaptive modification.

Proposal 6: For S&F satellite operation, RRC connection setup can be supported without any adaptive modification.

2.4.2 RRC Connection Reestablishment

In terrestrial network, RRC reestablishment is supported for CP solution. When the eNB receives the RRC connection reestablishment request message, it triggers the eNB CP relocation indication procedure including NAS-level security information received from the UE. Then, the MME performs authentication based on NAS-level security information.

If the UE initials RRC connection reestablishment towards the old eNB/satellite, where the RRC connection failed, e.g. due to RLF, this approach will naturally be supported with the architecture of split MME onboard.

If the UE initials RRC connection reestablishment towards the new eNB/satellite, which is different from the old eNB/satellite, the new eNB needs to forward RLF indication message towards the old eNB. After successful completion of the verification operation, the old eNB responds with the UE CP context indication message containing UE context information. Since ISL is unavailable, signalling exchange between the new eNB and the old eNB cannot be achieved directly. If signalling exchange is performed over MME-ground, multiple rounds of signalling exchange are needed and two rounds RRC connection reestablishment is needed, which will bring higher complexity from the UE point of view. Therefore, for the scenario of CP RRC connection reestablishment towards the new eNB, it is suggested that this scenario is not supported.

Proposal 7:  The CP RRC connection reestablishment is restricted to the old eNB/satellite.

2.5. Support UP Solution

2.5.1 RRC Connection Suspend

When the feeder link becomes unavailable, the eNB initials RRC connection suspend procedure, which is illustrated in Figure 1. When the eNB decides to suspend the RRC connection, it initiates the procedure and sends the UE context suspend request message to the MME. Since the split MME is deployed onboard, there is no barrier to signalling interaction between eNB and MME-onboard. The eNB will suspend UE context and send RRC connection release message to UE. In this message, some RRC resume related information such as release cause, resume identity and optional NCC will be carried together, which can be used for subsequent RRC resume.

Figure 1: RRC connection suspend procedure.

Proposal 8: For S&F satellite operation, RRC connection suspend can be supported without any adaptive modification.

In the scenarios of S&F mode, RRC release procedure will be initiated when the service link becomes unavailable, or the feeder link becomes unavailable. In the case, when the service link becomes unavailable, the UE performs RRC release, or the UE keeps in RRC connected mode with AS operation suspend, which is left to eNB implementation on the decision of the UE behavior.

In the previous discussion, some companies put forward an issue whether to introduce a new release cause for RRC release due to that feeder link becomes unavailable. Whether the existing release cause and the corresponding UE behavior suit the case of RRC release due to unavailable feeder link or not, should be assessed as an influencing factor. If the release cause indicates ‘rrc-Suspend’ or ‘other’, we find that the UE behavior upon leaving RRC connection described in the current spec is also accommodate to the case that RRC release due to unavailable feeder link. Therefore, we think it is not necessary to introduce a new release cause.

Proposal 9: It is not necessary to introduce a new release cause ‘feeder link becomes unavailable’.

2.5.2 RRC Connection Resume

Due to the unavailable ISL, the signaling interaction between the two eNBs for transferring UE context is not feasible. Whether the two options as follows can be supported should be taken into account.

Option 1: RRC connection resume is restricted to the eNB where the UE context is suspended

In this approach, the UE is restricted to initial RRC Connection Resume towards the eNB where its UE context was suspended. In the existing RRC connection resume procedure, upon receiving RRC connection resume request message that including resume ID, release cause, and authentication token, the eNB responds with an RRC Connection Resume message after successful completion of verification with resume ID and authentication token. The procedure of RRC connection resume will be performed directly without any requirements of signalling interaction between the two eNBs. It has small impact on the current spec. However, high latency is unavoidable since the UE has to wait for the eNB revisit its location and the revisit time may be around 12 hours. We also realize that it needs at least two rounds of connection establishment for one time data transmission in S&F mode, therefore, the latency will become more severe. In addition, if the UE can initial RRC connection resume towards any eNB without any restriction, it helps to balance the load between eNBs, and this will reduce the resume reject probability due to heavy load in one certain eNB. Therefore, if there are no technical problems, RRC connection resume should not be restricted to the eNB where the UE context is suspended.

Option 2: Both the eNB where the UE context is suspended and the other new eNBs can be supported for RRC connection resume

If it is allowed to perform RRC Connection Resume towards new eNB, the new eNB needs to fetch UE context from the old eNB. However, since the ISL is not available, the old eNB shall forward UE context to the other new eNBs via MME-ground after completing RRC suspend with the UE. Though such mechanism will result in some impacts on the legacy spec, it is feasible and acceptable.

Proposal 10: Both the eNB where the UE context is suspended and the other new eNBs can be supported for RRC connection resume.

In legacy, when the UE initials RRC connection resume towards a new eNB, the new eNB fetches UE context form the old eNB after the old eNB successfully verifies the UE by means of the Resume ID and authentication token. If proposal 6 is agreed, the existing procedure needs to be modified. For instance, the new eNB needs to fetch UE context from the old eNB in advance via MME-ground. Other than UE context, at least the resume ID associated with UE context and authentication token need to be provided for the new eNB.

Proposal 11: The new eNB fetches UE context, Resume ID associated with UE context, and authentication token in advance.

If the UE initials RRC connection resume towards a new eNB, which fetches and stores RRC connection resume related information in advance, the new eNB instead of the old eNB performs verification directly.

Proposal 12: The new eNB performs verification based on the stored the RRC connection resume related information.

3. Conclusions

In this contribution, we discussed the issues related to Store & Forward (S&F) satellite operation with split MME onboard. Based on the discussion, the following proposals are concluded:

Proposal 1: Assistance information (time info of service link, time info of feeder link) needs to be provided.

Proposal 2: Introduce a new barring bit for access control of R19 S&F capable UEs.

Proposal 3: Rel-19 S&F capable UEs shall ignore the legacy barring bit (cellBarred bit, cellBarred-NTN bit) and apply the new barring bit.

Proposal 4: Provide the information about the last time the feeder link was available for UE to decide whether to monitor paging.

Proposal 5: The issue how to monitor paging for multiple satellites pass scenario should be considered.

Proposal 6: For S&F satellite operation, RRC connection setup can be supported without any adaptive modification.

Proposal 7: The CP RRC connection reestablishment is restricted to the old eNB/satellite.

Proposal 8: For S&F satellite operation, RRC connection suspend can be supported without any adaptive modification.

Proposal 9: It is not necessary to introduce a new release cause ‘feeder link becomes unavailable’.

Proposal 10: Both the eNB where the UE context is suspended and the other new eNBs can be supported for RRC connection resume.

Proposal 11: The new eNB fetches UE context, Resume ID associated with UE context, and authentication token in advance.

Proposal 12: The new eNB performs verification based on the stored the RRC connection resume related information.

4. References

[1]	RP-240776, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2]	TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)

[3]	RP-221547, Summary for NB-IoT/eMTC support for Non-Terrestrial Networks (NTN)

[4]	TS36.300, Overall description