3GPP RAN WG2 Meeting #127bis	R2-2408283

# Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.8.5

Source:	HONOR

Title:	Discussion on regenerative payload

Document for:	Discussion and Decision

1. Introduction

In Rel-19 Non-Terrestrial Networks (NTN) for NR WID, the following objectives were achieved:

- Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3, RAN2] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In RAN2#127 meeting, the following agreements were made:

- RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario. - Send an LS to RAN1 and RAN4 asking whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for ta-Common or whether we should e.g. introduce negative values for ta-Common. Also indicate that in any case legacy UEs would have to rely on existing signalling and then on a minimum value equal to 0. - RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed. - We do not optimize UE power saving for SDT procedure in regenerative payload architecture

In this contribution, we will identify the issues involved in satellite switch with re-sync in regenerative architecture and discuss how to support satellite switch with re-sync.

2. Discussion

In transparent NTN architecture, the mechanism of satellite switch with re-sync has been specified as follows:

Upon both hard and soft satellite switch over in the quasi-Earth fixed scenario with the same SSB frequency and the same gNB, the satellite switch with re-sync procedure is supported. The satellite switch with re-sync avoids a L3 mobility for UEs in the cell by maintaining the same PCI on the geographical area covered by quasi-Earth fixed beam. CHO can be configured simultaneously with the satellite switch with re-sync procedure.

For soft satellite switch over, the UE can start synchronizing with the target satellite before the source satellite ends to serve the cell. It is not required for the UE to be connected to source satellite when the UE switches to target satellite.

For hard satellite switch over, the UE can only start synchronizing with the target satellite after the switch to the target satellite is initiated.

In legacy satellite switch with re-sync, since the gNB is deployed on the ground, the serving gNB remains unchanged. But, for regenerative architecture with gNB onboard, it is much different from that in transparent architecture. The satellite switch is accompanied by a change in the serving gNB.

If satellite switch with re-sync is performed in regenerative architecture, from the perspective of UE, the UE needs to perform synchronous towards the target satellite, which is similar as that in transparent architecture. From the perspective of gNB, it is necessary to perform signaling interaction between the source satellite and the target satellite via ISL, which seems feasible. In addition, since the satellite switch with re-sync can avoid a L3 mobility, it is beneficial to reduce signaling resulted from L3 HO. Therefore, we think that satellite switch with re-sync mechanism should also be supported in regenerative architecture.

Proposal 1: It is suggested to support satellite switch with re-sync mechanism in regenerative architecture.

2.1 PCI unchanged

In transparent architecture, upon satellite switch with re-sync, PCI remains unchanged. In the previous discussion on satellite switch with re-sync in regeneration architecture, an issue whether PCI can remain unchanged was raised. In transparent architecture, for hard satellite switch, since the target satellite starts to serve the cell after source satellite stopping serving the cell, there will be no confusion with the same PCI. This also applies to regeneration architecture. For soft satellite switch, SSB time offset can be configured to resolve the issue of PCI confusion. But for regenerative architecture, SSB time offset can be negotiated between source satellite and target satellite. From this point of view, it is feasible to perform satellite switch with PCI unchanged in regenerative architecture.

In addition, satellite switch with PCI changed will result in security key update, PDCP reestablishment, which is similar to operations in handover. PCI changed solution is not align with the intention of satellite switch with re-sync. However, if PCI remains unchanged, whether there is a security issue needs to be assessed by SA3. From RAN2 point of view, since the target gNB is a replica of source gNB, there are no serious security concerns. Therefore, we think it is feasible to remain PCI unchanged based on the analysis of the mentioned issues.

Proposal 2: PCI unchanged can be supported for satellite switch with re-sync mechanism in regenerative architecture.

2.2 Satellite Switch

During traditional HO, the source gNB decides to handover the UE, based on measurement report and RRM information. If it occurs in NTN network, besides measurement report and RRM information, some timing information, e.g., t-Service, t-ServiceStart, which is considered as satellite assistance information for NTN access contained in SIB19, can be used to assist HO decision. The gNB is naturally to be responsible to trigger re-sync. We also think that the UE could also trigger re-sync, based on satellite assistance information, measurement information and NW configuration information (e.g., satellite switch condition). If the execution is triggered by UE, the UE will perform re-sync with the target satellite without any signaling indication from the source satellite.

Proposal 3: Both NW and UE could trigger the satellite switch with re-sync.

2.3. Signalling Interaction

In the preparation phase of traditional handover, some necessary information, e.g., target cell ID, security related information, RRM configuration, etc., should be provided to the target gNB for preparing the handover via signalling interaction between two gNBs. In transparent architecture, in the procedure of satellite switch with re-sync, since the associated gNB remains unchanged, there is no signaling exchange between the two satellites. But for regeneration architecture, though satellite switch with re-sync is not a handover, signaling exchange between the two satellites is inevitable due to changed serving gNB.

In order to ensure that the UE can successfully access to the target satellite, the source satellite shall forward the re-sync related information (e.g., UE context, gNB context, RRC configuration, etc.) to the target satellite via ISL. The signalling in detail, is in the scope of RAN3. In addition, in the procedure of signalling exchange, whether there is a security issue needs to be assessed by SA3.

Proposal 4: The source satellite forwards the satellite switch with re-sync related information (e.g., UE context, gNB context, RRC configuration, etc.) to the target satellite via ISL.

2.4. Satellite Switch Completion

In transparent architecture, after performing synchronization with the new satellite, since the serving gNB remains unchanged, the signaling/data transmission can be performed immediately between UE and the new satellite. However, for regenerative architecture, the target satellite associated gNB needs to know whether satellite switch is completed based on the reception of a UL signaling, which is very similar to RACH-less HO.

In RACH-less HO, the target cell can allocate CG-based resource for UE in advance. Upon handover, the UE transmits the RRCReconfigurationComplete message using the configured CG-based resource. Alternatively, DG-based resource also can be applied for accessing to target cell, and dynamic scheduling resource shall be acquired by monitoring PDCCH. Correspondingly, both CG-based and DG-based resource allocation can be applied for satellite switch with re-sync.

Proposal 5: Both CG-based and DG-based resource allocation can be applied for satellite switch with re-sync.

3. Conclusions

In this contribution, we discussed the issues related to satellite switch with re-sync in regeneration architecture. Based on the discussion, the following proposals are concluded:

Proposal 1: It is suggested to support satellite switch with re-sync mechanism in regenerative architecture.

Proposal 2: PCI unchanged can be supported for satellite switch with re-sync mechanism in regenerative architecture.

Proposal 3: Both NW and UE could trigger the satellite switch with re-sync.

Proposal 4: The source satellite forwards the satellite switch with re-sync related information (e.g., UE context, gNB context, RRC configuration, etc.) to the target satellite via ISL.

Proposal 5: Both CG-based and DG-based resource allocation can be applied for satellite switch with re-sync.

4. References

[1]	RP-241036, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

[2]	TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)

[3]	TS 38.300, NR and NG-RAN Overall Description