3GPP RAN WG2 Meeting #127bis	R2-2408286

# Hefei, China, Oct 14th–18th, 2024

Agenda Item:	8.7.4.1

Source:	HONOR

Title:	Discussion on LCP enhancements

Document for:	Discussion and Decision

1. Introduction

In RAN2#127, the following agreements were agreed [1]:

RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

In this contribution, we focus on the LCP enhancements with delay information.

2 Discussion

In the last meeting, it was agreed to use additional priority configured to LCHs in case of these LCHs with delay-critical data. When the LCH becomes delay-critical, the additional priority should be higher than the legacy one to improve scheduling priority.

Proposal 1: An additional higher priority is configured to LCHs in case of these LCHs with delay-critical data.

There may be delay-critical data and non-delay critical data in one LCH at the same time. If the additional priority is configured to the LCH, there are two options to use it:

Option 1: the priority only applies to delay-critical data within the LCH.

Option 2: the priority applies to the whole LCH.

With the first option, UE needs to perform two rounds of LCH prioritization for selected UL grant, i.e. it first allocates resources for delay critical data from all logical channels based on additional priority, then repeats the same procedure with non-delay critical data based on legacy priority. Thus, the option 2 can be adopted.

Proposal 2: The additional priority applies to the whole LCH in case of the LCH with delay-critical data.

There are the following steps to allocate resources for LCH:

The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> step A: logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> step B: decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> step C: if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

The priority of LCH should be determined before resources allocation, so the step A and step C can use the same priority for one LCH, which can avoid the impact of the subsequent intra-UE prioritization.

Proposal 3: The additional priority of LCH is determined before resources allocation and used in whole resources allocation steps.

The current Bj maintains the allowed transmission size of the LCH j. For the step A, if current LCH has reached its Bj, the UE allocates resources to the next LCH though current LCH still has remaining delay-critical data. If there are no remaining resources in the step C, the remaining delay-critical data may not have chance to transmit. Therefore, RAN2 can consider the Bj enhancements of the LCH with delay-critical data.

Proposal 4: RAN2 considers the Bj enhancements of LCH with delay-critical data.

3 Conclusions

Proposal 1: An additional higher priority is configured to LCHs in case of these LCHs with delay-critical data.

Proposal 2: The additional priority applies to the whole LCH in case of the LCH with delay-critical data.

Proposal 3: The additional priority of LCH is determined before resources allocation and used in whole resources allocation steps.

Proposal 4: RAN2 considers the Bj enhancements of LCH with delay-critical data.

4. References

[1]	RAN2 #127 Chairman notes