3GPP RAN WG2 Meeting #127bis	R2-2408287

# Hefei, China, Oct 14th–18th, 2024

Agenda Item:	8.7.5

Source:	HONOR

Title:	Discussion on timely RLC enhancements

Document for:	Discussion and Decision

1. Introduction

In RAN2#126, the following agreements were agreed for timely RLC retransmissions [1]:

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic

In this contribution, we discuss the potential solutions related to RLC enhancements for ensuring timely RLC retransmissions.

2. Discussion

The existing mechanisms for RLC retransmission contain polling and status reporting. Reuse current mechanisms are to introduce smaller values of related timer (e.g., t-Reassembly, t-StatusProhibit, and t-PollRetransmit) and related threshold (e.g., pollbyte, pollPDU) for polling and status reporting. For the polling, introducing the smaller value can only trigger poll frequently. If the polling SN cannot satisfy the condition for triggering status report, the RLC Tx side should also wait for the status report. For the status reporting, the status report will be transmitted frequently with small value of t-Reassembly, then the RLF will be declared early. Therefore, reusing existing mechanisms cannot work well for timely RLC retransmission.

In the last meeting, there were provided three solutions to study and one is autonomous retransmission without status report.

There are two methods for autonomous retransmission without status report:

- Method 1: A timer can be introduced to control the RLC retransmission, e.g. the retransmission of the data can be triggered when the timer expires.

- Method 2: Current indicator from PDCP to RLC to notify delay-critical RLC SDU is used to trigger the corresponding RLC SDU retransmission.

For Method 1, a timer is introduced in RLC for retransmitting SDU. RLC entity starts the timer when receiving the RLC SDU. When the timer expires and the SDU has been sent, RLC retransmits the corresponding SDU. This timer is shorter than PDCP discard timer for avoiding retransmitting discarded SDU.

Fort Method 2, if a PDCP SDU has low remaining time, and if the corresponding PDCP Data PDU has already been submitted to lower layers, the delay-critical indication for the PDCP Data PDU is provided to RLC. This indication means the corresponding RLC SDU is about to time out, RLC can trigger the corresponding SDU retransmission when receiving the indication.

For the above two methods, both can be used to trigger timely RLC retransmission but method 2 reuses the current mechanism. So, RAN2 can apply method 2.

Proposal 1: Current indication of delay-critical RLC SDU from PDCP to RLC is reused to trigger the corresponding RLC SDU autonomous retransmission.

After the delay-critical RLC SDUs are retransmitted, the poll for these SDUs can be transmitted for confirming whether these SDUs are successfully received. Therefore, the polling can be enhanced considering delay-critical RLC SDUs.

Proposal 2: RAN2 considers enhancing polling based on delay-critical data.

3. Conclusions

Proposal 1: Current indication of delay-critical RLC SDU from PDCP to RLC is reused to trigger the corresponding RLC SDU autonomous retransmission.

Proposal 2: RAN2 considers enhancing polling based on delay-critical data.

4. References

[1]	RAN2 #126 Chairman Notes