3GPP TSG-RAN WG2 Meeting #127bis					R2-2408295

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.5.2

Source:	Samsung

Title:	On-demand SSB SCell Operation

Document for:	Discussion and Decision

# Introduction

The Rel-18 work item on network energy savings for NR specified techniques primarily for RRC Connected UEs such as SSB-less SCell operation, cell DTX/DRX mechanism, mechanisms to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, CHO procedure enhancement(s) etc.

Rel-19 work item aims to specify further network energy savings techniques such as on-demand SSB for SCell operation. WID of Rel-19 network energy saving specifies the following objective for on-demand SSB SCell operation:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

This contribution discusses the details on supporting on-demand SSB on SCell.

# Discussion

In the legacy NR operation, a UE can be configured with SSB based RRM measurement to discover SCell(s). UE reports the RRM measurement results to the gNB on the quality of the measured SSB. The gNB can further provide the SCell configuration by RRC to the UE. At the time of configuration, SCell can be initially activated or deactivated. If the SCell is deactivated, the gNB can activate it by SCell activation command by MAC CE. UE uses SCell’s SSB (or TRS if configured) for AGC and synchronization purpose to be ready for transmitting or receiving data on the activated SCell. After the SCell is activated, the UE can still use the SSB to get re-synchronized with the SCell, by its implementation.

An example of the legacy NR operation of a SCell is shown in Figure 1, wherein the SSB is transmitted by the SCell and is periodic. In case of SSB-less SCell, operation is same except that SSB measurements of a reference serving cell (SpCell or another SCell) are used for the SSB-less SCell.

Figure 1 Illustration of legacy NR procedure for the SCell transmitting SSB

The periodic transmission of SSB for the SCell operation affects the network energy consumption. For network energy savings following are agreed by RAN1 for SSB transmission on SCell:

  - Case #1: No always-on SSB on the cell. Upon on demand SSB trigger, SSBs are periodically transmitted.

  - Case #2: SSBs are periodically transmitted in SSB burst/window. On demand SSB can be additionally provided during the interval between SSB burst/window as and when needed. Note that periodic transmission can be at a longer periodicity to reduce energy consumption but affects performance ((re)sync/AGC delay, etc). Additional on demand SSB can improve performance.

Scenarios for on demand SSB

RAN1 has agreed that on demand SSB can be triggered for the following scenarios:

- Scenario 2: SCell is configured to a UE but before the UE receives SCell activation command

- Scenario 2A: When UE receives SCell activation command

Following scenarios for triggering on demand SSB are FFSs in RAN1

- Scenario 3A: After UE receives SCell activation command until SCell activation is completed

- Scenario 3B: When or after SCell activation is completed and SCell is activated

RAN1 has also agreed that for a cell supporting on-demand SSB SCell operation, L1 and/or L3 measurement based on on-demand SSB is supported for the cell.

RAN2 has agreed in RAN2#127 that

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated

For case #2, there can be two options for RRM measurements of SCell configured with on demand SSB. Its not clear which option is followed by UE.

- Option a) UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB.

- Option b) UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

SSBs transmitted during periodic transmission and on demand SSB transmission may be same or different. Considering that periodic transmission can be at a longer periodicity to reduce energy consumption but affects performance ((re)sync/AGC delay, etc), additional on demand SSB can improve performance. So it may be beneficial to follow option b).

Proposal 1: For case #2, RAN2 to discuss which option is followed by UE for RRM measurements of SCell configured with on demand SSB.

- Option a) UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB;

- Option b) UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

Currently, for L3 measurement (i.e. RRM), SCell can be configured with a measurement object for always on periodic SSB. It is indicated by field servingCellMO (of type MeasObjectId).

For supporting L3 measurement (i.e. RRM) based on on-demand SSB, SCell can be configured with a measurement object for on demand SSB. It can be indicated by new field onDemandSSBservingCellMO (of type MeasObjectId).

UE performs L3 measurement (i.e. RRM) based on onDemandSSBservingCellMO only when on-demand SSB is activated/transmitted.

Proposal 2: For supporting L3 measurement (i.e. RRM) based on on-demand SSB, SCell can be configured with a measurement object for on demand SSB. It can be indicated by new field onDemandSSBservingCellMO (of type MeasObjectId).

Signalling for on demand SSB:

RAN2 agreed the following in RAN2#127.

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

In our view, further optimisation (such as using new MAC CE for both OD-SSB transmission indication and SCell activation/deactivation) for scenario 2A are not needed. If network wants to activate OD-SSB transmission and SCell activation, it can do so by sending legacy MAC CE and new MAC CE in same TB.

Proposal 3: Further optimisation (such as using new MAC CE for both OD-SSB transmission indication and SCell activation/deactivation) for scenario 2A is not needed.

# Conclusion

Proposal 1: For case #2, RAN2 to discuss which option is followed by UE for RRM measurements of SCell configured with on demand SSB.

- Option a) UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB;

- Option b) UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

Proposal 2: For supporting L3 measurement (i.e. RRM) based on on-demand SSB, SCell can be configured with a measurement object for on demand SSB. It can be indicated by new field onDemandSSBservingCellMO (of type MeasObjectId).

Proposal 3: Further optimisation (such as using new MAC CE for both OD-SSB transmission indication and SCell activation/deactivation) for scenario 2A is not needed.

# References