3GPP TSG-RAN WG2 Meeting #127bis					R2-2408296

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.5.4

Source:	Samsung

Title:	Adaptation of common signal transmissions

Document for:	Discussion and Decision

# Introduction

RAN2 discussed the adaptation of paging occasions in RAN2#126 and made the following agreements:

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study

  - bundle paging frames and

  - extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

  - Option 1: Prevent the access of legacy UE via barring;

  - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

# Discussion

2.1 Adaptation of SSB in time domain

RAN1 has discussed adaptation of SSB in time domain and agreed the following:

- For adaptation of SSB in time-domain, Option 1 is supported

- Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s)

When pagingSearchSpace is zero and SS/PBCH block and CORESET multiplexing pattern is 2 or 3 , PMOs for paging are same as SSB occasions, PF is the radio frame in which SSBs are transmitted. This is ensured by configuring nAndPagingFrameOffset based on period/offset of SSB transmission. Figure 1 is an example illustration of this for SSB periodicity = 80ms, N = T/8 and PF_Offset = offset.

Figure 1

For pagingSearchSpace is zero and SS/PBCH block and CORESET multiplexing pattern is 2 or 3:

- In case SSB periodicity is changed, paging configuration needs to be changed to ensure that PF configured by paging configuration is the radio frame in which SSBs are transmitted as per new SSB periodicity. As shown in figure 2, PF in red circle is not a valid PF for SSB periodicity of 160ms.

  - Option 1: When SSB periodicity is changed, SIB1 can be updated by network to include new paging configuration.

  - Option 2: Two paging configuration can be signalled in SIB1, one corresponding to each SSB periodicity/SSB configuration. UE applies the configuration according to active SSB periodicity/SSB configuration.

For dynamic adaptation of SSB periodicity, option 2 seems more efficient.

SSB periodicity 1 = 80ms

Figure 2

Proposal 1: Two paging configurations can be signalled in SIB1, one corresponding to each SSB periodicity/SSB configuration. UE applies the configuration according to active SSB periodicity/SSB configuration.

2.2. Adaptation of paging occasions

Paging frame bundling

Example implementation 1 (Figure 1):

Figure 1

In the legacy paging frame is determined using the following equation

  - (SFN +PF_Offset) mod T = (T div N) * (UE_ID mod N)

The factor “(T div N)” distributes the Paging frames in the DRX cycle. For PF bundling, factor “(T div N)” can be removed from the equation and PF can be determined as follows:

  - (SFN +PF_Offset) mod T = UE_ID mod N

POs can be determined as in legacy. No change to PO formula.

Example implementation 2 (Figure 2):

Figure 2

PFs can be bundled over a short duration (D) periodically (see figure 2)

- #of PFs (N1) in duration D = D, D/2, D/4, D/8, ….

- Note that in current spec, D is T.

UE’s PF

- (SFN +PF_Offset) mod T = (D div N1) * (UE_ID mod N1). Note that in current spec, D is T.

- T is UE specific DRX cycle in multiples of X; T can be X; or T is max (UE specific DRX cycle and X)

Extension of N and Ns

An alternate approach for clustering POs at the beginning of DRX cycle is to extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. For example, let’s say 32 POs are needed in a DRX cycle to meet the paging load in a cell. Network can configure 1 PF in DRX cycle and 32 POs per PF. In this approach, UE can determine PF and PO using the legacy formulas by applying new values of N and Ns.

If cell supports paging adaptation, it configures new parameters N’ and Ns’. N’ and Ns’ configures extended values of number of paging frames and number of POs. Legacy UEs in the cell will apply legacy parameters N and Ns whereas NES UEs supporting paging adaptation applies N’ and Ns’.

Extension of N and Ns is simpler and does not require any change to legacy formulas. Additionally, each PO can be flexibly located by indicating starting PDCCH occasion number. However, in case of PF bundling such flexibility is not available as starting PDCCH occasion number for a PO in each PF is same.

Proposal 2: For adaptation of paging occasions in time domain, extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

Proposal 3: If cell supports paging adaptation, it configures new parameters N’ and Ns’. N’ and Ns’ configures extended values of number of paging frames and number of POs. Legacy UEs in the cell will apply legacy parameters N and Ns whereas NES UEs supporting paging adaptation applies N’ and Ns’

# Conclusion

Proposal 1: Two paging configurations can be signalled in SIB1, one corresponding to each SSB periodicity/SSB configuration. UE applies the configuration according to active SSB periodicity/SSB configuration.

Proposal 2: For adaptation of paging occasions in time domain, extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

Proposal 3: If cell supports paging adaptation, it configures new parameters N’ and Ns’. N’ and Ns’ configures extended values of number of paging frames and number of POs. Legacy UEs in the cell will apply legacy parameters N and Ns whereas NES UEs supporting paging adaptation applies N’ and Ns’

# References