3GPP TSG-RAN WG2 Meeting #127bis	R2-2408300

Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.8.2

Source:		Lenovo

Title:		Access control for NTN downlink coverage enhancement

Document for:		Discussion

- Introduction

The latest RAN2#127 meeting made the following agreements regarding downlink coverage enhancement in NTN:

RAN2#127 Agreements - From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects) - From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms. - RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity - If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

In this contribution we discuss issues and potential solutions in access control (cell barring) for downlink coverage enhancement in NTN.

- Discussion

To facilitate reducing gNB downlink transmission/uplink reception active time, cell DTX/DRX (i.e. active and non-active periods) and cell turning-off were introduced in Rel-18 NES. For Rel-19 NR NTN, it is expected to use Rel-18 NES mechanism as baseline and consider enhancements to satellite-beam level to facilitate flexible and dynamic power allocation among satellite beams for downlink coverage enhancement.

## NTN cell access control for Rel-19 NTN UEs

Due to the current working direction of SSB periodicity extension for NTN downlink coverage enhancement, the potential impacts were identified in RAN2#127 meeting focusing on the case that SSB periodicity is larger than 160ms (up to 640ms). If the SSB periodicity is extended to a value larger than 160ms, existing Rel-17 and Rel-18 NTN UEs as well as Rel-19 NTN UEs incapable of such enhancement will be severely impacted and therefore access control may be necessary.

Based on current RAN2 discussion, cellBarredNTN introduced in Rel-17 NR NTN can be reused to restrict Rel-17 & Rel-18 NTN UEs from accessing an NTN cell operating in downlink coverage enhancement (e.g., SSB periodicity is extended to a value larger than 160ms). However, the access control strategy for Rel-19 NTN UEs need further discussion, i.e., how to allow/restrict access of NTN UE capable/incapable of downlink coverage enhancement to an NTN cell operating in downlink coverage enhancement. It is obvious that the cellBarredNTN alone cannot facilitate the expected access control for Rel-19 NTN UEs. For example, if cellBarredNTN is used to prevent access of Rel-17 & Rel-18 NTN UEs to an NTN cell operating in downlink coverage enhancement, it has to be set to barred, which also prevents access of Rel-19 NTN UEs.

Observation 1: Using cellBarredNTN alone is not sufficient to implement cell barring of Rel-19 NTN UEs for downlink coverage enhancement.

Following this assumption, Rel-19 NTN UEs capable of downlink coverage enhancement needs a new scheme to avoid from being barred by cellBarredNTN set to barred because of operating in downlink coverage enhancement (while it may still need to be barred for other reasons when cellBarredNTN is set to barred). Generally, either explicit indication (e.g., new cell barring bit) or implicit indication (e.g., downlink coverage enhancement configuration) can be considered.

Observation 2: Explicit or implicit indication beyond cellBarredNTN can be considered to facilitate access control for Rel-19 NTN UEs in an NTN cell operating in downlink coverage enhancement.

For the option of explicit indication, a new barring bit (e.g., cellBarredNTN_DE) can be introduced, while for the option of implicit indication, an NTN downlink coverage enhancement operation indication or configuration can be used. In general, the following alternatives of NTN cell barring logic for NTN-capable UEs can be considered:

- If an NTN-capable UE is capable of downlink coverage enhancement (e.g., SSB periodicity extension larger than 160ms), it determines the barring status of the NTN cell:

- Alt. A1: controlled by cellBarredNTN_DE value. UE ignores the cellBarredNTN in SIB1 set as barred, and considers the cell as not barred if cellBarredNTN_DE is set as not barred, or considers the cell as barred if cellBarredNTN_DE is set as barred.

- Alt. A2: controlled by cellBarredNTN_DE presence. UE considers the cell as not barred if cellBarredNTN_DE is present (ignoring cellBarredNTN in SIB1 set as barred), or considers the cell as barred if cellBarredNTN_DE is absent (following cellBarredNTN in SIB1 set as barred).

- Alt. A3: controlled by downlink coverage enhancement operation indication or configuration presence. UE considers the cell as not barred if a downlink coverage enhancement operation indication is indicated or a configuration is configured, or considers the cell as barred if a downlink coverage enhancement operation indication or configuration is absent (following cellBarredNTN in SIB1 set as barred).

- If an NTN-capable UE is incapable of downlink coverage enhancement (e.g., SSB periodicity extension larger than 160ms), it determines the barring status of the NTN cell:

- Alt. B1: controlled by cellBarredNTN value (same as Rel-17 and Rel-18 UEs). UE considers the cell as not barred if cellBarredNTN is set as not barred, or considers the cell as barred if cellBarredNTN is set as barred.

- Alt. B2: controlled by cellBarredNTN_DE presence. UE considers the cell as barred if cellBarredNTN_DE is present (following cellBarredNTN in SIB1 set as barred).

- Alt. B3: controlled by downlink coverage enhancement operation indication or configuration presence. UE considers the cell as barred if a downlink coverage enhancement operation indication is indicated or a configuration is configured.

Proposal 1: RAN2 to discuss the following options for cell barring regarding NTN downlink coverage enhancement.

For UE capable of downlink coverage enhancement:

- Alt. A1: controlled by cellBarredNTN_DE value.

- Alt. A2: controlled by cellBarredNTN_DE presence.

- Alt. A3: controlled by downlink coverage enhancement operation indication or configuration presence.

For UE incapable of downlink coverage enhancement:

- Alt. B1: controlled by cellBarredNTN value (same as Rel-17 and Rel-18 UEs).

- Alt. B2: controlled by cellBarredNTN_DE presence.

- Alt. B3: controlled by downlink coverage enhancement operation indication or configuration presence.

## Cell reselection for Rel-19 NTN UEs

Based on the above discussions for NTN cell barring, Rel-19 NTN UEs also face the similar issues during the cell reselection procedures. More specifically, if a neighbour NTN cell is operating in downlink coverage enhancement, the corresponding cell barring scheme shall be considered by the UE when performing cell reselection. In particular, an NTN UE incapable of NTN downlink coverage enhancement shall avoid reselection to an NTN cell operating in downlink coverage enhancement.

Similar to the discussions for Proposal 1, the following alternatives of NTN cell reselection logic for NTN-capable UEs can be considered:

- If an NTN-capable UE is capable of downlink coverage enhancement (e.g., SSB periodicity extension larger than 160ms), it determines the barring status of a neighbour NTN cell for cell reselection:

- Alt. C1: controlled by neighbour cell’s cellBarredNTN_DE value. UE ignores its cellBarredNTN in SIB1 set as barred, and considers the neighbour cell as not barred if its cellBarredNTN_DE is set as not barred, or considers the cell as barred if its cellBarredNTN_DE is set as barred.

- Alt. C2: controlled by neighbour cell’s cellBarredNTN_DE presence. UE considers the neighbour cell as not barred if its cellBarredNTN_DE is present (ignoring cellBarredNTN in SIB1 set as barred), or considers the neighbour cell as barred if its cellBarredNTN_DE is absent (following its cellBarredNTN in SIB1 set as barred).

- Alt. C3: controlled by neighbour cell’s downlink coverage enhancement operation indication or configuration presence. UE considers the neighbour cell as not barred if a downlink coverage enhancement operation indication is indicated or a configuration is configured for the neighbour cell, or consider the cell as barred if a downlink coverage enhancement operation indication or configuration is absent in the neighbour cell.

- Alt. C4: controlled by a list of neighbour cells for downlink coverage enhancement. Instead of receiving and decoding the barring bit or configuration of each neighbour NTN cell, UE can be indicated with a list indicating all neighbour NTN cells operating in downlink coverage enhancement. UE considers the neighbour cell as not barred if it is in the list.

- If an NTN-capable UE is incapable of downlink coverage enhancement (e.g., SSB periodicity extension larger than 160ms), it determines the barring status of a neighbour NTN cell for cell reselection:

- Alt. D1: controlled by neighbour cell’s cellBarredNTN value (same as Rel-17 and Rel-18 UEs). UE considers the neighbour cell as not barred if its cellBarredNTN is set as not barred, or consider the neighbour cell as barred if its cellBarredNTN is set as barred.

- Alt. D2: controlled by neighbour cell’s cellBarredNTN_DE presence. UE considers the neighbour cell as barred if its cellBarredNTN_DE is present (following its cellBarredNTN in SIB1 set as barred).

- Alt. D3: controlled by neighbour cell’s downlink coverage enhancement operation indication or configuration presence. UE considers the neighbour cell as barred if a downlink coverage enhancement operation indication is indicated or a configuration is configured.

- Alt. D4: controlled by a list of neighbour cells for downlink coverage enhancement. Instead of receiving and decoding the barring bit or configuration of each neighbour NTN cell, UE can be indicated with a list indicating all neighbour NTN cells operating in downlink coverage enhancement. UE considers the neighbour cell as barred if it is in the list.

Proposal 2: RAN2 to discuss the following options for cell reselection regarding NTN downlink coverage enhancement.

For UE capable of downlink coverage enhancement:

- Alt. C1: controlled by neighbour cell’s cellBarredNTN_DE value.

- Alt. C2: controlled by neighbour cell’s cellBarredNTN_DE presence.

- Alt. C3: controlled by neighbour cell’s downlink coverage enhancement operation indication or configuration presence.

- Alt. C4: controlled by a list of neighbour cells for downlink coverage enhancement (whitelist).

For UE incapable of downlink coverage enhancement:

- Alt. D1: controlled by neighbour cell’s cellBarredNTN value (same as Rel-17 and Rel-18 UEs).

- Alt. D2: controlled by neighbour cell’s cellBarredNTN_DE presence.

- Alt. D3: controlled by neighbour cell’s downlink coverage enhancement operation indication or configuration presence.

- Alt. D4: controlled by a list of neighbour cells for downlink coverage enhancement (blacklist).

## Validation of access control status

The legacy cell barring schemes including cellBarredNTN are solely based on the corresponding bit status due to that the network type (e.g., NTN or TN) is fixed, and for cell reselection the barring status is fixed to 300s. UE can only re-acquire the SIB1 to understand if the cell barring status has changed.

Meanwhile, in Rel-19 NTN the access control could be time sensitive as the NTN cell may change between modes enabling and disabling downlink coverage enhancement (SSB periodicity extension). And it is possible that some prior information can be determined by network and even be indicated to UE to implement optimized access control e.g., based on flexible time duration in coordination with NTN downlink coverage enhancement operations.

Observation 3: The cell barring status regarding NTN downlink coverage enhancement is associated to the operation duration or cycle which can be flexible and pre-determined.

Therefore, if UE can be informed about the NTN downlink coverage enhancement operation duration or cycle, it may also determine the validity duration of a cell barring status determination, which can prevent UE from unnecessary SIB1 acquiring for cell barring status or allow UE to acquire SIB1 for cell barring status in time.

Proposal 3: RAN2 to consider cell barring enhancement based on the validity duration of cell barring status for NTN downlink coverage enhancement.

- Conclusion

In this contribution we discuss issues and potential solutions in access control for NTN downlink coverage enhancement in NTN, focusing on cell barring schemes. The following observations are given:

Observation 1: Using cellBarredNTN alone is not sufficient to implement cell barring of Rel-19 NTN UEs for downlink coverage enhancement.

Observation 2: Explicit or implicit indication beyond cellBarredNTN can be considered to facilitate access control for Rel-19 NTN UEs in an NTN cell operating in downlink coverage enhancement.

Observation 3: The cell barring status regarding NTN downlink coverage enhancement is associated to the operation duration or cycle which can be flexible and pre-determined.

And we propose:

Proposal 1: RAN2 to discuss the following options for cell barring regarding NTN downlink coverage enhancement.

For UE capable of downlink coverage enhancement:

- Alt. A1: controlled by cellBarredNTN_DE value.

- Alt. A2: controlled by cellBarredNTN_DE presence.

- Alt. A3: controlled by downlink coverage enhancement operation indication or configuration presence.

For UE incapable of downlink coverage enhancement:

- Alt. B1: controlled by cellBarredNTN value (same as Rel-17 and Rel-18 UEs).

- Alt. B2: controlled by cellBarredNTN_DE presence.

- Alt. B3: controlled by downlink coverage enhancement operation indication or configuration presence.

Proposal 2: RAN2 to discuss the following options for cell reselection regarding NTN downlink coverage enhancement.

For UE capable of downlink coverage enhancement:

- Alt. C1: controlled by neighbour cell’s cellBarredNTN_DE value.

- Alt. C2: controlled by neighbour cell’s cellBarredNTN_DE presence.

- Alt. C3: controlled by neighbour cell’s downlink coverage enhancement operation indication or configuration presence.

- Alt. C4: controlled by a list of neighbour cells for downlink coverage enhancement (whitelist).

For UE incapable of downlink coverage enhancement:

- Alt. D1: controlled by neighbour cell’s cellBarredNTN value (same as Rel-17 and Rel-18 UEs).

- Alt. D2: controlled by neighbour cell’s cellBarredNTN_DE presence.

- Alt. D3: controlled by neighbour cell’s downlink coverage enhancement operation indication or configuration presence.

- Alt. D4: controlled by a list of neighbour cells for downlink coverage enhancement (blacklist).

Proposal 3: RAN2 to consider cell barring enhancement based on the validity duration of cell barring status for NTN downlink coverage enhancement.