3GPP TSG-RAN WG2 Meeting #127bis	R2-2408305

Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.9.4

Source:		Lenovo

Title:		PWS broadcast support for NB-IoT in NTN

Document for:		Discussion

- Introduction

The latest RAN#105 meeting added the following objective to support PWS broadcast for NB-IoT UEs in NTN:

Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution we discuss issues and potential solutions to support PWS broadcast for NB-IoT in NTN.

- Discussion

The NTN with large coverage is well suited to provide services to more UEs with higher resource efficiency, e.g., via broadcast. Currently in Rel-19 the enhancements to support broadcast services in NR NTN are being studied while for IoT NTN the satellite operators and vendors also consider supporting PWS broadcast for IoT devices.

The current PWS mechanisms include ETWS and CMAS. For ETWS, a dedicated paging indication etws-Indication is used to notify UE to receive or update ETWS contents in LTE SIB 10 and SIB11. For CMAS, a dedicated paging indication cmas-Indication is used to notify UE to receive or update CMAS contents in LTE SIB 12. However, neither of the PWS mechanisms is not supported by NB-IoT UEs. Considering that RAN have decided to reuse LTE PWS mechanisms as much as possible, we further analysis the potential issues brought by NTN characteristics including large cell coverage, cell change due to satellite movement and UE requirement of uplink synchronization.

## PWS broadcast service area indication

In previous releases before Rel-19 the PWS mechanisms are designed for TN, and UE can simply judge whether it is in an emergency broadcast service area by its serving cell and the corresponding PWS broadcast configuration it receives. This mechanism is workable based on the assumption that a PWS broadcast service area is larger than a TN cell coverage so that the service area can be mapped to one or more TN cells. However, the diameter of an NTN cell (i.e. satellite beam footprint size) could be 1000km for LEO and 3500km for GEO, which could be much larger than an emergency broadcast service area. An NTN cell coverage may also across country border on one side of which an emergency broadcast service is not allowed or authorized. It is observed that, for ETWS there is no IE to describe a geographical area in the contents of legacy LTE SIB 10 and SIB11, while for CMAS as an optional IE warningAreaCoordinatesSegment was introduced in Rel-15 of LTE SIB12 to describe the alert area coordinates of a CMAS message.

Observation 1: At least for ETWS, the broadcast service area needs to be indicated to NB-IoT UE in NTN.

Regarding the area indication, a first alternative (also as mentioned in the WID) is based on NR NTN outcome, which is currently agreed as a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). By comparing the NR NTN outcome and the warningAreaCoordinatesSegment used for CMAS following ATIS-0700041, some differences are listed below:

Table 1 Broadcast area indication in Rel-19 NR NTN and in warningAreaCoordinatesSegment for CMAS

|  | Represented shapes | Number restriction | Granularity | Other function |
| --- | --- | --- | --- | --- |
| Rel-19 NR NTN | circle and polygon | N/A | radius: 10km, max 2560km coordinates: ~2.5m (20000km/223) |  |
| CMAS | circle and polygon | 10 shapes 100 coordinates | radius: 1km, max 64km coordinates: ~5m (20000km/222) | Geo-Fencing Maximum Wait Time |

It can be observed that, warningAreaCoordinatesSegment for CMAS can also represent circles and polygons with number restrictions. When representing circles, warningAreaCoordinatesSegment is with smaller granularity and smaller value range compared to Rel-19 NR NTN solutions.

Observation 2: The warningAreaCoordinatesSegment for CMAS can represent circles and polygons with number restrictions and smaller granularity and value range than radius.

Considering that the representation of polygons has not been decided for Rel-19 NR NTN, RAN2 can either wait for more progress or consider using warningAreaCoordinatesSegment in CMAS.

Proposal 1: At least for ETWS broadcast service area indication, RAN2 can wait for NR NTN progress or discuss whether warningAreaCoordinatesSegment for CMAS can be reused.

Meanwhile, for CMAS a potential issue is that whether the warningAreaCoordinatesSegment can fulfill the requirements of area indication in NTN, including number restrictions, granularity and value range.

Proposal 2: For CMAS broadcast service area indication, RAN2 can discuss whether warningAreaCoordinatesSegment is sufficient for NTN deployment or NR NTN solution may also be needed.

## Association of the PWS broadcast service area

Another issue for the PWS broadcast service area indication is the expected PWS operation related to the indicated area. In legacy PWS mechanisms there is no area indication in the paging message, meaning that UE in the cell will be notified to receive PWS SIBs anyway. If PWS broadcast service area indication is supported for NB-IoT UEs, the following options for area-based PWS broadcast service operation can be considered:

- Option 1: UE is paged with area indication, and determines whether to acquire the PWS SIBs based on whether it is in the indicated area.

- Option 2: UE is paged without area indication (as in legacy), and acquires the PWS SIBs including area indication to decide whether to process (e.g., decode and deliver to upper layers) the PWS contents in the SIBs.

Either option can facilitate area-based PWS broadcast service delivery to the UE and the difference is that whether UE needs to receive the PWS SIBs first.

Observation 3: The PWS broadcast service area indication can be associated to the paging indication or PWS contents in PWS SIBs to facilitate area-based delivery to NB-IoT UEs.

Proposal 3: RAN2 to discuss the association between PWS broadcast service area and paging indication or PWS contents in PWS SIBs.

- Conclusion

In this contribution we discuss issues and potential solutions to support PWS broadcast for NB-IoT in NTN. The following observations are given:

Observation 1: At least for ETWS, the broadcast service area needs to be indicated to NB-IoT UE in NTN.

Observation 2: The warningAreaCoordinatesSegment for CMAS can represent circles and polygons with number restrictions and smaller granularity and value range than radius.

Observation 3: The PWS broadcast service area indication can be associated to the paging indication or PWS contents in PWS SIBs to facilitate area-based delivery to NB-IoT UEs.

And we propose:

Proposal 1: At least for ETWS broadcast service area indication, RAN2 can wait for NR NTN progress or discuss whether warningAreaCoordinatesSegment for CMAS can be reused.

Proposal 2: For CMAS broadcast service area indication, RAN2 can discuss whether warningAreaCoordinatesSegment is sufficient for NTN deployment or NR NTN solution may also be needed.

Proposal 3: RAN2 to discuss the association between PWS broadcast service area and paging indication or PWS contents in PWS SIBs.