3GPP TSG-RAN WG2 Meeting #127b	R2-2408307

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.7.4

Source:		Lenovo

Title:			Enhanced delay status reporting for XR

Document for:	Discussion and Decision

- Introduction

In this contribution we discuss delay status reporting enhancements for XR traffic.

- Discussion

The Rel-19 Work Item on XR Phase 3 was agreed in RP-240791.

The Rel-19 XR Phase 3 objectives are as follows: Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2].

In RAN2#126 the following agreements were reached:

- Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included

In RAN2#127 the following agreements were reached:

- Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In RAN2#127 meeting, it was concluded to configure multiple remaining time thresholds for each LCG to report multiple pairs of remaining time and buffer size information. When multiple PDU sets in the LCG have different remaining times UE may report remaining time information on a PDU set level as in Rel-18 if DSR is triggered for an LCH and PDU set discard is configured.

Proposal 1: DSR reporting should be done on a PDU set level if PDU set discard is configured similar as Rel-18 principle.

The reporting thresholds can be separately configured from the DSR triggering threshold, e.g. remainingTimeThreshold, per LCG. In our view, the trigger threshold can be configured below or above a reporting threshold, there are no constraints on how the NW configures DSR triggering and reporting thresholds. There are two cases for reporting thresholds of an LCG:

- Case 1: reporting multiple delay status for data with remaining time below the trigger threshold. E.g., The trigger threshold is larger than the reporting thresholds.

- Case 2: reporting multiple delay status for data with remaining time below the trigger threshold, or above trigger threshold but below a reporting threshold. E.g., the trigger threshold is less than one of the reporting thresholds. This case extends the report to also consider non-delay critical data.

For case 1, if there are more than one PDU sets with different remaining time for an LCG, UE reports multiple pair of delay status (and associated buffer size information) with finer reporting granularity for the data with remaining time below the triggering threshold. This case resolves the reporting granularity issue of the Rel-18 DSR procedure.

For case 2, RAN2 needs to further discuss whether UE may report multiple pairs of remaining time information and buffer size regardless of whether the remaining time is less than the configured remainingTimeThreshold (DSR triggering threshold), so that gNB is provided with the status of both delay-critical and non-delay-critical data pending for transmission. Correspondingly, NW can allocate UL resources accounting for the detailed remaining delay information provided in the DSR, and UE performs resource allocation based on delay status, e.g. enhanced LCP, in response to receiving the UL grant. Therefore, it is beneficial to provide additional pairs of delay status information in an enhanced DSR even though this would increase the signalling overhead.

Therefore, we propose that no restrictions are introduced for the configuration of the DSR reporting and triggering thresholds, i.e., trigger threshold can be configured below or above reporting threshold. Here we assume, the minimum remaining time reported in R18 DSR is still reported in Rel-19, therefore reporting thresholds should include the Rel-18 reporting. UE accordingly reports a pair of remaining time information and corresponding buffer size based on the configured reporting threshold(s) for cases that a DSR has been triggered. The data with a remaining time being above the DSR trigger threshold (remainingTimeThreshold) and less than a configured reporting threshold is considered as non-delay critical data, while the data with remaining time below the DSR trigger threshold is considered as delay critical data as in the legacy. It is beneficial from uplink resource allocation perspective for the case that there is non-delay critical data ahead of the delay critical data in the buffer, that the non-critical data is calculated and reported separately in a DSR based on the configured reporting threshold(s).

Proposal 2: There are no restrictions on how NW configures the DSR triggering and reporting thresholds, i.e., trigger threshold can be below or above a report threshold, and reporting thresholds include the Rel-18 reporting. UE reports a pair of remaining time and corresponding buffer size based on the configured reporting thresholds.

Proposal 3: RAN2 to define delay critical data has remaining time below trigger threshold, non-delay critical data has remaining time below the report threshold and above the trigger threshold.

For the enhanced DSR format, we think that NW should configure if and for which LCG the DSR based reporting threshold(s) should be considered. For example, only for specific LCG(s) NW may configure reporting thresholds. If one LCG is configured with reporting thresholds, UE should use a new DSR format. For example, the new DSR format may use a one-bit field to indicate whether some further pair of remaining time information and buffer size information is present in the MAC CE for the associated LCG.

Proposal 4: If at least one LCG configured with report thresholds triggered DSR, UE should use new DSR format for all LCGs. A one-bit field (Extension flag) may indicate whether some further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

In R18, UE cancels a DSR if the data associated with the DSR has been discarded or has been transmitted. In R19, same principle should be followed but with some restrictions on the definition of the data associated with a DSR. UE cancels a DSR based on whether the delay critical data associated with the DSR has been discarded or has been transmitted. For example, if there is one pair of remaining time and buffer size information for delay critical data and one for non-delay critical data, and the delay critical data has been discarded and only non-delay critical data is associated with DSR, UE cancels the DSR. In addition, in Rel-18, when determining the data for triggering a DSR, the data should have not been transmitted or reported in a DSR. In Rel-19, RAN2 revisit whether the same restriction should be applied to the non-delay critical data which has been reported in an DSR.

Proposal 5: UE cancels a DSR if all the data associated with the DSR with a remaining time below the trigger threshold has been discarded or transmitted.

In Rel-18, PSI information is not included in a DSR. If it is reported in DSR, NW can be aware of PSI information of data associated with the reported delay status and may perform resource allocation considering also the importance of the data. Even when PSI level would be provided together with delay status information, the full potential will be only exploited if also the LCP procedure considers the PSI of the data, which is however not the in the scope of Rel-19. Therefore, we are not sure that there is sufficient benefit of providing PSI information within the new DSR report for the extra overhead. RAN2 can further discuss whether PSI should be included in new DSR format.

Proposal 6: RAN2 to further discuss whether there is a sufficiently large benefit to include PSI information in the new DSR format.

- Conclusion

In this contribution, we discuss delay status reporting enhancements. We have the following proposals:

Proposal 1: DSR reporting should be done on a PDU set level if PDU set discard is configured similar as Rel-18 principle.

Proposal 2: There are no restrictions on how NW configures the DSR triggering and reporting thresholds, i.e., trigger threshold can be below or above a report threshold, and reporting thresholds include the Rel-18 reporting. UE reports a pair of remaining time and corresponding buffer size based on the configured reporting thresholds.

Proposal 3: RAN2 to define delay critical data has remaining time below trigger threshold, non-delay critical data has remaining time below the report threshold and above the trigger threshold.

Proposal 4: If at least one LCG configured with report thresholds triggered DSR, UE should use new DSR format for all LCGs. A one-bit field (Extension flag) may indicate whether some further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

Proposal 5: UE cancels a DSR if all the data associated with the DSR with a remaining time below the trigger threshold has been discarded or transmitted.

Proposal 6: RAN2 to further discuss whether there is a sufficiently large benefit to include PSI information in the new DSR format.

- References

### TS 38.321 v18.0.0